3GPP TSG-RAN WG2 Meeting #127bis	R2-2408381

Hefei, China, 14th – 18th Oct, 2024

Agenda Item:	8.10.2

Source: 	China Unicom

Title:  	Discussion on MRO enhancement for LTM

Document for:	Agreement

Introduction

In the prior meeting, RAN2#127 and RAN3#125 reached consensus on several enhancements for MRO related to LTM as referenced [1] [2].

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled. We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit. Log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. RAN2 assumes this is already possible with existing spec.

Reuse the existing connection failure definition of too late handover, too early handover, handover to wrong cell for LTM failure case. Add a sentence for LTM handover. Take the existing detection mechanism descriptions as baseline. For failures due to inappropriate cell switch triggering, CU initiates analysis and may forwards the RLF report to the DU responsible for the failure. CU needs to forward the RLF related information to DU using existing Access and Mobility Indication procedure.

This document offers further analysis of MRO considerations for RA-based/RACH-less based cell switch indicators, indicators that identify whether a neighboring cell is an LTM candidate, and associated analytical insights.

Discussion

- 2.1	MRO enhancement for LTM

Access type

To analyze explicit and implicit indicators, it is crucial to determine whether the network can perform corresponding optimizations based on such indicators. According to specifications related to the LTM feature, if the UE successfully acquires a valid TA after receiving the LTM cell switch command, then the RACH-less LTM proceeds. If the UE fails to acquire any valid TA post-command, the RACH-based LTM is initiated. Thus, RACH-based LTM can be implicitly indicated by the post-command TA invalidity, which carries the same implications and overhead as an explicit 1-bit RACH-based LTM indicator. The network's reception of a 1-bit TA invalidity indicator or a 1-bit RACH-based LTM indicator is equivalent, as it indicates RACH-based LTM cell switch failure is potentially due to RA procedure failure. However, an indicator of acquired TA invalidation more directly assures the network of no TA issue, allowing it to focus optimizations on the RACH access procedure.

Proposal 1: Introduce an indicator to represent the acquired TA is invalid upon UE receipt of the LTM cell switch command to implicitly denote RACH-based LTM cell switch failure.

For RACH-less LTM, a single explicit RACH-less LTM indicator is insufficient, as it does not clarify whether failure stems from UE measurement-based early TA acquisition, PDCCH order-based early TA acquisition. Therefore, it is necessary to define implicit RACH-less LTM indicators, such as UE measurement-based TA acquisition and PDCCH order-based TA acquisition. With these indicators, the network may infer that RLF/HOF is due to an excessively prolonged UE self-measured TA procedure or an extended duration for the network to compute and transmit the TA to the UE.

Proposal 2: Introduce indicators for UE measurement-based RACH-less LTM failure and PDCCH order-based RACH-less LTM failure to implicitly imply RACH-less LTM cell switch failure.

Sub-optimal LTM cell switch

The network's objective in configuring UE measurement-based TA acquisition or transmitting PDCCH order-based TA via MAC CE is to reduce LTM cell switch time and service interruption. If the network, after sending the configuration or TA via MAC CE, discovers that RACH-based LTM, not RACH-less LTM, is triggered due to invalid TA, this indicates a sub-optimal LTM cell switch procedure, leading to the wastage of radio and signaling resources.

For sub-optimal LTM cell switches, it is necessary to log detailed reasons for TA invalidity in SHR/SPR. The detailed analysis includes the following scenarios:

Scenario 1: Invalid TA acquisition based on UE measurement, without configured TA from PDCCH order.

Scenario 2: Invalid TA acquisition based on PDCCH order, without configured UE-measured TA acquisition or with invalid self-measured TA acquired by UE

Scenario 3: Invalid TA based on PDCCH order, with valid TA measured by UE.

For Scenario 1, the network may optimize by employing PDCCH order-based TA acquisition methods. For Scenario 2, the network can directly optimize PDCCH order-based TA acquisition methods. For Scenario 3, the network can either optimize PDCCH order-based TA acquisition methods, or cease configuring such methods altogether and rely on UE measured TA acquisition method.

Proposal 3: Record TA validity indicators for UE measurement-based TA acquisition and PDCCH order-based TA acquisition in SHR.

Other information

The RAN2#127 meeting reached agreements on logging information to deduce ltmCandidate, considering if it is explicitly or implicitly signaled. If an explicit indicator is used, a 1-bit IE ltmCandidate is specified in RRC specs to indicate whether the neighboring cell is a candidate target cell for LTM cell switch, which is straightforward and simple. Moreover, no other implicit information can represent ltmCandidate information:

Proposal 4: Define and log a 1-bit ltmCandidate (similar to choCandidate) in SHR to explicitly indicate whether a neighboring cell is an LTM candidate cell or not.

Conclusion

RAN2 is kindly asked to discuss the following proposals:

Proposal 1: Introduce an indicator to represent the acquired TA is invalid upon UE receipt of the LTM cell switch command to implicitly denote RACH-based LTM cell switch failure.

Proposal 2: Introduce indicators for UE measurement-based RACH-less LTM failure and PDCCH order-based RACH-less LTM failure to implicitly imply RACH-less LTM cell switch failure.

Proposal 3: Record TA validity indicators for UE measurement-based TA acquisition and PDCCH order-based TA acquisition in SHR.

Proposal 4: Define and log a 1-bit ltmCandidate (similar to choCandidate) in SHR to explicitly indicate whether a neighboring cell is an LTM candidate cell or not.

References

- R2_127_Maint CovEnh SON-MDT eRedCapn (Mattias)_20240823_1000

- RAN3_125_agenda_20240823EOM