3GPP TSG-RAN WG2 Meeting #127bis	R2-2408383

Hefei, China, 14th – 18th Oct, 2024

Agenda Item:	8.2.4

Source: 	China Unicom

Title:  	Discussion on Random Access for Ambient IoT

Document for:	Agreement

Introduction

3GPP RAN has approved a new SID to study Ambient IoT (Internet of Things) in NR [1]. While some initial agreements have been reached regarding random access in TR 38.769, there are still many details that require further discussion [2]. During the previous meeting, RAN2#127 has made some agreements on random access for Ambient IoT as below [3].

- 1 RAN2 assumes that Msg3 transmission resource can explicitly be indicated in Msg2 (based on RAN1 agreements/design). Wait for RAN1 for further details. - 2 For 3-step CBRA Support fixed random ID size is 16 bit. The ID is randomly generated. - 3 Failure/success indication of D2R will be studied. FFS if it would be implicit or explicit and for which use case it is needed. FFS whether it is applied only to some cases. - 4 For 2step CBRA, RAN2 design will support msg2. Whether it is needed it is up to the reader. FFS when it is needed. For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2. FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices. - 5 In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2). FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes. - 6 FFS if reader assigns the AS ID for scheduling purposes.

In this contribution, we analyze the remaining issues and present our views about random access for Ambient IoT.

Discussion

- 2.1	Failure detection for D2R data transmission

During the previous meetings, RAN2 confirmed that slotted-ALOHA is the baseline for Ambient IoT random access and analyzed the basic steps for 3-step random access CBRA, 2-step CBRA. For 3-step CBRA, the first two messages were for contention resolution and the device sends the Msg3 to complete the random-access procedure.

The R2D transmission after D2R can be considered as the awareness of the Msg3 transmission failure/success. In this situation, If the device does not receive data from the reader, the device may not be able to determine whether the D2R (e.g. Msg3) message was sent successfully, and will not be able to quickly complete the behavior in the case of D2R data transmission failure.

The reader can always detect D2R transmission failures during timing-based as well as CRC-based methods. But for the device, if there is no subsequent information, then the device will not be able to tell whether its D2R message was sent successfully.

Observation 1: The reader can always detect whether the D2R message is sent successfully. But the device may not be able to tell whether its D2R message was sent successfully.

- 2.2	Need/when/how to feedback the failure/success indication

In cases where the reader can decide whether R2D data can be sent, the device can judge whether the previous D2R message was sent successfully by whether it received R2D data, which is an implicit way, but for some important D2R messages, it is necessary to design explicit indications.

To explicitly indicate success or failure, in our opinion, using a 1-bit indication with two code-points as “success” and “failure” is the better solution. In this case, when the device receives the indication, it can determine whether the D2R message was transmitted successfully or failed. If the device does not receive the indication, it can be determined that the transmission of the 1-bit indication failed.

For the case of 1-bit indication for failure indication (while its absence means success), the R2D transmission after D2R could be a failure indication of previous D2R, while the absent of R2D indicates the success reception of previous D2R.However, the disadvantage is that the device cannot distinguish the success indication or the failure indication fails to transmit.

Proposal 1: explicit indications are necessary, and it's better to use 1-bit indication with two code-points as “success” and “failure”.

- 2.3	Consequence of D2R data transmission failure

For the case of D2R transmission failure, if the D2R data is Msg1 or Msg3, re-access in another opportunity can be the baseline. Re-transmission solution may involve data decomposition, which is too demanding for the device. If we rely on the reader to indicate the retransmission part, then the reader needs to demodulate and decode as soon as possible and analyze which parts of the data need to be retransmitted, and give a clear indication, which also increases the complexity of the system.

Proposal 2: For the case of D2R transmission failure, if the D2R data is Msg1 or Msg3, re-access in another opportunity can be the baseline.

Conclusion

In this contribution, we discussed the issues related to failure handling aspects on Ambient IoT. Based on the discussion, the following observation and proposals are concluded:

Observation 1: The reader can always detect whether the D2R message is sent successfully. But the device may not be able to tell whether its D2R message was sent successfully.

Proposal 1: explicit indications are necessary, and it's better to use 1-bit indication with two code-points as “success” and “failure”.

Proposal 2: For the case of D2R transmission failure, if the D2R data is Msg1 or Msg3, re-access in another opportunity can be the baseline.

References

- RP-240826 New SID: Study on solutions for Ambient IoT (Internet of Things) in NR

- 3GPP TR 38.769 V1.0.0 Study on solutions for ambient IoT (Internet of Things)

- RP-242405 	Status Report to TSG