3GPP TSG-RAN WG2 Meeting #127bis	R2-2408389

Hefei, China, Oct. 14th – 18th, 2024

Agenda Item:	8.9.2

Source: 	CATT, Huawei, HiSilicon, CMCC, vivo

Title: 	Access Control for Store and Forward Operation

Document for: 	Discussion and Decision

Introduction

In RAN2#127 [1], the following agreements were achieved:

Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

There are two leftover issues on how to control the access of UEs with different capabilities on S&F operation. This contribution will discuss how to inform S&F capable UE of whether S&F operation is applied (especially whether an additional static indication for NW capability or not) and how to bar legacy UEs and still allow S&F capable UEs (w.r.t. the need of new barring bit).

Discussion

2.1	Details on operation mode indication and need of additional indication

Following the agreement below, the need of an indication for current network operation mode (i.e. normal IoT NTN operation vs. S&F operation) is concluded, but whether an additional static indication for the network capability on S&F is left as FFS.

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

The argument of the proponents to additionally introduce a static NW capability indication is that, for the satellite supporting S&F, the status of feeder link availability could switch from "available" to "unavailable" and vice versa. From our perspective, the static indication is not needed. When the feeder link is currently unavailable, it is straightforward that NW can only operate in S&F mode (and is thus capable of this feature). By contrast, when the feeder link is available, the NW should have already worked in the normal IoT NTN mode and provided service for UEs per legacy Rel-17/18 IoT NTN operations, since the normal IoT NTN operation can provide better communication performance than S&F operation.

As a result, we do not identify any necessity to make UE further know whether the NW is capable of S&F operation, in addition to in what mode the NW is currently operating.

Observation 1: There seems no additional benefit for the UE to know the NW's S&F capability, when the satellite indicates that it is operating in normal IoT NTN mode. This makes it not necessary to introduce another static indication on whether NW supports S&F feature, in addition to the indication already agreed on the current NW operation mode.

Hence, the UE only needs to know the NW current operation mode, meaning that one indication for current operation type is enough. From UE perspective, only when the indication is present, does it indicate that the serving satellite is operating in S&F mode; otherwise, it indicates the serving satellite is operating in normal IoT NTN mode. From network perspective, when the feeder link is not available, this indication is set as present; otherwise, it is absent. This is also aligned with SA2's conclusion shown as below [2].

1) When feeder link is not available and the network supports S&F operation, the network shall be able to inform UE(s) whether S&F Satellite operation is applied, (e.g. eNB broadcast support of S&F operation as part of System Information).

Regarding how the NW sets the presence/absence of this indication in terms of feeder link availability, it is the NW configuration option without the need of UE visibility, and thus can be left to NW implementation without need of extra Spec impact.

Proposal 1: No additional static indication of whether the NW supports S&F feature is needed.

Proposal 2: For the agreed S&F indication on NW operation mode: when the indication is present, it means the serving satellite is operating in S&F mode; otherwise, it means the serving satellite is operating in normal IoT NTN mode. How the NW sets presence/absence of this indication in terms of feeder link availability is up to NW implementation.

2.2	Need of a new barring bit for S&F capable UEs and detailed solution

Now we turn to address the below FFS on whether a new barring bit is needed for a Rel-19 S&F capable UE.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

At last meeting, some companies argued that they did not see the use case for a new barring bit, and were thus against introducing a new barring bit merely due to some general reasons like "protecting the NW". However, considering the characteristics of the S&F operation itself, there is indeed the use case that the buffer memory of the satellite has already been fully occupied or the satellite/cell is currently short of resources, and is thus unable to accept additional access from UEs. In this case, there certainly needs to be a way for the NW to perform the access control and bar the Rel-19 S&F capable UEs' access attempts.

Therefore, besides indicating the operation mode, whether a cell operating in S&F mode is barred or not also needs to be indicated to the UE. Solution-wise, we can further depend on the agreed indication (e.g., named as sf-CellBarred) to inform the Rel-19 S&F capable UEs of whether the cell is barred. Specifically, with the presence/absence of sf-CellBarred indicating whether S&F operation is being applied, the NW operating in S&F mode configures sf-CellBarred as present, and further sets the value of sf-CellBarred as "barred" or "not barred" to implement the AS-level access control as mentioned above.

Proposal 3: When the agreed S&F indication (e.g. sf-CellBarred) is present, it further functions as the S&F specific barring bit and indicates to the Rel-19 S&F capable UEs whether the cell operating in S&F mode is barred or not (i.e. with the two values of "barred" and "notBarred").

With the new barring bit introduced in Proposal 3, the intended behaviour for a Rel-19 S&F capable UE w.r.t. the new S&F specific barring bit and legacy barring bits are shown as follows:

- In case the sf-CellBarred is present (i.e. NW is operating in S&F mode), the UE determines whether the cell operating in S&F mode is barred or not depending on the new barring bit sf-CellBarred, and ignores existing cellBarred/cellBarred-NTN;

- Otherwise (i.e. NW is not operating in S&F mode but in legacy IoT NTN mode), the UE depends on the legacy cellBarred-NTN to perform the barring related operations.

So, the above UE behaviour is proposed as follows for the Rel-19 S&F capable UE:

Proposal 4: If the agreed S&F indication (e.g. sf-CellBarred) is present, the UE capable of S&F operation determines whether the cell operating in S&F mode is barred or not depending on the value of sf-CellBarred, and ignores existing cellBarred/cellBarred-NTN. Otherwise, the UE capable of S&F operation determines whether the cell is barred depending on the existing cellBarred-NTN.

In the Appendix, some informative texts are given to show how NW operating in S&F mode performs barring to the legacy IoT UEs, if regarded as needed. At the last meeting, RAN2 agreed to not further discuss whether/how to bar legacy IoT NTN UEs unless more SA2 input comes in, but confirmed if there is a need, the NW can rely on barring exiting mechanism to do so. Thus, the texts in the Appendix just aim at illustrating how the NW sets the new barring bits and legacy barring bits, in order to justify whether/how they jointly work to realize NW intended effect on AS-level access control.

Conclusion

In this contribution, access control for Store and Forward operation is discussed and the two FFS issues left from last meeting are addressed. The observation and proposals are listed as follows:

Observation 1: There seems no additional benefit for the UE to know the NW's S&F capability, when the satellite indicates that it is operating in normal IoT NTN mode. This makes it not necessary to introduce another static indication on whether NW supports S&F feature, in addition to the indication already agreed on the current NW operation mode.

Proposal 1: No additional static indication of whether the NW supports S&F feature is needed.

Proposal 2: For the agreed S&F indication on NW operation mode: when the indication is present, it means the serving satellite is operating in S&F mode; otherwise, it means the serving satellite is operating in normal IoT NTN mode. How the NW sets presence/absence of this indication in terms of feeder link availability is up to NW implementation.

Proposal 3: When the agreed S&F indication (e.g. sf-CellBarred) is present, it further functions as the S&F specific barring bit and indicates to the Rel-19 S&F capable UEs whether the cell operating in S&F mode is barred or not (i.e. with the two values of "barred" and "notBarred").

Proposal 4: If the agreed S&F indication (e.g. sf-CellBarred) is present, the UE capable of S&F operation determines whether the cell operating in S&F mode is barred or not depending on the value of sf-CellBarred, and ignores existing cellBarred/cellBarred-NTN. Otherwise, the UE capable of S&F operation determines whether the cell is barred depending on the existing cellBarred-NTN.

Reference

- RAN2 #127 meeting minutes

- 3GPP TR 23700-29: "Study on integration of satellite components in the 5G architecture ".

- Appendix

Last meeting, RAN2 made the following agreement:

RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

The rest of this appendix illustrates how the satellite operating in S&F mode can bar the legacy IoT NTN UEs by jointly using the legacy barring bits (including cellBarred and cellBarred-NTN) and the new barring bit sf-CellBarred, if the NW really sees a need to bar legacy IoT NTN UEs. There are two options on how to configure these barring bits by the NW operating in S&F mode:

- Option 1: cellBarred and cellBarred-NTN are set as "barred", sf-CellBarred is set as "barred" or "notBarred";

- Option 2: cellBarred is set as "barred", cellBarred-NTN is absent, sf-CellBarred is set as "barred" or "notBarred";

Accordingly, the behaviours of UEs with different S&F capabilities are shown in the following table.

Table A.1: Barring operations by a satellite operating in S&F mode

| NW Configuration Options UEs of different capabilities | Option 1: cellBarred is set as barred cellBarred-NTN is set as barred sf-CellBarred is set as barred or not barred | Option 2: cellBarred is set as barred cellBarred-NTN is absent sf-CellBarred is set as barred or not barred |
| --- | --- | --- |
| Legacy NTN UE and R19 NTN UE not capable of S&F | UE ignores cellBarred, and considers this cell is an NTN cell which is barred according to cellBarred-NTN. | UE considers this cell is a TN cell which is barred according to cellBarred. |
| UE support S&F operation | UE ignores cellBarred and cellBarred-NTN. UE considers this cell is in S&F operation which is barred or notBarred according to sf-CellBarred. | UE ignores cellBarred. UE considers this cell is in S&F operation which is barred or notBarred according to sf-CellBarred. |

Under both of these two NW configuration options, when a cell is operating S&F, the cellBarred-NTN is used to bar the connectivity of NTN UEs not capable of S&F operation. Same as legacy, the cellBarred is used to bar the connectivity of UE not capable of NTN. And the new barring bit sf-CellBarred further controls the access of the Rel-19 S&F capable UEs, either allowing their access per last-meeting agreement or prohibiting the access due to NW needs (e.g. buffer overload).

Since above options depends on proper NW setting of the new and legacy barring bits, no extra Spec impact is foreseen.