3GPP TSG-RAN WG2 Meeting #127bis	R2-2408402

Hefei, China, Oct 14th – 18th, 2024

Source:        ZTE Corporation

Title:           Initial Analysis on the RAN2 Impact for the R19 MIMO

Agenda item:   8.12.2

Document for:  Discussion and Decision

- Introduction

For the R19 MIMO topic, RAN1 has achieved many agreements and a LS was sent in [1] to RAN2 on part of the RRC parameters and also on the PL offset MAC CE.

1. Overall Description: Pertaining to the Rel-19 NR MIMO Ph5 WI, RAN1 would like to inform RAN2 about the following: - An initial list of RRC parameters (excel spreadsheet attached) - An agreement requiring MAC CE support: Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). “MAC-CE impact: MAC-CE updates the PL offset value(s) configured in one or more joint TCI states or UL TCI states”

In this contribution, we’d like to share our initial analysis on the RAN2 impact based on the RAN1’s progress.

- Discussion

In this chapter, we first discuss the UEIBM impact, and then analyse the impact from the Asymmetric DL sTRP/UL mTRP, at last we will give our consideration on the RRC parameters.

2.1 UE Initiated Beam Management (UEIBM)

For the UEIBM, RAN1 has discussed the Events definition (e.g. Event 1, 2 and 7), event trigger condition, UEIBR (UE initiated Beam Report) procedure (e.g. mode A and mode B) and also other related topics.

On the event trigger condition, a window would be introduced for the instance statistics, the related RAN1 agreement and FFS issue are listed as below:

R1-2405638 Moderator Summary #4 on UE-initiated/event-driven beam management Moderator (ZTE) Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

Observation 1: For the event trigger condition, whether the description on the instance number statistics in a window should be captured in RAN1 or RAN2 is still FFS.

On the UEIBR procedure, two procedure modes are defined as below, the main difference is that the mode B is based on the preconfigured UL resource, while in the mode A, the UE need to wait for the UL resource assignment before the 2nd UL message transmission.

Fig 1: UEIBR Procedure

Furthermore, for the step 1 of both the mode A and mode B, RAN1 has discussed more detail issues, and two alternatives are discussed in the RAN1 #118 meeting:

R1-2406045 Moderator Summary #2 on UE-initiated/event-driven beam management Moderator (ZTE) Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following - Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-B, down-select one of the following - Alt 1 (dedicated SR for mode-B): Introduce RRC parameter, e.g., reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeB-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above is at least applied to a single CC case.

From the above agreement, we can see that for the first UL message of both the mode A and mode B, whether the SR or the new UCI type would be used is still FFS.

Observation 2: For the first UL message of both the mode A and mode B, whether the SR or the new UCI type would be used is still FFS.

Based on the above observations, we need to wait for RAN1’s further progress on these 2 FFS issues. If the RAN1 decides to capture instances statistics within a window in RAN2 spec, or if the RAN1 select SR as the indication in the step 1, RAN2 can start the related discussion. Thus, we give our first proposal:

Proposal 1: On the UEIBR impact on the MAC layer procedure, wait for RAN1’s further progress.

2.2 Asymmetric DL sTRP/UL mTRP

About the asymmetric DL sTRP/UL mTRP, based on RAN1’s progress and updated WID[2], at least the below 3 topics need to be analysed in RAN2.

- Impact on the R17/18 Unified TCI state Activation/Deactivation MAC CE

- PL offset MAC CE

- 2TA with single DCI case

- Impact on the R17/18 Unified TCI state Activation/Deactivation MAC CE

Before we start the discussion on the Unified TCI state Activation/Deactivation MAC CE for the asymmetric DL sTRP/UL mTRP deployment scenario, we’d like to share the RAN1’s progress on the unified TCI state framework. During RAN1 discussion, the following framework has been confirmed:

For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM (inter-cell Beam management) and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

Based on this agreement, only single TRP was supported in R17, thus there would be no impact on the R17 “Unified TCI state activation/deactivation MAC CE”.

However, on the R18 unified TCI state activation/deactivation MAC CE, for both the joint and the Separate TCI states cases, there would be 2 TCI States indication for each DCI codepoint as in the Fig2.

Fig 2:R18 Unified TCI state MAC CE

When it comes to the asymmetric DL sTRP/UL mTRP deployment scenario, there is only one DL TCI state for each DCI codepoint. Then the question is if the R18 MACE CE is reused for the asymmetric case, how to indicate the DL TCI sated. On this issue, per the RAN1 discussion, one solution is that the network can only indicate one DL TCI state, e.g. always configure the applyIndicatedTCI-State for each downlink change to the first.

Proposal 2: RAN2 confirm that the existing R17 “Unified TCI state activation/deactivation MAC CE” and R18 “Enhanced TCI state activation/deactivation MAC CE for Joint TCI States/ Enhanced TCI state activation/deactivation MAC CE for Separate TCI States” can be reused for the asymmetric DL sTRP/UL mTRP deployment scenario. FFS on how to indicate only one DL TCI state for a DCI codepoint with R18 unified TCI state activation/deactivation MAC CE.

- PL offset MAC CE

A LS [1] was also sent to RAN2 on the path PL offset MAC CE, in which it indicates that a MAC CE would be needed for the PL offset indication.

Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). “MAC-CE impact: MAC-CE updates the PL offset value(s) configured in one or more joint TCI states or UL TCI states”

According to the RAN1 discussion, there is also agreement on the range of the PL offset

Agreement For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB - FFS: Extending the range to lower than -10dB - Step size is 4dB

Though there is a FFS on the lower range, according to the RAN1’s FL summary in [3], the -60dB may be set as the lowest value, then with step size 4dB, 5bits would be enough to indicate PL offset.

The other remaining issue is about whether the MAC CE indicates the PL offset only for the indicated TCI state(s) or for the activated TCI states. This may be further discussed in RAN1.

Observation 3: According to the LS, it’s not clear whether the MAC CE indicates the PL offset only for the indicated TCI state(s) or for the activated TCI states.

From RAN2 aspect, the MAC CE may need to be sent quite frequently if MAC CE only indicates the PL offset for the indicated TCI state(s). The more practical and efficient way is to indicate the PL offsets for the activated TCI state. However, as mentioned above, this issue may be further discussed in RAN1, to avoid the conflict between the RAN1 and RAN2, it’s better to wait for RAN1’s further progress on this issue. If the RAN1 doesn’t touch this issue this meeting, an LS can be sent to Ran1.

Proposal 3: Wait for RAN1’s further progress on the blow 2 issues:

- Number of bits for the PL offset indication;

- Whether the PL offset MAC CE only indicates the PL offset for the indicated TCI state(s) or indicates the PL offset for the activated TCI states.

Proposal 3a: If RAN1 doesn’t touch these 2 issues, a Reply LS can be sent to RAN1 for the confirmation.

At this stage, RAN2 still can have some high level discussion on the PL offset MAC CE, e.g. whether to adopt the same of different PL MAC CE for the R17/R18 unified TCI state framework.

As discussed above, for the R17, the unified TCI state framework was designed for the single TRP case, while in the R18, it can support multiple TRP case. For the multiple TRP case, it shall still allow the network to indicate PL offset for only one TRP, which is the same as the R17 framework. Thus a PL Offset MAC CE that can be compatible for both the R17/R18 unified TCI state framework is needed.

Proposal 4: The PL Offset MAC CE design shall be compatible for both the R17/R18 unified TCI state framework.

- 2TA with single DCI case

For the 2 TA case, in the Rel 18, the 2TA for mTRP was restricted for the mDCI case. For the the asymmetric DL sTRP/UL mTRP deployment scenario, the single DCI would be the basic case. Thus, in the last plenary meeting, it has also confirmed that “there is a need for extending the Rel-18 2TA specification for multi-DCI-based multi-TRP by removing the restriction that coresetPoolIndex needs to be configured (with legacy PRACH resources)” in the revised WID [2].

In RAN2, such kind of restriction shall also be removed from both the RRC and MAC spec. Considering that it would not cause too much specification work and this topic is quite separate from the other topics, this can be done in the RAN2 running CR review stage.

Proposal 5: On 2TA with single DCI case, RAN2 would remove the “the restriction that coresetPoolIndex needs to be configured” in the RRC and MAC running CR review stage.

2.3 RRC Parameters

In this chapter, we’d like to discuss the RRC parameters. In the LS [1], a RRC parameters table was also attached,. From which we can see that:

- Many parameters for the CSI enhancement were introduced, and some parameters are noted with “up to RAN2” also with part of FFS.

- There are some FFS issues for the UEIBM/3Tx/Asymmetric MTRP Topics.

Considering that there are still some FFS issues and this is the first Ran2 meeting, we can wait for a more stable version.

Proposal 6: For the RRC parameters implementation, Ran2 can wait for a more stable version.

- Conclusion and proposals

With the above analysis, we have the following proposals:

UEIBM

Observation 1: For the event trigger condition, whether the description on the instance number statistics in a window should be captured in RAN1 or RAN2 is still FFS.

Observation 2: For the first UL message of both the mode A and mode B, whether the SR or the new UCI type would be used is still FFS.

Proposal 1: On the UEIBR impact on the MAC layer procedure, wait for RAN1’s further progress.

Asymmetric DL sTRP/UL mTRP

Proposal 2: RAN2 confirm that the existing R17 “Unified TCI state activation/deactivation MAC CE” and R18 “Enhanced TCI state activation/deactivation MAC CE for Joint TCI States/ Enhanced TCI state activation/deactivation MAC CE for Separate TCI States” can be reused for the asymmetric DL sTRP/UL mTRP deployment scenario. FFS on how to indicate only one DL TCI state for a DCI codepoint with R18 unified TCI state activation/deactivation MAC CE.

Observation 3: According to the LS, it’s not clear whether the MAC CE indicates the PL offset only for the indicated TCI state(s) or for the activated TCI states.

Proposal 3: Wait for RAN1’s further progress on the blow 2 issues:

- Number of bits for the PL offset indication;

- Whether the PL offset MAC CE only indicates the PL offset for the indicated TCI state(s) or indicates the PL offset for the activated TCI states.

Proposal 3a: If RAN1 doesn’t touch these 2 issues, a Reply LS can be sent to RAN1 for the confirmation.

Proposal 4: The PL Offset MAC CE design shall be compatible for both the R17/R18 unified TCI state framework.

Proposal 5: On 2TA with single DCI case, RAN2 would remove the “the restriction that coresetPoolIndex needs to be configured” in the RRC and MAC running CR review stage.

Other

Proposal 6: For the RRC parameters implementation, Ran2 can wait for a more stable version.

- Reference

- R1-2407285  LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5   RAN1

- RP-242394   WID revision: NR MIMO Phase 5   Samsung

- R1-2407189	 Summary #1 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)