3GPP TSG-RAN WG2#127bis	R2-2408409

Hefei, China, Oct 14th – 18th, 2024

Source: 	Fujitsu

Agenda item:	8.5.2

Title: 	Discussion on on-demand SSB SCell operation

Document for:	Discussion and decision

# Introduction

In RAN2#127, RAN2 has started the on-demand SSB SCell discussion, and the following agreements were made [1]:

Agreements on OD-SSB Scenarios on OD-SSB: - RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B. OD-SSB transmission indication: - RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. - New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A. Measurement on OD-SSB: - Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1). - For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated. - RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements. - RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

In this contribution, we would like to discuss and provide our views on the on-demand SSB SCell operation from RAN2 perspective.

# Discussion

## On-demand SSB SCell

To reduce network energy consumption, on-demand SSB SCell should not be used as a normal cell. We assume the SCell is activated to handle user burst traffics in connected mode, and when the NW is expected the traffic is low, the NW should stop on-demand SSB transmissions and deactivate/release the SCell. Given the above use-case, on-demand SSB SCell should not be used as a PCell. Otherwise, the UEs may camp on this cell during on-demand SSB transmission periods and consequently the UE suddenly loses DL synchronisations due to stopping of on-demand SSB. It would case some waste in the idle/inactive UE behaviours.

Observation 1:	Even if the UE can receive SSB in the cell (always-on SSB or on-demand SSB), the cell should not be allowed to access from idle/inactive mode UEs as a PCell.

Therefore, for on-demand SSB SCell, some barring mechanism for idle/inactive UEs is used. In our understanding, because the UE in idle/inactive mode behaviours are the same, the Rel-19 NES UEs also should not camp on the on-demand SCell as well as legacy UEs. Thus, an existing barring mechanism (e.g., MIB barring) can work for this purpose.

Proposal 1:	An existing mechanism (e.g., MIB barring) indicates the on-demand SSB SCell is not used as a PCell for idle/inactive mode UEs. No specification is changed.

### On-demand SSB SCell configuration

Identification of on-demand SSB SCell:

From RAN1 agreements, the SCell with on-demand SSB transmission would have different SSB transmission mode from the legacy SCell, i.e., the SCell with on-demand SSB transmission may have no SSB transmission or have the always-on SSB transmission (e.g., with a long periodicity than usual) before the on-demand SSB transmission. In order for the UE not to measure the SCell with no always-on SSB to save the UE’s power, or for the UE to measure the SCell with always-on SSB with the relaxed measurement requirements, the UE may need to know which SCell is the SCell with on-demand SSB. Therefore, it is proposed that the gNB should indicate to the UE if the configured SCell is an SCell with on-demand SSB.

Proposal 2: 	The gNB indicates to the UE if the configured SCell is an SCell with on-demand SSB transmission.

NCD SSB configuration:

As agreed in RAN1#116-bis, it is up to gNB implementation whether always-on SSB on the cell is cell-defining SSB (CD SSB) or non-cell-defining SSB (NCD SSB). It means the gNB can transmit always-on NCD SSBs in the cell.

Agreement For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

However, NCD SSB has the same properties as the corresponding CD SSB as shown in TS 38.331.

nonCellDefiningSSB

If configured, the UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the CD-SSB of the serving cell (e.g. obtaining sync, measurements, RLM, BFD, beam management). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the "SSB" configured in the QCL-Info IE; the "ssb-Index" configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitly to this NCD-SSB.

The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE. In the MIB associated with this NCD-SSB, the systemFrameNumber field indicates the frame boundary and frame number of the NCD-SSB. The subCarrierSpacingCommon and dmrs-TypeA-Position field in the MIBs associated with CD-SSB and NCD-SSB in the same cell are configured with the same values, respectively.

RAN1 is discussing whether on-demand SSB is limited to NCD or not. Depending on the RAN1 conclusion, it would be some further RAN2 impacts on the current NCD SSB definitions in RRC due to the above relationship between CD and NCD SSB. Table.1 shows potential RAN2 impacts on the possible SSB type combinations between always-on SSB and on-demand SSB.

Table.1 - Possible SSB type combinations between always-on SSB and on-demand SSB

| Combination no. | Always-on SSB | On-demand SSB | Can NCD SSB use the same properties of corresponding CD SSB? |
| --- | --- | --- | --- |
| 1 | N/A | CD | - |
| 2 |  | NCD | No |
| 3 | CD | CD | - |
| 4 |  | NCD | Yes |
| 5 | NCD | CD | Maybe Yes |
| 6 |  | NCD | No |

From Table.1, at least for the combinations 2 and 6, the on-demand SSB cannot refer the same properties of corresponding CD SSB, because there is no corresponding CD SSB in the cell. For the combination 5, it is not straightforward way, but it is possible that the on-demand SSB (CD-SSB) is used as a corresponding SSB. On the other hand, considering network energy saving efficiency, it might be problematic because the periodicity of NCD SSB shall not be less than the periodicity of the serving cell’s CD SSB.

Observation 2:	If the on-demand SSB is NCD SSB in Case #1 (no always-on SSB in the cell), the UE may not identify the NCD on-demand SSB properties.

Observation 3:	If both the always-on SSB and the on-demand SSB in the cell are NCD SSB, the UE may not identify the NCD on-demand SSB properties.

In order to resolve the issues, two options are considered.

- Option1: the NW doesn’t support the cases where there is no corresponding CD SSB in the cell.

- Option2: NCD SSB configuration will be enhanced to include all SSB properties.

If RAN1 agrees the on-demand SSB is limited to NCD SSB and existing NCD SSB configuration is reused, the always-on SSB may be limited to CD SSB only (combination 4). It seems contradiction with the previous RAN1 agreement “whether it is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not”. Option 2 can support any combinations in Table.1 but specification impact is larger than Option 1.

Proposal 3:	RAN2 should discuss and downselect the following options, if there is no corresponding CD SSB in the cell.

- Option 1: the NW doesn’t support the cases where there is no corresponding CD SSB in the cell.

- Option 2: NCD SSB configuration will be enhanced to include all SSB properties.

L3 measurements on on-demand SSB:

In RAN2#127 meeting, to get precise measurement results, RAN2 agreed that the UE should measure the associated SSB only when on-demand SSB transmission is activated. Furthermore, NW can indicate the on-demand SSB transmission by MAC CE. On the other hand, currently SSB measurements are performed by SMTC windows configured by RRC. It means that a new mechanism could be necessary so that the SMTC periodicity should be quickly aligned to the on-demand SSB periodicity without RRC re-configuration. Otherwise, the UE may not measure the on-demand SSB correctly. The simplest way is that the UE should apply the SMTC window for on-demand SSB only when the on-demand SSB transmission is activated, and vise-versa. In our understanding, this can be applicable both Case #1 and Case #2, but at the last meeting, RAN2 has agreed this principle for Case #1 only. Therefore, we would like to confirm it is applicable to Case #2 as well.

Proposal 4:	For Case #2, the UE does not expect to measure on-demand SSB when on-demand SSB transmission is deactivated.

Proposal 5:	Regardless of Case #1 and Case #2, the UE should apply the SMTC window for on-demand SSB only when the corresponding on-demand SSB transmission is activated.

RAN2 considers other measurement configurations (e.g., information about time instance A/B) after RAN1 makes further progress on on-demand SSB transmission pattern.

### A new MAC CE format design

In the last meeting, RAN2 agreed new MAC CE is used to indicate on-demand SSB transmission. Besides that, RAN1 further agreed to configure multiple candidate values by RRC and indicate the applicable value by MAC CE as follows:

Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

Considering the above RAN1 agreement, a new MAC CE should include a new field to indicate the applicable value for on-demand SSB. The candidate values with index could be configured in SCell configuration by RRC.

Proposal 6:	A new MAC CE for on-demand SSB transmission can optionally include the index field per SCell, to indicate one of candidate value applied for the SCell.

At the last meeting, in case of scenario 2A, RAN2 discussed whether the new MAC CE also contains the serving cell index for SCell activation/deactivation (i.e., Ci field in SCell activation/deactivation MAC CE). Both separate and joint indications can work for SSB transmission and/or SCell activation/deactivation. However, separate indications may increase UE complexity. For instance, if one of MAC CE is received, whether the UE can wait for another MAC CE or not, i.e., if the UE received the on-demand SSB transmission MAC CE, the UE does not know it is the scenario 2 or 2A. Therefore, the UE may wait for another MAC CE for a while. On the other hand, joint indication can make the UE bahaviours simple, because two MAC CE can be received at the same time. From the above, it is preferred that NW can jointly indicate for SCell activation/ deactivation for scenario 2A.

Proposal 7:	In scenario 2A, NW jointly indicates on-demand SSB transmission and SCell activation in one MAC CE.

In summary, the following is the example of new MAC CE format design (with up to 7 SCells and 2 OD-SSB configurations). FFS: The maximum number of OD-SSB configuration. If NW supports up to 31 SCells and/or RAN1 requests more bits to indicate multiple configurations, a size of new MAC CE will be significantly large. Hence, it is reasonable to consider the message size reduction later.

Figure 1: Example of SSB activation/deactivation MAC CE

Proposal 8:	A new MAC CE has a valuable size and consists of the following fields:

    - Indication of on-demand SSB transmission,

    - On-demand SSB configuration index (optional for multiple configuration),

    - Ci field in SCell activation/deactivation MAC CE (optional for scenario 2A).

FFS: how to reduce the message size.

### Issues on Scenario #3A/#3B

RAN1 has not yet agreed to support scenario #3A/3B. If RAN1 agrees to support the on-demand SSB transmission after a reception of SCell activation command, potential use-cases and impacts on RAN2 are as follows:

- AGC and time/frequency synchronization:

  - RAN2 should discuss when/how the UE monitor the on-demand SSB to maintain AGC and time/frequency synchronization for the activated SCell.

- L3 measurements:

  - Besides cell measurements, RAN2 should discuss BFD and measurement event evaluation on the on-demand SSB.

However, these issues will be clarified after these scenarios are supported in RAN1. Thus, for the time being, RAN2 should wait for further RAN1 progress.

Proposal 9:	RAN2 to wait for further RAN1 progress on the scenario after a reception of SCell activation command.

# Conclusion

In this contribution, we have some discussions about on-demand SSB SCell operations, and the following proposals are made:

General aspects:

Observation 1:	Even if the UE can receive SSB in the cell (always-on SSB or on-demand SSB), the cell should not be allowed to access from idle/inactive mode UEs as a PCell.

Proposal 1:	An existing mechanism (e.g., MIB barring) indicates the on-demand SSB SCell is not used as a PCell for idle/inactive mode UEs. No specification is changed.

Identification of on-demand SSB SCell:

Proposal 2: 	The gNB indicates to the UE if the configured SCell is an SCell with on-demand SSB transmission.

NCD SSB configuration:

Observation 2:	If the on-demand SSB is NCD SSB in Case #1 (no always-on SSB in the cell), the UE may not identify the NCD on-demand SSB properties.

Observation 3:	If both the always-on SSB and the on-demand SSB in the cell are NCD SSB, the UE may not identify the NCD on-demand SSB properties.

Proposal 3:	RAN2 should discuss and downselect the following options, if there is no corresponding CD SSB in the cell.

- Option 1: the NW doesn’t support the cases where there is no corresponding CD SSB in the cell.

- Option 2: NCD SSB configuration will be enhanced to include all SSB properties.

L3 measurements on on-demand SSB:

Proposal 4:	For Case #2, the UE does not expect to measure on-demand SSB when on-demand SSB transmission is deactivated.

Proposal 5:	Regardless of Case #1 and Case #2, the UE should apply the SMTC window for on-demand SSB only when the corresponding on-demand SSB transmission is activated.

On-demand SSB activation/deactivation MAC CE:

Proposal 6:	A new MAC CE for on-demand SSB transmission can optionally include the index field per SCell, to indicate one of candidate value applied for the SCell.

Proposal 7:	In scenario 2A, NW jointly indicates on-demand SSB transmission and SCell activation in one MAC CE.

Proposal 8:	A new MAC CE has a valuable size and consists of the following fields:

    - Indication of on-demand SSB transmission,

    - On-demand SSB configuration index (optional for multiple configuration),

    - Ci field in SCell activation/deactivation MAC CE (optional for scenario 2A).

FFS: how to reduce the message size.

Issues on Scenario #3A and #3B:

Proposal 9:	RAN2 to wait for further RAN1 progress on the scenario after a reception of SCell activation command.

# References

- R2-2407571, Report from session on V2X/SL, R19 NES and MOB

# Annex -	RAN1 agreements on on-demand SSB SCell

<RAN1#118>

Agreement

- Update the previous RAN1 agreement as follows.

  - At least support L1 measurement based on on-demand SSB

    - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

      - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

      - The support of LTM is a separate discussion point

Agreement

For a cell supporting on-demand SSB SCell operation,

- Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

  - FFS: Whether to support RRC based signaling for other cases.

- Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A.

Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- Periodicity of the on-demand SSB

- FFS: Any other relevant parameters

Agreement

For a cell supporting on-demand SSB SCell operation, at least the following is supported

- On-demand SSB on the cell is not located on synchronization raster.

- On-demand SSB on the cell is non-cell-defining SSB

FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

Agreement

Support L3 measurement based on on-demand SSB

- Further work on L3 measurement is up to RAN2/RAN4

Agreement

LS to RAN2 for on-demand SSB SCell operation is agreed. Final LS in R1-2407438.

Agreement

The previous RAN1 agreement made in RAN1#117 is revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

  - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1

  - FFS: Whether the value of T is predefined or indicated/configured by gNB

  - (Working assumption) T=T_min

  - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

- Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

LS on timeline for On-demand SSB operation on SCell is agreed in R1-2407565.

<RAN1#117>

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell.

  - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell.

    - This DCI signaling does not provide SCell activation/deactivation.

    - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc.

  - FFS: Scenarios where the above signalings are applicable

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the following for on-demand SSB via higher layer RRC signaling is supported.

  - Frequency of the on-demand SSB

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst

  - Periodicity of the on-demand SSB

  - FFS: Whether more than one on-demand SSB configurations can be configured for the cell to UE

  - FFS: Whether the RRC is newly introduced or existing RRC is reused

Agreement

- At least support L1 measurement based on on-demand SSB

  - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

    - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

Agreement

For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

- Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission

  - The SSB time domain positions of on-demand SSB burst are configured by gNB.

- FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components

- Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

- FFS: Whether the value of T is predefined or indicated/configured by gNB

- FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB”

Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

Agreement

- For a cell supporting on-demand SSB SCell operation, at least the followings for on-demand SSB are known to UE.

  - Sub-carrier spacing of the on-demand SSB

  - Physical Cell ID of the on-demand SSB

  - Location of on-demand SSB burst

  - Downlink transmit power of on-demand SSB

  - FFS: Other parameters

  - FFS: Whether each of above parameters is configured/indicated explicitly or not

<RAN1#116-bis>

Agreement

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

- For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner.

  - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered.

- Notes:

  - Scenario #2A refers to

    - “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

  - Scenario #3A refers to

    - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

  - Scenario #3B refers to

    - “When SCell activation is completed and SCell is activated” or

    - “After SCell activation is completed and SCell is activated”

  - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications.

  - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not.

  - For on-demand SSB on the cell, downselect between the following alternatives

    - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not.

    - Alt-2: On-demand SSB is limited to non-cell-defining SSB.

      - FFS: Further limitations to on-demand SSB

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - L1 and/or L3 measurement based on on-demand SSB is supported for the cell.

    - FFS further details on L1 and/or L3 measurement

Agreement

The following agreement from RAN1#116 is modified (in red)

- For SSB burst(s) triggeredindicated by on-demand SSB SCell operation, study at least the following options.

  - Option 1: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A.

  - Option 1A: UE expects that on-demand SSB burst(s) is periodically transmitted from time instance A until gNB turns OFF the on demand SSB

  - Option 2: UE expects that on-demand SSB burst(s) is transmitted from time instance A to time instance B and not transmitted after time instance B.

  - Option 3: UE expects that on-demand SSB burst(s) is transmitted N times after time instance A and not transmitted after N on-demand SSB bursts are transmitted.

  - Option 4: UE expects that on-demand SSB burst(s) is transmitted with a periodicity from time instance A to time instance B and with the other periodicity after time instance B.

  - FFS: The combination of above options

  - FFS: How to define time instance A/B and the value of N per option

  - FFS: Each option is applicable to which Cases or Scenarios (as per the previous agreement)

Agreement

For a cell supporting on-demand SSB SCell operation, further study the following options.

- Option 1: Separate signaling between legacy/existing signaling (e.g., RRC, MAC CE) providing SCell activation/deactivation and signaling providing On-demand SSB transmission indication.

- Option 2: A single signaling in which both SCell activation/deactivation and On-demand SSB transmission indication are provided.

  - FFS: Details of the signaling

- Other options are not precluded.

- FFS: Details on On-demand SSB transmission indication

<RAN1#116>

Agreement

Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted.

FFS: Which scenario the above applies for

Agreement

- RAN1 to strive for a common design for on-demand SSB operation considering all applicable CA configurations.

Agreement

- For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios.

- Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321)

- Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321)

  - This does not preclude SCell for which activation is completed

  - FFS: The case where SCell activation is completed

FFS: Application timing between NW triggering message and on demand SSB transmission

Agreement

- Support on-demand SSB SCell operation triggered by gNB.

- FFS Details of associated signaling/indication/configuration provided to UE