3GPP TSG-RAN WG2#127bis	R2-2408410

Hefei, China, Oct 14th – 18th, 2024

Source: 	Fujitsu

Agenda item:	8.5.3

Title:	Discussion on on-demand SIB1 procedure for NES

Document for:	Discussion and decision

# Background

In RAN2#127 meeting, RAN2 made some agreements as follows [1].

Agreements on on-demand SIB1:

Scenarios:

- No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

- We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

- Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

- Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

- Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

- RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

OD-SIB1 SI conclusion:

- RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

In this contribution, we would like to discuss and provide our views on the on-demand SIB1 operation from RAN2 perspective.

# Discussion

## UL WUS procedures

In our understanding, the overall procedure of on-demand SIB1 request and acquisition is shown in the following figure:

Figure 1. On-demand SIB1 request and acquisition procedure

### UL WUS configuration

New SIB or existing SIB

In RAN2#126 meeting, on the transmission of UL WUS configuration, RAN2 agreed that Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbor NES cells, e.g., via new SIB or the existing SIB. From our perspective, the UL WUS configuration of a NES cell might not be changed frequently. If UL WUS configuration is transmitted in an existing SIB, the change of other parameters in the existing SIB would cause the NES UEs to re-acquire the SIB, even if the NES UEs may not be interested in the parameters. This would lead to additional UE efforts and power consumption. Moreover, as agreed by RAN2, one Cell A can provide the UL WUS configurations for more than one NES cells, so the size of the UL WUS configuration transmitted by Cell A might be relatively large if the number of the neighboring NES cells is large. In addition, the transmission period of UL WUS configuration can be also different from the period of the existing SIBs. Therefore, we prefer to transmit the UL WUS configuration of NES cells via a new SIB in Cell A.

Proposal 1: The UL WUS configuration of the NES cell is transmitted via a new SIB in Cell A.

In the last RAN2 meeting, the following was agreed on the UL WUS configuration of NES cell from NES cell:

- Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

As discussed in RAN#105 plenary meeting, an NES cell may provide the UL WUS configuration for its neighboring cells. For providing the UL WUS configuration for the neighboring cells, similar to Cell A, a new SIB can be used, and then the UEs that have already camped on the NES cell can acquire the new SIB to obtain the UL WUS configurations for the intended neighboring NES cells.

Proposal 2: The UL WUS configuration of the neighboring NES cell is transmitted via a new SIB in the NES cell.

For providing the NES cell’s own UL WUS configuration, we assume that SIB1 or the new SIB can be used. Currently, the RACH configurations for different features are provided in SIB1 and RAN2 has agreed that the UL WUS configuration includes at least RACH configuration, so it is straightforward to include the NES cell’s UL WUS configuration in its SIB1. On the other hand, the NES cell’s UL WUS configuration can also be included in the new SIB for providing the UL WUS configurations associated with the neighboring NES cells. We have no strong preference for SIB1 or the new SIB. RAN2 is suggested to discuss it.

Proposal 3: RAN2 to discuss whether the UL WUS configuration of the serving NES cell is transmitted via the new SIB or SIB1 in the NES cell.

Contents of UL WUS configuration

As mentioned, RAN2 agreed that the UL WUS configuration includes at least RACH configuration. In RAN1#117 meeting, it was agreed as a starting point to discuss the required parameters/contents inside the UL WUS configuration. In our opinion, as there are physical layer parameters, the content of the UL WUS configuration can be discussed in RAN1 and RAN2 can wait for RAN1 conclusion.

Proposal 4: RAN2 to wait for the RAN1 conclusion on the required parameters/contents inside the UL WUS configuration.

UL WUS configuration update

In RAN2#127 meeting, it was agreed that once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures. That is to say, the NES cell will apply the updated UL WUS configuration from the next modification period of the NES cell.

In RAN3#125 meeting, RAN3 agreed that NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn interface and it is FFS if Cell A broadcasts it right away or not. Therefore, when UL WUS configuration is updated in the NES cell, the updated UL WUS configuration would be sent from the NES cell to Cell A, and then Cell A would update its SIB (can be denoted as SIBx) including the updated UL WUS configuration right away or after a while. By the legacy SI modification procedure, the updated SIBx will be transmitted from the next modification period of Cell A.

If the next modification period of the NES cell is not aligned with the next modification period of Cell A, the UEs in Cell A may transmit UL WUS using the UL WUS configuration not aligned with the UL WUS configuration applied in the NES cell, and thus the UL WUS would not be received by the NES cell and the on-demand SIB1 request procedure would fail. In our view, if the out-of-sync between the updated UL WUS configuration lasts a longer time, the UL WUS transmission failure of UEs in Cell A will happen more frequently. It can be resolved the modification period of both Cell A and the NES cell is aligned. Otherwise, the legacy SI modification procedure may not be applicable. Therefore, RAN2 is suggested to discuss this issue.

Observation 1: From RAN2 perspective, if the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, UL WUS transmission from UEs in Cell A may fail.

Proposal 5: RAN2 to discuss the issue when the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell.

### Triggering condition for UL WUS

Other cell selection cases for triggering

It was agreed to use legacy intra/inter-frequency cell re-selection procedure as baseline for UL WUS triggering condition in RAN2#126 meeting. In RAN#105, only the Case 2 is supported for on-demand SIB1 operation, therefore it is obvious that the initial cell selection is not a trigger the UL WUS transmission. Besides that, for a case of recovery from out of coverage (to the NES cell), UL WUS configuration in Cell A may be changed during the UE is out of coverage, but the UE can't know that therefore it is natural not to support this procedure.

Proposal 6: Initial cell selection and recovery from out of coverage are not a trigger of the UL WUS transmission.

### UL WUS transmission

Random Access Response acquisition

RAN2 agreed that RAN2 assumes the UE is expected to receive the RAR for responding to the preamble transmission for Msg1-based on-demand SIB1 procedure as the baseline. According to this agreement, the UE will not be able to acquire SIB1 unless the UE successfully receives a RAR corresponding to the transmitted preamble. If the UE couldn’t receive RAR during RA procedure and the number of UL WUS retransmission is larger than preambleTransMax, this UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. The NES cell has to transmit RAR for each preamble transmitted from the UEs, so that the UEs will be able to acquire SIB1. It may increase the number of RARs and the number of UL WUS retransmissions. To reduce the signalling during SIB1 request procedure, the UE should check whether SIB1 is already transmitted from NES cell as the triggering condition and stopping condition for UL WUS procedure.

Proposal 7: The UE should check whether SIB1 is transmitted from NES cell as the triggering condition and stopping condition for UL WUS procedure.

### On-demand SIB1 reception

Failure case handling

In RAN2#126 meeting, upon random access procedure failure of on-demand SIB1 request, the UE will consider the cell as barred as follows.

UE behaviour on RACH failure of OD-SIB1 REQ

- As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

Based on the current UE behaviour, if the UE is unable to acquire SIB1, the UE may consider the cell as barred for up to 300s. If the cell is considered as barred, the UE may or may not reselect another cell on the same frequency depend on intraFreqReselection in MIB. If NES cell and Cell A are deployed on the same frequency, NW may set intraFreqReselection as “allowed” while if NES cell and Cell A are deployed on the different frequency, NW may set intraFreqReselection as “not allowed”. So, it is already supported to control cell level barring or frequency level barring.

Proposal 8: The legacy UE behaviour can be reused upon random access failure of on-demand SIB1 request, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

On-demand SIB1 reception

In RAN1#118 meeting, there are following agreements on PDCCH monitoring occasions for on demand SIB1:

Agreement At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options: - Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission - Option 2: starting time and duration are indicated in the UL WUS configuration Agreement For Case-2: For type 0 PDCCH monitoring occasions for on demand SIB1, on how the search space zero configuration is provided, RAN1 to down select from the following options: - Option 1: searchSpaceZero for on-demand SIB1 is provided from MIB on NES cell - Option 2: searchSpaceZero for on-demand SIB1 is provided from UL WUS configuration - Option 3: searchSpaceZero for on-demand SIB1 is provided from the RAR of UL WUS. Combination of multiple options is not precluded.

In our understanding, the configuration of CORESET #0/SS#0 and the time window for type 0 PDCCH monitoring occasions for RAR and on demand SIB1 can be further discussed by RAN1. RAN2 can wait for RAN1 conclusion and then discuss how to include the required configuration in the corresponding signalling.

Proposal 9: RAN2 to wait for the RAN1 conclusion on the configuration of RAR reception and SIB1 reception.

On-demand SIB1 update in NES cell

In the last RAN2 meeting, it was agreed that UE keeps SIB1 updated while on the NES cell via regular SI modification procedure. An issue that can be further discussed is how long the network should transmit the on-demand SIB1 from the network modification period, as shown in the following figure.

Figure 2. On-demand SIB1 update in the NES cell

In our view, the network can transmit the on-demand SIB1 during the whole next modification period (i.e., legacy SI update) or in a time window with length equal to the RAN1 defined time window for type 0 PDCCH monitoring occasions for on-demand SIB1 reception. Regardless of the options, the UE can assume the updated SIB1 is transmitted from the next modification period. If the UE is unable to acquire the updated SIB1, the UE should consider the NES cell as barred, because the UL WUS configuration may be obsoleted due to SI update.

Proposal 10: RAN2 to discuss how long the updated on-demand SIB1 is transmitted from the next modification period of the NES cell. The following options are considered:

- Option 1: the updated SIB1 is transmitted during whole next modification period,

- Option 2: the updated SIB1 is transmitted during time window for type 0 PDCCH monitoring occasions.

Proposal 11: The UE considers the NES cell as barred if the UE is unable to acquire the updated SIB1.

## Access restriction for legacy UEs

As agreed in RAN2#126, the legacy UEs need to be barred for accessing the Rel-19 NES cells.

Legacy UE impact

- NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

- How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

For this purpose, RAN2 agreed the following three options should be considered for access restriction in the normative phase.

- Option 1: Legacy UEs bar the on-demand SIB1 cell based on cellBarred bit set to barred in MIB. NES UE ignore the cellBarred bit. Additional barring bit in SIB1 to bar the on-demand SIB1 capable UE

- Option 2: Legacy UEs bar the on-demand SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting on demand SIB1 to list of excluded cells

As there are the existing barring mechanisms, all options can bar the legacy UEs for camping on the Rel-19 NES cells. Both MIB barring by options 1 and 2 can be applied to the cell selection and reselection cases, meanwhile option 3 would be used only for cell reselection case. It means that RAN2 needs to select one of the first two options (options 1 and 2) as baseline solution and discuss whether option 3 would be additionally needed.

Proposal 12: MIB barring in Rel-19 NES cell should be considered as baseline for legacy UE handling.

For option 1, it is already introduced for Rel-18 NES Cell DTX/DRX capable UE. As in Rel-18 NES, a new barring bit for the Rel-19 NES UE should be added in SIB1 to check the cell barring status. For option 2, it is still discussing in RAN1 (i.e., Kssb value in MIB of NES cell). Regardless the value of Kssb, to obtain the on-demand SIB1, it obviously requires NW should configure the actual ssb-SubcarrierOffset value in another place. We think the parameters for on-demand SIB1 reception are not changed frequently then it is useful to provide them in UL WUS configuration. If RAN1 agreed the Kssb value will always indicate absent of SIB1 in case of NES cell, it is enough to bar the legacy UEs and no need to use cellBarred. Therefore, RAN2 should wait for RAN1 progress.

Proposal 13: Whether/how to use cellBarred or ssb-SubcarrierOffset for barring of legacy UEs, RAN2 should wait for RAN1 progress.

For option 3, it is possible that the legacy UE does not consider the Rel-19 NES cell as candidate for cell re-selection when the Rel-19 NES cell is included in the current excluded cell list (i.e., IntraFreqExcludedCellList or InterFreqExcludedCellList). It is beneficial to reduce the attempts of SSB/SIB1 acquisition for legacy UEs. Hence, the excluded cell list should be considered.

Observation 2: Using excluded cell list for on-demand SIB1 cells would be beneficial to reduce the attempts of SSB/SIB1 acquisition.

However, Rel-19 NES UE should be allowed to access the Rel-19 NES cell even if the cell is listed in the excluded cell list. One solution is the Rel-19 NES UE can consider the listed cell in the excluded cell list as candidate if the corresponding UL WUS configuration is obtained from Cell A. If the UE does not have the corresponding UL WUS configuration, the cell(s) in the excluded cell list would not be considered as candidate. Another solution is that a new excluded cell list is additionally provided for Rel-19 NES UE to indicate the excluded cell(s).

Comparing the two solutions above, both solutions can work for the legacy UEs but the second solution may need more signalling overhead. For example, if NW would like to indicate cell #m is not considered as candidate for both the legacy and Rel-19 NES UEs, the NW has to include the cell #m into both the legacy and new excluded cell lists. Therefore, the first solution is preferred from the signalling overhead perspective.

Proposal 14: The Rel-19 NES UE can consider the cell in the legacy excluded cell list as candidate, if the UE has the UL WUS configuration corresponding to the cell in the excluded cell list.

# Conclusion

In this contribution, the following proposals and observations were made:

New SIB or existing SIB

Proposal 1: The UL WUS configuration of the NES cell is transmitted via a new SIB in Cell A.

Proposal 2: The UL WUS configuration of the neighboring NES cell is transmitted via a new SIB in the NES cell.

Proposal 3: RAN2 to discuss whether the UL WUS configuration of the serving NES cell is transmitted via the new SIB or SIB1 in the NES cell.

Contents of UL WUS configuration

Proposal 4: RAN2 to wait for the RAN1 conclusion on the required parameters/contents inside the UL WUS configuration.

UL WUS configuration update

Observation 1: From RAN2 perspective, if the UL WUS configuration update in Cell A is not synchronised with the UL WUS configuration update in the NES cell, UL WUS transmission from UEs in Cell A may fail.

Proposal 5: RAN2 to discuss the issue when the UL WUS configuration update in Cell A is not

synchronised with the UL WUS configuration update in the NES cell.

Other cell selection cases for triggering

Proposal 6: Initial cell selection and recovery from out of coverage are not a trigger of the UL WUS transmission.

Random Access Response acquisition

Proposal 7: The UE should check whether SIB1 is transmitted from NES cell as the triggering condition and stopping condition for UL WUS procedure.

Failure case handling

Proposal 8: The legacy UE behaviour can be reused upon random access failure of on-demand

SIB1 request, i.e., the NES UE should follow the intraFreqReselection in MIB of NES cell.

On-demand SIB1 reception

Proposal 9: RAN2 to wait for the RAN1 conclusion on the configuration of RAR reception and SIB1 reception.

On-demand SIB1 update in NES cell

Proposal 10: RAN2 to discuss how long the updated on-demand SIB1 is transmitted from the next modification period of the NES cell. The following options are considered:

- Option 1: the updated SIB1 is transmitted during whole next modification period,

- Option 2: the updated SIB1 is transmitted during time window for type 0 PDCCH monitoring occasions.

Proposal 11: The UE considers the NES cell as barred if the UE is unable to acquire the updated SIB1.

Access restriction for legacy UEs

Proposal 12: MIB barring in Rel-19 NES cell should be considered as baseline for legacy UE handling.

Proposal 13: Whether/how to use cellBarred or ssb-SubcarrierOffset for barring of legacy UEs,

RAN2 should wait for RAN1 progress.

Observation 2: Using excluded cell list for on-demand SIB1 cells would be beneficial to reduce the attempts of SSB/SIB1 acquisition.

Proposal 14: The Rel-19 NES UE can consider the cell in the legacy excluded cell list as candidate, if the UE has the UL WUS configuration corresponding to the cell in the excluded cell list.

# References

- R2-2407571, Report from session on V2X/SL, R19 NES and MOB