3GPP TSG-RAN WG2 Meeting #127b	 R2-2408415

Hefei, China, Oct 14-18, 2024

Agenda Item:	8.4.2

Source: 	NEC

Title:  	Discussion on LP-WUS in RRC_IDLE/INACTIVE

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#127 meeting, the following agreement related to LP-WUS in RRC_IDLE/INACTIVE are made:

Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ.

The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

In this contribution, we based on the achieved agreement, continue to discuss further issue related to configuration aspects, subgrouping, SI/PWS reception and mobility.

- 2 Discussion

- 2.1 Configuration aspects

For RRC_IDLE/INACTIVE UE, since there is no dedicated signalling from NW, signal strength/quality based entry/exit condition of LP-WUS monitoring is introduced and we need to further discuss whether this is mandatory or not due to the fact that same coverage between LR and MR is assumed. To our understanding, even though same coverage is assumed, the risk of LP-WUS false monitoring by LR is still higher than paging false monitoring by MR considering receiver capability, therefore, LP-WUS monitoring should only be performed when the signal strength/quality is good enough.

Proposal-1: the entry/exit condition of LP-WUS monitoring is mandatory present in the LP-WUS related configuration.

Furthermore, we believe that there could be some cases to also restrict UE to perform LP-WUS monitoring besides entry/exit condition, e.g., UL transmission, other feature (e.g., SDT/MBS) is ongoing during RRC_IDLE/INACTIVE, or latency requirement, etc. If these exceptional cases are existing, the UE is not allowed to enter LP-WUS monitoring mode or is not allowed to evaluate LP-WUS monitoring entry/exit condition. In a word, entry/exit condition could be the general one, but more exceptional cases can be FFS.

Proposal-2: In addition to entry/exit condition, some exception cases which may impact on whether the UE can enter LP-WUS monitoring mode should be considered.

Even though the system information is promising to provide LP-WUS configuration including entry/exit condition, but one question is that whether a unified entry/exit condition broadcasted by system information is suitable for all UEs in one cell considering different UE may have different LR capability, therefore a UE specified entry/exit condition of LP-WUS monitoring by dedicated RRC signalling can also be considered as an optional feature, here, since this is intended for RRC_IDLE/INACTIVE UE, RRCRelease is more suitable.

Furthermore, if the LP-WUS configuration is only provided by system information, it means all UE who support LP-WUS feature may use LP-WUS monitoring mode, which means there is no explicit indication to guide UE behaviour. One enhancement is that based on NW implementation, RRCRelease is used to explicitly indicate which UE could use LP-WUS monitoring mode. In this way, NW can coordinate UE between LP-WUS and paging monitoring so that avoid large amount of UEs monitoring LP-WUS which could reduce power saving gain.

Proposal-3: support dedicated RRC signalling (i.e., RRCRelease) to provide the UE-specific LP-WUS configuration, e.g., the UE-specific entry/exit condition of LP-WUS monitoring.

Proposal-3a: support dedicated RRC signalling to indicate whether the UE can perform LP-WUS monitoring or not.

- 2.2 Subgrouping

Last meeting we postponed the agreement related to subgrouping:

In UE_ID based subgrouping, a mechanism should be applied that allocates PEI and LP-WUS subgroups independently and separately (if UE is configured to use both PEI and LP-WUS and if UE support both) in order to reduce the false paging rate.

The intention is to confirm that we should differentiate subgroup ID between LP-WUS and PEI so that the UE who choose to monitor both signal could further reduce paging false alarm. Almost all company agree with this intention, the argument is from the bracket part. To our understanding, we already have agreement that whether the UE monitor both signal is based on UE implementation. Such a word “if UE is configured to use both PEI and LP-WUS could be a little bit ambiguous”, it could mean there is an explicit indication. So we suggest to remove the bracket part since it is already clear combining the previous agreement of UE implementation.

Proposal-4: agree the following agreement:

In UE_ID based subgrouping, a mechanism should be applied that allocates PEI and LP-WUS subgroups independently and separately in order to reduce the false paging rate.

Next we discuss how to achieve independent/separate subgrouping mechanism between LP-WUS and PEI.

There are 3 types for the subgrouping:

Type-1: Only CN based subgrouping is used for both LP-WUS and PEI

Type-2: Both CN based and UE-ID based subgrouping are used, i.e., UE-ID based for LP-WUS and CN based for PEI, vice versa

Type-3: Only UE-ID based subgrouping is used for both LP-WUS and PEI

For type-3 only UE-ID based subgrouping, it is promising to design LP-WUS subgrouping formula similar as PEI. Again question is that how to make sure that the UEs belonging to the same LP-WUS/PEI subgroup are assigned to different PEI/LP-WUS subgroup. In fact, one solution is that if different UE-ID based subgrouping number is configured between LP-WUS and PEI, according to the formula, the group of UEs belonging to the same one LP-WUS/PEI subgroup will be assigned to different PEI/LP-WUS subgroup by mod different subgroupsNumForUEID, which further reduce false alarm issue. Note that for this solution (i.e., different subgroupsNumForUEID configuration), although the gain is not so perfect compared with other orthogonality-like solution, it is very simple to be achieved as long as the NW broadcast different UE-ID based subgroup number between LP-WUS and PEI. Therefore, it is a promising solution, but instead of saying this is totally NW implementation, we would rather make some NOTEs for guidance since it is not easy to let future reader understand how to further reduce false alarm issue when the UE is monitoring both signal. For example, the NOTE could be “the UE is not required to monitor both LP-WUS and PEI if the UE-ID based subgroup number for LP-WUS and PEI is same”.

Proposal-4a: If UE-ID based subgrouping is used for both LP-WUS and PEI, it is up to NW implementation to broadcast different UE-ID based subgroup number for both LP-WUS and PEI. And a NOTE is needed, e.g., “the UE is not required to monitor both LP-WUS and PEI if the UE-ID based subgroup number for LP-WUS and PEI is same”.

When CN based subgrouping is involved, e.g., definition of type-1 and type-2 in this contribution, the network can, based on implementation, make a smart decision on differentiating LP-WUS and PEI subgrouping ID among UEs belonging to the same PEI/LP-WUS subgroup. Question for type-2 is that how CN is aware of UE-ID based subgrouping information, for example, if UE-1 and UE-2 are using UE-ID based subgrouping for LP-WUS, when CN assign PEI subgroup, does it know UE-1 and UE-2 are assigned for the same LP-WUS subgroup or not? It could be FFS.

Proposal-4b: If CN based subgrouping is used for both LP-WUS and PEI, it is up to NW implementation to differentiate CN based subgroup ID between UEs belonging to the same LP-WUS/PEI subgroup.

Proposal-4c: If both CN based and UE-ID based subgrouping are used simultaneously, it is up to NW implementation to differentiate CN based subgroup ID between UEs belonging to the same LP-WUS/PEI subgroup, FFS how CN is aware of UE-ID based subgrouping information.

- 2.3 LS from RAN1

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE.

When we look at the historical RAN1 discussion, the issue is raised about whether LR and MR can camp on different carriers. From RAN2 perspective, since RRC_IDLE/INACTIVE UE need to camp on initial downlink BWP, which means the MR should work on this BWP, therefore, it is promising that LR should also work on this same BWP. In fact, if different carrier between LR and MR are assumed, it would bring impact on what we are discussing currently, for example, we are discussing LP-WUS monitoring entry/exit condition by evaluating both MR and LR, and MR measurement relaxation/offloading, if LR is camping on different carrier, the LR result somehow cannot be used for determining some operations related to MR. Therefore, at least in this release, it is suggested to only support that LR and MR work on the same carrier during RRC_IDLE/INACTIVE.

Proposal-5: from RAN2 perspective, it is suggested to only support that LR and MR work on the same carrier for RRC_IDLE/INACTIVE at least in Rel-19.

- 2.4 SI/PWS reception

It has been agreed that “From RAN2 perspective, no new procedure is introduced for SI reception/updates”, which means if there is short message coming, the NW would firstly use LP-WUS to trigger UE to wake up, then the UE receive paging short message and proceed system information modification as legacy way. Basically this is same mechanism as PEI solution, but there is some difference, i.e., MR and LR transition is involved.

Based on the current achieved agreement, if the UE receive LP-WUS indication and turn on its MR, realising this paging is for short message (e.g., SI update or PWS reception). What is UE behaviour after SI/PWS reception? For normal UE, the UE will keep monitoring PEI/PO by using MR, but for LP-WUS capable UE, if there is no unicast paging, after reception of SI/PWS, keeping MR turned on is unnecessary, the LP-WUS monitoring UE should go back to LP-WUS monitoring mode again for power saving reason, i.e., turn off MR and keep using LR to monitoring LP-WUS.

Proposal-6: After SI acquisition / PWS reception during MR-ON, the LP-WUS monitoring UE should go back to LP-WUS monitoring mode, i.e., turn off MR and keep using LR to monitoring LP-WUS.

- 2.5 Mobility

Regarding mobility (i.e., cell reselection), since measurement results for both serving cell and neighbour cell can be derived during LP-WUS monitoring mode, it is expected that cell reselection can be supported for a LP-WUS capable UE during LP-WUS monitoring mode. And one of the reasons why mobility during LP-WUS monitoring mode should be supported is that we are expecting the UE could monitor LP-WUS as much as possible among cells as long as there is no paging/data/signal arrival even though the cell has already been changed, staying LP-WUS monitoring mode as much as possible is beneficial for UE power saving.

Fig.1 Cell reselection for LP-WUS capable UE

One example for intra frequency is that, in legacy the UE perform cell reselection based on the best cell. However if the best cell is not supporting LP-WUS feature meanwhile some other cells which support LP-WUS feature are also good enough for camping, by legacy way, the LP-WUS capable UE need to exist from LPWUS monitoring mode and camp on the best cell, which is not friendly for power saving. Therefore some solutions could be considered to let UE perform cell reselection to select one cell which supports LP-WUS feature as much as possible. For example, neighbour cell information can be provided by serving cell, which can inform UE of the information about which cell is providing LP-WUS configuration, i.e., LP-WUS supporting cell. Then the UE can, based on the neighbour cell information, optimize its cell reselection decision, of which details is FFS.

Proposal-7: RAN2 is suggested to consider optimization on cell reselection, e.g., introducing neighbour cell information to facilitate LP-WUS monitoring UE to camp on LP-WUS supporting cell as much as possible.

- 3 Conclusions

Proposal-1: the entry/exit condition of LP-WUS monitoring is mandatory present in the LP-WUS related configuration.

Proposal-2: In addition to entry/exit condition, some exception cases which may impact on whether the UE can enter LP-WUS monitoring mode should be considered.

Proposal-3: support dedicated RRC signalling (i.e., RRCRelease) to provide the UE-specific LP-WUS configuration, e.g., the UE-specific entry/exit condition of LP-WUS monitoring.

Proposal-3a: support dedicated RRC signalling to indicate whether the UE can perform LP-WUS monitoring or not.

Proposal-4: agree the following agreement:

In UE_ID based subgrouping, a mechanism should be applied that allocates PEI and LP-WUS subgroups independently and separately in order to reduce the false paging rate.

Proposal-4a: If UE-ID based subgrouping is used for both LP-WUS and PEI, it is up to NW implementation to broadcast different UE-ID based subgroup number for both LP-WUS and PEI. And a NOTE is needed, e.g., “the UE is not required to monitor both LP-WUS and PEI if the UE-ID based subgroup number for LP-WUS and PEI is same”.

Proposal-4b: If CN based subgrouping is used for both LP-WUS and PEI, it is up to NW implementation to differentiate CN based subgroup ID between UEs belonging to the same LP-WUS/PEI subgroup.

Proposal-4c: If both CN based and UE-ID based subgrouping are used simultaneously, it is up to NW implementation to differentiate CN based subgroup ID between UEs belonging to the same LP-WUS/PEI subgroup, FFS how CN is aware of UE-ID based subgrouping information.

Proposal-5: from RAN2 perspective, it is suggested to only support that LR and MR work on the same carrier for RRC_IDLE/INACTIVE at least in Rel-19.

Proposal-6: After SI acquisition / PWS reception during MR-ON, the LP-WUS monitoring UE should go back to LP-WUS monitoring mode, i.e., turn off MR and keep using LR to monitoring LP-WUS.

Proposal-7: RAN2 is suggested to consider optimization on cell reselection, e.g., introducing neighbour cell information to facilitate LP-WUS monitoring UE to camp on LP-WUS supporting cell as much as possible.

- 4 References