3GPP TSG-RAN WG2 Meeting #127b	 R2-2408418

Hefei, China, Oct 14-18, 2024

Agenda Item:	8.1.3

Source: 	NEC

Title:  	AIML MDT-based data collection for NW-side model

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#127 meeting, the following agreement related to NW-side data collection area made:

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network .

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

Agreements

- The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

- Data collection report will not be transmitted over SRB1.  FFS which SRB is used.

In this contribution, based on immediate MDT framework for NW-side data collection, we further consider the framework regarding logging/report type and configuration release.

- 2 Discussion

- 2.1 Considerations on data collection logging/report

For data collection report type, currently RAN2 has agreed to support periodical report, on-demand report. Event-based report should be further discussed.

To our understanding, data collection report is a long term procedure, large amount of data collection report signalling could make big impact on both UE and NW side. For example, for UE-side UL transmission resource, how to ensure the regular unicast data/signalling will not be impacted by data collection report? For UE-side power consumption, once the UE is detecting overheating or low battery, how to ensure the data collection report would not make thing even worse? For NW-side, it is not desirable that the gNB always need to receive huge amount of uninterrupted report from a large amount of UEs. Therefore, we think data collection report should have some limitation, besides on-demand report, event-based data collection report should be supported. And we can further study what type of event is needed.

Observation-1: it is beneficial that data collection should have some limitation by event-based report.

Last meeting we have one FFS that whether AS buffer event based reporting is supported. To our understanding, this is beneficial. Because we agreed that minimum AS layer memory size will be introduced to limit UE storage, the UE cannot store unlimited logged data. Once the logged data size reach to buffer size limitation, what is UE behavior? Does UE need to delete some of the previously logged data? Or the UE initiate data report? We think the former one is not efficient since it has already cost UE power consumption for logging but eventually not be reported. Therefore, AS buffer event based report should be supported. Furthermore, one more detail is that once the UE report the logged data, the buffer can be cleared so that UE can start a new round of data collection.

Proposal-1: support AS buffer event based report, i.e., when the logged data size reach to buffer size limitation, the UE report logged data to NW, and clear its buffer after report.

Now we discuss the common issue for both logging and report, for this actions, UE consumption and UL transmission resource is required. But noted that for a RRC_CONNECTED UE, data collection is not the only thing that the UE need to do. We need to balance between data collection and other operations performed by a RRC_CONNECTED UE. Therefore, some condition is needed to pause data collection logging/report. For example, upon the UE is detecting overheating (which is already specified in the spec), upon the UE is suffering from low battery, upon the UE is experiencing high traffic period, the UE is not allowed to perform data logging/report.

Proposal-2: some conditions should be considered to pause data collection logging/report, e.g., overheating, battery, high traffic period, etc.

- 2.2 Release condition of data collection configuration

We agreed that immediate MDT is the baseline to support data collection for both gNB-side and OAM-side. Immediate MDT configuration is reusing the existing measurement framework, which means the configuration is provided by RRCReconfiguration. If the data collection is done, the NW could explicitly release the configuration. Besides explicit way, we think some mechanism from logged MDT can be referred, i.e.,using maximum logging duration, or maximum data size required by NW could be considered to let UE release the configuration automatically, in this way, NW-UE signalling interaction could be reduced. For example, the NW could configure the UE to perform data collection for 12 hours (e.g., at night), when timer is expired, the data collection configuration could be released. For example, the NW could configure the UE to perform data collection up to X MB data, and the configuration could be released once the UE reported up to X MB data.

Proposal-3: maximum timer or maximum data size can be considered to release data collection configuration, i.e., when the UE has already reported data up to max timer/size, the UE release the configuration.

- 3 Conclusions

Proposal-1: support AS buffer event based report, i.e., when the logged data size reach to buffer size limitation, the UE report logged data to NW, and clear its buffer after report.

Proposal-2: some conditions should be considered to pause data collection logging/report, e.g., overheating, battery, high traffic period, etc.

Proposal-3: maximum timer or maximum data size can be considered to release data collection configuration, i.e., when the UE has already reported data up to max timer/size, the UE release the configuration.

- 4 References