3GPP TSG-RAN WG2 Meeting #127bis											R2-2408421

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.7.4.1

Source: 	Sharp

Title:  	Additional LCH Priority Handling and Prioritization

Document for: 	Discussion

# Introduction

In RAN#105, WID on Phase 3 XR has been updated to have a normative objective for additional LCH priority as follows [1]:

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This document discusses detailed procedures of additional LCH priority.

# Discussion

During the study phase for the scheduling enhancement, RAN2 made the following agreements on additional logical channel priority:

Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. The solution should consider impact on UE complexity (as already indicated in SI objective description) For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR).

The additional priority can be another logical channel priority value specific to a logical channel. Thus, from configuration perspective, it is natural that logicalChannelConfig includes the additional LCH priority.

Additional LCH priority is configured by logicalChannelConfig in an RRC message.

It may be possible that multiple additional LCH priority values are configured for different conditions. However, we see that it brings an unnecessary complexity without much gain. For simplicity, it would be desirable to have at most one additional LCH priority for a logical channel.

At most one additional LCH priority can be configured for each logical channel.

The next issue is whether a separate remaining time threshold is needed for the additional LCH priority. The threshold would determine whether the data needs to be treated urgently. The concept of Rel-18 DSR procedure can be simply reused. Hence, each logical channel can use the additional LCH priority when the logical channel has available delay critical data with remaining time smaller than the Rel-18 remaining time threshold.

Additional LCH priority is used when the logical channel has available delay-critical data for initial transmission with remaining time smaller than the Rel-18 remaining time threshold.

The next issue is fallback priority procedure to the legacy LCH priority. If all delay-critical data have been transmitted by MAC PDUs, there is no remaining delay-critical data waiting for the subsequent LCP. In this case, all remaining data are not delay-critical. There is no need to keep the additional priority.

When the legacy LCH priority is used when the logical channel does not have available delay-critical data for initial transmission with remaining time smaller than the Rel-18 remaining time threshold.

LCP is per LCH operation that each LCH participates in LCP to receive portion of a MAC PDU. There is no special handling for a particular data. Hence, the additional LCH priority applies to whole LCH when it has any delay-critical data.

Additional LCH priority applies to whole LCH.

The additional LCH priority was discussed as an LCP enhancement. However, LCH priority is used for intra-UE prioritization configured with Rel-16 lch-basedPrioritization. In Rel-16 lch-basedPrioritization, priority of an uplink grant is determined by using the LCH priority. Also, subclause 5.4.4 on SR uses priority of an SR i.e., the priority of the logical channel that triggered SR, for resource overlap among PUSCH and SR in Rel-16 lch-basedPrioritization. All those legacy intra-UE prioritization rules assume a single LCH priority. RAN2 needs to discuss how to determine those priorities when we have both legacy priority and additional priority.

RAN2 to discuss how to determine the priority of an uplink grant and priority of a logical channel which triggered an SR in lch-basedPrioritization (intra-UE prioritization).

# Conclusions

Based on the discussion given above, we have the following proposals:

- Additional LCH priority is configured by logicalChannelConfig in an RRC message.

- At most one additional LCH priority can be configured for each logical channel.

- Additional LCH priority is used when the logical channel has available delay-critical data for initial transmission with remaining time smaller than the Rel-18 remaining time threshold.

- When the legacy LCH priority is used when the logical channel does not have available delay-critical data for initial transmission with remaining time smaller than the Rel-18 remaining time threshold.

- Additional LCH priority applies to whole LCH.

- RAN2 to discuss how to determine the priority of an uplink grant and priority of a logical channel which triggered an SR in lch-basedPrioritization (intra-UE prioritization).

# Reference

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur)