3GPP TSG-RAN WG2 Meeting #127bis	R2-2408432

Hefei, China, 14 – 18 October 2024

Agenda Item:	7.4.1

Source:	Google

Title:	Discussion for Replying LS on inter-DU Early TA acquisition and Preamble Resources for PDCCH order in LTM

Document for:	Discussions & Approval

# 1	Discussion

RAN3 had been discussing the CFRA resource configuration for the PDCCH order in Inter-gNB-DU LTM since the completing stage of the Rel-18 LTM. As the discussion concerns the details of RACH operations as well as the usage of RA-RNTI and association for the TA information, there were no clear conclusion whether the current specification adequately supports the Early TA acquition and utilize of CFRA resources for inter-DU LTM scenario as effectively as it does for intra-DU LTM. The current messaging flows on the Early TA acquisition can be illustrated for the inter-DU LTM as below.

Figure 1: Illustration of Ealry TA acquisition for inter-DU LTM as per current specification

There are two questions from RAN3 specifically for inter-DU LTM:

Based on the previous RAN2 agreements, RAN3 had discussed the necessary parameters required for early TA acquisition for the inter-DU LTM scenario. RAN3 designed the Early UL Sync Configuration IE in F1AP including a RACH configuration (containing the RRC EarlyUL-SyncConfig IE) and a preamble index list assigned for the source gNB-DU. However, RAN3 lacks clarity on whether such design adequately supports the utilization of CFRA resources for inter-DU LTM scenario as effectively as it does for intra-DU LTM. Therefore, RAN3 seeks to understand the following two questions for early TA acquisition in inter-DU LTM: Question 1: Are shared preamble resources from the RAN2 agreement, consisting of the combination of SSB index, PRACH mask index, and preamble index, necessary, or is the preamble index list sufficient? Question 2: When using the RA-RNTI, is “SSB Positions In Burst” a necessary parameter to associate the TA value with a specific UE?

#### Discussion for Question 1 – shared preamble resource

The first question concerns about whether the preamble index list assigned by the candidate gNB-DU alone would be enough as it does for intra-DU LTM.

- RAN2#121bis-e (R2-2304553) From RAN2 perspective, to enable shared preamble resource among multiple UEs, it is beneficial that the information that identifies the allocated CFRA resource (i.e., SS/PBCH index, RACH occasion, and Random Access Preamble index) can be indicated in the PDCCH order (as legacy intra-cell PDCCH order).

As also indicated in the previous LS to RAN3, RAN2 has specified that the allocated CFRA resource can be indicated in the PDCCH order. This agreement is based on a RAN2 discussion [5]. For the intra-DU LTM, the same gNB-DU is in charge of the allocation of CFRA resource and sending the PDCCH order so that it can recognize the UE by the preamble index and the RACH occasion. For the inter-DU LTM, as the allocation is done by the candidate gNB-DU while the PDCCH order is done by the source gNB-DU, based on the current RAN3 specification, the source gNB-DU has to figure out the SSB index and the PRACH mask index to be indicated in the PDCCH order by itself. From the candidate gNB-DU side, based on the current RAN3 specification, since only a per-DU preamble index list is signalled to the source gNB-DU, it has to assume that all of the SSB index and PRACH mask index could be used by the source gNB-DU in the PDCCH order. Therefore, it can only allocate different preamble index(es) to different gNB-DU(s) so that it can differentiate which source gNB-DU to send back the TA value and the associated TA information (i.e., cell ID, preamble index, and RA-RNTI) for a UE. Although this design can support the early TA acquisition, it only utilizes the parameter ‘preamble index’ while there is no such limitation for intra-DU LTM.

The candidate gNB-DU can actually allocate the preamble resource that it can differentiate by the preamble index and the RACH occasion for the UE from different source gNB-DU to support early TA acquisition as in the intra-DU LTM. The preamble resource is not limited to only the ‘preamble index’ since it could be easily used up if it is the only utilizable parameter. Therefore, from our perspective, the shared preamble resource can consist of the combination of SSB index, PRACH mask index, and preamble index and it is up to implementation how the candidate gNB-DU allocates it. In our understanding, RAN2 did not intend to limit the preamble resource to only the preamble index and we suggest RAN3 taking it into account for better utilization of the shared preamble resource in the RAN3 inter-DU signalling design.

Observation 1: RAN2 did not intend to restrict the preamble resource solely to the preamble index in the inter-DU scenario.

Proposal 1: For inter-DU LTM, shared preamble resources as agreed upon by RAN2 can be formed by combining SSB index, PRACH mask index, and preamble index.

#### Discussion for Question 2 – SSB Positions In Burst for the source gNB-DU

The second question concerns about whether (and how) the ‘ssb positions in burst’ plays a role in the RA-RNTI calculation for a UE at the source gNB-DU. To answer this question

- RAN2#123-bis (R2-2311332) RAN2 understands that the source DU needs to know the early RACH configuration for each candidate cell, so that the source cell can know how to set the PDCCH order information for early RACH; The candidate DU provides the TA value and its associated information to the source DU via the CU, e.g. preamble index, RO information (i.e. RA-RNTI) and candidate cell identity, so that the source DU can identify the UE. RAN3 can design the necessary network signalling.

RAN3 had implemented the above agreements by including a RACH Configuration IE including the EarlyUL-SyncConfig (i.e., the early RACH configuration as the above agreement) in a UE Context Modification Request message and including the cell ID, preamble index, and the RA-RNTI in a DU-CU TA Information Transfer message and CU-DU TA Information Transfer message from the candidate gNB-DU to the source gNB-DU via the gNB-CU. These two agreements are based on a RAN2 discussion [6]. As shown in the example scenario 1 in the discussion [6], the source DU instructs two UEs in the same source cell to send preambles to the same candidate cell. The source gNB-DU triggers different UEs to use either different a preamble index or a different RO (i.e., different SSB index and/or PRACH mask index), the additional information along with the TA value can be the preamble index and the RO information. Therefore, there should be a way for the source gNB-DU to associate with the TA values and the different UEs by the preamble index and/or the RO information.

To associate the RO information, the source gNB-DU needs to know the exact PRACH occasion used by the UE during the PDCCH ordered CFRA so that it can calculate the corresponding RA-RNTI. The exact PRACH occasion would be one of the one or more PRACH occasion(s) indicated by the PDCCH order for the UE as the PRACH mask index could allow more than one PRACH occasion of a SSB. Similar to the candidate gNB-DU or the UE, the source gNB-DU needs to have a mapping between the SSB indexes to PRACH occasions. The EarlyUL-SyncConfig provides for the source gNB-DU to construct the mapping several parameters such as the ssb-PerRACH-Occasion, the prach-ConfigurationIndex, the msg1-FDM, etc. Additionally, the ssb-PositionsInBurst also serves as one of the key parameters for this mapping according to the TS 38.213 Section 8.1. According to the SSB index, the PRACH mask index and this mapping, the source gNB-DU is able to calculate the one or more RA-RNTI(s) corresponding to the allowed PRACH occasion(s) as indicated in the PDCCH order. Therefore, when it receives the TA information along with the TA value, it can associate the UE using the TA information (i.e., the cell ID, the preamble index, and the RA-RNTI). The value of the received RA-RNTI should be equal to the value of one of the RA-RNTI(s) so that the source gNB-DU knows which TA value is for which UE. An example illustration of SSB indexes to valid RO mapping is as below.

Figure 2: Example illustration for the PRACH occasion and SSB Index mapping

It is worth noting that the RA-RNTI is not always necessary for the source gNB-DU to associate the TA value and a UE. For example, the source gNB-DU only triggers one UE at a time to perform the PDCCH ordered CFRA and it does not trigger the next UE using the same preamble index to the same cell until it receives the TA value with the cell ID and the preamble index. In such cases, the RA-RNTI may not be vital for the association. However, for other examples, as long as the RO information is needed as a differentiation, the RA-RNTI serves as a way for the source gNB-DU to associate the TA value and the UE.

Proposal 2: The 'SSB Positions In Burst' parameter is essential for the source gNB-DU to calculate the RA-RNTI, which in turn allows for associating a TA value with a UE when RACH occasion differentiation is required.

If the above proposals are agreeable, RAN2 can provide a reply LS based on a draft reply LS as in [9].

Proposal 3: RAN2 to reply RAN3 considering the draft reply LS in R2-2408433.

# 3	Conclusion

Observation 1: RAN2 did not intend to restrict the preamble resource solely to the preamble index in the inter-DU scenario.

Proposal 1: For inter-DU LTM, shared preamble resources as agreed upon by RAN2 can be formed by combining SSB index, PRACH mask index, and preamble index.

Proposal 2: The 'SSB Positions In Burst' parameter is essential for the source gNB-DU to calculate the RA-RNTI, which in turn allows for associating a TA value with a UE when RACH occasion differentiation is required.

Proposal 3: RAN2 to reply RAN3 considering the draft reply LS in R2-2408433.

# 4	Reference

[1] R2-2407924/R3-244793, LS on Early TA acquisition for the inter-DU scenario

[2] R2_121bis-e Chair Notes EOM

[3] R2 123bis Session Notes eMob mIAB LPWUS

[4] R2 125bis Report from session on Mobility Enh and Mobile IAB

[5] R2-2302750, Discussion on the early TA acquisition, Intel Corporation

[6] R2-2310999, RRC aspects for LTM, Huawei, HiSilicon

[7] R2-2304553, Reply LS on L1 measurement RS configuration and PDCCH ordered RACH for LTM, Fujitsu, CATT

[8] R3-244177, Discussion for Early TA acquisition and CFRA resource configurations in LTM, Google

[9] R2-2408433, [Draft] Reply LS on Early TA acquisition for the inter-DU scenario

# Appendix

TS 38.213, Section 8.1 – the SSB index to PRACH occasion mapping logic

SS/PBCH block indexes provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon are mapped to valid PRACH occasions in the following order where the parameters are described in [4, TS 38.211]. - First, in increasing order of preamble indexes within a single PRACH occasion - Second, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions - Third, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot - Fourth, in increasing order of indexes for PRACH slots An association period, starting from frame 0, for mapping SS/PBCH block indexes to PRACH occasions is the smallest integer number in the set determined by the PRACH configuration period according Table 8.1-1 such that NTxSSB SS/PBCH block indexes are mapped at least once to the PRACH occasions within the association period, where a UE obtains NTxSSB from the value of ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon. If after an integer number of SS/PBCH block indexes to PRACH occasions mapping cycles within the association period there is a set of PRACH occasions or PRACH preambles that are not mapped to NTxSSB SS/PBCH block indexes, no SS/PBCH block indexes are mapped to the set of PRACH occasions or PRACH preambles. An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec. PRACH occasions not associated with SS/PBCH block indexes after an integer number of association periods, if any, are not used for PRACH transmissions. … Table 8.1-1: Mapping between PRACH configuration period and SS/PBCH block to PRACH occasion association period PRACH configuration period (msec); Association period (number of PRACH configuration periods). 10; {1, 2, 4, 8, 16}. 20; {1, 2, 4, 8}. 40; {1, 2, 4}. 80; {1, 2}. 160; {1}.

TS 38.212, Sec 7.3.1.2.1 – the content of PDCCH order

DCI format 1_0 is used for the scheduling of PDSCH in one DL cell. The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats - 1 bits - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment - bits where is given by Clause 7.3.1.0 If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows: - Random Access Preamble index - 6 bits according to ra-PreambleIndex in Clause 5.1.2 of [8, TS38.321] - UL/SUL indicator - 1 bit. If the value of the "Random Access Preamble index" is not all zeros and if the UE is configured with supplementaryUplink in ServingCellConfig in the cell, this field indicates which UL carrier in the cell to transmit the PRACH according to Table 7.3.1.1.1-1; otherwise, this field is reserved - SS/PBCH index - 6 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the SS/PBCH that shall be used to determine the RACH occasion for the PRACH transmission; otherwise, this field is reserved. - PRACH Mask index - 4 bits. If the value of the "Random Access Preamble index" is not all zeros, this field indicates the RACH occasion associated with the SS/PBCH indicated by "SS/PBCH index" for the PRACH transmission, according to Clause 5.1.1 of [8, TS38.321]; otherwise, this field is reserved - Cell indicator - log2C+1 bits indicating the cell for the corresponding PRACH transmission if the UE is configured with higher layer parameter EarlyUlSyncConfig, where C is the number of candidate cells configured with higher layer parameter EarlyUlSyncConfig; 0 bit otherwise. The bit field index 0 of the cell indicator field is mapped to the serving cell, and other bit field indexes are mapped to the candidate cells configured with higher layer parameter EarlyUlSyncConfig according to an ascending order of a candidate identity configured by ltm-CandidateId, with the bit field index 1 mapped to the candidate cell with the smallest candidate identity. …

TS 38.321 – RA-RNTI calculation and PRACH Mask Index/ ra-ssb-OccasionMaskIndex values

### 5.1.3 Random Access Preamble transmission … The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted or the RA-RNTI associated with the last valid PRACH occasion in the set of PRACH occasions (as specified in TS 38.213 [6]) for Msg1 repetition, is computed as: RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier). ## 7.4 PRACH Mask Index values Table 7.4-1: PRACH Mask Index values PRACH Mask Index/ msgA-SSB-SharedRO-MaskIndex/ ssb-SharedRO-MaskIndex; Allowed PRACH occasion(s) of SSB. 0; All. 1; PRACH occasion index 1. 2; PRACH occasion index 2. 3; PRACH occasion index 3. 4; PRACH occasion index 4. 5; PRACH occasion index 5. 6; PRACH occasion index 6. 7; PRACH occasion index 7. 8; PRACH occasion index 8. 9; Every even PRACH occasion. 10; Every odd PRACH occasion. 11; Reserved. 12; Reserved. 13; Reserved. 14; Reserved. 15; Reserved.

TS 38.473

#### 9.2.1.24 DU-CU TA INFORMATION TRANSFER This message is sent by the gNB-DU to inform the gNB-CU about TA information. Direction: gNB-DU gNB-CU IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; ignore. Transaction ID; M; 9.3.1.23; YES; reject. DU to CU TA Information List; 1; YES; ignore. >DU to CU TA Information Item IEs; 1 .. <maxnoofTAList>; EACH; ignore. >>Candidate Cell ID; M; NR CGI 9.3.1.12; -. >>TA Value; M; INTEGER (0..4095); Indicates the TA value as defined in TS 38.213 [31].; -. >>Preamble Index; M; INTEGER (0..63); -. >>RA-RNTI; M; INTEGER (0..65535, ...); RA-RNTI as defined in TS 38.321 [16].; -. >>Source gNB-DU ID; M; gNB-DU ID 9.3.1.9; -. >>Tag ID Pointer; O; OCTET STRING; Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8].; -. #### … #### 9.3.1.328 Early UL Sync Configuration This IE indicates the early UL sync configurations for the UE. IE/Group Name; Presence; Range; IE type and reference; Semantics description. RACH Configuration; M; OCTET STRING; Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [8].. LTM gNB-DUs List; 0..1; This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s).. >LTM gNB-DUs Item IEs; 1..< maxnoofLTMgNBDUs>. >>LTM gNB-DU ID; M; gNB-DU ID 9.3.1.9. >>Preamble Index List; O; 9.3.1.329. Range bound; Explanation. maxnoofLTMgNBDUs; Maximum no. of gNB-DUs allowed to be configured with LTM towards one UE, the maximum value is 8.. #### 9.3.1.329 Preamble Index List This IE indicates the list of preamble indexes to be used for the UE. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Preamble Index Item IEs; 1..< maxnoofLTMCells>; -. >Preamble Index; M; INTEGER (0..63); -. Range bound; Explanation. maxnoofLTMCells; Maximum no. of Cells configured LTM allowed towards one UE, the maximum value is 8..