3GPP TSG-RAN WG2 Meeting #127bis	R2-2408447

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.4.2

Source:	Huawei, HiSilicon

Title:	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Document for:	Discussion and Decision

# Introduction

The WID [1] has the following objective related to the procedure and configuration of LP-WUS indicating paging monitoring triggerd by LP-WUS for IDLE/INACTIVE modes:

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

In this contribution, we discuss the related issues considering the progress in RAN1 and RAN2 so far.

# Discussion

## Configuration Aspects of LP-WUS

### Dedicated configuration in RRC Signalling

In RAN2-127, the FFS from the agreement in RAN2-125bis was discussed but nothing was concluded.

RAN2#125bis Agreement The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed.

The LP-WUS related configuration should be cell specific since it is for IDLE/INACTIVE state. The configuration includes both configuration for LP-SS and LP-WUS thus covering both OOK-based and OFDM-based receivers respectively. Providing dedicated configuration in RRC signalling for example in RRC Release limits the scenarios in which LP-WUS can be used. For example, when an LP-WUS UE is provided with dedicated configuration and if the UE selects a new cell due to mobility, the new cell is not aware of the dedicated configuration of the LP-WUS for the UE and thus the UE still needs to rely on the system information from the cell. Thus, there is no need for dedicated configuration.

Proposal 1: Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

### Enabling of LP-WUS

If LP-WUS functionality is enabled for all the UEs supporting LP-WUS in a camped cell, for example, when a UE receives LP-WUS configuration via SIB from the camped cell, it can use LP-WUS functionality. It is simple but may lead to a lot of UEs using LP-WUS, which means the false paging probability increases and causes more transitions from LP-WUR to MR. This will reduce the power saving gain or even result in negative power saving gain considering the maximum number of subgroups for LP-WUS discussed in RAN1 won't exceed 256. Hence there can be UE/group specific control for enabling LP-WUS.

To reduce false paging probability, the network (CN or RAN) decides whether a LP-WUS capable UE is allowed to use the LP-WUS functionality based on the conditions in the network (e.g., radio resources) and/or the characteristics of the UEs (e.g., paging probability, shorter DRX cycle, etc.). For example, the CN uses NAS signaling to indicate whether a specific UE can use LP-WUS functionality, which is similar to Rel-17 CN-assigned subgrouping method. As an alternative, without impacting CN, the RAN can indicate UEs that can use LP-WUS functionality via SIB. Once the UE is enabled to use LP-WUS functionality, it uses the LP-WUS related configuration via SIB.

Proposal 2: The network indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality, the detailed signalling is FFS.

### Configuration for different types of LP-WUR

RAN1 has been discussing to support both OOK-based and OFDM-based receiver for LP-WUS monitoring. Since the OFDM-based LP-WUR has stronger capability for LP-WUS signal reception and processing, it is possible for NW to configure a shorter LP-WUS to wake up only OFDM-based receiver to reduce the resource overhead. In this case, the UE with OFDM-based receiver can benefit from the low-complexity operation on LP-WUS, e.g., to cancel any OOK sequence/pattern detection or implement termination of LP-WUS procedure by early detection. The UE with OOK-based receiver will skip the current LP-WUS for obtaining more power saving gain. Thus, the NW should be able to support the scenario that only UEs with OFDM-based LP-WUR are paged, e.g., by sending a shorter LP-WUS that cannot be received by OOK-based receiver. This information can be implicitly indicated by the absence

Proposal 3: NW should configure whether only OFDM-based LP-WUR is supported in a cell.

### LP-WUS entry/exit condition

In the latest RAN2’s meeting [2], the following agreements are reached on the entry/exit condition for different types of LP-WUR.

RAN2#127 Agreement Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

RAN2 has agreed that the thresholds of LP-WUS entry/exit condition should be separately configured for different types of receiver. Considering the LP-WUS cell coverage cannot reach the full coverage of legacy cell as specified in the WID [1], the feasibility of LP-WUS feature may be greatly affected if the entry/exit condition is absent in the LP-WUS configuration, i.e., to leave when to enter/exit LP-WUS mode to UE implementation, since UE will spend plenty of time and resource overhead to decide whether LP-WUS can be reliably used. Therefore, it is beneficial that the thresholds of LP-WUS entry/exit condition are always carried in the LP-WUS related configuration as a reference for UE to determine the LP-WUS coverage with more precision. Thus, we propose that the thresholds of LP-WUS entry/exit conditions for a specific type of receiver should be always configured if supported by the network.

Proposal 4: Thresholds of LP-WUS entry/exit conditions for OOK and OFDM based receivers are always configured when the respective LP-WUS configuration is provided by the network.

RAN2 has also made a working assumption on whether the entry condition can be also based on the measurement result of LP-WUR in the latest RAN2#127 meeting [2],

RAN2#127 Working Assumption Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

Firstly, we would like to clarify that the above WA is only related to entry condition. To avoid ambiguity, we suggest to remove the ‘/exit’ from the WA.

UE usually performs RRM measurements by MR as per legacy behavior. Thus, the serving cell measurements by MR are always available and good enough. As the hardware capability of MR is better than that of LP-WUR, the measurements by LP-WUR do not provide any additional benefit for LP-WUS entry decision. On the other hand, using both MR and LP-WUS measurements can make the LP-WUS entry procedure complicated, introducing much specification impact. Hence, the measurement by LP-WUR is not needed for LP-WUS entry condition. We suggest RAN2 not to discuss or confirm this WA anymore, and to wait for RAN1/4’s further input on this issue.

Proposal 5: Whether the measurement by LP-WUR is needed or not for LP-WUS entry condition should wait for RAN1/4’s further input.

After entering LP-WUS mode, the regular measurement on SSB and/or LP-SS can ensure that UE will exit LP-WUS mode immediately after the exit condition is fulfilled. The baseline of LP-WUS exit condition should be that the serving cell measurement by LP-WUR (e.g., based on SSB and/or LP-SS depending on type of the receiver) is below configured threshold(s).

In addition to exit condition, the UE should turn on MR when it receives LP-WUS indication to monitor paging, or has data to transmit in UL.

Proposal 6: RAN2 to consider that the serving cell measurement by LP-WUR below configured threshold(s) as the baseline for LP-WUS exit condition. No need to introduce any additional conditions.

One important thing to consider while discussing entry/exit condition is to avoid frequent switching between LP-WUR and MR (i.e., ping-pong behaviour). Ping-pong can happen when a UE with mobility moves back and forth at the edge of LP-WUS coverage. It can also happen when the UE is stationary due to inconsistent measurement results over a short period of time (e.g., measurement deviation or instantaneous blockage of the channel). As ping-pong between LP-WUR and MR can severely impact the UE power saving gain, RAN2 needs to consider avoiding ping-pong e.g., by prolonging the decision of entry/exit condition prior to entry/exit.

Proposal 7: RAN2 to consider how to avoid ping-pong behavior for LP-WUS entry/exit procedure.

## Issues for paging monitoring triggered by LP-WUS

The primary purpose of LP-WUS in RRC_IDLE/INACTIVE is to indicate paging monitoring. RAN1 made the following related agreements in RAN1#116 [2]:

RAN1#116 Agreements - For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not. - It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up. - FFS: support of UE monitoring dynamic PO - Conclusion For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

Observation 1: Both options of UE monitoring legacy paging DCI/paging and monitoring PEI after MR wakes up are supported in RRC_IDLE/INACTIVE.

LP-WUS subgroup design is one of the main issues to be considered for LP-WUS paging design and RAN2 has agreed [4][5] that Rel-17 PEI subgrouping method can be reused.

RAN2#125bis Agreement The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups. RAN2#126 Agreement RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

### LP-WUS and PEI

When LP-WUS and PEI are both configured and used, there are two cases of potential subgrouping types possible for LP-WUS and PEI.

Case 1 - Same type of subgrouping method used for LP-WUS and PEI: In this case, both LP-WUS and PEI use either CN assigned subgrouping or UE_ID based subgrouping. For CN assigned subgrouping, CN can ensure proper subgrouping of the LP-WUS and PEI by implementation to reduce the power consumption. For UE_ID based subgrouping, it is straightforward to support it without CN control.

Case 2 - Different type of subgrouping method used for LP-WUS and PEI: In this case, CN assigned subgrouping is applied for LP-WUS and UE_ID based subgrouping is applied for PEI and vice-versa. We do not see any functional issues other than a slight impact on the power saving gain. Hence what type of subgrouping to apply for LP-WUS and PEI can be left to NW implementation.

Proposal 8: If LP-WUS and PEI are used together, it is up to NW to decide which types of subgrouping (i.e. CN assigned subgrouping or UE_ID based subgrouping) are applied.

### LP-WUS subgrouping

In practice, not all UEs in a subgroup are paged. When the UEs that are not paged in the subgroup wake-up (i.e., false alarm wake-up), power consumption for these UEs increases. If LP-WUS subgrouping is used together with the existing subgrouping for PEI, the false alarm wake-up for the UEs can be reduced. In this method, referred to as two-level subgrouping, UEs in the first level subgrouping monitor LP-WUS. And the UEs in the first level subgrouping are further distributed to different subgroups in the second level subgrouping.

The two-level UE_ID based subgrouping is shown in Figure 1. UE1 and UE2 are placed in the same group assigned with group ID 1 in LP-WUS as the first-level subgrouping. Then in the second-level subgrouping in PEI, UE1 and UE2 are placed in different groups. When UE1 is paged, UE2 does not need to monitor paging based on its PEI so that its paging false alarm can be further reduced.

Figure 1: Illustration of 2-level subgrouping

We calculate paging false alarm numerically and present the results in Table 1 when different number of UEs in one PO use LP-WUS with/without monitoring PEI. A brief description of calculating paging false alarm rate is summarized in Appendix. We assume the number of subgroups for both PEI and LP-WUS is 8, the paging probability of UE is 1% and the subgrouping of LP-WUS is independent or orthogonal to PEI subgrouping. The paging false alarm rate for arbitrary UE comes from the probability that UE receives the invalid paging when the subgroup of this UE is indicated by LP-WUS to wake up to monitor legacy PO/PEI.

Table 1. Paging false alarm rate for LP-WUS with/without monitoring PEI

| Number of UEs in one PO | Paging false alarm rate |  |
| --- | --- | --- |
|  | LP-WUS only | LP-WUS + PEI |
| 32 | 74.6% | 2.9% |
| 64 | 87.1% | 6.7% |
| 96 | 91.2% | 10.4% |
| 128 | 93.3% | 13.9% |
| 256 | 96.5% | 26.5% |

As seen from the numerical results, the UE can achieve a great reduction of paging false alarm rate after receiving LP-WUS with using PEI as the second-level independent grouping.

Observation 2: If UEs in the same subgroup indicated by LP-WUS are further placed in different subgroups by PEI, false paging probability can be further reduced.

Proposal 9: When using LP-WUS and PEI together, to obtain additional power saving gain, UEs from the same LP-WUS subgroup can be divided to different PEI subgroups.

To achieve the two-level subgrouping described above, especially when UE_ID based subgrouping is used for both LP-WUS and PEI, the UE_ID based subgrouping for LP-WUS should be independent of UE_ID based subgrouping for PEI subgrouping. For UE_ID based subgrouping for LP-WUS, the formula for UE_ID based subgrouping for PEI in TS 38.304 can be taken as base line and different bits can be used for LP-WUS and PEI subgrouping by adding new parameter(s). To be more specific, the design should make sure the bits used for LP-WUS subgrouping do not overlap with the bits used for PEI subgrouping. This can be addressed by introducing a value K multiplied with N*Ns in the formula of UE_ID based subgrouping as below:

SubgroupID = (floor(UE_ID/(N*Ns*K)) mod subgroupsNumForUEID_LP-WUS) + (subgroupsNumPerPO_LP-WUS - subgroupsNumForUEID_LP-WUS)

Similar to the function of N*Ns used in UE_ID based subgrouping formula of PEI, the effect of K is to separate the UEs in one LP-WUS subgroup into different PEI subgroups. Otherwise, UEs in one LP-WUS subgroup may be in the same PEI subgroup, resulting no further paging false alarm reduction by using PEI.

Proposal 10: The UE_ID based subgrouping formula in TS 38.304 can be reused for LP-WUS subgrouping with introducing new parameter(s), to ensure that the LP-WUS subgrouping can use different bits than the PEI subgrouping in the truncated UE ID, i.e.,

SubgroupID = (floor(UE_ID/(N*Ns*K)) mod subgroupsNumForUEID_LP-WUS) + (subgroupsNumPerPO_LP-WUS - subgroupsNumForUEID_LP-WUS),

where:

- N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state;

- Ns: number of paging occasions for a PF;

- UE_ID: 5G-S-TMSI mod X_LP-WUS, X_LP-WUS = 8092 * maximum number of LP-WUS subgroups;

- subgroupsNumForUEID_LP-WUS: number of subgroups for LP-WUS UE_ID based subgrouping in a PO, which is broadcasted in system information

- K: configurable value which is equal to or larger than 8;

## SI reception procedure considering LP-WUS

UE should reuse the legacy system information acquisition procedure to monitor short message in paging DCI for change of system information as well as ETWS and CMAS notification. SI changes from UE’s camped cell may involve critical configuration updates, e.g., LP-WUS configuration. In order to ensure that UEs monitoring LP-WUS do not experience any issues because they miss any SI updates, the consensus is that LP-WUS should be able to wake up all the UEs to receive SI update as well as ETWS/CMAS notification.

Proposal 11: LP-WUS should be able to wake up all UEs to monitor paging for SI update/ETWS/CMAS notification.

When SI changes, the network can reuse LP-WUS indicating paging monitoring and wake up UEs in all the subgroups. Then the UE will use MR to receive paging DCI that indicates SI change and acquires the updated SI in the following modification period. RAN1 has already agreed in RAN1#118 meeting [3] that a LP-WUS for IDLE/INACTIVE mode should indicate a codepoint value to carry subgroup information:

Agreement For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS: - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported - Value of X is larger than 1. FFS on additional details of X.

The number of radio resources required to wake up all the UEs can be very large, for example, up to 256 or 65536 LP-WUSs are required to wake up all the UEs using LP-WUR (depending on 8 or 16 bits are used for subgrouping info respectively).

Observation 3: A large amount of radio resources are required to wake up all the UEs using codepoint way.

To minimize radio resources, a straight forward way is to introduce a common LP-WUS and all the UEs using LP-WUR can monitor a common LP-WUS occasion, as specified in NB-IoT. If the common LP-WUS is received, all the UEs irrespective of their subgroups will transit to MR.

Proposal 12: Introduce a common LP-WUS to wake up all the UEs in RRC_IDLE/INACTIVE to monitor paging for SI update/ETWS/CMAS notification.

## RAN1 LS and LP-WUS prioritization in cell reselection

RAN1 has sent an LS [6] to RAN2 and RAN4 to check if there is any issue and specification impact in the case UE may not support LP-WUS reception on all the bands supported by the UE.

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

It’s feasible that UE supports LP-WUS reception only on some of the bands supported by the UE. As LP-WUS related configuration is provided via system information, the UE can determine whether to use LP-WUS reception based on NW configuration and its UE capabilities. From NW side, it needs to know whether the current band for paging transmission is supported by the UE for LP-WUS reception. This can be supported via existing UE paging related capability procedure i.e., UE indicates supported bands for LP-WUS reception as paging capability and gNB forwards radio paging capability to CN.

Observation 4: It is feasible to support LP-WUS reception only on some of the bands supported by the UE. In specification, UE should be able to report supported band(s) for LP-WUS reception to the network.

Based on RAN1’s LS, we can know in practical deployment, even if the LP-WUS is configured from NW perspective, it cannot be guaranteed that UE can use LP-WUS in all the cells/frequencies supporting LP-WUS. UE can use LP-WUS on a cell only if LP-WUS is enabled in the cell and cell’s band matches with UE capabilities. Otherwise, UE cannot use LP-WUS and power saving gain cannot be obtained.

Observation 5: Even UE supporting LP-WUS camps on a cell with LP-WUS reception enabled from NW perspective, it is possible that it still cannot use LP-WUS if the cell’s band does not match with UE capabilities, which does not provide any power saving gain.

Thus, it would be beneficial for LP-WUS capable UEs to camp on the matched frequency supporting LP-WUS as often as possible in order to obtain more power saving gain. For example, NW broadcasts the frequencies of neighbor cells that supporting LP-WUS, so that UE can prioritize the neighbor cells on which UE is able to use LP-WUS during cell (re)selection procedure. One possible implementation is to adjust the priority used for cell (re)selection, e.g., LP-WUS capable UEs may regard its supported frequency with LP-WUS enabled to be the higher priority. Especially, camping on a lower frequency is beneficial due to smaller path loss to obtain better coverage performance, thus if there are more than one neighbor cells on which UE is able to use LP-WUS, the UE can take a lower frequency among them as higher priority.

Proposal 13: UE supporting LP-WUS prioritizes its supported frequency with LP-WUS enabled in the cell (re)selection procedure. FFS on prioritization of lower frequency if multiple frequencies support LP-WUS.

# Conclusion

In this contribution, we discuss procedure and configuration of LP-WUS for IDLE/INACTIVE modes, and have the following proposals.

Configuration Aspects of LP-WUS

Proposal 1: Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

Proposal 2: The network indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality, the detailed signalling is FFS.

Proposal 3: NW should configure whether only OFDM-based LP-WUR is supported in a cell.

Proposal 4: Thresholds of LP-WUS entry/exit conditions for OOK and OFDM based receivers are always configured when the respective LP-WUS configuration is provided by the network.

Proposal 5: Whether the measurement by LP-WUR is needed or not for LP-WUS entry condition should wait for RAN1/4’s further input.

Proposal 6: RAN2 to consider that the serving cell measurement by LP-WUR below configured threshold(s) as the baseline for LP-WUS exit condition. No need to introduce any additional conditions.

Proposal 7: RAN2 to consider how to avoid ping-pong behavior for LP-WUS entry/exit procedure.

Issues for paging monitoring triggered by LP-WUS

Observation 1: Both options of UE monitoring legacy paging DCI/paging and monitoring PEI after MR wakes up are supported in RRC_IDLE/INACTIVE.

Proposal 8: If LP-WUS and PEI are used together, it is up to NW to decide which types of subgrouping (i.e. CN assigned subgrouping or UE_ID based subgrouping) are applied.

Observation 2: If UEs in the same subgroup indicated by LP-WUS are further placed in different subgroups by PEI, false paging probability can be further reduced.

Proposal 9: When using LP-WUS and PEI together, to obtain additional power saving gain, UEs from the same LP-WUS subgroup can be divided to different PEI subgroups.

Proposal 10: The UE_ID based subgrouping formula in TS 38.304 can be reused for LP-WUS subgrouping with introducing new parameter(s), to ensure that the LP-WUS subgrouping can use different bits than the PEI subgrouping in the truncated UE ID, i.e.,

SubgroupID = (floor(UE_ID/(N*Ns*K)) mod subgroupsNumForUEID_LP-WUS) + (subgroupsNumPerPO_LP-WUS - subgroupsNumForUEID_LP-WUS),

where:

- N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state;

- Ns: number of paging occasions for a PF;

- UE_ID: 5G-S-TMSI mod X_LP-WUS, X_LP-WUS = 8092 * maximum number of LP-WUS subgroups;

- subgroupsNumForUEID_LP-WUS: number of subgroups for LP-WUS UE_ID based subgrouping in a PO, which is broadcasted in system information

- K: configurable value which is equal to or larger than 8;

SI reception procedure considering LP-WUS

Proposal 11: LP-WUS should be able to wake up all UEs to monitor paging for SI update/ETWS/CMAS notification.

Observation 3: A large amount of radio resources are required to wake up all the UEs using codepoint way.

Proposal 12: Introduce a common LP-WUS to wake up all the UEs in RRC_IDLE/INACTIVE to monitor paging for SI update/ETWS/CMAS notification.

LP-WUS prioritization in cell reselection

Observation 4: It is feasible to support LP-WUS reception only on some of the bands supported by the UE. In specification, UE should be able to report supported band(s) for LP-WUS reception to the network.

Observation 5: Even UE supporting LP-WUS camps on a cell with LP-WUS reception enabled from NW perspective, it is possible that it still cannot use LP-WUS if the cell’s band does not match with UE capabilities, which does not provide any power saving gain.

Proposal 13: UE supporting LP-WUS prioritizes its supported frequency with LP-WUS enabled in the cell (re)selection procedure. FFS on prioritization of lower frequency if multiple frequencies support LP-WUS.

# References

RP-241824 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

RAN2#125-bis chairman notes;

RAN2#126 chairman notes;

RAN2#127 chairman notes;

RAN1#118 chairman notes;

R1-2407559 LS on LP-WUS operation in IDLE/INACTIVE mode;

# Appendix

## Calculation of paging false alarm rate

- In this appendix, we provide a brief description on how we obtain the results of paging false alarm rate.

- Case 1: LP-WUS only

- For UEs that are paged in a LP-WUS subgroup, the conditional probability that n UEs are paged can be expressed as:

P1 = Pn UEs are pagedSubgroup is paged = CNLn pn(1-p)NL-n1 -(1-p)NL ,

- where p is the per-UE paging probability and NL is number of UEs in the subgroup of LP-WUS. Among NL UEs, there are n UEs that are paged, the conditional probability that a target UE is included can be expressed as:

P2 = PTarget UE is pagedn of NL UEs are paged = CNL-1n-1 CNLn = nNL

- Combining the above 2 equations, we can obtain the probability that a target UE is paged in a certain subgroup as following:

Ptarget =  n=1NLP1×P2 =n=1NLCNLn pn(1-p)NL-n1 -(1-p)NL×nNL

- The paging false alarm rate of a certain UE after being indicated by LP-WUS to monitor PO can be expressed as:

- PFAR, Case 1=1-  Ptarget =1-n=1NLCNLn pn(1-p)NL-n1 -(1-p)NL×nNL

- Case 2: LP-WUS with PEI

- The difference between Case 1 and 2 is that for Case 2, once UEs are indicated by LP-WUS to wake up, they will be filtered by PEI with another independent subgrouping. The probability of one certain PEI subgroup being indicated for paging can be regarded as:

PG= P(One certian PEI Subgroup is paged) = 1 - 1 - pNP,

- where NP is the number of UEs in the subgroup number of PEI. Similar to Case1, the paging false alarm rate of a certain UE after being indicated by LP-WUS to monitor PEI and PO can be expressed as:

- PFAR, Case 2= PG×(1-  Ptarget) =( 1 - 1 - pNP)×1-n=1NLCNLn pn(1-p)NL-n1 -(1-p)NL×nNL

- It can be shown that compared to the formula of Case 1, UE monitoring LP-WUS with PEI can obtain a reduction of 1 - pNP off in FAR.