3GPP TSG-RAN WG2 #127bis	R2-2408448

Hefei, China, August 2024

Agenda Item:	8.2.2

Source:	OPPO

Title:	Discussion on AIoT functionalities

Document for:	Discussion, Agreement

Introduction

This paper will discuss some of the issues of stage-2 AIoT functionalities as follows:

- Segmentation;

- Remaining message size of energy status;

- Visibility of AIoT information;

- Temporary device ID;

- L2 repetitions.

Discussion

Issue-1: segmentation

In last RAN2 meeting, whether to support segmentation was discussed and an LS (R2-2407831) was approved to sent to RAN1/SA2 to check the supported TBS and the application payload size, so that RAN2 can further make decision on whether segmentation is needed or not.

We understand RAN2 will wait for the feedback from RAN1/SA2 before it can make the decision in this meeting, though it would be good that we can give our analysis and the initial view from RAN2 perspective:

- In the TR 38.848, in section 5.5, it’s captured that the expected message size to be transmitted/received from the AIoT device is approximately 1000 bits. We understand that this message size can be regarded as the typical upper layer packet size to be further handled in L2, which we can also wait for the response from SA2.

- The maximum TBS size to be supported may have impacts on the coverage, based on our initial assessment, with 1000 bits TBS size, the required coverage of AIoT device can still be supported. This is also under the evaluation by RAN1, thus we can still wait for the response from RAN1.

- L2 buffer is needed when segmentation is supported, because the segmented parts needs to be buffered before they get the resources to transmit. Thus it may potentially increase the complexity and cost of the AIoT devices.

With above, we propose:

Segmentation is not supported in L2, unless RAN2/SA2 indicates that segmentation is necessary.

Issue-2: message and energy status report

In last meeting, RAN2 agreed to study the need and use of energy status report from the device to reader in case the device does not have energy for the follow up messages, also to study the need of simple message to report the remaining data size.

Agreements

-	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

-	RAN2 will study the need and use of a simple message “size” reporting to the reader

In our view, the AIoT devices can communicate with the reader based on the energy harvested from external energy sources. Before it’s charged, the status of the AIoT devices are in a state of “shut down”, i.e., neither transmitting nor receiving to/from the reader. The energy charging state, the energy storage and also the available energy for the communication may have impacts on the procedure of the communication between reader and the devices. Thus, we may need to consider the mechanism of reporting the available energy of the devices, so that the reader can be aware of that for further controlling the devices.

There are several ways of reporting the energy status from the devices to the reader:

- The device can report whether it has the capability of energy storage. Some types of AIoT devices may have the capability of energy storage, e.g., active transmitting enabled devices. While some types of AIoT devices may not be able to store the energy, e.g., purely based on energy harvesting. Thus, it would be good to enable the reader to be aware of different types with respective of the energy storage.

- Furthermore, the device can also be enabled to report the available energy for the follow-up messages. The consideration is that the reader may need such information to decide whether to further schedule the devices or hold on the current transmission. We can further study on how the information is reported, e.g., in the form of remaining transmission time or just a simple indication whether the device can support the follow-up messages due to energy issue. The available energy reporting can be enabled in an on-demand way, e.g., requested by the reader, or together with other information, e.g., message reporting.

The AIoT devices can be enabled to report the capability of energy storage. Some forms of available energy can be further studied, e.g., the remaining time or simple indication to indicate whether the devices can support the follow-up messages.

With respective of the remaining message size reporting, we think it’s beneficial to enable the AIoT devices to indicate the remaining message size to the reader. It’s beneficial to the reader for the following up scheduling. However, it also depends on the message size to be transmitted, e.g., if the upper layer packet size is somehow a fixed message size. Then such indication may not be necessary as the reader can estimate the resources to be schedule to the AIoT devices, thus the complexity in the AIoT devices can be further reduced. Otherwise, some simple way of indication remaining message size can be supported.

It’s beneficial to support the remaining message size reporting, with the assumption that the upper layer packet size is non fixed.

Issue-3: visibility of AIoT information

For the visibility of AIoT information, some progresses were made in last RAN2 meeting. We agreed that at least the service type, e.g., inventory and command, should be visible to the reader. The detailed agreements are as follows:

Agreements

- At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

2	RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

There are several types of command, e.g., read/write/disable, we think the reader should be aware of the detailed types of command, so that it can be enabled with different behaviours. For example, if the command type is read, the reader should expect the follow-up message (i.e., the information to be read from the AIoT device) to be received from the device. Then if the command type is write/disable, the reader should send the message to the device instead of expecting receiving something from the device. Thus, we propose:

The reader should be visible of the command type, i.e., read/write/disable.

The information to be visible by the reader should be transferred via the interface between the core and the RAN, or the external AF and the RAN. Then, it would be good that RAN3 should be involved because the interface design is in their scope. Furthermore, we also need to inform SA2 about the information to be visible so that they can evaluate the feasibility.

Send LS to RAN3 and SA2 for the information to be visible by the reader.

Issue-4: temporary device ID

For the temporary ID to be used during the communication between device and reader, as we proposed in last meeting, we think the temporary ID is needed. Actually, similar mechanism is used in RFID system, i.e., RN16. During RACH procedure, the temporary ID can be obtained/reused, e.g., for the contention based RACH, the random ID transmitted in msg1 can be reused as the temporary device ID after the reader echoes the ID by sending msg2. While for contention free RACH, the reader can allocate a temporary device ID in the paging message, which is also feasible from our perspective.

Temporary device ID is needed during the communication between reader and AIoT devices. The temporary device ID can be obtained/reused during the RACH procedure.

Issue-5: L2 repetitions

In last meeting, some companies (R2-2406459, R2-2406818) proposed to support a L2 repetition mechanism, in order to handle the case when the message is not successfully received by the device/reader. While some other companies think such repletion mechanism can be handled by implementation (R2-2406751), e.g., for the R2D case, when the reader is aware that the message is not successfully received by the AIoT device, it can transmit the same message again by implementation.

In our view, we think we don’t need to support a mechanism of L2 repetition. For R2D case, the reader can handle the case of transmission when reader is aware of message is not successfully received by the AIoT device, thus it can trigger another transmission of the same message again by implementation. For the D2R case, the reader can also trigger the paging procedure when the expected information is not received by the reader during the inventory procedure or the command procedure.

No need to support L2 repetition mechanism, as it can be handled by reader implementation.

- Conclusion

We have the following proposals:

Proposal 1	Segmentation is not supported in L2, unless RAN2/SA2 indicates that segmentation is necessary.

Proposal 2	The AIoT devices can be enabled to report the capability of energy storage. Some forms of available energy can be further studied, e.g., the remaining time or simple indication to indicate whether the devices can support the follow-up messages.

Proposal 3	It’s beneficial to support the remaining message size reporting, with the assumption that the upper layer packet size is non fixed.

Proposal 4	The reader should be visible of the command type, i.e., read/write/disable.

Proposal 5	Send LS to RAN3 and SA2 for the information to be visible by the reader.

Proposal 6	Temporary device ID is needed during the communication between reader and AIoT devices. The temporary device ID can be obtained/reused during the RACH procedure.

Proposal 7	No need to support L2 repetition mechanism, as it can be handled by reader implementation.

Reference

- R1-24xxxx, xxxx