3GPP TSG-RAN WG2 Meeting #127bis	R2-2408453

HEFEI CHINA 14 Oct 2024-18 Oct 2024

Source: 	Vodafone

Title: 		Inventory procedure without device ID

Document for: Decision

Agenda Item: 8.2.2

## Introduction

During RAN WG1#127, the agreement was reached to support Proximity determination:

Conclusion

Proximity determination is concluded to be feasible with either of the two solutions below. Potential specification impact or not will not be determined in the Rel-19 study item.

Solution 1

For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission:

then the device is determined as near to the reader based on measurements at the reader side

Solution 2

For proximity determination, if reader successfully receives D2R transmission from the device in response to R2D transmission then the device is determined as near to the reader

At the same time Vodafone proposed in R2-2406582 to cover the scenarios where customers are inventorying groups of devices or all devices and would only like to know the number of the devices which have arrived or have left, are present or are missing under a particular location/locations.

## Discussion

As it can be seen from the decision made by RAN WG1 both proposed solutions enable the lean inventory procedure proposed in R2-2406582, as in each the reader determines that a particular device is “near” just from the reception of a D2R transmissions.

The only difference between the two solutions proposed by RANWG1 is whether the determination is also based on measurements on the D2R transmission or not, which in our assumption is just a means to make the proximity determination more precise.

Going back to our proposal on the lean inventory procedure, the following figure summarizes the communications flows required to realize it:

Figure 1: Lean inventory procedure

In order to minimise specification and device impact, it is proposed to re-use the current design assumptions as much as possible. In order to perform proximity determination and/or device counting procedure, the following steps are needed:

- trigger paging message to 1, multiple or group of devices

- the UE will answer with MSG 1 including random ID

- If no contention resolution is needed, MSG 2 does not need to be sent.

Based on the successful reception of the D2R transmission (MSG1) the device is deemed near according to the proximity procedure, with or without the need for measurements of this message. This would not mean that the proximity determination cannot be achieved through the successful reception of other types of D2R transmission. With this framework, one can also consider a given device to be “discovered” for inventory counting purposes as per the original use case envisioned for this procedure. As it can be seen from above, the impact to the standard is limited to:

- Addition of a “cause value” to the paging

- Skipping of MSG2 in case of no contention

- No D2R transmission after MSG2 (in case it is mandatory message even there is no contention resolution)MSG1 can be totally re-used as per today assumptions for 3step RACH

Observation 1: In order to support proximity determination and/or device counting, the impact to the specification is very limited.

Proposal 1: It is proposed to call the procedure for proximity and counting reasons a lean inventory procedure and it is proposed to support A-IOT lean inventory procedure within the A-IOT study item and include it into  TP 38.769.

Knowledge of device ID:

- Some may argue, that using the procedure described above, the system does not get a knowledge about particular devices as MSG1 does not contain device ID. This is true, if 3 step RACH is a baseline. On the other hand, if there is a wish to know the exact device ID, the device ID could be included into MSG 1  as  foreseen for 2 step RACH anyway.

Observation 2: The lean inventory procedure can be realised with or without transmitting device ID within MSG1

Need for MSG2:

In our understanding the main reason for MSG2 is to allocate resources for MSG3 and provide the UE Random ID as confirmation about successful contention resolution.

As there is no follow up communication from the device to the reader, the resources are not needed and if there is no contention resolution, MSG2 also does not need to be sent. The device should assume after timer x, that the MSG1 was received successfully and operation ends.

If MSG2 is received the UE should re-transmit MSG 1. The details of retransmission (same or different paging occasion, etc) can be decided separately in RAN2 but even it is agreed for other reasons to send MSG2 all the times, we can make it optional for the case paging indicates “Lean Inventory”. Also a separate cause value in paging would be useful from operational perspective.

Observation 3: With the cause value in initial paging set to “Lean Inventory”, the transmission of MSG2 upon reception of MSG1 can be optional.

Proposal 2: RAN2 to agree that MSG2 is optional if the paging is triggered with the cause value “Lean Inventory”.

## Conclusions

In this contribution we have presented the following observations and proposals:

Observation 1: In order to support proximity determination and/or device counting, the impact to the specification is very limited.

Observation 2: The lean inventory procedure can be realised with or without transmitting device ID within MSG1

Observation 3: With the cause value in initial paging set to “Lean Inventory”, the transmission of MSG2 upon reception of MSG1 can be optional.

Proposal 1: It is proposed to call the procedure for proximity and counting reasons a lean inventory procedure and it is proposed to support A-IOT lean Inventory procedure within the A-IOT study item and include it into  TP 38.769.

Proposal 2: RAN2 to agree that MSG2 is optional if the paging is triggered with the cause value “Lean Inventory”.