3GPP TSG-RAN WG2 Meeting #127bis	R2-2408454

HEFEI, CHINA 14.10.2024-18.10.2024

Source: 	Vodafone, Deutsche Telekom

Title: 		Barring and SIB1-less case 2

Document for: Decision

Agenda Item: 8.5.3

Introduction

During the last 3GPP TSG RAN Meeting #105 the updated scope for SIB1-less operation was approved:

The main idea of the case 2 is that the WUS configuration is delivered by the cell A, providing the UE all the needed information to request SIB1 once it reselects to SIB1-less cell. This approach foresees that the UE is selecting to the Cell A first and only after receiving the configuration is able to reselect and operate under SIB1-less cell. The aim of this paper is to discuss the barring operation for this scenario.

Discussion

Normal operation:

Once UE is switched on and during the PLMN selection it finds a suitable cell.

If  Cell A is selected first, it will receive over broadcasted or even dedicated way a WUS configuration to request SIB1 for the SIB1-less cell. This configuration will most likely include PCI and ARFCN of the SIB1-less cell, so that UE can understand during the reselection if it has a valid WUS configuration for the cell it is reselecting too. This seems to be feasible. The only clarification which might be needed for such scenario is to state that Cell A might also provide WUS configuration for itself. This is useful addition for the scenario where the gNB would like to become a SIB1-less cell after operating as Cell A.

Proposal 1: It is proposed to clarify that Cell A can also transmit the WUS configuration for itself in the case it intends to become a SIB1-less cell.

If the SIB1-less cell is selected first, as of today, the UE will bar such a cell for 5 minutes due to the lack of SIB1:

In our view, there are several ways of operation with one of them being

“Do nothing”:

The UE selects the SIB1-less cell, bars the cell and selects Cell A which provides a valid WUS configuration. With this configuration, the UE will try to reselect to the SIB1-less cell.

If reselection from Cell A to SIB1-less cell happens within 5 Minutes after SIB1-less cell was declared as barred, the SIB1 cell is still barred and reselection will not work. More likely the UE will not even try to reselect to such a cell, even it has a valid WUS configuration:

Observation 1: Barring behaviour as of today would lead to not availability of reselection to SIB1-less cell within 5 minutes and reduce the usefulness of case 2. On top of that it might lead to the additional battery consumption from the UE as the UE can only understand PCI once it is synchronized to the cell.

In order to solve the problem, we could see several ways of operation:

- The Rel 19 UE who implemented SIB1-less operation and try to camp on SIB1-less cell first, will understand that this is the SIB1-less cell (RAN1 is currently discussing the exact mechanism of how it could be signalled). In this case the UE could just not bar the cell, even if it does not transmit SIB1.

- The UE bars the cell as of today, but once reselecting to the Cell A and receiving the WUS, it would declare the SIB1-less cell as not barred anymore.

- The UE bars the cell, but the barring time is not 5 minutes, but less, e.g. 2 minutes.

The solution 1 has in our view some disadvantages, e.g. If the UE is not barring SIB1-less cell, it might be stuck in this one and try to acquire SIB1. Moreover, due to the additional information identifying that the cell is SIB1-less cell, it might select another cell (Cell B) which does not provide  WUS configuration and will try to reselect to the SIB1-less cell again, which will not work and consume additional UE battery. In addition, this solution would require changes in specification on the UE behaviour (barring of the cell lacking SIB1) which was specified and used since many years.

Solution 3 would probably work, but at the same time, it would be not easy to find a right value for barring time. Please note, that this is not a configurable timer, but the one specified in 38.304.

Observation 2: Solution 1 and Solution 3 have several disadvantages and could be de-prioritised during the discussion.

Overall, Solution 2 seems to be most straightforward behaviour to overcome this issue.

Proposal 2: It is proposed to standardise the barring behaviour for the Rel 19 UEs camping on SIB1-less cell for the first time and to decide if the solution 2 as above is the right way to go.

Conclusions

Proposal 1: It is proposed to clarify that Cell A can also transmit the WUS configuration for itself.

Observation 1: Barring behaviour as of today would lead to not availability of reselection to SIB1-less cell within 5 minutes and reduce the usefulness of case 2. On top of that it might lead to the additional battery consumption from the UE as the UE can only understand PCI once it synchronized to the cell.

Observation 2: Solution 1 and Solution 3 have several disadvantages and could be de-prioritised during the discussion.

Proposal 2: It is proposed to standardise the barring behaviour for the Rel 19 UEs camping on SIB1-less cell for the first time and to decide if the solution 2 as above is the right way to go.

References:

# RP-242354: Revised WID: Enhancements of network energy savings for NR