3GPP TSG RAN WG2 #127bis		R2-2408456

Hefei, China, October 14th – 18th, 2024

Title:	Discussion on LCM for UE-sided model for BM

Agenda item:	8.1.2.2

Source:	Google

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#127 meeting, there were some agreements and FFSs for UE-sided model for BM case. Some FFSs were captured in the LS [1] to RAN1 and RAN2 can wait for the related progress in RAN1.

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

In this paper, we further discuss the remaining FFSs and other issues not dependent on RAN1 and give our considerations.

# 2 Discussion

## Supported functionality information

According to the progress in the LS [1] of the [POST127] [019] email discussion, RAN2 has made the following signalling procedure on applicable functionality reporting for beam management:

Figure 1 Applicable functionality reporting

As shown in figure 1, the UE will report the supported functionalities by UECapablityInformation message in step 2. To enable the identification of the supported functionalities between UE and NW, it is an intuitive idea to introduce the functionality identification (ID) for each supported functionality.

Proposal 1: UE reports the functionality identification for supported functionalities in UECapablityInformation message.

In RAN1#111 meeting, there was the working assumption on the definition of functionality identification. It is preferred to wait for the progress on the functionality identification in RAN1 and avoid duplicated discussion of it in RAN2.

Working Assumption

| Terminology | Description |
| --- | --- |
| Functionality identification | A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE Note: Information regarding the AI/ML functionality may be shared during functionality identification. FFS: granularity of functionality |

Proposal 2: RAN2 waits for the progress on the definition of functionality identification in RAN1.

## Applicable functionality information

### Content of applicable functionality information

The reporting of applicable functionality information can assist the NW to identify the one(s) which can be chosen for activation. If the functionality ID is defined and introduced, it is straightforward to explicitly report the functionality ID of the applicable functionality, as previously reported in the supported functionalities.

As mentioned in the above section, the UE initially reports the supported functionalities to the NW. Subsequently, it reports the applicable functionality information. The alternative solution may be to report implicit functionality information for the applicable functionalities. For the implicit functionality information, it can be mapped to the supported functionality list and finally used to identify the actual functionality information. Technically speaking, for the implicit reporting scheme, there may be less signalling overhead. For example, a bitmap can be reported as the applicable functionality information. For each bit in the bitmap, it corresponds to a specific functionality in the supported functionality reported in step 2. With the above knowledge, the NW can still deduce the explicit functionality ID of the applicable functionality information.

Proposal 3: For the content of applicable functionality reporting, the following two options can be considered:

- Opt 1: explicit functionality ID of the applicable functionality

- Opt 2: implicit functionality information (e.g., bitmap) mapped to the supported functionalities

### Signalling for applicable functionality reporting

As response to NW-side additional condition requesting applicable functionality reporting in step 3, there is an FFS on applicable functionality reporting via UAI or RRCReconfigurationComplete, etc.

When the UE receives the request for applicable functionality reporting in RRC Reconfiguration message, the UE should respond an RRC Reconfiguration Complete message as a response. Upon the request, the UE starts the applicability checking of the related functionalities internally. If the UE can get the applicable functionality information when preparing the RRC Reconfiguration Complete message, it can promptly report it to the NW via the RRC Reconfiguration Complete message. This enables faster activation of the related functionality between UE and NW. Moreover, this may potentially avoid the need for a separate UAI message.

Additionally, if the applicable functionality information changes, UAI message should be used to report the new applicable functionality information. This offers flexibility and efficiency by reporting the applicable functionality in both UAI and RRCReconfigurationComplete.

Proposal 4: Both UAI and RRCReconfigurationComplete messages are used for the applicable functionality reporting.

Furthermore, to enable the flexible requesting for the applicable functionality reporting, the NW can configure the request in the RRC Resume and Reestablishment messages. Correspondingly, the UE can indicate the applicable functionality information in the RRC Resume/Reestablishment Complete messages.

Proposal 5: RRC Resume and Reestablishment procedures can be used for requesting and reporting of the applicable functionality information.

## Consistency of inference configuration between UE and NW

Based on the agreements related to figure 1, it is known that the NW may provide the inference configuration to UE, e.g., in step 3 and step 5 if need, or in step 5 only. The content of the inference configuration is still FFS as asked in Q6 in the LS [1] to RAN1.

From RAN2 perspective, the inference configuration guides the UE to derive necessary inputs for the inference operation of the activated functionality at UE side. Therefore, the inference configuration provided by the NW should be supported at least for the activated functionality at UE side. When the UE receives the proper inference configuration, it can activate the functionality and perform related inference operation.

Observation 1: The inference configuration provided by the NW is crucial for inference operation at UE side.

It should be also noted that if the NW provides the unsuitable inference configuration, the UE may consider the configuration is incompatible. Furthermore, the UE may declare failure between UE and NW. This will extremely downgrade the UE experience and system performance.

Observation 2: The unsuitable inference configuration may cause connection failure between UE and NW.

Therefore, it is desirable to ensure the consistency of inference configuration between UE and NW. The possible solution is that the UE proactively reports the required or supported inference configuration to the NW. One possible option is that the UE reports the required/supported inference configuration(s) in step 2, alongside the supported functionalities. This allows the NW to be aware of all possible inference configurations supported by the functionalities at UE side. Furthermore, the NW can take it into account for the inference configuration in subsequent step(s).

The alternative option is to report the required/supported inference configuration in step 4, along with the applicable functionality reporting. This provides the NW with precise requirements on the inference configuration. Correspondingly, the NW can accurately provide the inference configuration for the functionality to be activated.

Proposal 6: To ensure the consistency of inference configuration between UE and NW, the UE reports the supported inference configuration(s) to the NW, e.g., in step 2 or step 4.

## Model availability reporting

It was agreed in RAN2 for UE to determine the applicable functionality information based on the model availability in device.

-	UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

For the applicable functionalities, it refers to the functionalities that the UE has at least one model and is ready to apply for inference.

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

However, some functionalities may meet both NW-side additional conditions (if provided) and UE-side additional conditions but lack a locally available model. For these functionalities, they can be considered as candidate functionalities.

Observation 3: There are candidate functionalities at UE side, which satisfies both NW-side (if provided) and UE-side additional conditions but without available model.

In previous meetings, there were some discussions on the necessity to report the model availability information to the NW. Some companies argued that the applicable functionality reporting is sufficient for the future functionality-based LCM. From this perspective, there is no need to report model availability information. Nevertheless, for candidate functionalities, we believe that the reporting of the model availability information may be beneficial, e.g., for the mode transfer/delivery.

Observation 4: Model availability information can be used for model transfer/delivery operation.

For instance, if a candidate functionality has met both NW-side (if provided) and NW-side additional conditions but lacks a local model at UE side, the UE can inform NW of this unavailability information of the candidate functionality. If there is the valid model for the candidate functionality at NW side, the model transfer/delivery can be initiated to the UE. With these operations, the model(s) becomes available for the candidate functionality at UE side. Correspondingly, this converts the candidate functionality into an applicable one, ready for inference. This will optimize the functionality utilization.

Proposal 7: UE reports to candidate functionality information (those meeting NW- (if provided) and UE-side additional conditions but lacking a local model) to the NW.

Proposal 8: Introduce model availability information reporting for the candidate functionalities.

# 3 Conclusion

In this contribution, we discussed the potential solutions on the remaining FFSs and gave more considerations on the applicable functionality reporting schemes. The related observations and proposals are summarized below:

Observation 1: The inference configuration provided by the NW is crucial for inference operation at UE side.

Observation 2: The unsuitable inference configuration may cause connection failure between UE and NW.

Observation 3: There are candidate functionalities at UE side, which pass the checking of NW-side additional conditions (if provided) and UE-side additional conditions but without available model.

Observation 4: Model availability information can be used for model transfer/delivery operation.

Proposal 1: UE reports the functionality identification for supported functionalities in UECapablityInformation message.

Proposal 2: RAN2 waits for the progress on the definition of functionality identification in RAN1.

Proposal 3: For the content of applicable functionality reporting, the following two solutions can be considered:

- Opt 1: explicit functionality ID of the applicable functionality

- Opt 2: implicit functionality information, e.g., bitmap, which is mapped to the functionality ID of the supported functionalities

Proposal 4: Both UAI and RRCReconfigurationComplete messages are used for the applicable functionality reporting.

Proposal 5: RRC Resume and Reestablishment procedures can be used for requesting and reporting of the applicable functionality information.

Proposal 6: To ensure the consistency of inference configuration between UE and NW, the UE reports the supported inference configuration(s) to the NW, e.g., in step 2 or step 4.

Proposal 7: UE reports to candidate functionality information (those meeting NW- (if provided) and UE-side additional conditions but lacking a local model) to the NW.

Proposal 8: Introduce model availability information reporting for the candidate functionalities.

# 4 Reference

- [1] R2-2407848, LS on applicable functionality reporting for beam management UE-sided model