3GPP TSG-RAN WG2 Meeting#127bis                                	R2-2408459

Hefei, China, October 14th – 18th, 2024

Agenda item:	8.8.2

Source: 	Google

Title: 	DL coverage enhancement at system level

Document for:	Discussion and Decision

# Introduction

One key research area (i.e., the area for improvement) in the Rel-19 NR-NTN (i.e., NR_NTN_Ph3) is the downlink coverage enhancement, where the enhancement can be considered from both the link level and the system level perspectives, as described in the following Rel-19 NR-NTN WID [1].

DL coverage enhancements can be considered at both - Link level to improve the link margin of selected physical channels in order to accommodate the EIRP reduction in FR1-NTN. A link margin improvement for physical channels (e.g. PDSCH and PDCCH) may be considered without impact on SSB design. - System level to support an efficient dynamic and flexible power sharing between beams or different beam pattern/size (i.e., wide or narrow) across the satellite foot print for FR1-NTN and FR2-NTN.

The Rel-19 NR-NTN discussion was kicked off in RAN2 since RAN2#125bits meeting, and some progress was made in RAN2#126 and RAN2#127 meeting [2][3] with regard to the DL coverage enhancement.

Agreements in RAN2#126: - Based on the solution being investigated in RAN1, RAN2 will further discuss whether/how legacy UEs might operate in a cell supporting DL coverage enhancements. - RAN2 assumes that both EFC and EMC are supported Agreements in RAN2#127: - From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects) - From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms. - RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity. - If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

In this paper, we discuss the support of system-level DL coverage enhancement from the signaling perspective, including whether to extend the RRC parameters related to the SSB periodicity. Based on the discussion, several way forwards are proposed in this paper to facilitate the discussion in RAN2 while waiting for more inputs from RAN1.

# Discussion

## The necessity of extending the SSB periodicity

RAN1 has investigated the effect of increasing the SSB periodicity (as a consequence of introducing the beam hopping mechanism) on improving the coverage ratio, and observed that there might be a need to increase the SSB periodicity beyond the existing maximum value (160ms) to support the coverage ratio up to 96.8%. In addition, RAN1 also agreed to extend the default value (up to 640ms) regarding the SSB periodicity assumed by the UE during the initial access [4].

Agreement in RAN1#118: As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms}

Note that the above agreement is about the UE assumed value for SSB periodicity during the initial access, and therefore has nothing to do with the actual RRC signaling. In fact, in the reply LS R1-2407538 [5], RAN1 indicated the extension of the SSB periodicity is still under RAN1’s discussion.

Answer to Q1 (Reply LS R1-2407538): According to the updated WID (RP-241667), SSB enhancement other than SSB periodicity extension is not considered. Thereby, RAN1 understanding is that enhancements to the existing SSB patterns, e.g. SSB position in burst, SSB index number, are not within the scope. The extension of the SSB periodicity is still under RAN1’s discussion.

The necessity of increasing the SSB periodicity is due to the introduction of the beam hopping mechanism. For example, the satellite illustrated in Figure 1 provides a number beam footprints (grouped into four clusters) on the ground to increase the coverage and activates only one of the four beam footprint clusters at one time due to the power constraint. Although the satellite is transmitting the SSB burst every 20 ms, the SSB burst is transmitted in turn among the four footprint clusters. Therefore, for each beam footprint (e.g., beam footprint i), 75% of the SSB bursts are absent because the satellite is transmitting SSB bursts in other footprint clusters. From the beam footprint perspective, it is equivalent that the SSB burst is transmitted once every 80 ms (i.e., ssb-periodicityServingCell = 80 ms). The actual SSB periodicity may go beyond 160 ms (the maximum value in ssb-periodicityServingCell), if there are more than eight footprint clusters provided by the satellite in the example.

Figure 1. The increased SSB periodicity due to the beam hopping among the four beam clusters.

In view of this, several companies proposed in RAN2#127 to extend the maximum value for ssb-periodicityServingCell, and also for other SSB related parameters such as the SMTC, mgrp, and nAndPagingFrameOffset. However, even without being informed of the extended SSB periodicity due to beam hopping (i.e., even with the original SSB periodicity), the UE should be able to know the presence/absence of an SSB burst based on the DTX configuration of the cell/beam that the UE is camping on. Figure 2 below illustrates the same satellite using the same beam hopping sequence as that in Figure 1, but the network additionally signals the DTX pattern for each beam footprint. In Figure 2, although ssb-periodicityServingCell is still 20 ms, with the aid of the DTX configuration, the UE is able to only receive the SSB during the DTX ON period while ignoring the SSBs in the DTX OFF period. The DTX configuration is almost the same for different beam footprints, except for the DTX offset value that is different for different beam footprints in different beam footprint clusters.

Figure 2. The absence/presence of an SSB burst is known to the UE based on the DTX configuration.

The increased SSB periodicity due to the beam hopping can be known to the UE based on the original SSB periodicity (i.e., the SSB periodicity before beam hopping) plus the DTX configuration.

Therefore, with the aid of the DTX configuration, the existing values for ssb-periodicityServingCell and for other SSB related parameters/IEs can remain the same, as the UE is able to figure out the actual periodicity of the SSB burst by itself.

The existing values for ssb-periodicityServingCell, SSB-MTC, SSB-MTC4, and mgrp can remain the same if the DTX pattern is signaled to support the beam hopping mechanism.

In addition, combining the SSB periodicity with the DTX configuration has the merit of offering more flexible beam hopping options, as illustrated in Figure 3. In Figure 3, the beam hopping is conducted between two footprint clusters, and each footprint cluster is activated for a duration of 40 ms followed by a deactivated duration of 40 ms. Such a beam hopping option can be easily configured by the DTX configuration with the DTX ON duration equal to 40 ms and with the DTX cycle length equal to 80 ms. However, it is not possible to use the extended SSB periodicity values to configure such a beam hopping option as the SSB periodicity is changing over time (i.e. , the SSB periodicity is {20ms, 60ms, 20ms, 60ms, …}).

Figure 3. The absence/presence of an SSB burst is known to the UE based on the DTX configuration.

The DTX configuration offers more flexible beam hopping options, due to the configurable ON duration in the DTX configuration.

If the SSB periodicity is to be extended, we may also need to extend the number of SMTCs for the purpose of neighbor cell measurement. Consider the example in Figure 1, since the SSB occurrence is shifted by 20 ms per beam footprint cluster, we need n SMTCs if there are n footprint clusters. However, if the UE is aware of the DTX configurations for it neighbor cells, there is no need to extend the existing SMTC list as the UE can adjust the SMTC for a neighbor cell based on the DTX configuration of the neighbor cell.

Based on the above observations, we think as long as the DTX configuration (of the service cell, the neighbor cell, or both) is provided to the UE, there is no need to extend the existing values for ssb-periodicityServingCell as well as for other SSB related parameters/IEs. There is also no need to extend the existing SMTC list due to the same reason.

RAN2 neither extends the values for ssb-periodicityServingCell (and other SSB related parameters) nor extends the SMTC list, but introduces the DTX configuration instead to support the system level DL coverage enhancement.

## The provision of the DTX configuration

The introduction of the N1 beam footprints (i.e., the number of the beam footprints not providing service) will have impact to the UE (both the legacy and Rel-19 UE) in the idle or inactive state. This is because the camped cell may not transmit anything to the UE at the time when the UE wakes up at its paging occasions. One may argue that the network can configure a paging cycle length and the number of paging frames (that are evenly distributed within the paging cycle) in a way that the paging frame always aligns with the occurrence of the SSB. However, as the maximum interval between two adjacent paging frames is now 160ms (according to nAndPagingFrameOffset), aligning the paging frame with the occurrence of SSB through the paging configuration is not possible when the DTX cycle (i.e., the actual SSB periodicity) is more than 160 ms.

The beam hopping mechanism may have the impact to the UE in idle/inactive state, as when the UE wakes up at its paging occasions, it may not detect anything including the SSB burst.

The WID pointed out that the Rel-18 Network Energy Saving (NES) techniques should be considered as the baseline for signaling the beam hopping pattern. However, the cell DTX/DRX configuration in Rel-18 NES cannot be directly employed here to signal the beam hopping pattern, as the cell DTX/DRX configuration in Rel-18 NES is only applicable to the UE in the connected state, and the cell will still transmit common control signals even in the non-active period of a DTX cycle. As the beam hopping mechanism may impact the legacy UE as well as the Rel-19 UE in the idle/inactive state, the cell DTX/DRX configuration shall be also applicable to the UEs in the idle/inactive state, and shall cover the state where the satellite beam does not transmit anything. Therefore, we think the beam hopping pattern (i.e., the cell or beam DTX configurations) can be provided in the system information, so that at least the Rel-19 UE is able to determine when to wake up and monitor the paging message based on the beam hopping pattern and the paging configuration.

The DTX configuration can be provided in the system information for idle/inactive UEs, and covers the Tx state where the beam/cell is not transmitting anything.

Regarding whether the DTX configuration is provided to the UE on a per-beam or a per-cell basis, we think this is relevant to how the network/operator maps the satellite beams into an NR beam or an NR cell. According to R1-2400303 [6], there are five possible options for mapping the satellite beams into an NR beam or NR cell.

- Single NR cell per satellite beam and single NR beam cell

- Multi-Satellite beam NR cell and single NR beam cell: Cell splitting

- Multi-Satellite beam cell and multi NR beam cell

- Multi NR cells per satellite beam

- Multi NR cells split across satellite beams

If the DTX configuration is provided on a per-cell basis, it works for Option 1 and Option 4 above, but does not work for the other options. On the other hand, if the DTX configuration is provided on a per-beam basis, it works for all the options except for Option 2, and may result in some duplicated signaling in Option 4 (i.e., not optimized for Option 4). In fact, the cell DTX configuration can be treated as a special case of the beam DTX configuration, in which all the NR beams within the same cell share the same beam DTX configurations.

Providing beam DTX configuration can support most of the mapping options between the satellite beam and the NR beam/cell, and the cell DTX configuration can be considered as a special case of the beam DTX configure.

Therefore, we think the provision of the DTX configuration on a per-beam basis can be considered as the baseline, where each DTX configuration can be associated to an SSB index. It can be further discussed whether the DTX configuration can be associated to a CSI-RS resource ID.

The DTX configuration can be a beam-specific configuration, and can be associated to an SSB index.

To address the issue of having duplicated DTX configurations while providing the beam-specific DTX configuration in Option 4, one possible handling is to provide a DTX configuration but not associate it with any beam index. The UE would consider such a DTX configuration (not associated with any beam index) begin applicable to all the beams within the same cell.

If a DTX configuration is not associated with an SSB index, it is applicable for the entire cell.

## The impact on the RRC_CONNECTED UE

When the UE is in the connected state and the serving beam or the serving cell is under the OFF state (i.e., the state when the beam or cell is not transmitting anything), the UE is not required to monitor the PDCCH for saving energy. In order to further save the UE power and to prevent the UE from declaring the RLF or the beam failure while the beam/cell is not transmitting anything, we think the connected UE should refrain itself from performing the RLM and BFD, and may also refrain from performing the serving cell measurement during the beam/cell off periods.

The connected UE is not required to monitor the PDCCH, perform the RLM/BFD, and perform the serving cell measurement during the OFF periods configured by the DTX configuration.

As adopting P4 above may have some impact to the specification, an alternative approach to introduce minimum specification impact is to rely on the c-DRX configuration. Since the UE being configured with c-DRX would comply with the timing requirement (for measurement, RLM, and BFD) that is based on the c-DRX cycle length, the UE is able to avoid monitoring/measuring the RS during the beam/cell OFF period if every c-DRX on-duration would fall within or overlap with the DTX active duration. To ensure the c-DRX on-duration would align with the DTX active duration, a stage-2 description similar to the one for the Rel-18 NES might be needed. That is, the UE in the connected state is expected to receive a c-DRX configuration whose the cycle length is equal to one or multiple times of the cell/beam DTX cycle length.

The connected UE is expected to receive a c-DRX configuration whose the cycle length is equal to one or multiple times of the DTX cycle length.

# Conclusion

In this paper, we discussed the key issues regarding the extension of the SSB periodicity and the signaling of the DTX configuration, and have the following observations.

Observation 1	The increased SSB periodicity due to the beam hopping can be known to the UE based on the original SSB periodicity (i.e., the SSB periodicity before beam hopping) plus the DTX configuration.

Observation 2	The existing values for ssb-periodicityServingCell, SSB-MTC, SSB-MTC4, and mgrp can remain the same if the DTX pattern is signaled to support the beam hopping mechanism.

Observation 3	The DTX configuration offers more flexible beam hopping options, due to the configurable ON duration in the DTX configuration.

Observation 4	The beam hopping mechanism may have the impact to the UE in idle/inactive state, as when the UE wakes up at its paging occasions, it may not detect anything including the SSB burst.

Observation 5	Providing beam DTX configuration can support most of the mapping options between the satellite beam and the NR beam/cell, and the cell DTX configuration can be considered as a special case of the beam DTX configure.

Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

Proposal 1	RAN2 neither extends the values for ssb-periodicityServingCell (and other SSB related parameters) nor extends the SMTC list, but introduces the DTX configuration instead to support the system level DL coverage enhancement.

Proposal 2	The DTX configuration can be provided in the system information for idle/inactive UEs, and covers the Tx state where the beam/cell is not transmitting anything.

Proposal 3	The DTX configuration can be a beam-specific configuration, and can be associated to an SSB index.

Proposal 4	If a DTX configuration is not associated with an SSB index, it is applicable for the entire cell.

Proposal 5	The connected UE is not required to monitor the PDCCH, perform the RLM/BFD, and perform the serving cell measurement during the OFF periods configured by the DTX configuration.

Proposal 6	The connected UE is expected to receive a c-DRX configuration whose the cycle length is equal to one or multiple times of the DTX cycle length.

# Reference

RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

R2-2406202, Report of 3GPP TSG RAN WG2 meeting #126, ETSI MCC.

Draft report of 3GPP TSG RAN WG2 meeting #127, ETSI MCC.

Draft Report of 3GPP TSG RAN WG1 meeting #118, ETSI MCC.

R1-2407538, Draft Reply LS on DL coverage enhancements, China Mobile.

R1-2400303, Discussion on NR NTN Downlink coverage enhancements, Thales.