3GPP TSG-RAN WG2 Meeting#127bis                                	R2-2408461

Hefei, China, October 14th – 18th, 2024

Agenda item:	8.5.3

Source: 	Google

Title: 	Open issues for the on-demand SIB1 transmission

Document for:	Discussion and Decision

# Introduction

The second phase (i.e., Release-19) of the NES WI was kicked off in February 2024, and the revised Work Item Description (WID) RP-242354 [1] was updated after the overall progress was checked at the checkpoint in the RAN#105 meeting. The WID indicates that 3GPP will specify the procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, with the focus on the scenario where the UE obtains UL WUS configuration from Cell A, transmits UL WUS on NES Cell, and receives on-demand SIB1 from NES Cell (i.e., Case 2).

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

Based on the current progress in RAN2 and RAN1, this paper discuss further details on the procedure and possible enhancements from the signaling perspective, to support the Case 2 on-demand SIB1 procedure.

# Discussion

## The reception of the RAR

RAN2 has agreed in the RAN2#126 meeting [2] that the UE is expected to receive the RAR responding to the UL WUS, which is then followed by RAN1 in the RAN1#118 meeting [3] as follows.

Agreement in RAN2#126 RAR monitoring for MSG1 based OD-SIB1 REQ - RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

Agreement in RAN1#118 RAN1 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

As the RAR message is merely served as an acknowledgement to the UL WUS that the UE has transmitted earlier, the contents (i.e., the temporary C-RNTI, the uplink grant, and the TAC value) within the RAR message are not really useful and hence can be ignored by the UE.

The UE ignores the TAC value, temporary C-RNTI, and uplink grant in the RAR message acknowledging an UL WUS.

Alternatively, we can adopt a similar mechanism used in confirming the completeness of the MSG1-based other SI request procedure, where the UE considers the MSG1-based other SI request procedure as being completed successfully, when the UE receives an RAPID MAC sub-header matching the PRACH preamble transmitted by the UE earlier.

UE considers the RACH procedure of an UL WUS transmission is completed successfully, upon receiving an RAPID MAC sub-header matching the PRACH preamble of the UL WUS.

To monitor the RAR message after transmitting the UL-WUS, UE has to use a specific RNTI to decode the PDCCH, where the specific RNTI can be the RA-RNTI as used in the legacy RACH procedure, or a common RNTI that is common to all UEs no matter which PRACH resource the UE picked to transmit the UL-WUS. In our view, using the common RNTI can facilitate reducing the signal overheads as the network is able to use a single RAR to acknowledge multiple UL-WUS transmitted by different UEs at different time, and hence is more preferable.

RAN2 to discuss whether the UE shall use the RA-RNTI as in legacy or use a new common RNTI, to monitor the RAR responding to the preamble transmission for the on-demand SIB1 procedure.

## Monitoring window for SIB1 reception

RAN1 has agreed in the RAN1#117 meeting [4] that the UE shall monitor the type 0 PDCCH monitoring occasions for the on-demand SIB1 transmission after transmitting the UL WUS, and the PDCCH monitoring occasions will last for a time window. In RAN1#118 [3], RAN1 further discussed two options for indicating the starting time and duration of the PDCCH monitoring occasion for on-demand SIB1 reception, but no conclusion is reached.

Agreement in RAN1#117 At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, after UE transmits the UL WUS in idle/inactive mode, RAN1 assumes following as a starting point: - Option 1: One or more type 0 PDCCH monitoring occasions for on demand SIB1 within a time window Agreement in RAN1#118 At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options: - Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission - Option 2: starting time and duration are indicated in the UL WUS configuration

No matter which signal option is adopted eventually by RAN1, it only makes sense to start the SIB1 monitoring window after the UE has received the RAR message acknowledging the UL WUS that the UE has transmitted earlier. However, it might be beneficial (in terms of UE power saving) for UE to delay the start of the SIB1 monitoring window for a few slots/symbols after the last slot/symbol of the RAR message, and how much delay should be applied is up to RAN1 decision.

UE starts a SIB1 monitoring window after receiving the RAR message acknowledging the UL WUS transmission. The exact timing on when to start the SIB1 monitoring window is up to RAN1.

From RAN2 perspective, what is more important is the UE behavior after the expiry of the SIB1 monitoring window. In our view, if UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, the UE shall consider the cell as being barred and pick the next candidate cell to evaluate the cell reselection criteria again. This is to avoid UE stranding too long in a cell that does not intend to response to the UL WUS immediately, otherwise the long waiting time will prolong the cell reselection procedure, prevent the UE from reselecting to another cell, and eventually compromise the idle/inactive mode mobility performance. This behavior also aligns with the following RAN2 agreement made in RAN2#126 [2].

Agreement in RAN2#126 Barring for R19 NES UE - If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE considers the cell as being barred.

Moreover, we think the mobility state of a UE may have impact on how swiftly the UE shall make the cell reselection decision. Usually, if a UE has higher mobility, we would want the UE to make the cell reselection decision faster, otherwise the UE may soon leave the coverage of the original serving cell and lose it connectivity to the network. Therefore, we think the SIB1 monitoring window should cater to different UEs with different mobility states (as defined in TS 38.304), and the simplest way to make this happen is to let the UE scale the SIB1 monitoring window by a factor based on its mobility state.

UE can scale the SIB1 monitoring window based on the mobility state of the UE.

## Area scope of the UL WUS configuration

RAN1 has agreed in the RAN1#117 meeting [4] that a UL-WUS configuration can apply to either a single NES cell or multiple NES cells, and a unified configuration format is needed for supporting these two cases.

Agreement in RAN1#117 From RAN1 point of view, the following is feasible. It is up to RAN2 to decide whether/how to support it. At least for Case 2 (Option 1+B+X) design, a unified configuration format that can support both Option 2 (i.e. a UL-WUS configuration applies to multiple NES cells) and Option 3 (i.e. a UL-WUS configuration applies to a single NES cell).

Apart from the RAN1 agreement, we think it is beneficial from the network perspective to use a single UL WUS configuration across multiple cells, as it saves the configuration burdens for the network/operator and has a potential merit of reducing the signaling overheads. It is even more needed in the case when the UE is configured with the UL WUS via the RRC Release message, as the UE may move to another cell after entering the idle/inactive state.

The network can apply the same uplink WUS configuration to more than one cells.

RAN2 agreed that the PCI and frequency of a NES Cell can be used to associate a UL WUS configuration with the NES cell. That means for each UL WUS configuration there can be at least one PCI (and one frequency) being included in the UL WUS configuration.

Agreement in RAN2#126 Information of WUS configuration - Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

To extend to the cases where the same UL WUS configuration is applied to multiple cells, the simplest way is to include a PCI list containing more than one PCIs in the UL WUS configuration. However, if these cells happen to be within the same SI area, the same tracking area, or the same PLMN, there are more efficient ways to signal the UL WUS configuration. For instance, the UL WUS configuration may include an SI area ID, a TAI, or a PLMN ID within it instead of including a cell list. Alternatively, we may also index each UL WUS configuration with a configuration ID, and then associate this configuration ID to the cells listed in SIB3 or SIB4.

RAN2 to consider an efficient and unified method to signal the UL WUS configuration that can be applied to multiple cells. For instance, an UL WUS configuration can be associated with a list of PCIs, an SI area ID, a TAI, or a PLMN ID.

## Validity of the UL WUS configuration

Considering a scenario where a UE obtain an UL-WUS configuration for an NES cell from Cell A, and then the UE moves to Cell B that does not provide any UL-WUS configuration. Later, when the UE camping on Cell B decides to reselect the NES cell, should the UE use the UL-WUS configuration that the UE has obtained from Cell A to request the on-demand SIB1 transmission from the NES cell, or should the UE refrain from requesting the on-demand SIB1 transmission from the NES cell? We think the UL-WUS configuration is a sort of semi-static configuration that do not change quite often, and hence it is reasonable for the UE to keep the UL-WUS configuration for a period even if the UE has reselected another cell. On the other hand, to make the design simpler without introducing a new validity duration/timer/mechanism for the UL-WUS configuration, we can also force the UE to discard the UL-WUS configuration upon reselecting another cell.

RAN2 to discuss whether the UE should keep or release the UL-WUS configuration that the UE has acquired earlier from a cell, upon moving to another cell not providing the UL-WUS configuration.

## The provision of the UL-WUS configuration in the LTE cell

Since a Rel-19 UE camping on an LTE cell may measure and then reselect an NR cell based on the NR frequency information provided in SIB24, it is possible for such a UE to reselect an NR NES cell that does not transmit SIB1. Without the UL-WUS configuration being provided in the LTE cell, the UE reselecting the NR NES cell will have to bar the NR NES cell due to the absence of SIB1, and then have to reselect another cell that may consume quite considerable time and energy from the UE. To address the issue, one way is to refrain the LTE cell from listing the NR frequency of the NES cell in SIB24, but this would also prevent the UE from reselecting other non-NES cells operating in the same NR frequency. A more desirable approach is to provide the UL-WUS configuration also in the LTE cell, so that the Rel-19 UE can use this configuration to request the on-demand SIB1 transmission from the NR NES cell.

The UL-WUS configuration can be also provided in an E-UTRAN cell.

## Access control for legacy and Rel-19 UEs

In RAN2#127 meeting [5], three options were discussed for the network to bar the legacy UE from accessing the NES cell not transmitting SIB1, using the legacy mechanism. However, as the legacy UEs include the UEs capable of certain advanced features before Rel-19 (e.g., Rel-17 NTN, Rel-17 RedCap, etc.) and the barring mechanism for these UEs relies on additional barring indications resided in SIB1, using the cellBarred indication in MIB (Option 1) has limited effects in preventing the legacy UEs from accessing the on-demand SIB1 cell.

Agreement in RAN2#127 Access restriction for legacy UEs - Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded. - Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB. - Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB. - Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Using the legacy barring mechanism (i.e., cellBarred indication in MIB) has limited effects in preventing the legacy UEs from accessing the on-demand SIB1 cell.

In view of this, we think we can already rule out Option 1. As to the comparison between Option 2 and Option 3, since Option 2 is feasible to all cases including the cell selection and reselection cases while Option 3 only works for the cell reselection cases, we think Option 2 can be taken as the baseline for restricting the legacy UEs from accessing the NES cell.

As the baseline, legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

More importantly, the Rel-19 UE capable of the on-demand SIB1 procedure shall be allowed to access the NES cell (i.e., request the on-demand SIB1 transmission from the NES cell) as long as the UE has a valid UL WUS configuration for the NES cell, even if the cell is barred/excluded by the legacy signal (e.g., MIB, SIB3, SIB4).

Rel-19 UE is allowed to transmit UL WUS to an NES cell as long as the UE has a valid UL WUS configuration for the NES cell, even if the cell is barred/excluded by the legacy signal (e.g., MIB, SIB3, SIB4).

## The provision of the neighbouring cell information

Since the idle/inactive UE may need to check the PLMN ID and certain feature-specific barring indication (e.g., cellbarredNTN-r17, cellBarredRedCap1Rx-r17, etc.) in SIB1 to determine whether the UE can camp on the cell or not, the entire cell reselection process may take much longer time if the cell the UE attempts to reselect to is an NES cell not transmitting SIB1. The worst case is that the UE may find an NES cell not suitable for cell reselection after requesting and acquiring the on-demand SIB1 from the cell.

The UE may find an NES cell not suitable for cell reselection after requesting and acquiring the on-demand SIB1 from the cell.

Therefore, we think it is beneficial if the PLMN ID and the feature-specific barring indications of an NES cell can be provided to UEs in other mean (e.g., provided through the SIB4/5 in the neighbor cells). In addition, if the UL WUS configuration can be applicable across multiple cells within a specific region (e.g., TAI, SI Area), it is better if the tracking area code (TAC) as well as the SI area identity of a NES cell can be provided to the UEs in the same way as the PLMN ID, so that a UE can know whether the UL-WUS configuration is applicable in that NES cell.

If SIB1 is not transmitted in an NES cell, the cell access related information (e.g., PLMN IDs and feature-specific barring indications) of the NES cell can be provided in the neighboring cells.

## Impact on the cell reselection

It has been agreed in RAN2 that the UE should first perform the cell reselection procedure, and then request the on-demand SIB1 transmission from the cell UE picks, if the picked cell happens to be an NES cell not transmitting SIB1.

Agreement in RAN2#126 Triggering condition of WUS transmission - If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission. - After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

In the beginning of the cell reselection procedure, the UE perform the serving and neighboring cell measurement  by measuring the SSBs transmitted by the serving and neighboring cells. As the MIB payload within a SSB is able to indicate (through the ssb-SubcarrierOffset IE, pdcch-ConfigSIB1 IE, or the reserved bit) whether a cell is transmitting SIB1 or not, and RAN1 assumes the always-on SSB is transmitted on the NES cell with on-demand SIB1, the UE should be able to know whether a neighboring cell is transmitting SIB1 or not upon performing the measurement on that cell.

Agreement in RAN1#117 For further study of on-demand SIB1 in idle/inactive mode, it is assumed that always-on SSB is transmitted on the NES cell with on-demand SIB1.

UE is able to know whether a neighboring cell is transmitting SIB1 or not upon performing the measurement on the neighboring cell.

Given that the UE is able to know whether a neighboring cell is transmitting SIB1 or not while measuring the neighboring cell, reselecting to the neighboring cell having the highest ranking but not transmitting SIB1 may not be a best practice as it may introduce overhead and consume more power at the UE compared to reselecting to another cell with already-activated SIB1 transmissions. To achieve good balance among the mobility performance, the signaling overheads, and the power consumption, we may consider to introduce an NES version of the rangeToBestCell parameter, which allows UE to reselect to a non-NES cell among the non-NES cells whose R value is within rangeToBestCell of the R value of the highest ranked cell that happens to be an NES cell.

The network can configure whether the UE is allowed to reselect to a non-NES cell whose ranking is smaller than but within a ranking margin of the ranking of the best-ranked NES cell.

# Conclusion

In this paper, we discuss several open issues for enabling the on-demand SIB1 mechanism based on the current progress in RAN1 and RAN2. Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

The reception of the RAR:

Proposal 1	The UE ignores the TAC value, temporary C-RNTI, and uplink grant in the RAR message acknowledging an UL WUS.

Proposal 2	UE considers the RACH procedure of an UL WUS transmission is completed successfully, upon receiving an RAPID MAC sub-header matching the PRACH preamble of the UL WUS.

Proposal 3	RAN2 to discuss whether the UE shall use the RA-RNTI as in legacy or use a new common RNTI, to monitor the RAR responding to the preamble transmission for the on-demand SIB1 procedure.

Monitoring window for SIB1 reception:

Proposal 4	UE starts a SIB1 monitoring window after receiving the RAR message acknowledging the UL WUS transmission. The exact timing on when to start the SIB1 monitoring window is up to RAN1.

Proposal 5	If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE considers the cell as being barred.

Proposal 6	UE can scale the SIB1 monitoring window based on the mobility state of the UE.

Area scope of the UL WUS configuration:

Observation 1	The network can apply the same uplink WUS configuration to more than one cells.

Proposal 7	RAN2 to consider an efficient and unified method to signal the UL WUS configuration that can be applied to multiple cells. For instance, an UL WUS configuration can be associated with a list of PCIs, an SI area ID, a TAI, or a PLMN ID.

Validity of the UL WUS configuration:

Proposal 8	RAN2 to discuss whether the UE should keep or release the UL-WUS configuration that the UE has acquired earlier from a cell, upon moving to another cell not providing the UL-WUS configuration.

The provision of the UL-WUS configuration in the LTE cell:

Proposal 9	The UL-WUS configuration can be also provided in an E-UTRAN cell.

Access control for legacy and Rel-19 UEs:

Observation 2	Using the legacy barring mechanism (i.e., cellBarred indication in MIB) has limited effects in preventing the legacy UEs from accessing the on-demand SIB1 cell.

Proposal 10	As the baseline, legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

Proposal 11	Rel-19 UE is allowed to transmit UL WUS to an NES cell as long as the UE has a valid UL WUS configuration for the NES cell, even if the cell is barred/excluded by the legacy signal (e.g., MIB, SIB3, SIB4).

The provision of the neighbouring cell information:

Observation 3	The UE may find an NES cell not suitable for cell reselection after requesting and acquiring the on-demand SIB1 from the cell.

Proposal 12	If SIB1 is not transmitted in an NES cell, the cell access related information (e.g., PLMN IDs and feature-specific barring indications) of the NES cell can be provided in the neighboring cells.

Impact on the cell reselection:

Observation 4	UE is able to know whether a neighboring cell is transmitting SIB1 or not upon performing the measurement on the neighboring cell.

Proposal 13	The network can configure whether the UE is allowed to reselect to a non-NES cell whose ranking is smaller than but within a ranking margin of the ranking of the best-ranked NES cell.

# Reference

RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson.

R2-2406202, Report of 3GPP TSG RAN WG2 meeting #126, ETSI MCC.

Draft Report of 3GPP TSG RAN WG1 meeting #118, ETSI MCC.

Draft Report of 3GPP TSG RAN WG1 meeting #117, ETSI MCC.

Draft report of 3GPP TSG RAN WG2 meeting #127, ETSI MCC.