3GPP TSG-RAN2 #127-bis	   						        R2-2408462

# Hefei, China, 14th - 18th October 2024

Agenda Item:	7.24.2.2

Source:  LG Uplus, SK Telecom, Telecom Italia, China Unicom, NTT Docomo, ETRI

Title: 	Coexistence issue for NES and NCR

Document for:	Discussion

Introduction

In 3GPP Rel-18, the Network Energy Savings (NES) WI and the Network Controlled Repeaters (NCR) WI have been introduced. While both WIs aim to enhance network efficiency and performance, the simultaneous implementation of NES and NCR has revealed a critical issue. The problem lies in the periodical Discontinuous Transmission (DTX) and Discontinuous Reception (DRX) cycles of the gNB, which are inefficiently managed by NCR. This inefficiency results in improper NCR on/off cycles and unnecessary NCR mobilities, undermining the intended energy savings and degrading network performance. This contribution aims to explain these issues in detail and suggest possible solutions to ensure that NES and NCR can coexist effectively and achieve their goals in Rel-18 networks.

Discussion

Coexistence issue for NES and NCR

The current NCR are designed to operate in an implicit manner for gNB DRX/DTX as a NES operation, particularly in situations like off when Radio Link Failure (RLF) occurs. This means that their activation and deactivation are not explicitly signalled or controlled directly by the network but are instead determined by specific conditions or triggers, such as the quality of the radio link.

When a serving gNB goes into DRX/DTX mode, NCR would maintain connection with the gNB until RLF is declared by an elapsed time such as N311 subframes and T310. NCR would consume an unintended power during the time even though there is no radio transmission and reception with the serving gNB. In a special case the T310 longer than DRX/DTX off duration, NCR could not obtain any power saving.

As for a serving gNB coming back from DRX/DTX mode, NCR would continue waste energy by reestablishment since it does not know the exact on duration.

Observation 1: If a gNB is dynamically switched on or off in response to varying traffic patterns, the NCR cannot adjust its state rapidly. As a result, the NCR would face delays in responding to changes, potentially undermining the intended energy saving of the network.

As explained, slow NCR’s switching on/off caused by the uncertainty of implicit mechanism will suffer from wasting energy. It underscores the need for a more concrete mechanism to explicitly manage the NCR attachment to gNBs, where the action of NCR access link according to DTX/DRX operation of the serving gNB should be specified.

Proposal 1: It is proposed to specify the operation of NCR access link according to DTX/DRX of the serving gNB.

Based on the current specification, if NCR lost the connection due to DTX of a serving gNB, it would declare RLF and initiate reestablishment to other gNB in on state. However, the situation could be unintended deployment and affect interference increase of neighbour cells. When the original gNB coming back from DRX/DTX mode, the system performance degradation could grow severer. Or, NCR could be handover to the original gNB but, at the next DTX/DRX period, it would also continue to temporarily change serving gNB. It is quite redundant procedures in energy saving and signalling perspectives.

Observation 2: When a gNB is turned off, a NCR might connect to an unintended temporary source gNB. This unintended attachment would lead to unstable mobility within the network, as the NCR may continuously switch between different gNBs based on their dynamic on/off states.

This instability is problematic because it can degrade the quality of service experienced by users. The frequent and unintended switching caused by the lack of robust mobility mechanism in managing the NCR’s connections to gNBs can lead to increased latency, dropped connections, and an overall reduction in network reliability. A NCR device should maintain the connection with the serving gNB while the serving gNB is on the DTX/DRX mode.

Proposal 2: It is proposed to specify the mobility mechanism of a NCR MT, where the NCR MT maintain the connection with the serving gNB while the serving gNB is on the DTX/DRX mode.

Conclusion

In this contribution, the below observations on simultaneous implementation of NES and NCR are shown;

Observation 1: If a gNB is dynamically switched on or off in response to varying traffic patterns, the NCR cannot adjust its state rapidly. As a result, the NCR would face delays in responding to changes, potentially undermining the intended energy saving of the network.

Observation 2: When a gNB is turned off, a NCR might connect to an unintended temporary source gNB. This unintended attachment would lead to unstable mobility within the network, as the NCR may continuously switch between different gNBs based on their dynamic on/off states.

Based on the above observations, we believe spec modification on NCR is required. We kindly suggest as following;

Proposal 1: It is proposed to specify the operation of NCR access link according to DTX/DRX of the serving gNB.

Proposal 2: It is proposed to specify the mobility mechanism of a NCR MT, where the NCR MT maintain the connection with the serving gNB while the serving gNB is on the DTX/DRX mode.

References

- TS38.300

- TS38.321

- TS38.331