3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408473

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	7.24.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on intra-band EN-DC channel spacing

Document for:	Discussion and Decision

1. Introduction

RAN2 received a LS [1] from RAN4 on intra-band EN-DC. In this contribution, we discuss about the intra-band EN-DC channel spacing issue according to RAN4 LS.

2. Discussion

According to current TS 38.306, the contiguity capability for intra-band EN-DC can be indicated through intraBandENDC-Support. The UE can indicate either contiguous (i.e. through absence of the field), non-contiguous, or both contiguous and non-contiguous capability for one (NG)EN-DC combination. In 38.331, there are separate bits defined for “non-contiguous” and “both”. In other words, the UE is able to declare support of either one of these two capabilities separately.

MRDC-Parameters ::= SEQUENCE {

singleUL-Transmission               ENUMERATED {supported}              OPTIONAL,

dynamicPowerSharingENDC             ENUMERATED {supported}              OPTIONAL,

tdm-Pattern                         ENUMERATED {supported}              OPTIONAL,

ul-SharingEUTRA-NR                  ENUMERATED {tdm, fdm, both}         OPTIONAL,

ul-SwitchingTimeEUTRA-NR            ENUMERATED {type1, type2}           OPTIONAL,

simultaneousRxTxInterBandENDC       ENUMERATED {supported}              OPTIONAL,

asyncIntraBandENDC                  ENUMERATED {supported}              OPTIONAL,

...,

[[

dualPA-Architecture                 ENUMERATED {supported}              OPTIONAL,

intraBandENDC-Support               ENUMERATED {non-contiguous, both}   OPTIONAL,

ul-TimingAlignmentEUTRA-NR          ENUMERATED {required}               OPTIONAL

]]

}

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| intraBandENDC-Support Indicates whether the UE supports intra-band (NG)EN-DC with only non-contiguous spectrum, or with both contiguous and non-contiguous spectrum for the (NG)EN-DC combination as specified in TS 38.101-3 [4]. If the UE does not include this field for an intra-band (NG)EN-DC combination the UE only supports the contiguous spectrum for the intra-band (NG)EN-DC combination. If intrabandENDC-Support-UL is absent and the band combination supports intra-band (NG)EN-DC only in DL, this field indicates the DL capability. If intrabandENDC-Support-UL is absent and the band combination supports intra-band (NG)EN-DC in DL and UL, this field indicates the common capability for both DL and UL. If intrabandENDC-Support-UL is included, intraBandENDC-Support indicates the DL capability. | BC | No | N/A | N/A |

According to RAN4 LS, from RAN4 perspective, the UE supporting ‘non-contiguous’ should not preclude the support of ‘contiguous’ in the same band, since support of ‘non-contiguous’ is kind of more stringent implementation. However, such restriction is not reflected in current specs. The UE already in the market is not expected to follow such requirement in any conformance test. It is impossible to do any NBC change now in RAN2.

The issue came out when the usable spectrum is only contiguous spectrum in some regions. Operators are motivated to deploy the contiguous EN-DC combinations for better system performance. In such combinations, the UEs reporting “non-contiguous” in intrabandENDC-Support may not be able to access to corresponding cells.

To address the issue, RAN4 agreed to introduce a new capability to indicate support of intra-band non-contiguous spectrum with nominal channel spacing. In last RAN4 meeting, the applicable requirement for the new capability was introduced in [2]. Based on RAN4 spec, there are now three types of UE capability:

1. UE indicating "Non-contiguous" (existing): Can meet the requirement for non-contiguous EN-DC when it is configured with EN-DC combination with channel spacing larger than the nominal value.

2. UE indicating "Contiguous" (existing): Can meet the requirement for contiguous EN-DC when it is configured with EN-DC combination with channel spacing less than or equal to nominal value.

3. New UE type: Can meet the requirement for non-contiguous EN-DC when it is configured with EN-DC combination with channel spacing equal to or larger than nominal value.

It can be seen from the above that the new UE capability is different from existing capability. For existing UE types (i.e. indicating “contiguous” or “both” in existing capability field), with the channel spacing equal to nominal value, the requirement for contiguous EN-DC should be applied; however, for new UE type, in this case, the requirement for non-contiguous EN-DC should be applied. In this way, it brings less testing complexity for legacy “non-contiguous” UE to support the new capability.

Proposal 1: Introduce new capability to indicate support of intra-band non-contiguous (NG)EN-DC with nominal channel spacing.

Besides, in current spec, there is another capability intrabandENDC-SupportUL defined when there is different contiguity capability between DL and UL. This capability is only applicable for special EN-DC combinations in RAN4 spec, e.g. DL DC_(n)48CA, UL DC_48A_n48A. If the UE supports same contiguity between DL and UL, the legacy field intrabandENDC-Support will be applied. Moreover, for the inter-band EN-DC combination with multiple intra-band components, it was also agreed to introduce a list to indicate the contiguity capability for each component.

There are three options to handle the cases where different contiguity is supported between DL/UL or in multiple intra-band EN-DC components in a single BC:

- Opt1: Preclude the applicability of the new capability for the BCs with different contiguity between DL/UL or in multiple intra-band ENDC components;

- Opt2: Introduce the new capability in a finer granularity, i.e. per DL/UL direction per intra-band ENDC component;

- Opt3: Keep the new capability in per-BC level, and clarify the new capability only applies to the DL/UL direction of the intra-band ENDC component with “non-contiguous” indicated in the existing capability.

To have a more future-proof mechanism with less signaling overhead, we suggest to go with option3. For example, when “non-contiguous” is supported in both DL and UL (i.e. intrabandENDC-Support is set to “non-contiguous”) and the new capability is signalled, the new capability applies for both DL and UL. When “contiguous” is supported in DL and “non-contiguous” is supported in UL (i.e. intrabandENDC-Support is set to “contiguous” and intrabandENDC-SupportUL is set to “non-contiguous”), and the new capability is signalled, the new capability only applies for UL. In other words, for nominal channel spacing, the “non-contiguous” requirement should be applied for UL, and “contiguous” requirement should be applied for DL.

In conclusion, the new capability should be introduced in per-BC level, which is applicable only for the DL and/or UL intra-band EN-DC component(s) with “non-contiguous” indicated in the existing capability in the BC. The UE indicating support of the new capability shall indicate support of “non-contiguous” in at least one among intrabandENDC-Support/intrabandENDC-SupportUL/ intrabandENDC-Support-DL-v1790/ intrabandENDC-Support-UL-v1790.

The corresponding CRs are provided in [3][4].

Proposal 2: The new capability is in per-BC level, which is only applicable for the DL and/or UL intra-band EN-DC component(s) with “non-contiguous” indicated in the existing capability in the BC. The UE indicating support of the new capability shall indicate support of “non-contiguous” in at least one among intrabandENDC-Support/intrabandENDC-SupportUL/intrabandENDC-Support-DL-v1790/ intrabandENDC-Support-UL-v1790.

Proposal 3: Agree the CRs in [3][4].

3. Conclusion

In this contribution, we have following proposals:

Proposal 1: Introduce new capability to indicate support of intra-band non-contiguous (NG)EN-DC with nominal channel spacing.

Proposal 2: The new capability is in per-BC level, which is only applicable for the DL and/or UL intra-band EN-DC component(s) with “non-contiguous” indicated in the existing capability in the BC. The UE indicating support of the new capability shall indicate support of “non-contiguous” in at least one among intrabandENDC-Support/intrabandENDC-SupportUL/intrabandENDC-Support-DL-v1790/ intrabandENDC-Support-UL-v1790.

Proposal 3: Agree the CRs in [3][4].

4. Reference

[1] R4-2410693, LS on intra-band EN-DC channel spacing.

[2] R4-2414423, R18 Cat-F CR 38.101-3 channel spacing for intra-band EN-DC[Intra-Band_EN-DC_Channelspacing]

[3] R2-2408474, Introduction of new capability for intra-band EN-DC channel spacing (38.331)

[4] R2-2408475, Introduction of new capability for intra-band EN-DC channel spacing (38.306)