3GPP TSG RAN WG2 #127bis	   												                        R2-2408482

Hefei, China, October 14th – 18th, 2024

Agenda item:	8.2.2

Source:	ETRI

Title:	Discussion on A-IoT functionality

Document for:	Discussion and Decision

# Introduction

This contribution discusses the functionality of A-IoT, with a particular focus on message size/status indication and energy status report.

# Discussions

## Message size/status indication

During the last RAN2 meeting, the following agreement was made regarding message size/status indication[1].

- RAN2 will study the need and use of a simple message “size” reporting to the reader

The traffic characteristics for each use case considered in A-IoT are as follows.

- Inventory use case: The reader identifies individual devices from the selected group using a slotted ALOHA with query and acknowledgement. In inventory use case, the major role is to transmit device ID from device to reader. The EPC (Electronic Product Code) ID is commonly used as a device ID in RFID specification[3], and its size varies from 96 to 496 bits in length. However, the 96-bit EPC is the default length and is commonly used in most applications.

- Command use case: Once a device is identified, the reader can choose to perform various operations, ranging from reading or writing to a specified memory bank using a read or write command, to even permanently disabling the device. The device’s response to the read command is expected to involve variable-length data transmission. In contrast, commands like write or disable are responses to specific requests, so the transmission is expected to be short and fixed length, such as an ACK/NACK.

Proposal 1: For Rel-19 A-IoT use cases, both variable and fixed message sizes may be considered for D2R transmission depending on the use cases.

The reader needs to know the data size in D2R transmission to schedule radio resources effectively. In A-IoT, the methods for determining data size for scheduling can be categorized as follows.

- Service-based mechanism: The reader allocates radio resources based on the expected data size according to the service type. In A-IoT, certain data sizes can be predetermined. For example, consider an A-IoT system performing an inventory service using a 3-step CBRA with a 96-bit EPC. In this case, the reader can determine that the size of msg3 is 96 bits if it contains only the device ID. To achieve this, the reader receives the service type and data size for each service from the CN and allocates resources accordingly. This mechanism can be used to indicate fixed-size messages.

- BSR-like mechanism: The device requests resource allocation based on the data size required by the higher layer. To achieve this, the device sends the data size as control information, such as MAC CE, to the reader, and the reader allocates radio resources. This mechanism can be used to indicate messages of any size.

In the service-based mechanism, the message size can be pre-configured based on each service, and radio resources can be allocated accordingly. In this case, scheduling can be done either by using an index that indicates the predefined message size or without an exchange the actual message size. However, this requires exchanging information such as the service type and message size between the CN and AS. In contrast, a BSR-like mechanism exchanges the requested data size, which may result in transmission delays, increased energy consumption, and added device complexity due to the signaling procedures. Based on the above discussion, we believe it is necessary to consider both types for message size/status indication.

Proposal 2: RAN2 to discuss the service- based and BSR-like mechanisms for the message size/status indication.

## Energy status report

During the last RAN2 meeting, we reached the following agreement regarding energy status report[1].

- RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages. FFS details on signaling

The following are the benefits of introducing the energy status report.

- Halting unnecessary actions: If a device transmits its energy status to the reader, the reader can halt unnecessary procedures. For example, the reader will stop transmitting messages to the device upon receiving an energy status report indicating that the device has little to no remaining energy. This allows the reader to avoid unnecessary actions directed at the device.

- Handling of scheduling priority: Based on the energy status of a device, the reader can prioritize the scheduling order among devices. For example, if a device has low energy capacity but its service can be completed within the remaining energy, that service is given higher scheduling priority than others.

Considering energy status report, following issues should be taken into account.

The first issue is device complexity. For the energy status report, both the device and the reader should be capable of storing information such as operational status, which may increase complexity. Additionally, the complexity of measuring available energy could also pose a challenge for devices using this mechanism.

The second issue is resuming a procedure on a discharged device once it has recharged. While the reader can detect when a device powers off during operation, it cannot predict when the device will be recharged, making it difficult to determine the right time to use this information. Therefore, it is necessary to design a resumption procedure that signals when the device has been recharged.

Proposal 3: When designing the energy status report, device complexity and the actions required to resume procedures should be taken into account.

The energy status report can be transmitted from the device to the reader as part of the control information. The AS protocol in A-IoT consists of both MAC and PHY layers [2]. Therefore, if the energy status report is introduced, one of these two layers could be considered for this function. This is similar to the PHR function in 5G NR, where PHR is managed by the MAC layer, making it reasonable for the MAC layer to handle this function in A-IoT as well.

Proposal 4: An energy status report can be transmitted through a control element in the MAC layer.

The content of the energy status report includes the available energy on the device. To indicate this, either the actual remaining energy or the percentage of charged energy relative to the device's total energy capacity can be used. Since the energy status report is used for scheduling and device status indication, both options are applicable.

Proposal 5: The energy status report message includes information about the remaining energy available to the device. This can be represented either as the actual amount left or as a percentage of the total capacity.

The energy status can be reported either on an event basis or in a periodic manner. In event-based reporting, the energy status is transmitted when a specific event occurs, whereas in periodic reporting, it is sent at predetermined time intervals. Both mechanisms require configurations from the reader. Event-based reporting involves configuring a specific event to trigger the report, while periodic reporting requires setting the time interval for transmission. This configuration can be included in the paging message.

Proposal 6: The configuration for energy status reporting can be transmitted to the device via a paging message from the reader.

# Conclusions

Based on the discussion in the contribution, we have the following proposals:

Proposal 1: For Rel-19 A-IoT use cases, both variable and fixed message sizes may be considered for D2R transmission depending on the use cases.

Proposal 2: RAN2 to discuss the service- based and BSR-like mechanisms for the message size/status indication.

Proposal 3: When designing the energy status report, device complexity and the actions required to resume procedures should be taken into account.

Proposal 4: An energy status report can be transmitted through a control element in the MAC layer.

Proposal 5: The energy status report message includes information about the remaining energy available to the device. This can be represented either as the actual amount left or as a percentage of the total capacity.

Proposal 6: The configuration for energy status reporting can be transmitted to the device via a paging message from the reader.

# References

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #127”, August 2024.

- 3GPP TR 38.769, Study on solutions for Ambient IoT (Internet of Things) in NR, Sep., 2024.

- EPC Radio-Frequency Identity Protocols Generation-2 UHF RFID, 2013.