3GPP TSG-RAN2#127bis	R2-2408489

Hefei, China, October 14-18, 2024

Agenda item:		8.4.2

Source:		Lenovo

Title:		 Procedure and Configuration of LP-WUS in RRC Idle/ Inactive

Document for:		Discussion and Decision

- Introduction

In RAN#103 meeting, WID of Rel-19 Low-power wake-up signal and receiver for NR (LP-WUS/ WUR) [1] was discussed and confirmed. Currently, the UE’s Main Radio (MR) needs to periodically monitor PDCCH once per DRX cycle, which dominates the power consumption in periods with no signaling or data traffic. If the UE’s MR can wake up only when they are triggered (e.g. paging), power consumption could be dramatically reduced. This can be achieved by using a wake-up signal to trigger the main radio and a separate receiver which has the ability to monitor wake-up signal with ultra-low power consumption. Main radio works for data transmission and reception, which can be turned off or set to deep sleep unless it needs to monitor PDCCH [2].

Furthermore, the following agreements were made during RAN2 #126 meeting:

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions. The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE: - - LP-SS configuration - - LP-WUS configuration - - Entry/exit condition for LP-WUS monitoring (FFS if it always configured) Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. From RAN2 perspective, no new procedure is introduced for SI reception/updates.

This contribution looks at the possible RRC Idle/ Inactive procedures for an LP-WUR enabled UE including entry and exit conditions for LR activation, subgrouping and possible paging enhancements.

- Discussion

  - Entry and Exit Conditions

It has been agreed in previous meetings that the baseline entry/exit conditions for LR to monitor for LP-WUS will be based on the serving cell measurements and/or serving cell quality (e.g., RSRP or RSRQ) from MR. Whether these conditions are always configured is still FFS. As per the WID objectives, it has been captured that the target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for msg3. Some companies have pointed out that this means it is expected for LR coverage to match that of MR and that an entry/exit condition need not be always configured. In our view, although the LR coverage shall match that of the MR’s PUSCH coverage for msg3, this is still lesser than the MR’s DL coverage, i.e., the LR shall have a smaller coverage with respect to the MR’s DL coverage. Furthermore, since the LP-WUR architecture may vary from one UE to another (OOK-based or OFDM-based LR), it is necessary to always provide an entry/ exit condition in the SIB for a cell supporting LP-WUS.

The entry/exit condition for LP-WUS monitoring is always configured in SIB when the cell supports LP-WUS.

RAN2 is yet to discuss if any other entry/exit conditions may be configured on top of the RSRP thresholds. For example, the UE’s mobility state, paging probabilities etc. may be additionally configured to limit UEs from transitioning too frequently from MR to LR and vice-versa.

RAN2 to discuss the need for any additional entry/exit conditions such as the UE’s mobility state, UE’s paging probabilities etc.

  - LP-WUS Subgrouping

When the UE is in RRC_IDLE/ RRC_INACTIVE Mode, the network may page the UE to notify it about incoming data, call requests and/ or network updates. When the MR is in ultra-deep sleep mode, it needs to be woken up before the UE can be paged. The LP-WUS can be used to wake-up the MR for the purpose of PDCCH monitoring and paging. The following was previously agreed in RAN1 #116:

For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

On the basis of the above agreement, a cell may now consist of four UE types by combining the support of LP-WUS and PEI monitoring: legacy UEs that are only capable of PO monitoring, UEs that are capable of only PEI monitoring, UEs that are capable of both LP-WUS and PEI monitoring but only monitors for LP-WUS, and UEs capable of both LP-WUS and PEI monitoring and also monitors for both. Since it is understood that LP-WUS will support subgrouping and it will reuse the PEI subgrouping methods as a baseline, LP-WUS subgrouping must be designed in a way that it does not negatively impact the false alarm rate and thus the power saving gains. That is, when a UE is capable of monitoring both LP-WUS and PEI (provided the cell is configured with both), the LP-WUS subgrouping should be designed such that, when used in tandem with PEI, it can reduce the false alarm rate lower than legacy PEI methods. To enable such a 2-level subgrouping without increasing the payload size of the LP-WUS signal, we think that the maximum number of subgroups supported by LP-WUS should be 8, as is in legacy PEI. Furthermore, if the LP-WUS also supports up to 8 subgroups at maximum (as in PEI), LP-WUS subgrouping may be used to replace legacy PEI functionality for the case when PEI is not supported or the UE is not configured with PEI, making LP-WUS subgrouping more flexible.

Observation 1  LP-WUS Subgrouping should be designed such that it does not negatively impact false alarm rate and thus the power saving gain.

The maximum number of subgroups that can be configured for LP-WUS subgrouping is 8.

As the discussion is already ongoing in RAN1 as to how the payload will be designed, we think it is sufficient for RAN2 to discuss the maximum number of subgroups supported for LP-WUS and allow RAN1 to continue the discussion on how this subgrouping information may be carried in the LP-WUS signal. Furthermore, we think RAN2 should focus the discussion on how this LP-WUS subgrouping may be configured in the cell, e.g., by including some additional Subgrouping Configuration in the SIB signalling and what this configuration may include, although RAN2 may need to wait further on RAN1 inputs before finalizing the exact details.

RAN2 to discuss the inclusion of a LP-WUS Subgrouping Configuration in the SIB signalling.

Another aspect that RAN2 should discuss is how this 2-level subgrouping needs to be configured such that the false alarm rates are not increased. One possible solution is to ensure orthogonality between the LP-WUS subgrouping and PEI subgrouping. For example, by ensuring that the UEs within a PEI subgroup are divided into different LP-WUS subgroups (i.e., a 1:1 mapping is not used between PEI subgroup and LP-WUS subgroup), the number of UEs that wake-up to monitor for PEI (if PEI monitoring is configured and implemented) will be reduced achieving reduced false alarm rates while PO monitoring. How to achieve orthogonality may be left for FFS. Figures 1, 2 and 3 show how the false alarm rates can be reduced by configuring orthogonal subgrouping between LP-WUS and PEI. Here, the total number of UEs in the cell is 80 (assuming all UEs are monitoring for both LP-WUS and PEI), the number of PEI subgroups configured is 8, and the number of LP-WUS subgroups configured are 5, 4 and 3 respectively.

Figure 1: False Alarm Reduction with 8 PEI Subgroups and 5 LP-WUS Subgroups

Figure 2: False Alarm Reduction with 8 PEI Subgroups and 4 LP-WUS Subgroups

Figure 3: False Alarm Reduction with 8 PEI Subgroups and 3 LP-WUS Subgroups

RAN2 to discuss the configuration of 2-level subgrouping to reduce false alarm rates in comparison to legacy PEI.

  - Paging monitoring triggered by LP-WUS

As agreed in RAN1#116 meeting [3]: for the cases where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not. And in the RAN1#117 meeting [4], the association between LO and PO are also discussed, 3 potential options are given from RAN1’s conclusions. It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup:

- Option 1: UEs monitoring the same PO monitor the same LO.

- Option 2: UEs corresponding to different POs monitor the same LO.

- Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.

Based on RAN1’s conclusions, network may indicate UEs in each sub-group to monitor specific PO(s) (or PEI, if configured) after being woken up by LP-WUS. However, in some exceptional cases, UE may fail to receive the paging message after receiving the LP-WUS indicating wake-up. For example, there was no paging transmitted during the target PO(s), or UE misses the target PO/PEI(s) due to e.g., potential transition time. Or, UE has been falsely alarmed by the LP-WUS targeted to other UEs.  RAN2 is suggested to discuss the UE behaviors for the cases that UE fails to receive paging after being woken up by LP-WUS. From our understanding, if UE keeps monitoring paging, then the energy consumption can be aggravated. UE may enter LP-WUS monitoring mode in case no paging is received in target PO(s) as configured after being woken up by LP-WUS. And whether the LP-WUS entry conditions need to be re-evaluated before entering the LP-WUS monitoring mode can be further studied.

UE may enter LP-WUS monitoring mode in case no paging is received in target PO(s) after being woken up by LP-WUS. FFS whether entry condition is re-evaluated before entering the LP-WUS monitoring mode.

# Conclusion

This document discusses some procedures for a LR capable UE in the RRC_IDLE/ RRC_INACTIVE Mode. Following are the proposals and observations made in this document:

- The entry/exit condition for LP-WUS monitoring is always configured in SIB when the cell supports LP-WUS.

- RAN2 to discuss the need for any additional entry/exit conditions such as the UE’s mobility state, UE’s paging probabilities etc.

- Observation 1  LP-WUS Subgrouping should be designed such that it does not negatively impact false alarm rate and thus the power saving gain.

- The maximum number of subgroups that can be configured for LP-WUS subgrouping is 8.

- RAN2 to discuss the inclusion of a LP-WUS Subgrouping Configuration in the SIB signalling.

- RAN2 to discuss the configuration of 2-level subgrouping to reduce false alarm rates in comparison to legacy PEI.

- UE may enter LP-WUS monitoring mode in case no paging is received in target PO(s) after being woken up by LP-WUS. FFS whether entry condition is re-evaluated before entering the LP-WUS monitoring mode.

# References

[1] 3GPP RAN#103, RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] 3GPP TR 38.869 v18.0.0: Study on low-power wake up signal and receiver for NR (Release 18)

[3] 3GPP TSG RAN WG1#116, Chair’s Notes,

[4] 3GPP TSG RAN WG1#117, Chair’s Motes