3GPP TSG-RAN2#127bis	R2-2408490

Hefei, China, October 14-18, 2024

Agenda item:		8.4.4

Source:		Lenovo

Title:		 LP-WUS in RRC Connected Mode

Document for:		Discussion and Decision

- Introduction

Currently, the UE’s Main Radio (MR) needs to periodically monitor PDCCH once per DRX cycle, which dominates the power consumption in periods with no signaling or data traffic. If the UE’s MR can wake up only when they are triggered (e.g. paging), power consumption could be dramatically reduced. This can be achieved by using a wake-up signal to trigger the main radio and a separate receiver which has the ability to monitor wake-up signal with ultra-low power consumption. Main radio works for data transmission and reception, which can be turned off or set to deep sleep unless it needs to monitor PDCCH [1]. The following agreements were made at the RAN1 #116bis and RAN2 #126 meetings:

Agreements at RAN1 #116bis: - Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, at least consider the following： - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 2: A codepoint value corresponding to one or part of UE identity, e.g., C-RNTI - Option 3: A codepoint value corresponding to [one or more] UEs - Option 4: Multiple codepoint values with each corresponding to [one or more] UE(s) - Option 5: Multiple bit blocks with each corresponding to [one or more] UE(s) - Combination of above options are not precluded. - FFS how to carry LP-WUS information, e.g., by encoded bits (with/without CRC) and/or by OOK sequence selection for ‘ON-OFF’ pattern for OOK symbols of LP-WUS. - FFS how to carry LP-WUS information by overlaid OFDM sequences. - It doesn’t preclude considering the configuration where a single candidate overlaid OFDM sequence is used. - FFS details of LP-WUS information to trigger PDCCH monitoring (e.g., whether above is applicable to one or more serving cells) Agreements at RAN1 #118: - For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. • LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. • UE is configured with legacy C-DRX configurations as Rel-18 • UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) o FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration • LP-WUS triggers the start of a timer during which UE monitors PDCCH o FFS the timer is existing timer and/or new timer • UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected o drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL • No impact on RRM/RLM/BFD measurement requirements is assumed • For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) o Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up o Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not • UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration • Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. • Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. • FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE - Note: Above can be revisited considering RAN2 decisions. Agreements at RAN2 #127: - For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

This contribution looks at the possible RRC Connected Mode procedures for an LP-WUR enabled UE including activation/ deactivation and the use of LP-WUS in Connected Mode DRX.

- Discussion

  - Activation/ Deactivation of LP-WUS Monitoring

RAN1 has already agreed in previous meetings that gNB RRC singaling will be used to enable/ disable PDCCH monitoring triggered by LP-WUS. And for further activation/ deactivation of LP-WUS monitoring after it has been enabled, RAN1 has provided four options as mentioned above.

- RAN1 has agreed that gNB RRC signalling is used to enable/disable PDCCH monitoring triggered by LP-WUS.

In our view, the activation/deactivation of LP-WUS is related to the traffic arrival and channel conditions, we think at least some implicit activation/ deactivation methods need to be discussed. For example, LP-WUS monitoring may be deactivated when some UL data becomes available for transmission in the HARQ buffers. Additionally, gNB can configure LP-WUS deactivation timer, i.e., a duration after activation and/or configure a LP-WUS activation timer, i.e., a duration after deactivation. We also think it would be beneficial to discuss other activation/ deactivation methods so that network may have more control of when the UE monitors for LP-WUS.

Furthermore, RAN1 has agreed that the gNB is aware of LP-WUS monitoring in the UE. We think that some explicit indication may be used for the purpose of network awareness, i.e., the UE indicates to the network when it is actively monitoring for LP-WUS or when it stops monitoring for the LP-WUS (e.g., due to fulfilling the LR exit conditions). For example, some dedicated RRC signalling may be used for this purpose.

In the case that LP-WUS monitoring is enabled, RAN2 to consider additional conditions to activate/deactivate the LP-WUS monitoring.

UEs in RRC_CONNECTED state should report its LP-WUS monitoring status to gNB, e.g., by dedicated RRC signalling.

  - LP-WUS for Connected-Mode DRX

As per the R16 power saving item, in RRC Connected Mode, the MR may be configured with a DCI 2_6 (Wake-Up Signal). This DCI format 2_6 is monitored outside of the DRX active time to decide whether the MR needs to be woken up in the next drx-onDurationTimer. RAN2 should discuss how the UE may use the LP-WUS to wake up the MR such that the power saving gain achieved by using the LR must be higher than the power saving gain achieved by using only the DCI format 2_6, at the least.

- RAN1 has made the working assumption to support Option 1-1 and Option 1-2 for RRC_CONNECTED state in RAN1#118 meeting [2]. Based on the working assumption, in the last RAN2#127 meeting, it has been agreed that for option 1-2, after LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer, and when the timer is running, the UE monitors PDCCH. Whether the timer is newly defined timer or legacy timer needs to be further studied. From our point of view, a newly defined timer can better adapt to the service triggered by LP-WUS, and we may call the new timer triggered by LP-WUS as LR-specific timer, e.g., LR-specific on-duration timer. The newly defined timer has separate start conditions from legacy C-DRX related timers and is also configured and operated independently from legacy timers. The detail of the LR specific timer is shown in figure 1.

- Figure 1: One example of LR-specific timer.

- However, there’s a risk that data transmission has not ended when the timer expires, which may degrade the system performance. In the case that UE performs PDCCH monitoring during the new timer and the PDCCH indicates a new UL, DL or SL transmission for the MAC entity, how the new timer to be extended to guarantee the completion of the new data transmission needs to be discussed. For one solution, legacy drx-Inactivity timer can be reused to solve the issues since the PDCCH monitoring duration triggered by the LP-WUS is regarded as active time. For another solution, an additional inactivity like timer e.g., LR-specific inactivity timer is introduced on top of the above newly defined timer as shown in figure 2. In our view, reusing the existing drx-Inactivity timer is a simpler solution and would have lesser specification impact. Furthermore, it would align with RAN 1’s agreement to not impact the other timers in C-DRX. Hence, it is suggested that RAN2 discuss the issue and consider whether legacy drx-Inactivity timer can be reused on top of the newly defined timer to extend the active time triggered by LP-WUS and guarantee that data can be transmitted completely.

- Figure 2: One example of Inactivity timer

For option 1-2, support a newly defined timer for PDCCH monitoring triggered by LP-WUS. The newly defined timer is configured and operated independently from legacy timers.

RAN2 to discuss whether legacy drx-Inactivity timer can be reused on top of the newly defined timer to extend the active time triggered by LP-WUS.

- Additionally, in RAN1#118 meeting, whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE was discussed and needs further study. From our point of view, the combination of Option 1-1 and Option 1-2 can guarantee periodic service and non-periodic service simultaneously without degrading power saving gain. In this case, when receiving LP-WUS outside the legacy C-DRX active time, UE may start PDCCH monitoring immediately if Option 1-2 is applied or wait till legacy drx-onDurationTimer is initiated if Option 1-1 is applied. The resource of detected LP-WUS (e.g., detected time or specific LO configured at a time offset from the start of drx-onDuration Timer) can be used to determine the start point of PDCCH monitoring in this case. For example, if the detected time is within a time window before the drx-onDurationTimer or the time between detected LP-WUS and start of drx-onDurationTimer is lower than a minimum time gap value, UE starts PDCCH monitoring when the associated drx-onDurationTimer is started; otherwise, UE starts PDCCH monitoring immediately according to the new timer, when LP-WUS is detected. In another example, network may provide multiple LP-WUS configuraions (e.g., configuration 1 is for option 1-1 and configuration 2 is for option 1-2) to the UE, UE determines which configuration is used based on the resource of detected LP-WUS.

In the case that option 1-1 and option 1-2 are configured simultaneously for the same UE, the start time of PDCCH monitoring corresponds to the resource of detected LP-WUS, e.g., detected time.

  - MR wake-up due to UL data

During the Study Item Phase, it had been agreed in RAN2 that MR needs to wake-up for PDCCH monitoring when the MR performs some UL data transmission [3].

- Agreements: UL transmission by MR also triggers PDCCH monitoring by MR.

This also means that when new UL data becomes available for transmission, it can be considered as an implicit indication to wake-up the MR, if the MR had previously been asleep. This implicit and/ or explicit indication to wake-up may result in a frequent wake-up of MR whenever new data becomes available for transmission after LP-WUS activation leading to a reduction in the power saving gain offered by configuring the LP-WUR.

- Frequent wake-up of MR caused by UL transmission can lead to a reduction in the power saving gain offered by LP-WUR configuration.

Hence, some enhancements may be considered in the wake-up behavior of the MR to optimize the power saving gain even when some UL transmission needs to take place. For example, one way to avoid reduction in the power saving gain is by delaying the MR wake-up (where wake-up refers to transmission of UL data and the consequent monitoring of PDCCH) when new UL data becomes available for transmission (especially if the data that arrived is not urgent), i.e., the wake-up may be postponed until absolutely necessary.

Hence, RAN 2 should discuss how MR sleep time may be prolonged when UL data becomes available for transmission.

RAN2 to discuss means of improving MR sleep time, e.g., by delaying (non-urgent) UL transmissions.

  - CSI Reporting

At present, the UE may frequently report CSI to the network in one of three modes – Periodic, Semi-Persistent and Aperiodic. This implies that the MR may need to remain awake for long periods to transmit such reports to the network, which can significantly diminish the power saving gain offered by using the LR when it is configured. Hence, it may be beneficial to consider some restrictions to CSI reporting such as reduced reporting or reduced measurement when LP-WUS monitoring is activated. Hence, we think it would be beneficial for RAN2 to discuss possible restrictions or relaxations to CSI reporting when LP-WUS is activated.

RAN2 to discuss the restriction or relaxation of CSI reporting when LP-WUS is activated.

  - Others

- SPS and CG impacts

- For UEs in RRC_Connected state, the resources for SPS and CG are configured by the network to UEs for potential downlink and uplink transmission. The UE may perform DL reception and UL transmission on the configured SPS and CG resources. LP-WUS impacts on the configured SPS/CG needs to be identified when UE performs LP-WUS monitoring.  It is suggested that RAN2 discuss potential UE behaviours on the configured SPS/CG resources when performing LP-WUS monitoring in RRC_Connected state. For example, in the case of UE performs LP-WUS monitoring, i.e., UE MR enters sleep state, whether the configured SPS/CG resource remains valid, and whether UE MR wakes up on the configured SPS/CG resource needs to be clarified.

RAN2 to discuss UE behaviours on the configured SPS/CG resources when performing LP-WUS monitoring in RRC_Connected state.

- Dual DRX Groups

- Serving cells of a MAC entity may be configured by RRC with two DRX groups, each DRX group has separate DRX parameters to be applied for FR1 and FR2. The DRX parameters that are separately configured for each DRX group are: Then RAN2 needs to identify that for UEs configured with LP-WUS configuration, whether common LP-WUS configuration or per DRX group LP-WUS configuration are provided to UE. In legacy, DRX configuration is provided in the MAC cell group configuration, while DCP (if configured) is provided in PCG configuration. Considering that the solutions introduced for DCP mechanism is taken as baseline for LP-WUS design in Option 1, it is proposed that LP-WUS configuration is configured per-DRX group for UEs configured with dual DRX groups.

LP-WUS configuration is configured per-DRX group for UEs configured with dual DRX groups.

- Conclusion

This document discusses some procedures for a LR capable UE in the RRC Connected Mode. Following are the proposals and observations made in this document:

- Observation 1: RAN1 has agreed that gNB RRC signalling is used to enable/disable PDCCH monitoring triggered by LP-WUS.

- Proposal 1: In the case that LP-WUS monitoring is enabled, RAN2 to consider additional conditions to activate/deactivate the LP-WUS monitoring.

- Proposal 2: UEs in RRC_CONNECTED state should report its LP-WUS monitoring status to gNB, e.g., by dedicated RRC signalling.

- Proposal 3: For option 1-2, support a newly defined timer for PDCCH monitoring triggered by LP-WUS. The newly defined timer is configured and operated independently from legacy timers.

- Proposal 4: RAN2 to discuss whether legacy drx-Inactivity timer can be reused on top of the newly defined timer to extend the active time triggered by LP-WUS.

- Observation 2: Frequent wake-up of MR caused by UL transmission can lead to a reduction in the power saving gain offered by LP-WUR configuration.

- Proposal 6: RAN2 to discuss means of improving MR sleep time, e.g., by delaying (non-urgent) UL transmissions.

- Proposal 7: RAN2 to discuss the restriction or relaxation of CSI reporting when LP-WUS is activated.

- Proposal 8: RAN2 to discuss UE behaviours on the configured SPS/CG resources when performing LP-WUS monitoring in RRC_Connected state.

- Proposal 9: LP-WUS configuration is configured per-DRX group for UEs configured with dual DRX groups.

- References

[1] 3GPP TR 38.869 v0.2.0: Study on low-power wake up signal and receiver for NR (Release 18)

[2] 3GPP TSG RAN WG1#118, Chair’s Notes

[3] R2 123bis Session Notes: Report from session on Mobility Enh, Mobile IAB and LP-WUS