3GPP TSG-RAN WG2 Meeting #127bis	R2-2408498

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.6.2

Source: 	OPPO

Title:  	Discussion on open issues for inter-CU LTM

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to provide our considerations on open issues for inter-CU LTM.

Discussion

LTM fast failure recovery

In R18, the LTM fast failure recovery is supported for RLF and T304 expiry after LTM cell switch. It facilitates a more efficient failure recovery mechanism by leveraging the preconfigured LTM candidate cell configurations, which is also needed in R19.

LTM fast recovery for RLF and T304 expiry after LTM cell swich is supported in R19.

For R19 LTM, both intra-CU and inter-CU LTM configurations can be configured, it is questionable whether all these LTM candidates can be applied for fast recovery.

Given that the security key update is not required for intra-CU LTM, at least the intra-CU LTM configurations can be used for fast recovery, i.e., if an intra-CU (which is compared to the source cell) LTM candidate cell is available during cell reselection, the configuration can be applied and the source cell security info is reused.

For inter-CU LTM configurations, their applicability for fast recovery depends on the security key update approach. For example, if the security key update information needs to be carried in LTM cell switch MAC CE, fast recovery to an inter-CU LTM candidate cell may not be feasible. How to perform the security key update is still under discussion in SA3 and LTM fast recovery may not be considered during their study. If proposal1 is agreed, it is necessary to send and LS to SA3 to inform them of the case that the LTM configuration can be applied for failure recovery and to ask them to take this case into account when making decision on security key update approach.

The intra-CU LTM configurations can be applied for fast failure recovery after RLF and LTM failure.

Whether the inter-CU LTM configurations can be applied for fast failure recovery depends on the mechanism for security update.

Send an L3 to SA3 to inform the case that the LTM configuration can be applied for fast failure recovery and ask them to take the case into account when studying the security key update approach.

For R18 LTM fast recovery, the UE should perform PDCP continuation for SRB(s) after LTM cell switch failure to avoid keystream reuse issue. For instance, if the COUNT value “N” is used for transmitting the RRCReconfigurationComplete message during the first LTM attempt, the COUNT value should be continued from “N+1” for the second LTM attempt (i.e., LTM failure recovery) due to consecutive LTM attempts w/o security key update.

For R19 LTM fast recovery, the two LTM attempts can be inter-CU or intra-CU as summarized in the table below (assuming that the fast failure recovery to an inter-CU LTM candidate cell is possible), which may result in different UE behaviour upon T304 expires.

|  | First LTM attempt | LTM fast recovery |
| --- | --- | --- |
| Case1 | Intra-CU LTM | Intra-CU LTM |
| Case2 | Intra-CU LTM | Inter-CU LTM |
| Case3 | Inter-CU LTM | Intra-CU LTM |
| Case4 | Inter-CU LTM | Inter-CU LTM |

- Case1/Case2: If the two LTM attempts are both intra-CU LTM as in Case1, the same behaviour as R18 should be followed upon T304 expires, i.e., the UE revert back to the source configuration except for the PDCP state variables for SRB(s). If an inter-CU LTM candidate cell is selected for fast recovery as in Case2, the security key update and PDCP re-establishment shall be performed, during which procedure the TX_NEXT for SRB(s) will be reset to the initial value. For Case2, the PDCP continuation for SRBs is not required upon the failure of the first LTM attempt while it also doesn’t matter if the same behaviour as for case1 is performed upon T304 expires. Since the type of LTM candidate cell selected for fast recovery is not predictable, the PDCP continuation should be executed after intra-CU LTM failure in order to support the possible occurrence of Case1.

- Case3/Case4: If the first attempt is inter-CU LTM, the UE will perform PDCP re-establishment and security key update during LTM execution. If an intra-CU LTM candidate cell is selected for fast failure recovery as in Case3, the security key of the source cell should be applied which is different from the one used for the first LTM attempt and the PDCP state variables for SRB(s) need to continue from the status of source. If an inter-CU LTM candidate cell is selected for fast failure recovery as in Case4, a new security key which is different from the one used for the source cell and the first LTM attempt will be applied. For both cases, there is no keystream reuse issue even if the UE revert back to the source cell configuration including the PDCP state variables for SRBs upon T304 expires.

For LTM fast recovery after intra-CU LTM failure, the UE revert back to the source cell configuration except for the PDCP state variables for SRB(s) upon T304 expires.

For LTM fast recovery after inter-CU LTM failure, the UE revert back to the source cell configuration upon T304 expires.

Coexistence issue

For various types of LTM configurations including inter-MN LTM, intra-MN LTM, inter-SN LTM and intra-SN LTM, further discussion is needed on their co-existence. Our view on the issue is provided as follows.

Inter-MN LTM and inter-SN LTM

The co-existence of inter-MN LTM and inter-SN LTM is not supported as per WID.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

Inter-CU LTM at MN/SN and intra-CU LTM at SN/MN.

In our understanding, the inter-CU LTM configured in one node (i.e., MN or SN) can coexist with the intra-CU LTM configured in the other node (i.e., SN or MN). However, the LTM configuration in one node might be impacted by the LTM execution at the other node as analyzed below.

- The SCG can be released during MCG LTM execution. If SCG is released, the SCG LTM configuration (either inter-SN or inter-SN) should also be released.

- The execution of inter-CU LTM at one node may require a configuration update at the other node, which would impact the validity of the intra-CU LTM configuration at the other node. For example, there is a case that the bearer type change is performed during inter-CU LTM, e.g., from MCG bearer to SCG bearer, but the candidate cells for intra-CU LTM are prepared based on the context that it is an MCG bearer. The bearer type change is not expected during intra-CU LTM as discussed in R18, and if intra-CU LTM is executed after inter-CU LTM, it will lead to error bearer type change. To handle the issue, it is preferred to rely on the NW to guarantee the validity of LTM configuration, e.g., the LTM configuration can be released or updated by explicit NW signalling.

Intra-MN LTM and intra-SN LTM

The co-existence of intra-MN LTM and intra-SN LTM has been agreed in R18.

Based on the analysis above, we have the following proposals:

The following co-existence cases are supported:

  - Inter-MN LTM and intra-SN LTM.

  - Intra-MN LTM and inter-SN LTM.

  - Intra-MN LTM and intra-SN LTM.

If the SCG is released at MCG LTM execution, the SCG LTM configuration should be released.

- Obervation1  The intra-CU LTM configuration configured in one node (i.e., MN or SN) might be invalid due to the execution of inter-CU LTM at the other node (i.e., SN or MN).

It is up to the NW to guarantee the validity of the LTM configuration for case that the serving cell configuration is updated due to the LTM execution at the other NW node (MN or SN).

LTM configuration release

The R18 LTM configuration is autonomously released by the UE in the cases as summarized in the table below.

| RRC procedure | RRCRelease/go to IDLE | MR-DC release | Re-establishment (Case1: attemptLTM-Switch not configured Case2: attemptLTM-Switch configured but LTM candidate cell is not selected during T311 |
| --- | --- | --- | --- |
| R18 MCG LTM config | Autonomous release | \ | Autonomous release |
| R18 SCG LTM config | Autonomous release | Autonomous release | Autonomous release |

For RRC re-establishment, it is still not clear how the R19 LTM configuration can be applied for fast recovery as discussed in section 2.1. The handling of LTM configuration for this case can be studied when further progress is made for R19 LTM failure recovery. For other cases, the same behaviour can be simply followed for R19 LTM configuration

The R19 inter-CU LTM configuration is autonomously by the UE for the following cases:

  - Reception of RRCRelease.

  - Go to IDLE.

  - MR-DC release (only for inter-CU SCG LTM config, if configured).

Conclusion

Based on the discussion in section2, we have following observation proposals:

Proposal 1	LTM fast recovery for RLF and T304 expiry after LTM cell swich is supported in R19.

Proposal 2	The intra-CU LTM configurations can be applied for fast failure recovery after RLF and LTM failure.

Proposal 3	Whether the inter-CU LTM configurations can be applied for fast failure recovery depends on the mechanism for security update.

Proposal 4	Send an L3 to SA3 to inform the case that the LTM configuration can be applied for fast failure recovery and ask them to take the case into account when studying the security key update approach.

Proposal 5	For LTM fast recovery after intra-CU LTM failure, the UE revert back to the source cell configuration except for the PDCP state variables for SRB(s) upon T304 expires.

Proposal 6	For LTM fast recovery after inter-CU LTM failure, the UE revert back to the source cell configuration upon T304 expires.

Proposal 7	The following co-existence cases are supported:

a.	Inter-MN LTM and intra-SN LTM.

b.	Intra-MN LTM and inter-SN LTM.

c.	Intra-MN LTM and intra-SN LTM.

Proposal 8	If the SCG is released at MCG LTM execution, the SCG LTM configuration should be released.

Obervation1  The intra-CU LTM configuration configured in one node (i.e., MN or SN) might be invalid due to the execution of inter-CU LTM at the other node (i.e., SN or MN).

Proposal 9	It is up to the NW to guarantee the validity of the LTM configuration for case that the serving cell configuration is updated due to the LTM execution at the other NW node (MN or SN).

Proposal 10	The R19 inter-CU LTM configuration is autonomously by the UE for the following cases:

a.	Reception of RRCRelease.

b.	Go to IDLE.

c.	MR-DC release (only for inter-CU SCG LTM config, if configured).