3GPP TSG-RAN WG2 Meeting #127bis	                                R2-2408500

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.6.4

Source: 	OPPO

Title:  	Discussion on conditional LTM

Document for:	Discussion and Decision

Introduction

In the latest RAN plenary meeting, the WID for further enhancement of NR mobility is updated, and conditional LTM objective is modified as following:

In this contribution, we discuss the open issues for supporting conditional LTM.

Discussion

Conditional LTM is introduced in Rel-19 which aims to achieve high robustness by enabling the procedure to be executed without explicit signalling exchange with source cell beforehand. As indicated in WID, conditional LTM is limited to intra-CU scenario, which is same as Rel-18 LTM. To have a whole picture on how to support CLTM with considering Rel-18 intra-CU LTM as baseline, the key components for R18 intra-CU LTM have been sorted out as below. And in this the following section, we will analyse the additional part for supporting CLTM.

|  | R18 Intra-CU LTM | Additional part for supporting Conditional LTM |
| --- | --- | --- |
| HO preparation/ LTM config | √ | - Event config preparation |
| Early DL sync | √ | √ |
| Early UL sync | √ | - Event triggered early sync - early sync with RAR - Candidate TA maintenance |
| NW configured L1 measurement reporting | √ | √ |
| Event triggered L1 measurement reporting |  | √ |
| Early TCI activation/deactivation | √ | √ |
| LTM cell switch | √ | - Event triggered LTM execution - Threshold based CG resource selection - CG based RACH-LESS CLTM |

CLTM configuration

The most significant difference between LTM and CLTM is the trigger of the LTM cell switch execution. For candidate cell preparation, Rel-18 LTM candidate cell preparation procedure can be reused for CLTM. Besides, the event configuration for CLTM candidate cell evaluation can also be prepared during the CLTM candidate cell preparation period.

And to support subsequent CLTM, the event configuration for CLTM should also be provided by each serving cell/candidate cell considering the event configurations for CLTM candidate evaluation should use different threshold/parameters for the events given that the radio propagations between the new serving cell and its neighbours are different from those corresponding to previous source serving cell.

The R18 LTM preparation procedure is reused for conditional LTM candidate preparation.

Event configuration for CLTM is generated by each candidate cell.

Early sync

For NW triggered intra-CU LTM, the early UL synchronization is triggered by PDCCH order, UE performs PRACH transmission without receiving RAR. Then, the candidate cell indicates the computed TA to source cell and the source cell forward the TA to UE through LTM cell switch MAC CE. For CLTM, explicit NW signalling which triggering cell switch execution is omitted to guarantee a robust HO procedure, that is, there is no LTM cell switch MAC CE delivered to UE.

To support early sync procedure for CLTM, RAR can be reused for TA provision. And there are two way for the RAR reception:

- Option 1: RAR is transmitted by candidate cell

- Option 2: RAR is transmitted by serving cell

In our understanding, both options work. While option 2 have more benefit as the serving cell service will not be impacted due to RAR monitoring. As the RACH is performed towards a candidate cell, and UE is maintaining the connection with serving cell, monitoring RAR on candidate cell may cause additional data interruption for serving cell. The RA response for early sync could be delivered to UE by serving cell in CLTM.

In addition to TA, legacy RAR also includes the fields to indicate TC-RNTI and UL grant. While the candidate cell configuration provided to UE already includes C-RNTI for the candidate cell, there is no necessary to provide redundant RNTI to UE. Regarding to UL grant of candidate cell, it is waste of resources to provide candidate cell resource before cell switch since UE will only perform data transmission with current serving cell instead of candidate cell. Thus, there is no need to include the field of UL grant (candidate cell) and TC-RNTI in RAR for the RACH triggered for early synchronization. RAN2 can further study whether to introduce new RAR format or reuse legacy Absolute Timing Advance Command MAC CE/Timing Advance Command MAC CE.

PDCCH ordered early sync to candidate cell(s) is supported for CLTM.

For CLTM, RA response is delivered to UE by serving cell for early sync procedure.

Besides PDCCH ordered early sync, there is another alternative for triggering the early sync procedure, i.e. event triggered early sync. UE can evaluate the candidate cell/RSs and trigger early sync to a triggered cell/beam to acquire the TA value. In R19 inter-CU LTM, companies also reach consensus that the event configuration can be used for early sync scenario since UE have the real-time L1 measurement of the candidate cells/RSs.

Event triggered early sync to candidate cell(s) is supported for CLTM.

Furthermore, if RAR reception is supported for early sync to candidate cell, it is questionable whether and how the TA maintenance is supported.

For serving cells, maintenance of uplink time alignment relies on a TA timer that is associated with a TAG. UE starts/restarts the corresponding TA timer if TA is indicated by NW. For early sync procedure, if RAR including TA information is provided to UE through RAR, the TA maintenance associated to candidate cell should be studied.

One straightforward way is to reuse TAG for TA indication and maintenance. Similar as serving cell TA maintenance, UE starts or restarts the timeAlignmentTimer associated with this TAG when a timing advance command is received in a random access response message for a candidate cell belonging to a TAG. In that way, TAG ID should be unique at least among serving cell(s) and candidate cells.

RAN2 study whether the TA maintenance for candidate cell is supported for CLTM.

L1 measurement reporting and early TCI activation

In Rel-18, NW configured L1 measurement reporting for candidate cells are introduced, and we further introduce event triggered L1 measurement reporting in Rel-19 for signalling overhead reduction. From CLTM perspective, we do see the need on performing L1 measurement reporting to assist NW to make decision for NW triggered procedures. For example, NW can trigger the early sync procedure by sending PDCCH order which indicates the proper contention free PRACH resources as well as beam info according to the received L1 measurement results. Similar for early TCI state activation/deactivation, NW need to know well about the radio conditions of each candidate TCI state . And both NW configured and event triggered L1 reporting can be supported by CLTM.

Both NW configured and event triggered L1 measurement reporting are supported for CLTM.

With the received measurement result, NW can also send early TCI state activation/deactivation MAC CE to UE to reduce the interruption time caused by SSB/CSI-RS fine tracking.

NW triggered early TCI state activation/deactivation is supported for CLTM.

CLTM execution

As in NW triggered intra-CU LTM, both RACH-based and RACH-less cell switch can be supported for CLTM execution. For RACH-less CLTM, as the cell switch execution is triggered by UE itself, NW may not be able to schedule a dynamic grant to UE immediately upon cell switch execution. Thus, only CG based RACH-less CLTM is supported if TA is available.

Only CG based RACH-less CLTM is supported if TA is available.

And for CG resource selection for RACH-less CLTM, UE can follow the way in RACH-less HO in NTN and mIAB. UE selects a configured uplink grant for initial uplink transmission according to the configured threshold.

RSRP Threshold based CG resource selection is used for RACH-less CLTM.

Regarding the trigger event granularity, i.e. cell level or beam level, we understand the agreement in R19 inter-CU LTM is also valid for CLTM. At least beam level event is support for CLTM.

RAN2 study whether to introduce cell level event for LTM execution evaluation.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	The R18 LTM preparation procedure is reused for conditional LTM candidate preparation.

Proposal 2	Event configuration for CLTM is generated by each candidate cell.

Proposal 3	PDCCH ordered early sync to candidate cell(s) is supported for CLTM.

Proposal 4	For CLTM, RA response is delivered to UE by serving cell for early sync procedure.

Proposal 5	Event triggered early sync to candidate cell(s) is supported for CLTM.

Proposal 6	RAN2 study whether the TA maintenance for candidate cell is supported for CLTM.

Proposal 7	Both NW configured and event triggered L1 measurement reporting are supported for CLTM.

Proposal 8	NW triggered early TCI state activation/deactivation is supported for CLTM.

Proposal 9	Only CG based RACH-less CLTM is supported if TA is available.

Proposal 10	RSRP Threshold based CG resource selection is used for RACH-less CLTM.

Proposal 11	RAN2 study whether to introduce cell level event for LTM execution evaluation.

Reference

[1] RP-234036 New WID-R19 Mobility Enhancements-v5.docx