3GPP TSG-RAN WG2 Meeting #127bis	R2-2408507

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.6.4

Source:	Panasonic

Title: 	Considerations for conditional LTM

Document for:	Discussion

# Introduction

As per WID in [1], Rel-19 work item for mobility enhancements specifies to support intra-CU conditional L1/L2 triggered mobility (LTM), aiming to provide support for conditional cell switch and subsequent conditional cell switch, where UE evaluates execution conditions to perform cell switch operation during mobility.

In this contribution, some of the considerations to support conditional LTM is discussed.

# Discussion

## Components of conditional LTM

Figure-1 shows a typical sequence for a condition handover procedure as evaluated and executed by UE. It could be categorized into four components as follows.

- Conditional candidate preparation and configuration.

- Execution condition evaluation.

- DL and UL synchronization.

- Handover completion.

Figure-1: Components of conditional handover executed at UE

This document is divided into sections based on the above listed components to discuss some of the open issues for the individual components of condition LTM, because the components in existing conditional handover feature can be adapted or optimized for the conditional LTM procedure in Rel-19 NR.

### Conditional candidate preparation for LTM

Candidate cells for lower layer mobility is also an integral part of Rel-18 LTM (focussing on intra-CU LTM), since one or more LTM candidate cells are cumulated and configured at UE earlier before even the actual handover decision is made towards one target cell among LTM candidate configurations. Here the conditional LTM feature also requires a similar candidate preparation phase and configurations, and therefore the candidate preparation and configuration could be reused from the R-18 LTM (or extent as per R-19 LTM in future).

Proposal 1: Conditional LTM to reuse LTM configuration procedure as in R-18 LTM (for intra-CU candidate cells) to configure LTM candidate cells to UE, including the LTM candidate IDs which are indicated to uniquely identify each LTM candidate cell configuration at UE.

### Conditional LTM execution phase

Conditional handover currently supports following execution conditions to evaluate and perform handover decisions at UE, defined based on channel quality measurements of the candidate cells.

CondEvent A3: Conditional candidate becomes amount of offset better than source.

CondEvent A4: Conditional candidate becomes better than absolute threshold.

CondEvent A5: Source becomes worse than absolute threshold1 AND Conditional candidate becomes better than another absolute threshold2.

The above events designed for conditional handover feature in NR is defined at cell level evaluation because these conditions are evaluated at RRC layer, and now for conditional LTM the requirement can be changed to evaluate such event conditions at the lower layer, hence beam specific events may be needed. We can also further discuss about the usage of cell level event evaluation (or beam consolidation for cell level evaluation) as necessary, since UE must evaluate to perform switching from source cell to target cell with best conditions (for example, the best target cell to switch can be determined if more beams are evaluated to satisfy event conditions).

Proposal 2: Beam specific events are defined for conditional LTM as the baseline. Like the LTMx events defined for measurement evaluation and reporting.

Proposal 3: Legacy conditional handover conditions such as CondEvent-A3, CondEvent-A4 and CondEvent-A5 can be considered as baseline to define event configuration for condition LTM.

- FFS: whether and how to consolidate beam specific conditions to cell level conditions

- FFS:  the need for any new event definitions specific to conditional LTM.

### Early synchronization phase

As seen in figure-1, when UE evaluates execution conditions for conditional handover and is triggered for one of the event conditions to switch from source cell to target cell, DL and UL synchronization is performed before the first data is communicated with the target cell. This introduces interruption in services provided to the UE during handover.

Now similar to R-18 LTM, to reduce interruption during cell switch operation, early synchronization concept can be introduced into conditional LTM where the UE performs early DL and UL synchronization for one of more LTM candidate cells, before deciding to switch towards one target when execution conditions are satisfied.

Proposal 4: RAN2 to discuss the feasibility of Rel-18 DL and UL early synchronization if used for conditional LTM. The following aspects needs to be clarified when UE is configured to evaluate the execution conditions for conditional LTM.

- How TCI state activation is achieved towards one or more candidate cells.

- How TA acquisition can be achieved using RACH procedure with one or more candidate cells.

Further if UE performs cell switch decision during conditional event evaluation, then the TCI state activation and TA acquisition should be performed by UE before executing conditional cell switch. Since conditional cell switch would include execution conditions to evaluate and perform such decisions, it is required that UE must be provided with similar execution conditions to perform early DL and UL synchronization procedure with those beams/ cells suitable for early synchronization. For example, when UE evaluates execution conditions as defined for conditional cell switch operation, one or more candidate beams must be evaluated to perform early synchronization operation and acquire TA value, so that RACH-less cell switch operation is possible when event trigger conditions satisfy for the target cell to perform cell switch.

Proposal 5: Conditional LTM to configure execution conditions at UE to evaluate and perform early synchronization with candidate beams.

### Conditional LTM completion phase

Conditional LTM execution and completion phase can be same as in R-18 LTM, but instead of LTM cell switch command in DL indicated to trigger the cell switch operation, UE here decides to trigger cell switch towards the candidate beam for which the event condition has been satisfied.

Further, the indication to reset RLC layer can be reused from R-18 LTM, like using the ltm-NoResetID-r18 value form the candidate configurations.

Proposal 6: Conditional LTM to support RACH-less LTM during condition cell switch towards the target cell.

Proposal 7: For intra-CU conditional LTM, reuse the IE ltm-NoResetID-r18 to determine whether RLC layer to be re-established after candidate configuration is applied for conditional cell switch.

# Conclusions

Conditional LTM preparation phase

Proposal 1: Conditional LTM to reuse LTM configuration procedure as in R-18 LTM (for intra-CU candidate cells) to configure LTM candidate cells to UE, including the LTM candidate IDs which are indicated to uniquely identify each LTM candidate cell configuration at UE.

Conditional LTM execution

Proposal 2: Beam specific events are defined for conditional LTM as the baseline. Like the LTMx events defined for measurement evaluation and reporting.

Proposal 3: Legacy conditional handover conditions such as CondEvent-A3, CondEvent-A4 and CondEvent-A5 can be considered as baseline to define event configuration for condition LTM.

- FFS: whether and how to consolidate beam specific conditions to cell level conditions

- FFS:  the need for any new event definitions specific to conditional LTM.

Early DL and UL synchronization

Proposal 4: RAN2 to discuss the feasibility of Rel-18 DL and UL early synchronization if used for conditional LTM. The following aspects needs to be clarified when UE is configured to evaluate the execution conditions for conditional LTM.

- How TCI state activation is achieved towards one or more candidate cells.

- How TA acquisition can be achieved using RACH procedure with one or more candidate cells.

Proposal 5: Conditional LTM to configure execution conditions at UE to evaluate and perform early synchronization with candidate beams.

Conditional LTM completion

Proposal 6: Conditional LTM to support RACH-less LTM during condition cell switch towards the target cell.

Proposal 7: For intra-CU conditional LTM, reuse the IE ltm-NoResetID-r18 to determine whether RLC layer to be re-established after candidate configuration is applied for conditional cell switch.

# References

[1]		RP-242356; Revised Work Item: NR mobility enhancements Phase 4.