3GPP TSG-RAN WG2 Meeting #127bis                               R2-2408508

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:		8.11.2

Source:				Charter Communications

Title:					SBFD RACH configuration for initial random access

Document for:		Discussion and decision

# Introduction

According to the revised WID [1] on the evolution of NR duplex operation – Subband Full duplex (SBFD), enhancements for random access for SBFD-aware UE should be considered and supported under the following objective:

  - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

In this contribution, focus is on supporting random access in SBFD symbols for SBFD-aware UEs in RRC_IDLE/INACTIVE mode and provide analysis on the various configurations options agreed in RAN1, and then discuss the possible approaches to convey the RACH configurations for initial random access in SBFD symbols.

# SBFD RACH Configuration Options

In RAN#105, RAN1 reported some progress and agreements on the PRACH configuration options, namely, Option 1 and Option 2. The agreements are as follows [2]:

Agreement

For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following:

- For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon.

- For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration.

Agreement

For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, select one from the following alternatives.

- Alt 1: The parameter msg1-FrequencyStart in rach-ConfigCommon is applied with no reinterpretation.

- Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

- Alt 2-2: Use a fixed value (e.g., 0) for the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

- Alt 2-3: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs is equal to mod (msg1-FrequencyStart, bandwidth of UL usable PRBs).

- Alt 2-4: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRB is msg1-FrequencyStart*scaling factor, where the scaling factor=UL usable PRB sizeUL BWP size.

- Note: Other alternatives are not precluded

While Option 1 with Alt 1-1 supports the use of single RACH configuration based on existing RACH configuration parameters such as msg1-FrequencyStart and msg1-FDM etc., Option 2 will support additional RACH configuration in addition to the legacy RACH configuration.

If Alternative 1 in Option 1 with Alt 1-1 is selected, as shown in Figure 1, configuring single RACH configuration for both SBFD and non-SBFD UEs for initial access could lead to invalid ROs (those ROs that fall outside of the SBFD UL symbol) due to msg1-FrequencyStart offset configured for legacy RACH without considering the SBFD-aware UEs whose the FDMed ROs may overlap with DL symbol, thereby causing invalid ROs. Therefore, if using Option 1, existing RACH configuration parameters (msg1-FrequencyStart and msg1-FDM) must be configured to prevent invalid ROs.

UL

SSB2

RO2

SSB1

RO1

RO4

SSB4

RO3

SSB3

UL Initial BWP

X

UL

DL

DL

DL

X

DL

DL

time

frequency

UL

Invalid ROs for RACH in SBFD symbol.

RACH resource offset indicated by

msg1-FrequencyStart

for both SBFD and non-SBFD legacy RACH. The ROs may be invalid for SBFD UEs.

Figure 1. Illustration of RACH configuration signaling with Option 1 Alt 1

Although Figure 1 illustrates invalid ROs scenario, it is just an example and other alternatives may also lead to invalid ROs. If the existing RACH parameters must be used for single RACH configuration, invalid ROs are inevitable. Therefore, provide a fallback mechanism for SBFD UEs to use non-SBFD RACH resource when RACH fails on SBFD UL symbols.

Observation 1: Some of the configuration alternatives for Option 1 Alt 1-1 may lead to invalid ROs for SBFD UEs.

Proposal 1: RAN2 to discuss how to handle invalid ROs case when using the single RACH configuration alternatives in Option 1 Alt 1-1.

# Indicating RACH Configuration for SBFD

In RAN2#127, RAN2 agreed that SBFD RACH configuration can be indicated to the UEs via SIB1 and the detailed signaling is FFS [2]. For Option 1 Alt 1-1 configuration, existing RACH configuration parameters could be used with no reinterpretation to ndicate RACH frequency resource offset in SBFD slot. The benefit of reusing these existing parameters is less signaling overhead and lower impact on specification.

Proposal 2: For Option 1 Alt 1-1, RAN2 to use legacy RACH configuration parameters (e.g., msg1-FrequencyStart and msg1-FDM).

For some of the alternatives in Option 1, SBFD-aware UEs may find invalid ROs in SBFD symbols. In this case SBFD-aware UE is allowed to use the ROs in non-SBFD symbols for the random access.

Proposal 3: For Option 1 Alt 1-1, RAN2 to discuss whether to use ROs in non-SBFD symbols if there is no valid ROs in SBFD symbols.

In the case single RACH configuration for Option 1 Alt 1-1 does not confine the valid ROs to the UL usable PRBs, possible solution is to limit the value of msg1-FDM to keep the frequency offset of the lowest ROs within the usable UL SBFD PRBs. This could have limited impact on specification.

Proposal 4: For Option 1 Alt 1-1, RAN2 to limit the number of PRACH transmission occassions FDMed to confine ROs within UL SBFD usable PRBs.

For Alternative 2-4 in Option 1 Alt 1-1, reduce the overhead on the UE-side by allowing the network (NW) to configure the scaling factor but it is upto RAN2 to decide if UE implementation should calculate the scaling factor to determine the frequency offset of the lowest RO. Alternatively, a new parameter (such as ra-RO-ScalingFactorSBFD-r19) maybe introduced by RAN2 for the NW to signal the value of the scaling factor along with other parameters in the RACH configuration such as the rach-ConfigGeneric of RACH-ConfigCommon. This is a possible option since the NW knows the UL usable PRB and initial UL BWP sizes a priori.

Proposal 5: For Alternative 2-4 in Option 1 Alt 1-1, RAN2 to discuss if calculation of the scaling factor is upto UE implementation.

Proposal 6: RAN2 to discuss introducing new single RACH configuration parameter for configuring the scaling factor by the network.

# Conclusions

As a result of the discussion in this contribution, we have made the following observation and proposals the following:

Observation 1: Some of the configuration alternatives for Option 1 Alt 1-1 may lead to invalid ROs for SBFD UEs.

Proposal 1: RAN2 to discuss how to handle invalid ROs case when using the single RACH configuration alternatives in Option 1 Alt 1-1.

Proposal 2: For Option 1 Alt 1-1, RAN2 to use legacy RACH configuration parameters (e.g., msg1-FrequencyStart and msg1-FDM).

Proposal 3: For Option 1 Alt 1-1, RAN2 to discuss whether to use ROs in non-SBFD symbols if there is no valid ROs in SBFD symbols.

Proposal 4: For Option 1 Alt 1-1, RAN2 to limit the number of PRACH transmission occassions FDMed to confine ROs within UL SBFD usable PRBs.

Proposal 5: For Alternative 2-4 in Option 1 Alt 1-1, RAN2 to discuss if calculation of the scaling factor is upto UE implementation.

Proposal 6: RAN2 to discuss introducing new single RACH configuration parameter for configuring the scaling factor by the network.

# References

- RP‑241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- RP-242034, “Evolution of NR duplex operation: Sub-band full duplex (SBFD),” Status report, Huawei, Samsung