3GPP TSG-RAN WG2 Meeting #127bis	R2-2408514

# Hefei, China, Oct 14th– 18th, 2024

Source:	ZTE Corporation

Title:	Clarification on NR PDCP related capability

Agenda item:	5.1.3.2

Document for:	Discussion & Decision

# Introduction

For LTE PDCP, only 18bit SN length is supported, but for NR PDCP, the UE mandatorily supports 18bit SN length, in addition, the UE can also report whether it supports 12bit short SN. For LTE connects to 5GC (i.e. eLTE), the UE can also be configured with NR PDCP and corresponding functionality can be applied (e.g. shortSN).

In current LTE/NR specs, separate UE capabilities are defined for indicating the support of NR PDCP shorts to LTE eNB and NR gNB. However, we observed some inter-operability issue during IoT test, in this contribution, we discuss the observed issue and ask for clarification.

# Discussion

For NR UE, the shortSN capability is reported via UE-NR-Capability->PDCP-Parameters->shortSN, its definition in TS 38.306 is given below. Because this capability is defined within NR capability container, it is transparent to (e)LTE eNBs.

#38.306

| shortSN Indicates whether the UE supports 12 bit length of PDCP sequence number. | UE | Yes | No |
| --- | --- | --- | --- |

For eLTE UE, a separate UE capability is defined: UE-EUTRA-Capability-v1510-IEs->PDCP-ParametersNR-r15->sn-SizeLo-r15, its definition is the same as shortSN defined in TS 38.306. Because this capability is defined within LTE capability container, it is transparent to NR gNBs.

#36.306

### 4.3.1A NR PDCP Parameters NR PDCP capabilities: the definition of rohc-Profiles-r15, rohc-ContextMaxSessions-r15, rohc-ProfilesUL-Only-r15, rohc-ContextContinue-r15, outOfOrderDelivery-r15 and sn-SizeLo-r15 are the same as supportedROHC-Profiles, maxNumberROHC-ContextSessions, uplinkOnlyROHC-Profiles, continueROHC-Context, outOfOrderDelivery and shortSN defined in TS 38.306 [32]. ims-VoiceOverNR-PDCP-MCG-Bearer-r15 indicates whether the UE supports IMS voice over NR PDCP with only MCG RLC bearer. ims-VoiceOverNR-PDCP-SCG-Bearer-r15 indicates whether the UE supports IMS voice over NR PDCP with only SCG RLC bearer when configured with EN-DC. ims-VoNR-PDCP-SCG-NGENDC-r15 indicates whether the UE supports IMS voice over NR PDCP with only SCG RLC bearer when configured with NGEN-DC. NOTE: In this release of specification, IMS voice over split bearer is not supported for (NG)EN-DC.

Observation 1	For NR PDCP shortSN, separate UE capabilies are defined, one is for eLTE UE, the other is for NR UE, the two capabilies are defined in TS 36.331 and TS 38.331 separately.

In our IoT test, we observed that for a given UE, the UE reports different values for the two UE capabilies. (e.g. the UE indicates “support” for sn-SizeLo-r15 in EUTRA capability, but indicates “not support” for shortSN in NR capability). We understand this means that the UE does not support shortSN when the UE is in NR SA or NR-NR DC. However, it is still unclear whether short SN can be configured in following cases, and RRC reestablishment happened due to the potential mismatch between the UE and the network.

- Case 1: LTE UE connects to 5GC and the UE is configured with NR PDCP;

- Case 2: UE is configured with (NG)EN-DC:

  - Case 2a: MN terminated MCG bearer or MN terminated SCG bearer;

  - Case 2b: SN terminated SCG bearer or SN terminated SCG bearer;

- Case 3: UE is configured with NE-DC:

  - Case 3a: MN terminated MCG bearer or MN terminated SCG bearer;

  - Case 3b: SN terminated SCG bearer or SN terminated SCG bearer

For Case 1, since the UE is only connected to 4G eNB, the network can only decode the LTE capability container, so it should be straightforward that the sn-SizeLo-r15 capability should be checked.

Proposal 1	RAN2 confirm when SA UE connects to LTE+5GC and configured with NR PDCP, the network checks LTE capability (i.e. sn-SizeLo-r15) to determine whether it can configure NR PDCP short SN to the UE.

For Case 2 and Case 3, based on TS37.340, in EN-DC, MN can decide whether to use LTE PDCP or NR PDCP for MN terminated MCG bearer, for all the other bearer types, only NR PDCP can be configured. In case of MR-DC with 5GC(e.g. NGEN-DC, NE-DC), NR PDCP is always used for all bearer types.

According to the signalling design in TS38.331, PDCP SN length configuration is defined in PDCP-Config which is included in radioBearerConfig.

PDCP-Config ::=         SEQUENCE {

drb                     SEQUENCE {

discardTimer            ENUMERATED {ms10, ms20, ms30, ms40, ms50, ms60, ms75, ms100, ms150,    ms200, ms250, ms300, ms500, ms750, ms1500, infinity}       OPTIONAL, -- Cond Setup

pdcp-SN-SizeUL          ENUMERATED {len12bits, len18bits}            OPTIONAL, -- Cond Setup1

pdcp-SN-SizeDL          ENUMERATED {len12bits, len18bits}            OPTIONAL, -- Cond Setup2

***ignore non-related part***

}

For MR-DC, the MN is responsbile for generating the radioBearerConfig for MN terminated MCG/split/SCG bearers, the SN is responsible for generating the radioBearerConfig for SN terminated SCG/split/MCG bearers. So, MN and SN need to check the capability individually. More specifically

- For Case 2a, MN(LTE) checks LTE capability (i.e. sn-SizeLo-r15);

- For Case 2b, SN(NR) checks NR capability (i.e. shortSN);

- For Case 3a, MN(NR) checks NR capability (i.e. shortSN);

- For Case 3b, SN(LTE) checks LTE capability (i.e. sn-SizeLo-r15)

To avoid inter-operability issue, we repectfully ask RAN2 to confirm above understanding.

Proposal 2	For UEs in (NG)EN-DC, RAN2 confirm the MN(LTE) checks LTE capability (i.e. sn-SizeLo-r15) to determine whether it can configure NR PDCP short SN for MN terminated MCG/split/SCG bearers to the UE; the SN(NR) checks NR capability (i.e. shortSN) to determine whether it can configure NR PDCP short SN for SN terminated MCG/split/SCG bearers to the UE.

Proposal 3	For UEs in NE-DC, RAN2 confirm the MN(NR) checks NR capability (i.e. shortSN) to determine whether it can configure NR PDCP short SN for MN terminated MCG/split/SCG bearers to the UE; the SN(LTE) checks LTE capability (i.e. sn-SizeLo-r15) to determine whether it can configure NR PDCP short SN for SN terminated MCG/split/SCG bearers to the UE.

If P1~P3 are agreeable, we think it is necessary to clarify it in specification, the current definition of UE capabilities in TS 36.306 and TS 38.306 are too vague, that would be the reason that UE and network have different implementations.

Regarding the spec change, we notice that in addition to short SN capability, several NR PDCP related capabilities are defined and reported to LTE, we think the same principle also applies to other capabilities. More specifically, the concerned capability are highlighted in yellow below:

(Note: IMS capabilities are not involved, because the applicable scenario is already captured in spec)

#36.331---NR PDCP capabilities

PDCP-ParametersNR-r15 ::=		SEQUENCE {

rohc-Profiles-r15					ROHC-ProfileSupportList-r15,

rohc-ContextMaxSessions-r15			ENUMERATED {

cs2, cs4, cs8, cs12, cs16, cs24, cs32,

cs48, cs64, cs128, cs256, cs512, cs1024,

cs16384, spare2, spare1}			DEFAULT cs16,

rohc-ProfilesUL-Only-r15				SEQUENCE {

profile0x0006-r15						BOOLEAN

},

rohc-ContextContinue-r15			ENUMERATED {supported}				OPTIONAL,

outOfOrderDelivery-r15				ENUMERATED {supported}				OPTIONAL,

sn-SizeLo-r15						ENUMERATED {supported}				OPTIONAL,

ims-VoiceOverNR-PDCP-MCG-Bearer-r15	ENUMERATED {supported}				OPTIONAL,

ims-VoiceOverNR-PDCP-SCG-Bearer-r15	ENUMERATED {supported}				OPTIONAL

}

Consider that in TS 36.306, the definition of NR PDCP capabilities is referring to the definition of corresponding capabilities in TS 38.306, so, we think clarification can be made in TS 38.306, we have provided the CRs for TS 38.306 in [1].

Proposal 4	RAN2 confirm that the same principle also applies to other NR PDCP capability that reported to LTE (i.e. rohc-Profiles-r15, rohc-ContextMaxSessions-r15, rohc-ProfilesUL-Only-r15, rohc-ContextContinue-r15, outOfOrderDelivery-r15), and agree the CRs in [1].

# Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following observations and proposals:

Observation 1	For NR PDCP shortSN, separate UE capabilies are defined, one is for eLTE UE, the other is for NR UE, the two capabilies are defined in TS 36.331 and TS 38.331 separately.

Proposal 1	RAN2 confirm when SA UE connects to LTE+5GC and configured with NR PDCP, the network checks LTE capability (i.e. sn-SizeLo-r15) to determine whether it can configure NR PDCP short SN to the UE.

Proposal 2	For UEs in (NG)EN-DC, RAN2 confirm the MN(LTE) checks LTE capability (i.e. sn-SizeLo-r15) to determine whether it can configure NR PDCP short SN for MN terminated MCG/split/SCG bearers to the UE; the SN(NR) checks NR capability (i.e. shortSN) to determine whether it can configure NR PDCP short SN for SN terminated MCG/split/SCG bearers to the UE.

Proposal 3	For UEs in NE-DC, RAN2 confirm the MN(NR) checks NR capability (i.e. shortSN) to determine whether it can configure NR PDCP short SN for MN terminated MCG/split/SCG bearers to the UE; the SN(LTE) checks LTE capability (i.e. sn-SizeLo-r15) to determine whether it can configure NR PDCP short SN for SN terminated MCG/split/SCG bearers to the UE.

Proposal 4	RAN2 confirm that the same principle also applies to other NR PDCP capability that reported to LTE (i.e. rohc-Profiles-r15, rohc-ContextMaxSessions-r15, rohc-ProfilesUL-Only-r15, rohc-ContextContinue-r15, outOfOrderDelivery-r15), and agree the CRs in [1].

# Reference

[1] R2-2408515~R2-2408518 Correction on NR PDCP capabilities    ZTE Corporation,  CR  TS 38.306;