3GPP TSG-RAN WG2 Meeting #127bis	R2-2408515

# Hefei, China, Oct 14 – 18, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1178 | rev | - | Current version: | 15.26.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NR PDCP capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2024-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For LTE UE connects to 5GC, the UE can be configured with NR PDCP. In addition, NR PDCP can be configured in case of MR-DC. Based on the current specifications, the NR PDCP related UE capabilities (e.g. sn-SizeLo-r15) are defined separately in LTE capability and NR capability, therefore, the UE is allowed to set the capabilites into different values to LTE and NR. For example, in LTE capabilities, the UE can indicate it supports short SN (12 bits) for NR PDCP, but in NR capabilities, the UE can indicate it does not support short SN (12 bits) for NR PDCP. Currently, the definitions of NR PDCP capabilities in TS 36.306 and TS 38.306 are too vague, it is unclear whether and which capabilties cover the below cases, thus IoT problem occurs. - Case 1: LTE UE connects to 5GC and the UE is configured with NR PDCP; - Case 2: UE is configured with (NG)EN-DC: - Case 2a: MN terminated MCG bearer or MN terminated SCG bearer; - Case 2b: SN terminated SCG bearer or SN terminated SCG bearer; - Case 3: UE is configured with NE-DC: - Case 3a: MN terminated MCG bearer or MN terminated SCG bearer; - Case 3b: SN terminated SCG bearer or SN terminated SCG bearer To solve the IoT problem, the CR is provided to clarify the applicable scenarios for NR PDCP capabilities that reported to LTE and NR. |  |  |  |  |  |  |  |  |
|  |  | “ |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify in section 4.2.4 that the NR PDCP capabilies (i.e. supportedROHC-Profiles, maxNumberROHC-ContextSessions, uplinkOnlyROHC-Profiles, continueROHC-Context, outOfOrderDelivery, shortSN) defined in TS 38.331 are applicable to NR SA, NR-DC, SN terminated bearer in (NG)EN-DC and MN terminated bearer in NE-DC; - Clarify in section 4.2.4 that the NR PDCP capabilities (i.e. rohc-Profiles-r15, rohc-ContextMaxSessions-r15, rohc-ProfilesUL-Only-r15, rohc-ContextContinue-r15, outOfOrderDelivery-r15, sn-SizeLo-r15) defined in TS 36.331 are applicable to eLTE SA UE, MN terminated bearer in (NG)EN-DC and SN terminated bearer in NE-DC. Impact analysis Impacted 5G architecture options: LTE connects to 5GC, NR SA, NR-DC, (NG)EN-DC, NE-DC Impacted functionality: NR PDCP configuration Inter-operability: - If the network is implemented according to the CR and the UE is not, or If the UE is implemented according to the CR and the network is not, the UE may assume the NR PDCP configuration is wrong and trigger RRC establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - In case the UE reports different value for NR PDCP capability in LTE capability and NR capability, it is unclear whether and which NR PDCP capabilites should be checked when configuring radio bearer with NR PDCP. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

4.2	UE Capability Parameters

***ignore non-related part***

4.2.4	PDCP Parameters

The capabilities supportedROHC-Profiles, maxNumberROHC-ContextSessions, uplinkOnlyROHC-Profiles, continueROHC-Context, outOfOrderDelivery and shortSN specified in this specification are applicable to NR SA, NR-DC and the following cases:

-	SN terminated SCG/split/MCG bearer when configured with (NG)EN-DC;

-	MN terminated MCG/split/SCG bearer when configured with NE-DC.

The corresponding NR PDCP capabilities rohc-Profiles-r15, rohc-ContextMaxSessions-r15, rohc-ProfilesUL-Only-r15, rohc-ContextContinue-r15, outOfOrderDelivery-r15 and sn-SizeLo-r15 specified in TS 36.331 [17] are applicable to the following cases:

-	Radio bearers with NR PDCP when MR-DC is not configured;

-	MN terminated MCG/split/SCG bearer when configured with (NG)EN-DC;

-	SN terminated SCG/split/MCG bearer when configured with NE-DC.

| Definitions for parameters | Per | M | FDD-TDD DIFF |
| --- | --- | --- | --- |
| continueROHC-Context Defines whether the UE supports ROHC context continuation operation where the UE does not reset the current ROHC context upon PDCP re-establishment, as specified in TS 38.323 [16]. | UE | No | No |
| maxNumberROHC-ContextSessions Defines the maximum number of header compression context sessions supported by the UE, excluding context sessions that leave all headers uncompressed. | UE | No | No |
| outOfOrderDelivery Indicates whether UE supports out of order delivery of data to upper layers by PDCP. | UE | No | No |
| pdcp-DuplicationMCG-OrSCG-DRB Indicates whether the UE supports CA-based PDCP duplication over MCG or SCG DRB as specified in TS 38.323 [16]. | UE | No | No |
| pdcp-DuplicationSplitDRB Indicates whether the UE supports PDCP duplication over split DRB as specified in TS 38.323 [16]. | UE | No | No |
| pdcp-DuplicationSplitSRB Indicates whether the UE supports PDCP duplication over split SRB1/2 as specified in TS 38.323 [16]. | UE | No | No |
| pdcp-DuplicationSRB Indicates whether the UE supports CA-based PDCP duplication over SRB1/2 and/or, if (NG)EN-DC is supported, SRB3 as specified in TS 38.323 [16]. | UE | No | No |
| shortSN Indicates whether the UE supports 12 bit length of PDCP sequence number. | UE | Yes | No |
| supportedROHC-Profiles Defines which ROHC profiles from the list below are supported by the UE: - 0x0000 ROHC No compression (RFC 5795) - 0x0001 ROHC RTP/UDP/IP (RFC 3095, RFC 4815) - 0x0002 ROHC UDP/IP (RFC 3095, RFC 4815) - 0x0003 ROHC ESP/IP (RFC 3095, RFC 4815) - 0x0004 ROHC IP (RFC 3843, RFC 4815) - 0x0006 ROHC TCP/IP (RFC 6846) - 0x0101 ROHC RTP/UDP/IP (RFC 5225) - 0x0102 ROHC UDP/IP (RFC 5225) - 0x0103 ROHC ESP/IP (RFC 5225) - 0x0104 ROHC IP (RFC 5225) A UE that supports one or more of the listed ROHC profiles shall support ROHC profile 0x0000 ROHC uncompressed (RFC 5795). An IMS voice capable UE shall indicate support of ROHC profiles 0x0000, 0x0001, 0x0002 and be able to compress and decompress headers of PDCP SDUs at a PDCP SDU rate corresponding to supported IMS voice codecs. | UE | No | No |
| uplinkOnlyROHC-Profiles Indicates the ROHC profile(s) that are supported in uplink-only ROHC operation by the UE. - 0x0006 ROHC TCP (RFC 6846) A UE that supports uplink-only ROHC profile(s) shall support ROHC profile 0x0000 ROHC uncompressed (RFC 5795). | UE | No | No |

End of change