3GPP TSG-RAN WG2 Meeting #127bis                        R2-2408519

Hefei, China, Oct 14th- 18th, 2024

Source: 				ZTE Corporation

Title:            Clarifiction on band selection procedure

Agenda item:		7.0.2.11

Document for: 	Discussion and decision

- Introduction

In this contribution, we discussed the UE band selection procedure in TS 38.331.

- Discussion

Similar to LTE, in NR, one NR frequency can belong to more than one frequency bands, in case an NR cell operates on a frequency that belongs to more than one frequency bands, the network can broadcast a list of bands in system information of the cell, which is called MFBI (multiple frequency band indicator) cell.

When a UE camps on a MFBI cell and decides to trigger RRC connection establishment, the UE will select one of listed band that supported by the UE. The rule of band selection is specified in TS 38.331. From network perspective, the network can also determine which band has been selected by the UE according to the same rule after receiving UE’s radio capability (e.g. UE’s supported band list). To avoid inter-operability issue, the network and the UE should have the same understanding on which band will be selected, otherwise, problems may occur in the follow-up operations (e.g. CA/DC configuration). For example, the network indicates band 1 and band 2 in multiBandInfoList, the network assumes that UE has selected band 1, but the UE actually selects band 2. From network perspective, when network wants to configure CA or DC, the network will check the UE capability to find suitable band combination which contains band 1, e.g. BC {band 1+band 3}. But when network reconfigures the UE, the reconfiguration failure may happen if the UE does not support BC {band 2+band 3}. On the other hand, upon RRM measurement configuration, the network determines gap requirements based on the target measured band and current operating band or band combination, if the network and UE have different understanding on which band(s) are currently used, it is possible the gap configuration does not match the need, e.g. the network may not provide gap but the UE assumes gap is needed to perform the measurements.

Observation 1: When UE access an MBFI cell, the network and the UE should have the same understanding on which band will be selected by the UE, otherwise, RRC configuration failure may occur, e.g. when configuring CA/DC band combination.

In LTE spec, the multiBandInfoList was introduced as the extension of frequencyBandIndicator IE. In the field description, it is clearly stated that the first supported band should be selected by the UE (assuming frequencyBandIndicatorPriority is not configured).

multiBandInfoList A list of additional frequency band indicators, as defined in TS 36.101 [42], table 5.5-1 and TS 36.102 [113], table 5.2-1, for NTN capable UE that the cell belongs to. If the UE supports the frequency band in the freqBandIndicator field it shall apply that frequency band. Otherwise, the UE shall apply the first listed band which it supports in the multiBandInfoList field. If E-UTRAN includes multiBandInfoList-v9e0 it includes the same number of entries, and listed in the same order, as in multiBandInfoList (i.e. without suffix). See Annex D for more descriptions. The UE shall ignore the rule defined in this field description if freqBandIndicatorPriority is present and supported by the UE.

However, in NR spec, the similar statement only exists in the field description for UL, not for DL.

FrequencyInfoDL-SIB field descriptions

frequencyBandList List of one or multiple frequency bands to which this carrier(s) belongs. If frequencyBandList-v1760 is present, it shall contain the same number of entries, listed in the same order as in frequencyBandList (without suffix).

FrequencyInfoUL-SIB field descriptions

frequencyBandList Provides the frequency band indicator and a list of additionalPmax and additionalSpectrumEmission values as defined in TS 38.101-1 [15], table 6.2.3.1-1, TS 38.101-2 [39], table 6.2.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1. The UE shall apply the first listed band which it supports in the frequencyBandList field. If frequencyBandList-v1760 is present, it shall contain the same number of entries, listed in the same order as in frequencyBandList (without suffix).

frequencyBandListAerial Provides the frequency band indicator and a list of additionalPmax and additionalSpectrumEmission values for aerial UE as defined in TS 38.101-1 [15], clause 6.2K. If the field is present, the aerial UE shall apply the first listed band which it supports in the frequencyBandListAerial field. If the field is absent, frequencyBandList applies.

Observation 2: In LTE spec, the field description is clear that the first supported band should be selected by the UE, but such statement does not exist in the field description of NR DL frequencyBandList IE.

Actually, in NR, after introducing different device types and cell barring mechanisms, the selection of band becomes more complex and confusing, for example, an NR cell can indicate whether separate cell barring indicator for 1Rx and 2Rx RedCap UEs. But the 1Rx or 2Rx capability is per-band defined (not per UE). Similarly, an NR cell can also indicate whether 2Rx XR UE is allowed or not, and the Rx capability for XR capable UE is also per-band level defined.

Despite above field description, in current TS 38.331, the band seletion rule for DL and UL is specified as below:

TS 38.331 section 5.2.2.4.2: 5> select the first frequency band in the frequencyBandList, for FDD from frequencyBandList for uplink, or for TDD from frequencyBandList for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present, and for (e)RedCap UEs in FDD, if the halfDuplexRedCapAllowed is not present, for which the UE supports full-duplex FDD operation;

We can see that UE will select the first band from the broadcasted band list for which it supports the emission requirements. The only exception is for (e)RedCap UE, for which the RedCap UE can further check its HD-FDD capability for each band, if the cell does not support HD-FDD (i.e. halfDuplexRedCapAllowed is not present), then the UE can select the first band which the UE supports FD-FDD operation. This new mechanism was added based on agreed RAN2 CR [1].

Observation 3: Based on the text procedure in NR RRC spec, the UE should select the first band that UE supports the emission requirements, the only exception is for (e)RedCap UE, where the UE can further check its FD-FDD capability.

However, such exception does not apply to other cases, such as Rx number and supported channel bandwidth values. For instance, the NR cell broadcasts band 1 and band 2 in SIB1, a RedCap UE only supports 1 Rx operation for band 1, but it supports 2Rx operation for band 2, the cell indicates that the cell is barred for 1Rx UEs, then it is unclear whether the UE should treat the cell as barred or the UE can select band 2 for access. because the text procedure in section 5.5.4.2 is confusing. The detailed problems are shown below:

(Note: the text procedure for band selection is highlighted in yellow, it is executed after the main procedures)

##### #TS 38.331 ##### 5.2.2.4.2 Actions upon reception of the SIB1 Upon receiving the SIB1 the UE shall: 1> store the acquired SIB1; ***skip non-related part*** 1> if the UE is a RedCap UE and it is in RRC_IDLE or in RRC_INACTIVE, or if the RedCap UE is in RRC_CONNECTED while T311 is running: 2> if intraFreqReselectionRedCap is not present in SIB1: 3> consider the cell as barred in accordance with TS 38.304 [20]; 3> perform barring as if intraFreqReselectionRedCap is set to allowed, upon which the procedure ends; 2> else: 3> if the cellBarredRedCap1Rx is present in the acquired SIB1 and is set to barred and the UE supports 1 Rx branch; or 3> if the cellBarredRedCap2Rx is present in the acquired SIB1 and is set to barred and the UE supports 2 Rx branches; or 3> if the halfDuplexRedCapAllowed is not present in the acquired SIB1 and the UE supports only half-duplex FDD operation: 4> consider the cell as barred in accordance with TS 38.304 [20]; 4> perform barring based on intraFreqReselectionRedCap as specified in TS 38.304 [20], upon which the procedure ends; 1> if the UE is a 2Rx XR UE and is in RRC_IDLE or in RRC_INACTIVE, or if the 2Rx XR UE is in RRC_CONNECTED while T311 is running: 2> if the cellBarred2RxXR is present in the acquired SIB1: 3> consider the cell as barred in accordance with TS 38.304 [20]; 3> if the intraFreqReselection2RxXR is present in the acquired SIB1: 4> perform barring based on intraFreqReselection2RxXR as specified in TS 38.304 [20] upon which the procedure ends; 3> else: 4> perform barring as if intraFreqReselection2RxXR is set to allowed upon which the procedure ends; 1> if the UE supports nes-CellDTX-DRX and it is in RRC_IDLE or in RRC_INACTIVE, or if the UE supporting nes-CellDTX-DRX is in RRC_CONNECTED while T311 is running: 2> if cellBarred in the acquired MIB is set to barred: 3> if cellBarredNES is absent in the acquired SIB1: 4> consider the cell as barred in accordance with TS 38.304 [20]; 4> perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends; 1> if the UE is an eRedCap UE and it is in RRC_IDLE or in RRC_INACTIVE, or if the eRedCap UE is in RRC_CONNECTED while T311 is running: 2> if intraFreqReselection-eRedCap is not present in SIB1: 3> consider the cell as barred in accordance with TS 38.304 [20]; 3> perform barring as if intraFreqReselection-eRedCap is set to allowed upon which the procedure ends; 2> else: 3> if the cellBarred-eRedCap1Rx is present in the acquired SIB1 and is set to barred and the UE supports 1 Rx branch; or 3> if the cellBarred-eRedCap2Rx is present in the acquired SIB1 and is set to barred and the UE supports 2 Rx branches; or 3> if the halfDuplexRedCapAllowed is not present in the acquired SIB1 and the UE supports only half-duplex FDD operation: 4> consider the cell as barred in accordance with TS 38.304 [20]; 4> perform barring based on intraFreqReselection-eRedCap as specified in TS 38.304 [20] upon which the procedure ends; 1> if the cellAccessRelatedInfo contains an entry of a selected SNPN or PLMN and in case of PLMN the UE is either allowed or instructed to access the PLMN via a cell for which at least one CAG ID is broadcast: 2> in the remainder of the procedures use npn-IdentityList, trackingAreaCode, and cellIdentity for the cell as received in the corresponding entry of npn-IdentityInfoList containing the selected PLMN or SNPN; 1> else if the cellAccessRelatedInfo contains an entry with the PLMN-Identity of the selected PLMN: 2> in the remainder of the procedures use plmn-IdentityList, trackingAreaCode, trackingAreaList, and cellIdentity for the cell as received in the corresponding PLMN-IdentityInfo containing the selected PLMN; 1> if the UE in RRC_INACTIVE is configured for feature(s) that it does not support in current serving cell: 2> not use the corresponding configuration in current serving cell; NOTE 0: The requirement above applies only to UE that indicates different support of UE capabilities for TN and NTN. 1> if in RRC_CONNECTED while T311 is not running: 2> disregard the frequencyBandList, if received, while in RRC_CONNECTED; 2> forward the cellIdentity to upper layers; 2> forward the trackingAreaCode to upper layers, if included; 2> forward the trackingAreaList to upper layers, if included; 2> forward the received posSIB-MappingInfo to upper layers, if included; 2> apply the configuration included in the servingCellConfigCommon; 2> if the UE has a stored valid version of a SIB or posSIB, in accordance with clause 5.2.2.2.1, that the UE requires to operate within the cell in accordance with clause 5.2.2.1: 3> use the stored version of the required SIB or posSIB; 2> else: 3> acquire the required SIB or posSIB requested by upper layer as defined in clause 5.2.2.3.5; NOTE 1: Void. 1> else: 2> if the UE supports one or more of the frequency bands indicated in the frequencyBandList or frequencyBandListAerial for downlink for TDD, or one or more of the frequency bands indicated in the frequencyBandList or frequencyBandListAerial for uplink for FDD, and they are not downlink only bands, and 2> if the UE is IAB-MT or wide area NCR-MT (see TS 38.106 [79]) or supports at least one additionalSpectrumEmission in the nr-NS-PmaxList or nr-NS-PmaxListAerial for a supported band in the downlink for TDD, or a supported band in uplink for FDD, and 2> if the UE supports an uplink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15], TS 38.101-2 [39], and TS 38.101-5 [75]) which - is smaller than or equal to the carrierBandwidth (indicated in uplinkConfigCommon for the SCS of the initial uplink BWP or, for (e)RedCap UE, of the RedCap-specific initial uplink BWP if configured), and which - is wider than or equal to the bandwidth of the initial uplink BWP or, for (e)RedCap UE, of the RedCap-specific initial uplink BWP if configured, and 2> if the UE supports a downlink channel bandwidth with a maximum transmission bandwidth configuration (see TS 38.101-1 [15], TS 38.101-2 [39], and TS 38.101-5 [75]) which - is smaller than or equal to the carrierBandwidth (indicated in downlinkConfigCommon for the SCS of the initial downlink BWP or, for (e)RedCap UE, of the RedCap-specific initial downlink BWP if configured), and which - is wider than or equal to the bandwidth of the initial downlink BWP or, for (e)RedCap UE, of the RedCap-specific initial downlink BWP if configured, and 2> if frequencyShift7p5khz is present and the UE supports corresponding 7.5kHz frequency shift on this band; or frequencyShift7p5khz is not present, and 2> if the UE is neither a RedCap nor an eRedCap UE, or for TDD if the UE is an (e)RedCap UE, or for FDD if the UE is an (e)RedCap UE and halfDuplexRedCapAllowed is present, or if the UE is an (e)RedCap UE and the (e)RedCap UE supports full-duplex FDD operation on this band: 3> if neither trackingAreaCode nor trackingAreaList is provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list: 4> consider the cell as barred in accordance with TS 38.304 [20]; 4> perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20]; 3> else if UE is IAB-MT but not a mobile IAB-MT and if iab-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list nor the selected SNPN nor the registered SNPN nor SNPN of the equivalent SNPN list: 4> consider the cell as barred in accordance with TS 38.304 [20]; 3> else if UE is NCR-MT and if ncr-Support is not provided: 4> consider the cell as barred in accordance with TS 38.304 [20]; 3> else if UE is a mobile IAB-MT and if mobileIAB-Support is not provided for the selected PLMN nor the registered PLMN nor PLMN of the equivalent PLMN list nor the selected SNPN nor the registered SNPN nor SNPN of the equivalent SNPN list: 4> consider the cell as barred in accordance with TS 38.304 [20]; 3> else: 4> apply a supported uplink channel bandwidth with a maximum transmission bandwidth which - is contained within the carrierBandwidth indicated in uplinkConfigCommon for the SCS of the initial uplink BWP or, for (e)RedCap UEs, RedCap-specific initial uplink BWP, if configured, and which - is wider than or equal to the bandwidth of the initial BWP for the uplink or, for a (e)RedCap UE, of the RedCap-specific initial uplink BWP if configured; 4> apply a supported downlink channel bandwidth with a maximum transmission bandwidth which - is contained within the carrierBandwidth indicated in downlinkConfigCommon for the SCS of the initial downlink BWP or, for (e)RedCap UEs, RedCap-specific initial downlink BWP, if configured, and which - is wider than or equal to the bandwidth of the initial BWP for the downlink or, for a (e)RedCap UE, of the RedCap-specific initial downlink BWP if configured; 4> if the UE is aerial UE and it supports at least one frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, for which SIB1 includes nr-NS-PmaxListAerial and the UE supports at least one of the additionalSpectrumEmission values in the nr-NS-PmaxListAerial: 5> select the first frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, which the UE supports and for which SIB1 includes nr-NS-PmaxListAerial and the UE supports at least one of the additionalSpectrumEmission values in the nr-NS-PmaxListAerial; 4> else if the UE is aerial UE and it supports at least one frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, for which SIB1 does not include nr-NS-PmaxListAerial and the UE supports at least one of the additionalSpectrumEmission values in the nr-NS-PmaxList within frequencyBandList for the same NR frequency band number: 5> select the first frequency band in the frequencyBandListAerial, for FDD from frequencyBandListAerial for uplink, or for TDD from frequencyBandListAerial for downlink, which the UE supports and for which SIB1 does not include nr-NS-PmaxListAerial and the UE supports at least one of the additionalSpectrumEmission values in the nr-NS-PmaxList within frequencyBandList for the same NR frequency band number; 4> else: 5> select the first frequency band in the frequencyBandList, for FDD from frequencyBandList for uplink, or for TDD from frequencyBandList for downlink, which the UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present, and for (e)RedCap UEs in FDD, if the halfDuplexRedCapAllowed is not present, for which the UE supports full-duplex FDD operation; 4> if the UE is aerial UE and SIB1 includes nr-NS-PmaxListAerial for the selected frequency band within frequencyBandListAerial in uplinkConfigCommon for FDD or in downlinkConfigCommon for TDD but the UE capable of nr-NS-PmaxListAerial does not support any of the additionalSpectrumEmission values in the nr-NS-PmaxListAerial for the selected frequency band: 5> consider the cell as barred in accordance with TS 38.304 [20]; 5> perform barring as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, or intraFreqReselection-eRedCap for eRedCap UEs, or intraFreqReselection2RxXR for 2Rx XR UEs is set to notAllowed, upon which the procedure ends; 4> forward the cellIdentity to upper layers; 4> forward the trackingAreaCode to upper layers; 4> forward the trackingAreaList to upper layers, if included; ***ignore non-related part***

As we can see, most barring check operations are related to the selected band, but in text procedure, they are executed before band selection. So, it is unclear what’s the exact UE behaviour. In summary, the concern mainly involves:

- Case 1: 1Rx, 2Rx barring bit check for RedCap and eRedCap UEs

- Case 2: 2Rx XR barring bit check for XR UEs

- Case 3: Supported channel bandwidth checking

- Case 4: 7.5kHz freq shift checking

For Case 1, RAN2 discussed the issue in RAN2#126 and made below agreements:

For R18: Barring determination is done based on the 1Rx/2Rx support on the selected band instead of whether the UE is equipped with 1Rx branch or 2Rx branches. For R18: eRedCap UE supporting both 1Rx and 2Rx operation does not consider the cell as barred if cellBarred-eRedCap1Rx or cellBarred-eRedCap2Rx is set to “not barred”.

At the same time, CR[2] was agreed and the field descriptions have been updated. Note that the conclusion implies that band selection procedure will not be impacted. And the cell barring bit is only applicable to the selected band. This means that, if the cell broadcasts 2 bands (band 1 and band 2 in order) and the cell indicates cell is barred for 1Rx RedCap UE, but the UE only supports 1Rx for band 1, the UE should consider the cell as barred.

cellBarred-eRedCap1Rx Value barred means that the cell is barred for an eRedCap UE supporting 1 Rx branch on the selected frequency band as specified in clause 5.2.2.4.2, as defined in TS 38.304 [20]. This field is ignored by non-eRedCap UEs. An eRedCap UE supporting 2 Rx on the selected frequency band as specified in clause 5.2.2.4.2 shall ignore this field when cellBarred-eRedCap2Rx is set to notBarred.

cellBarred-eRedCap2Rx Value barred means that the cell is barred for an eRedCap UE supporting 2 Rx branches on the selected frequency band as specified in clause 5.2.2.4.2, as defined in TS 38.304 [20]. This field is ignored by non-eRedCap UEs. An eRedCap UE supporting 1 Rx on the selected frequency band as specified in clause 5.2.2.4.2 shall ignore this field when cellBarred-eRedCap1Rx is set to notBarred.

cellBarredRedCap1Rx Value barred means that the cell is barred for a RedCap UE supporting 1 Rx branch on the selected frequency band as specified in clause 5.2.2.4.2, as defined in TS 38.304 [20]. This field is ignored by non-RedCap UEs. A RedCap UE supporting 2 Rx on the selected frequency band as specified in clause 5.2.2.4.2 shall ignore this field when cellBarredRedCap2Rx is set to notBarred.

cellBarredRedCap2Rx Value barred means that the cell is barred for a RedCap UE supporting 2 Rx branches on the selected frequency band as specified in clause 5.2.2.4.2, as defined in TS 38.304 [20]. This field is ignored by non-RedCap UEs. A RedCap UE supporting 1 Rx on the selected frequency band as specified in clause 5.2.2.4.2 shall ignore this field when cellBarredRedCap1Rx is set to notBarred.

Observation 4: Based on current spec, the (e)RedCap 1Rx/2Rx barring check does not impact the band selection procedure.

However, such clarification is currently missing for Case 2,3,4, so, it may cause different UE implementation and result in inter-operability issue. To avoid such problem, RAN2 is asked to clarify the expected UE behaviour and update the spec accordingly.

Proposal 1: RAN2 confirm that in MBFI cell, the UE first selects a band and then determines whether the cell is barred according to the selected band and UE’s capabilities for the selected band.

Proposal 2: RAN2 confirm the band selection procedure in MBFI cell as follow:

- UE always selects the first band that UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present; For (e)RedCap UEs in FDD, if the halfDuplexRedCapAllowed is not present, the UE selects the first band for which the UE supports full-duplex FDD operation (as specified in TS 38.331 section 5.2.2.4.2);

Proposal 3: RAN2 confirm the barring checking procedure in MBFI cell as follow:

- The cell is barred if cellBarred2RxXR is present and the UE is 2Rx XR on the selected band;

- The cell is barred if the UE does not support any channel bandwidth that fulfills the requirements in section 5.2.2.4.2 for the selected band;

- The cell is barred if frequencyShift7p5khz is present and the UE does not support 7.5kHz frequency shift for the selected band.

The CR to TS 38.331 is provided in [1].

Proposal 4: Agree the RRC CR in [3].

- Conclusion

In this contribution, proposals and observations are:

Observation 1: When UE access an MBFI cell, the network and the UE should have the same understanding on which band will be selected by the UE, otherwise, RRC configuration failure may occur, e.g. when configuring CA/DC band combination.

Observation 2: In LTE spec, the field description is clear that the first supported band should be selected by the UE, but such statement does not exist in the field description of NR DL frequencyBandList IE.

Observation 3: Based on the text procedure in NR RRC spec, the UE should select the first band that UE supports the emission requirements, the only exception is for (e)RedCap UE, where the UE can further check its FD-FDD capability.

Observation 4: Based on current spec, the (e)RedCap 1Rx/2Rx barring check does not impact the band selection procedure.

Proposal 1: RAN2 confirm that in MBFI cell, the UE first selects a band and then determines whether the cell is barred according to the selected band and UE’s capabilities for the selected band.

Proposal 2: RAN2 confirm the band selection procedure in MBFI cell as follow:

- UE always selects the first band that UE supports and for which the UE supports at least one of the additionalSpectrumEmission values in nr-NS-PmaxList, if present; For (e)RedCap UEs in FDD, if the halfDuplexRedCapAllowed is not present, the UE selects the first band for which the UE supports full-duplex FDD operation (as specified in TS 38.331 section 5.2.2.4.2);

Proposal 3: RAN2 confirm the barring checking procedure in MBFI cell as follow:

- The cell is barred if cellBarred2RxXR is present and the UE is 2Rx XR on the selected band;

- The cell is barred if the UE does not support any channel bandwidth that fulfills the requirements in section 5.2.2.4.2 for the selected band;

- The cell is barred if frequencyShift7p5khz is present and the UE does not support 7.5kHz frequency shift for the selected band.

Proposal 4: Agree the RRC CR in [3].

- Reference

[1] R2-2313726  Correction for the selected band for HD-FDD capability checking by RedCap UE   TS38.331  CR4430 r1  v17.6.0;

[2] R2-2406142  Correction on RedCap 1Rx and 2Rx barring  TS38.331  CR4632 r5   v18.1.0

[3] R2-2408636  Correction on band selection procedure  TS 38.331  CR5023   v18.3.0