3GPP TSG-RAN WG2 Meeting #127bis                                R2-2408525

Hefei, China, Oct 14th – 18th, 2024

Source: 	ZTE Corporation

Title:            Discussion on L1 event triggered measurement reporting

Agenda item:	8.6.3

Document for:	Discussion and decision

Introduction

At last meeting, we discussed event-triggered L1 measurement reporting for LTM and made the following agreement:

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

In this contribution, we discussed some remaining issues for L1 event triggered measurement reporting.

Discussions

Event evaluation

At last meeting, RAN2 discussed how to define the serving and candidate beam used for LTM event evaluation. It’s agreed that the current beam (i.e. a beam corresponding to the indicated TCI state) is used for the serving cell and any beam in candidate RS configuration can be used for the candidate cell. Besides, we have made the work assumption that the same RS type is used for serving and candidate cell for event evaluation at RAN2#126 meeting [2].

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

However, it’s possible that the current serving beam corresponding to the indicated TCI state (e.g. CSI-RS beam) is not the same type of RS for configured candidate beams of candidate cells (e.g. SSB beam).

In LTM event evaluation, the current serving beam corresponding to the indicated TCI state (e.g. CSI-RS beam) may not be the same type of RS for configured candidate beams of candidate cells (e.g. SSB beam).

In this case, the UE needs to use the same type of RS for beam associated with QCL RS provided in the indicated TCI state for the serving cell evaluation, i.e. the same RS type as the candidate beam.

- RAN2 confirms the working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. The RS for current beam of serving cell is same as or associated with QCL RS provided in the indicated TCI-state, i.e. using the same type of RS as the candidate beam.

For LTM event evaluation, TTT mechanism can be applied. It requires the beam level measurement result of the cell continuously satisfies the entering condition of the LTM event to trigger the L1 measurement reporting. But it’s unclear whether the TTT is evaluated per beam or per cell.

Taking the event LTM3 evaluation as an example, in case that the TTT is applied per beam, the UE shall start the TTT for a candidate beam whose quality meets the entering condition of the LTM event. The UE can evaluate multiple TTTs in parallel, if multiple beams of the candidate cell meet the entering condition. If one evaluated beam’s quality falls below the entering condition during the TTT is running, the UE shall stop the event evaluation for the beam and reset the TTT. Considering that the L1 measurement result of a specific beam may fluctuate larger than L3 measurement result, e.g. due to the lack of filtering, the TTT criteria may be not easily satisfied continuously. Thus, the triggering of the L1 report and LTM cell switch may be delayed due to the TTT reset, e.g. caused by the beam level measurement result fluctuation. Besides, the change of the current beam of the serving cell may also reset the LTM event evaluation frequently.

Considering that the L1 measurement result of a specific beam may fluctuate larger than L3 measurement result, it may be not easy to meet the TTT criteria continuously for a specific beam. If the TTT is evaluated per beam, the triggering of the L1 measurement report and LTM cell switch may be delayed due to the TTT reset.

In case that the TTT is evaluated per cell, the UE shall continue the event evaluation if at least one beam of the cell meets the entering condition during the TTT is running, e.g. the change of the best beam shall not impact the event evaluation for a cell. In this way, the triggering of L1 measurement reporting shall not be delayed as long as there is at least one beam of the cell meeting the entering condition while the TTT is running.

If the TTT is evaluated per cell, the triggering of L1 measurement reporting shall not be delayed as long as there is at least one beam of the cell meeting the entering condition while the TTT is running.

An example of the TTT evaluation (per beam vs. per cell) is shown as below:

Figure 1. An example of the TTT evaluation (per beam vs. per cell)

As shown in the Figure 1, two candidate beams of a cell meet the threshold of the LTM event in different time points. If the TTT is evaluated per beam, the TTT shall be reset if the beam becomes worse than the threshold, and restart when the beam becomes better than the threshold again. The L1 reporting shall be triggered only when a beam continuously meets the threshold. But if the TTT is evaluated per cell, the TTT shall keep running as long as there is a beam of the cell can meet the threshold, which follows the similar logic as the TTT for L3 measurement evaluation. Thus, the L1 reporting can be triggered faster than beam-specific TTT evaluation. Since one of main advantages for LTM is to support fast mobility triggering, timely measurement reporting is important for the fast triggering. Thus, we prefer to apply TTT per cell.

- For the LTM event evaluation, TTT is evaluated per cell, i.e. not per beam. The TTT shall keep running as long as at least one beam of the cell meets the entering condition of the LTM event.

Besides, it’s agreed that beam specific offset can be applied for LTM event evaluation at RAN2#126 meeting [2], but it’s FFS whether cell specific offset is applied as well.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In the current RRM event evaluation, both cell specific offset and MO specific offset can be applied to adjust the threshold for serving cell and/or neighbour cell. Correspondingly, both cell specific offset and beam specific offset can be considered for LTM event evaluation. Especially, considering that beams from multiple cells may be grouped in one measurement resource set for the LTM event, the cell specific offset is beneficial for the more flexible adjustment of the event threshold applicable to different cells.

- The cell specific offset can be applied for LTM event evaluation.

Measurement resource and report configuration

At last meeting, we discussed the signaling of measurement resource and report configuration and made some general principle as baseline:

5. R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

In this section, we further discuss the detailed signaling design for L1 event triggered measurement configuration.

In Rel-18 LTM, the LTM CSI resource configuration (i.e. LTM-CSI-ResourceConfig) has been introduced for L1 measurement. And each resource set configuration is defined to include one or more SSB resources from one or more LTM candidate cells, wherein the SSB resource configuration is provided in each LTM candidate configuration (i.e. LTM-Candidate).

LTM-CSI-ResourceConfig-r18 ::= SEQUENCE { ltm-CSI-ResourceConfigId-r18 LTM-CSI-ResourceConfigId-r18, ltm-CSI-SSB-ResourceSet-r18 LTM-CSI-SSB-ResourceSet-r18, ... } LTM-CSI-SSB-ResourceSet-r18 ::= SEQUENCE { ltm-CSI-SSB-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index, ltm-CandidateIdList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18, ... }

LTM-Candidate-r18 ::= SEQUENCE { ltm-CandidateId-r18 LTM-CandidateId-r18, ltm-CandidatePCI-r18 PhysCellId OPTIONAL, -- Need M ltm-SSB-Config-r18 LTM-SSB-Config-r18 OPTIONAL, -- Need M ... } LTM-SSB-Config-r18 ::= SEQUENCE { ssb-Frequency-r18 ARFCN-ValueNR, subcarrierSpacing-r18 SubcarrierSpacing, ssb-Periodicity-r18 ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL, -- Need R ssb-PositionsInBurst-r18 CHOICE { shortBitmap BIT STRING (SIZE (4)), mediumBitmap BIT STRING (SIZE (8)), longBitmap BIT STRING (SIZE (64)) } OPTIONAL, -- Need R ss-PBCH-BlockPower-r18 INTEGER (-60..50) OPTIONAL, -- Need R ... }

Before discussing how to provide the L1 measurement resource configuration for LTM event, we think it’s important to firstly clarify the granularity of the event application, i.e. the granularity of measurement resource associated with one event configuration. The following options can be considered:

- Option 1: the event is applied per RS or beam;

- Option 2: the event is applied per cell;

- Option 3: the event is applied per frequency;

- Option 4: the event is applied per CSI resource set.

RAN2 can firstly clarify the granularity of the event application (i.e. the granularity of measurement resource associated with one event configuration), e.g. per beam, per cell, per frequency or per CSI resource set.

The option 1 is the finest granularity of event application. In this option, a beam specific resource configuration list can be defined, e.g. a list of RS measurement resource/object. And each RS measurement resource/object can be associated with an event/report configuration. In option 2, one event configuration is applied for all configured beams in a cell. We can introduce a list of cell specific resource configuration or simply reuse the SSB/CSI-RS resource configuration within the LTM candidate configuration, e.g. LTM-SSB-Config. In option 3, one event configuration is applied for all configured beams and cells in the same frequency. The L3 MO similar configuration can be introduced. In option 4, one event configuration is applied for all beams and cells within one CSI resource set (e.g. LTM-CSI-ResourceConfig). In this way, the NW can flexibly adjust the granularity of event application by including one or more RS(s) from one or more cell(s) in one CSI resource set, e.g. including one RS from one candidate cell (i.e. per RS), all measured RSs from one candidate cell (i.e. per cell), or all measured RSs from candidate cells in the same frequency (i.e. per frequency) in one CSI resource set.

The NW can flexibly adjust the granularity of event application by including one or more RS(s) from one or more cell(s) in one CSI resource set, e.g. including one RS from one candidate cell (i.e. per RS), all measured RSs from one candidate cell (i.e. per cell), or all measured RSs from candidate cells in the same frequency (i.e. per frequency) in one CSI resource set.

Thus, option 4 is preferred to allow flexible granularity of event application. And the similar signaling structure for R18 LTM CSI resource configuration can be considered. In the existing CSI resource configuration signaling, the SSB resource set is mandatory present. And it’s agreed that both SSB and CSI-RS can be taken as the measurement resource for event-triggered measurement. If we directly extend the current signaling to include CSI-RS resources, there must be an SSB resource set associated with the CSI-RS resource even if the NW just wants to configure CSI-RS resources for the LTM event. This seems not a desired signaling design.

The SSB resource set is mandatory present in the current LTM CSI resource configuration signaling. If the current signaling is directly extended to include CSI-RS resources, there must be an SSB resource set associated with the CSI-RS resource even if the NW just wants to configure CSI-RS resources for the LTM event.

Thus, we can further discuss the detailed ASN.1 design for the measurement resource once CSI-RS resource configuration is introduced.

- The similar R18 LTM CSI resource configuration structure is reused as the measurement resource configuration for LTM event, but the detailed ASN.1 design can be discussed later once CSI-RS resource configuration is introduced.

For Rel-18 LTM L1 measurement, one LTM CSI resource configuration is associated with one LTM report configuration by including the LTM-CSI-ResourceConfigId into the LTM-CSI-ReportConfig. In order to associate the measurement resource configuration and the report configuration for LTM event, there are two options that can be considered:

- Option 1: including the LTM event configuration into the LTM-CSI-ReportConfig and reusing the LTM-CSI-ResourceConfigId to associate the measurement resource configuration with the report configuration;

- Option 2: introducing an additional LTM event report configuration (e.g. parallel to LTM-CSI-ReportConfig) and a measurement ID to associate the measurement resource configuration and the LTM event report configuration.

The current LTM-CSI-ReportConfig includes multiple mandatory IEs for UCI based measurement reporting, e.g. ltm-ReportConfigType, ltm-ReportContent. But such IEs shall not be required if MAC CE based reporting is used for event-triggered L1 measurement reporting.

LTM-CSI-ReportConfig-r18 ::= SEQUENCE { ltm-CSI-ReportConfigId-r18 LTM-CSI-ReportConfigId-r18, ltm-ResourcesForChannelMeasurement-r18 LTM-CSI-ResourceConfigId-r18, ltm-ReportConfigType-r18 CHOICE { periodic-r18 SEQUENCE { reportSlotConfig-r18 CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUCCH-r18 SEQUENCE { reportSlotConfig-r18 CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUSCH-r18 SEQUENCE { reportSlotConfig-r18 CSI-ReportPeriodicityAndOffset, reportSlotOffsetList-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-2-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-1-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), p0alpha P0-PUSCH-AlphaSetId }, aperiodic-r18 SEQUENCE { reportSlotOffsetList-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-2-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-1-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128) }, ... }, ltm-ReportContent-r18 LTM-ReportContent-r18, ... } LTM-ReportContent-r18 ::= SEQUENCE { nrOfReportedCells-r18 ENUMERATED {n1,n2,n3,n4}, nrOfReportedRS-PerCell-r18 ENUMERATED {n1,n2,n3,n4}, spCellInclusion-r18 ENUMERATED {true} OPTIONAL -- Need R }

Besides, the current LTM-CSI-ResourceConfig only includes SSB based measurement resources, which is also mandatory present. We agreed that CSI-RS based measurement resource is also supported for event-triggered L1 measurement reporting. Thus, the additional LTM CSI resource configuration ID may also be required in the LTM-CSI-ReportConfig to enable the event-triggered L1 measurement reporting based on CSI-RS resources only.

The current LTM-CSI-ReportConfig includes multiple mandatory IEs for UCI based measurement reporting, e.g. ltm-ReportConfigType, ltm-ReportContent. Such IEs shall not be required if MAC CE based reporting is used for event-triggered L1 measurement reporting.

Since we prefer to use MAC CE carry the event-triggered measurement report, it would be cleaner to introduce an additional LTM event report configuration, i.e. option 2.

- An additional report configuration is introduced for LTM event configuration, e.g. parallel to LTM-CSI-ReportConfig.

In addition, one measurement resource configuration can be associated with different event configuration, e.g. different event type. And one report configuration can be applicable to different measurement resource configurations as well. In order to associate the report configuration with the measurement resource configuration flexibly, a measurement ID can be introduced to associate one measurement resource configuration with one report configuration, similar to MeasId in L3 measurement configuration.

- A measurement ID is introduced to associate one measurement resource configuration with one report configuration for LTM event, e.g. similar to MeasId.

Regarding the detailed parameters included in the report configuration, it’s agreed that TTT, hysteresis for entering/leaving, and/or beam specific offset can be applied for LTM event evaluation. Thus, the NW can provide the threshold/offset of the LTM event, TTT, and hysteresis parameters in the measurement report configuration. In the legacy L3 event report configuration, there are several additional parameters related to the event/report configuration, e.g. reportOnLeave, useAllowedCellList, rsType, reportInterval, reportAmount, reportQuantityCell, maxReportCells, etc. We can further discuss whether such parameters are applicable to LTM event-triggered measurement report as well.

reportOnLeave

In L3 measurement reporting, the reportOnLeave is used to indicate whether to initiate the measurement reporting procedure when the leaving condition of the event is fulfilled. It can help the NW to know the signal quality deterioration of the concerned beam/cell in time, so that the NW can decide to, e.g., cancel the triggering of handover or adjust the target cell. The similar mechanism is also beneficial for the measurement report for the LTM. We think it can be applicable to event-triggered L1 measurement report. And it is configurable by the NW.

- The reportOnLeave is applicable to the event-triggered L1 measurement report and configurable by the NW.

useAllowedCellList

In L3 measurement, the NW can provide the allowed and excluded cell lists in the measurement object configuration, and indicate whether only the cells included in the allowed list of the associated measurement object are applicable for the event in the report configuration, i.e. via useAllowedCellList. For event-triggered L1 measurement report, the Rle-18 LTM CSI resource configuration (i.e. LTM-CSI-ResourceConfig) can be reused as the measurement resource. And one LTM-CSI-ResourceConfig can include one or more beams from one or more candidate cells in one resource set. But the NW may want to make the LTM event only applicable to some specific beams or cells in the resource set. Besides, the current CSI resource configuration is common for all candidate cells, i.e. the UE uses the common CSI resource configuration for L1 measurement in the subsequent LTM. But the candidate beams or cells to be measured/reported may be changed when the UE switches to different candidate cells, e.g. only consider some specific neighbour beams/cells.

The Rle-18 LTM CSI resource configuration (i.e. LTM-CSI-ResourceConfig) can include one or more beams from one or more candidate cells in one resource set. The NW may want to make the LTM event only applicable to some specific beams or cells in the resource set.

Thus, it would be beneficial to reuse the similar mechanism for the allowed cell list, to allow the UE reporting L1 measurements only for some specific cells and/or beams, e.g. avoid the unnecessary measurement reporting. And we can further discuss whether the allowed cell and/or beam list is provided in the measurement resource configuration (i.e. similar to L3 measurement) or the measurement report configuration.

- RAN2 to discuss whether the allowed cell/beam list is applicable to the event-triggered measurement report. FFS whether the allowed cell/beam list is provided in the measurement resource configuration or the measurement report configuration.

rsType

At last meeting, we agreed that both SSB and CSI-RS are supported as the measurement resource for event-triggered measurement report. It’s possible to configure the SSB resources and CSI-RS resources in one measurement resource configuration, but the NW wants to enable the event-triggered measurement report for only one type of measurement resource. Thus, it would be more flexible to allow the NW indicate which RS is used to provide the measurement result for the LTM event evaluation in the report configuration.

- The RS used to provide the measurement results for the LTM event evaluation is configurable by the NW, e.g. include the rsType in the report configuration.

reportInterval, reportAmount

In L3 measurement, the reportInterval and reportAmount are applicable to the L3 event-triggered measurement report, which is used for event-triggered periodical reporting. Namely, once the reporting is triggered by an event, the UE can periodically send the subsequent measurement report up to a certain amount as configured by reportAmount, as long as there is cell(s) remained in the triggered cell list. For event-triggered L1 measurement report, we think the subsequent periodical reporting is also useful for the NW, e.g. to identify the best beam change. But the subsequent measurement report shall also increase the signaling overhead if a lot of reporting is triggered by the event. Thus, it would be better to allow the NW controlling the report interval and report amount, i.e. similar to L3 measurement.

- The reportInterval and reportAmount are applicable to the event-triggered measurement report and configurable by the NW.

reportQuantityCell

In L3 measurement reporting, the reportQuantityCell is used to indicate the measurement quantities to be included in the measurement report. In Rel-18 LTM measurement report, only L1-RSRP is included. Some companies proposed that L1-SINR could also be included in the measurement report, e.g. useful for the neighbor cell scheduling. Besides, whether to support L1-SINR is also under the RAN1 MIMO discussion. In our understanding, it can be up to RAN1 decide whether to support L1-SINR, and then RAN2 can discuss whether to indicate the reportQuantityCell/Beam in the report configuration.

It’s up to RAN1 whether other measurement qunatities (e.g. L1-SINR) are supported for event-triggered measurement report.

maxReportCells, maxNrofRS-IndexesToReport

In L3 measurement reporting, the maxReportCells is used to indicate the max number of non-serving cells to include in the measurement report, and the maximum value is 8. The maxNrofRS-IndexesToReport is used to indicate the number of RS indexes to include in the measurement report, and the maximum value is 32. For Rel-18 LTM L1 measurement reporting, the maximum number of cells (i.e. nrOfReportedCells) and the maximum number of beams per cell (i.e. nrOfReportedRS-PerCell) to include in the measurement report are also controlled by the NW, based on the UE capability. The maximum value of each item is 4, so the maximum total number of beams to include in one measurement report is limited to 16.

In Rel-18 LTM L1 measurement reporting, the maximum number of cells (i.e. nrOfReportedCells) and the maximum number of beams per cell (i.e. nrOfReportedRS-PerCell) to include in the measurement report are controlled by the NW, based on the UE capability. The maximum total number of beams to include in one measurement report is limited to 16.

The similar mechanism can be followed in the event-triggered L1 measurement report. Namely, the maximum number of cells and the maximum number of beams per cell to include in the measurement report are configurable by the NW, if supported by the UE. Considering that MAC CE is preferred to be used to carry the event-triggered L1 measurement report, larger and more flexible message size can be used to carry more measurement results, compared to the UCI based solution. We can further discuss whether to extend the maximum total number of beams to include in the measurement report, e.g. 32.

- For event-triggered L1 measurement report, if supported by the UE, the maximum number of cells and the maximum number of beams per cell to include in the measurement report are configurable by the NW, similar to Rel-18 LTM L1 measurement. FFS whether to extend the maximum number, e.g. maximum total number of beams is 32.

An example of RRC signalling on the measurement configuration for LTM event is provided in the Annex for information.

Information in the measurement report

In Rel-18 LTM L1 measurement report, the L1-RSRP and the corresponding candidate beam and cell information are included. The candidate beam and cell information are implicitly indicated by the entry index (i.e. SSBRI) in the LTM-CSI-ResourceConfig, which indicates the association between the SSB index and the LTM-CandidateId. For event-triggered measurement report, the similar information should also be included considering the L1 measurement results for multiple beams and cells can be included in one measurement report. If we agree to reuse the LTM CSI resource configuration as the measurement resource for event-triggered reporting, the entry index (i.e. SSBRI) in the LTM-CSI-ResourceConfig can also be reused to indicate the candidate beam and cell information in the measurement report. An alternative way is to explicitly include the beam ID and candidate cell ID in the report. The details can be further discussed once the signaling of measurement configuration is decided.

In Rel-18 LTM L1 measurement report, the L1-RSRP and the corresponding candidate beam and cell information are included, wherein the candidate beam and cell information are implicitly indicated by the entry index (i.e. SSBRI) in the LTM-CSI-ResourceConfig.

Besides, the information of the triggered event is also required for event-triggered measurement report. With this information, the NW can know the measurement report is triggered for which event and the related configuration for the event, e.g. threshold value. Regarding how to indicate the information of the triggered event, it’s also related the signaling design of the measurement report configuration. If a L1 measurement ID is introduced to link the measurement resource with the measurement report configuration, it’s simply to include the L1 measurement ID in the report. Alternatively, if we reuse LTM-CSI-ReportConfig for the event-triggered measurement report, where the associated LTM-CSI-ResourceConfigId is included, the report configuration ID can implicitly indicate the triggered event.

- The content of the event-triggered measurement report at least includes the following information:

Beam-level measurement results, e.g. L1-RSRP;

The corresponding beam information and cell information, e.g. implicitly indicated by entry index (i.e. SSBRI) in the LTM-CSI-ResourceConfig, or explicitly indicated by beam ID and cell ID;

Information for the triggered event, e.g. L1 measurement ID, or report configuration ID.

Another issue is whether to include the L1 measurement result of the serving cell in the event-triggered measurement report. In L3 measurement reporting, the measurement result of serving cell configured with servingCellMO shall always be reported to the NW. And the servingCellMO is mandatory present for the SpCell. It means that the L3 measurement reporting shall always include the measurement results for the SpCell. the In Rel-18 LTM L1 measurement reporting, if the UE supports to always include SpCell results in L1 measurement report (i.e. based on the UE capability currentSpCellInclL1-Report-r18), the NW can explicitly indicate whether to include the L1 measurement result of the SpCell in the reporting, i.e. via the spCellInclusion in the LTM-CSI-ReportConfig.

In Rel-18 LTM L1 measurement reporting, based on the UE capability (i.e. currentSpCellInclL1-Report-r18), the NW can explicitly indicate whether to include the L1 measurement result of the SpCell in the report. But for L3 measurement reporting, the measurement result of the SpCell is mandatory included.

In our view, it’s beneficial to include the measurement results of the SpCell (e.g. the measurement result of the best beam or the beam corresponding to the indicated TCI-state) in the event-triggered reporting, e.g. to decide whether the triggering of LTM is required. RAN2 can further discuss whether to mandatory or optionally (e.g. based on the explicit indication by the NW) include the measurement result of the SpCell. And whether to reuse Rel-18 capability can be discussed later.

- If supported by the UE, the measurement result of the SpCell can be included in the event-triggered measurement report. RAN2 to discuss how to include it, e.g. mandatory include it, or optionally include it based on the explicit indication by the NW. Whether to reuse Rel-18 capability can be discussed later.

Conclusion and Proposals

In this contribution, we discussed the event-triggered L1 measurement reporting with the following observations and proposals:

Observation 1:	In LTM event evaluation, the current serving beam corresponding to the indicated TCI state (e.g. CSI-RS beam) may not be the same type of RS for configured candidate beams of candidate cells (e.g. SSB beam).

Observation 2:	Considering that the L1 measurement result of a specific beam may fluctuate larger than L3 measurement result, it may be not easy to meet the TTT criteria continuously for a specific beam. If the TTT is evaluated per beam, the triggering of the L1 measurement report and LTM cell switch may be delayed due to the TTT reset.

Observation 3:	If the TTT is evaluated per cell, the triggering of L1 measurement reporting shall not be delayed as long as there is at least one beam of the cell meeting the entering condition while the TTT is running.

Observation 4:	RAN2 can firstly clarify the granularity of the event application (i.e. the granularity of measurement resource associated with one event configuration), e.g. per beam, per cell, per frequency or per CSI resource set.

Observation 5:	The NW can flexibly adjust the granularity of event application by including one or more RS(s) from one or more cell(s) in one CSI resource set, e.g. including one RS from one candidate cell (i.e. per RS), all measured RSs from one candidate cell (i.e. per cell), or all measured RSs from candidate cells in the same frequency (i.e. per frequency) in one CSI resource set.

Observation 6:	The SSB resource set is mandatory present in the current LTM CSI resource configuration signaling. If the current signaling is directly extended to include CSI-RS resources, there must be an SSB resource set associated with the CSI-RS resource even if the NW just wants to configure CSI-RS resources for the LTM event.

Observation 7:	The current LTM-CSI-ReportConfig includes multiple mandatory IEs for UCI based measurement reporting, e.g. ltm-ReportConfigType, ltm-ReportContent. Such IEs shall not be required if MAC CE based reporting is used for event-triggered L1 measurement reporting.

Observation 8:	The Rle-18 LTM CSI resource configuration (i.e. LTM-CSI-ResourceConfig) can include one or more beams from one or more candidate cells in one resource set. The NW may want to make the LTM event only applicable to some specific beams or cells in the resource set.

Observation 9:	It’s up to RAN1 whether other measurement qunatities (e.g. L1-SINR) are supported for event-triggered measurement report.

Observation 10:	In Rel-18 LTM L1 measurement reporting, the maximum number of cells (i.e. nrOfReportedCells) and the maximum number of beams per cell (i.e. nrOfReportedRS-PerCell) to include in the measurement report are controlled by the NW, based on the UE capability. The maximum total number of beams to include in one measurement report is limited to 16.

Observation 11:	In Rel-18 LTM L1 measurement report, the L1-RSRP and the corresponding candidate beam and cell information are included, wherein the candidate beam and cell information are implicitly indicated by the entry index (i.e. SSBRI) in the LTM-CSI-ResourceConfig.

Observation 12:	In Rel-18 LTM L1 measurement reporting, based on the UE capability (i.e. currentSpCellInclL1-Report-r18), the NW can explicitly indicate whether to include the L1 measurement result of the SpCell in the report. But for L3 measurement reporting, the measurement result of the SpCell is mandatory included.

Event evaluation

Proposal 1:	RAN2 confirms the working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. The RS for current beam of serving cell is same as or associated with QCL RS provided in the indicated TCI-state, i.e. using the same type of RS as the candidate beam.

Proposal 2:	For the LTM event evaluation, TTT is evaluated per cell, i.e. not per beam. The TTT shall keep running as long as at least one beam of the cell meets the entering condition of the LTM event.

Proposal 3:	The cell specific offset can be applied for LTM event evaluation.

Measurement resource and report configuration

Proposal 4:	The similar R18 LTM CSI resource configuration structure is reused as the measurement resource configuration for LTM event, but the detailed ASN.1 design can be discussed later once CSI-RS resource configuration is introduced.

Proposal 5:	An additional report configuration is introduced for LTM event configuration, e.g. parallel to LTM-CSI-ReportConfig.

Proposal 6:	A measurement ID is introduced to associate one measurement resource configuration with one report configuration for LTM event, e.g. similar to MeasId.

Proposal 7:	The reportOnLeave is applicable to the event-triggered L1 measurement report and configurable by the NW.

Proposal 8:	RAN2 to discuss whether the allowed cell/beam list is applicable to the event-triggered measurement report. FFS whether the allowed cell/beam list is provided in the measurement resource configuration or the measurement report configuration.

Proposal 9:	The RS used to provide the measurement results for the LTM event evaluation is configurable by the NW, e.g. include the rsType in the report configuration.

Proposal 10:	The reportInterval and reportAmount are applicable to the event-triggered measurement report and configurable by the NW.

Proposal 11:	For event-triggered L1 measurement report, if supported by the UE, the maximum number of cells and the maximum number of beams per cell to include in the measurement report are configurable by the NW, similar to Rel-18 LTM L1 measurement. FFS whether to extend the maximum number, e.g. maximum total number of beams is 32.

Information in the measurement report

Proposal 12:	The content of the event-triggered measurement report at least includes the following information:

-	Beam-level measurement results, e.g. L1-RSRP;

-	The corresponding beam information and cell information, e.g. implicitly indicated by entry index (i.e. SSBRI) in the LTM-CSI-ResourceConfig, or explicitly indicated by beam ID and cell ID;

-	Information for the triggered event, e.g. L1 measurement ID, or report configuration ID.

Proposal 13:	If supported by the UE, the measurement result of the SpCell can be included in the event-triggered measurement report. RAN2 to discuss how to include it, e.g. mandatory include it, or optionally include it based on the explicit indication by the NW. Whether to reuse Rel-18 capability can be discussed later.

References

R2-2407571  Report from session on V2X/SL, R19 NES and MOB  Vice Chairman (Samsung)

R2-2405701  Report from session on V2X/SL, R19 NES and MOB  Vice Chairman (Samsung)

- Annex

- –	CSI-MeasConfig

The IE CSI-MeasConfig is used to configure CSI-RS (reference signals) belonging to the serving cell in which CSI-MeasConfig is included, channel state information reports to be transmitted on PUCCH on the serving cell in which CSI-MeasConfig is included and channel state information reports on PUSCH triggered by DCI received on the serving cell in which CSI-MeasConfig is included. See also TS 38.214 [19], clause 5.2.

CSI-MeasConfig information element

-- ASN1START

-- TAG-CSI-MEASCONFIG-START

CSI-MeasConfig ::=                  SEQUENCE {

nzp-CSI-RS-ResourceToAddModList     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource   OPTIONAL, -- Need N

nzp-CSI-RS-ResourceToReleaseList    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToAddModList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL, -- Need N

nzp-CSI-RS-ResourceSetToReleaseList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need N

csi-IM-ResourceToAddModList         SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource           OPTIONAL, -- Need N

csi-IM-ResourceToReleaseList        SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId         OPTIONAL, -- Need N

csi-IM-ResourceSetToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet     OPTIONAL, -- Need N

csi-IM-ResourceSetToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId   OPTIONAL, -- Need N

csi-SSB-ResourceSetToAddModList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSet   OPTIONAL, -- Need N

csi-SSB-ResourceSetToReleaseList    SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSets)) OF CSI-SSB-ResourceSetId OPTIONAL, -- Need N

csi-ResourceConfigToAddModList      SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfig

OPTIONAL, -- Need N

csi-ResourceConfigToReleaseList     SEQUENCE (SIZE (1..maxNrofCSI-ResourceConfigurations)) OF CSI-ResourceConfigId

OPTIONAL, -- Need N

csi-ReportConfigToAddModList        SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig  OPTIONAL, -- Need N

csi-ReportConfigToReleaseList       SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfigId

OPTIONAL, -- Need N

reportTriggerSize                   INTEGER (0..6)                                                            OPTIONAL, -- Need M

aperiodicTriggerStateList           SetupRelease { CSI-AperiodicTriggerStateList }                            OPTIONAL, -- Need M

semiPersistentOnPUSCH-TriggerStateList    SetupRelease { CSI-SemiPersistentOnPUSCH-TriggerStateList }         OPTIONAL, -- Need M

...,

[[

reportTriggerSizeDCI-0-2-r16        INTEGER (0..6)                                                            OPTIONAL -- Need R

]],

[[

sCellActivationRS-ConfigToAddModList-r17  SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-Config-r17   OPTIONAL, -- Need N

sCellActivationRS-ConfigToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofSCellActRS-r17)) OF SCellActivationRS-ConfigId-r17 OPTIONAL  -- Need N

]],

[[

ltm-CSI-ReportConfigToAddModList-r18   SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfig-r18

OPTIONAL, -- Need N

ltm-CSI-ReportConfigToReleaseList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ReportConfigurations-r18)) OF LTM-CSI-ReportConfigId-r18

OPTIONAL  -- Need N

]],

[[

ltm-Event-ReportConfigToAddModList-r19   SEQUENCE (SIZE (1..maxNrofLTM-Event-ReportConfigurations-r19)) OF LTM-Event-ReportConfig-r19

OPTIONAL, -- Need N

ltm-Event-ReportConfigToReleaseList-r19  SEQUENCE (SIZE (1..maxNrofLTM-Event-ReportConfigurations-r19)) OF LTM-Event-ReportConfigId-r19

OPTIONAL,  -- Need N

ltm-MeasIdToAddModList-r19   SEQUENCE (SIZE (1..maxNrofLTM-MeasIds-r19)) OF LTM-MeasIdConfig-r19

OPTIONAL, -- Need N

ltm-MeasIdToReleaseList-r19  SEQUENCE (SIZE (1..maxNrofLTM-MeasIds-r19)) OF LTM-MeasId-r19

OPTIONAL  -- Need N

]]

}

-- TAG-CSI-MEASCONFIG-STOP

-- ASN1STOP

- –	LTM-Event-ReportConfig

The IE LTM-Event-ReportConfig is used to configure event-triggered L1 measurement report on the cell in which the LTM-Event-ReportConfig is included.

LTM-Event-ReportConfig information element

-- ASN1START

-- TAG-LTM-Event-REPORTCONFIG-START

LTM-Event-ReportConfig-r19 ::=      SEQUENCE {

ltm-Event-ReportConfigId-r19                     LTM-Event-ReportConfigId-r19,

ltm-Event-TriggerConfig-r19                     LTM-Event-TriggerConfig-r19

...

}

LTM-EventTriggerConfig-r19 ::=                      SEQUENCE {

LTM-eventId-r19                                     CHOICE {

LTM-eventA2-r19                                     SEQUENCE {

a2-Threshold-r19                                MeasTriggerQuantity,

hysteresis-r19                                  Hysteresis,

timeToTrigger-r19                               TimeToTrigger

},

LTM-eventA3-r19                                     SEQUENCE {

a3-Offset-r19                                   MeasTriggerQuantityOffset,

hysteresis-r19                                  Hysteresis,

timeToTrigger-r19                               TimeToTrigger,

},

LTM-eventA4-r19                                     SEQUENCE {

a4-Threshold-r19                                MeasTriggerQuantity,

hysteresis-r19                                  Hysteresis,

timeToTrigger-r19                               TimeToTrigger,

},

LTM-eventA5-r19                                     SEQUENCE {

a5-Threshold1-r19                               MeasTriggerQuantity,

a5-Threshold2-r19                               MeasTriggerQuantity,

hysteresis-r19                                  Hysteresis,

timeToTrigger-r19                               TimeToTrigger,

},

...,

},

...

}

-- TAG-LTM-EVENT-REPORTCONFIG-STOP

-- ASN1STOP

- –	LTM-Event-ReportConfigId

The IE LTM-Event-ReportConfigId is used to identify an LTM-Event-ReportConfig.

LTM-Event-ReportConfigId information element

-- ASN1START

-- TAG-LTM-EVENT-REPORTCONFIGID-START

LTM-Event-ReportConfigId-r19 ::=            INTEGER (0..maxNrofLTM-Event-ReportConfigurations-1-r19)

-- TAG-LTM-EVENT-REPORTCONFIGID-STOP

-- ASN1STOP

- –	LTM-MeasId

The IE LTM-MeasId is used to identify an LTM measurement configuration, i.e., linking of a measurement resource and a reporting configuration.

LTM-MeasId information element

-- ASN1START

-- TAG-LTM-MEASID-START

LTM-MeasId-r19 ::=                          INTEGER (1.. maxNrofLTM-MeasIds-r19)

-- TAG-LTM-MEASID-STOP

-- ASN1STOP

- –	LTM-MeasIdConfig

The IE LTM-MeasIdConfig is used to configure the association between a measurement resource and a reporting configuration.

LTM-MeasIdConfig information element

-- ASN1START

-- TAG-LTM-MEASIDCONFIG-START

LTM-MeasIdConfig ::=                  SEQUENCE {

ltm-measId-r19                              LTM-MeasId-r19,

ltm-measObjectId-r19                        LTM-CSI-ResourceConfigId-r18,

ltm-reportConfigId-r19                      LTM-Event-ReportConfigId-r19,

...

}

-- TAG-LTM-MEASIDCONFIG-STOP

-- ASN1STOP