3GPP TSG-RAN2#127bis								             R2-2408533

Hefei, China, Oct 14th – 18th, 2024

Title: 	MAC signalling for data rate control for XR applications

Source: 	ZTE Corporation, Sanechips

Agenda item:	8.7.6

Document for:	Discussion and decision

# Introduction

In RAN#105 meeting, Revised WID on XR (eXtended Reality) for NR Phase 3 has been agreed and the following objective has been included:

- Specify uplink congestion signalling [RAN2]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

In this paper, we will discuss the related issue and give our proposals.

# Discussion

In the current NR MAC specification, Recommended bit rate MAC CE is supported to indicate the recommended bit rate for the UE for a specific logical channel and a specific direction (either uplink or downlink).  However, this feature is not tailored for XR. Specifically, the following shortcomings for the recommended bit rate MAC CE needs to be addressed for this mechanism to be reused for XR applications:

- Support of XR data rates:

Recommended bit rate MAC CE covers only data rates up to 8000 kb/s. The XR applications on the other hand needs much higher data rates (going up to tens of Mbps or even higher). The upper bound for the recommended bit rate MAC CE should hence be adapted to cover the higher range for the XR applications. Although, the network can configure a multiplication factor for the signalled recommended bit rate, only multipliers of 40, 70, 100 and 200 are supported which don’t provide enough granularity for the XR applications in question.

- Granularity of signalled recommended bit rates:

Currently the Recommended bit rate MAC CE uses only 6 bit field and hence there are only a total of 64 code points available. The granularity of this field is not sufficient to cover the wide range of traffic profiles and data rates that might be generated by various codec modes for XR applications. It is hence better to define either a new MAC CE to cover the suitable range of traffic with sufficient granularity for XR applications or define finer values for the bitRateMultiplier to enable better granularity and peak data rates for XR.

- QoS flow specific indication:

Currently the Recommended bit rate MAC CE indicates the recommended bit rate per LCID. However, in case of XR, different QoS flows may be mapped to the same radio bearer and in order to better control the upper layer throughput it may be better to extend this mechanism to indicate the throughput on a per QoS flow basis rather than LCID.

Based on the above, we propose to enhance the recommended bit rate MAC CE to cover XR applications in Rel-19 as follows:

- Support finer granularity of recommended bit rates to cover all the possible XR bit rates.

- Support recommended bit rates per QoS flow.

Proposal 1: enhance the recommended bit rate MAC CE to cover XR applications in Rel-19 as follows:

- Support finer granularity of recommended bit rates to cover all the possible XR bit rates.

- Support recommended bit rates per QoS flow.

# DL rate control

The existing recommended bit rate MAC CE works for both UL and DL rate control. Whilst UL rate control is negotiated between the lower layers and application layer within the UE, the DL rate control requires application-level signalling between the UE and the application server. This is done by the UE by initiating end-to-end bit rate adaptation with its peer (UE or application server) using application-level signalling. Such signalling is feasible already with the existing recommended bit rate MAC control mechanism. Thus, by reusing this existing mechanism it is also possible to control the DL bit rates for XR applications. Thus, it is proposed not to artificially restrict the new rate control mechanism for RAN level recommended bit rate to just UL.

Proposal 2: The enhanced recommended bit rate MAC CE should support both UL and DL bit rates for XR applications

# Conclusion

Proposal 1: enhance the recommended bit rate MAC CE to cover XR applications in Rel-19 as follows:

- Support finer granularity of recommended bit rates to cover all the possible XR bit rates.

- Support recommended bit rates per QoS flow.

Proposal 2: The enhanced recommended bit rate MAC CE should support both UL and DL bit rates for XR applications

# References

- RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3	RAN#105

- 3GPP TS 38.321, V18.2.0 (2024-06), NR; Medium Access Control (MAC) protocol specification