3GPP TSG-RAN2#127bis								             R2-2408537

Hefei, China, Oct 14th – 18th, 2024

Title: 	Number of UEs in RRC_INACTIVE with data transmission

Source: 	ZTE Corporation, Sanechips

Agenda item:	7.0.2.4

Document for:	Discussion and decision

# Introduction

SA5 has sent an LS to RAN2 (see S5-245138), asking us whether there is a possibility to specify the number of UEs in RRC_INACTIVE state with data transmissions. In this contribution we propose a possible reply back to SA5.

# Background

SA5 are working on 5G performance measurements and KPIs for various features that are defined as part of 5G advanced. As part of this work, SA5 has the WI to specify performance measurements and KPIs for SDT feature (see SP-241155).

The intention of the LS is to understand whether it is possible to define a mechanism for the network to count the number of UEs in RRC_INACTIVE state with data transmission (i.e. the number of SDT UEs) as one of the network KPIs.

When SDT is initiated, the UE sends RRCResumeRequest to the network. SDT procedure is initiated with either a transmission over RACH (configured via system information) or over Type 1 CG resources (configured via dedicated signalling in RRCRelease). Thus, for MO-SDT, the network knows the SDT cause either by the RACH resource used for sending the RRCResumeRequest or by the reception of RRCResumeRequest via the type 1 CG resource configured for SDT. When MT-SDT is initiated by the UE, a resume cause indicating MT-SDT is included in the RRCResumeRequest. Thus, the network can keep track of number of UEs in a given cell that have SDT ongoing at any given point in time in the cell. So, no new mechanism is needed to keep count of this on the network side and no new UE procedure is needed either for this.

Based on the above, it is proposed to reply to SA5 as follows:

Proposal: Reply to SA5 that:

SDT procedure is initiated by sending RRCResumeRequest. For MO-SDT, SDT procedure is initiated with either a transmission over RACH resource configured for SDT or over Type 1 CG resources. Thus, for MO-SDT, the network knows the SDT cause either by the RACH resource used for sending the RRCResumeRequest or by the reception of RRCResumeRequest via the type 1 CG resource configured for SDT. When MT-SDT is initiated by the UE, a resume cause indicating MT-SDT is included in the RRCResumeRequest. Thus, it is possible for the network to keep track of the number of UEs in a given cell that have SDT ongoing (both for MO-ST and MT-SDT) at any given point in time in the cell.

# Conclusion

Proposal: Reply to SA5 that:

SDT procedure is initiated by sending RRCResumeRequest. For MO-SDT, SDT procedure is initiated with either a transmission over RACH resource configured for SDT or over Type 1 CG resources. Thus, for MO-SDT, the network knows the SDT cause either by the RACH resource used for sending the RRCResumeRequest or by the reception of RRCResumeRequest via the type 1 CG resource configured for SDT. When MT-SDT is initiated by the UE, a resume cause indicating MT-SDT is included in the RRCResumeRequest. Thus, it is possible for the network to keep track of the number of UEs in a given cell that have SDT ongoing (both for MO-ST and MT-SDT) at any given point in time in the cell.

# References

- S5-245138 	LS on Number of UEs in RRC_INACTIVE state with data transmission, SA5

- SP-241155	Revised WID for 5G performance measurements and KPIs phase 4