3GPP TSG-RAN WG2 Meeting #127bis	R2-2408549

Hefei, China, 14 – 18 Oct 2024

Agenda Item:	8.2.4

Source:	NEC

Title:	Random Access for Ambient IoT device

Document for:	Discussion, Decision

# Introduction

In the last RAN2 meeting, the following agreements were reached for the random access procedure for Ambient IoT device:

Agreement for 3 step RA

- For 3-step CBRA Support fixed random ID size is 16 bit.  The ID is randomly generated.

- Failure/success indication of D2R will be studied. FFS if it would be implicit or explicit and for which use case it is needed.  FFS whether it is applied only to some cases.

Agreement for 2 step RA

- for 2step CBRA, RAN2 design will support msg2.  Whether it is needed it is up to the reader.  FFS when it is needed.  For 2-step CBRA (when mgs2 is needed), the random ID (fixed 16bits) is also included in A-IoT Msg1, and is echoed in A-IoT Msg2.   FFS if there will be devices support only 2-step RA and any other optimizations will be needed for such devices.

- In contention-free access, the A-IoT device directly sends the upper layer data (e.g. device ID) in its very first D2R message after being triggered (i.e. skip contention resolution Msg1/2).   FFS if a short AS ID is also included in the message and what type of ID for scheduling purposes.

- FFS if reader assigns the AS ID for scheduling purposes

There was post meeting email discussion on the random access procedure for Ambient IoT device: [POST127][033][AIoT] Random Access (Huawei).

This paper intends to discuss the controversial issues following the email discussion above.

# Discussion

## D2R data transmission failure

During the email discussion, there was a discussion on the consequence of D2R data transmission failure, and the following options were listed on the table

Option 1: Re-transmit the D2R data

Option 2: Re-access in another opportunity (i.e. retry the random access)

Option 3: No particular action

Option 4: Follow Reader’s paging/triggering message

We think for the general D2R data transmission, for simplifying device behavior, in case the R2D provides the D2R scheduling for this device (within the timing relationship), device shall follow the received R2D to transmit the D2R regardless it is initial transmission or re-transmission. With this said, the Option 1 may be applicable for dedicated D2R transmission. However, in the case of CBRA msg 1, Option 2 may be the best option, ie., Re-access in another opportunity.

Proposol-1: In case of D2R data transmission failure, device shall follow the received R2D to transmit the D2R re-transmission for general D2R data transmission, and Re-access in another RO opportunity for CBRA MSG1.

## AIoT Device Re-Access

During the email discussion there was a discussion on where/when to perform the re-access during device access failure. There are several options proposed during the email discussion:

Option 1: In the same access occasion

Option 2: In the following access occasion of the same access round

Option 3: In the next access round

Option 4: In the next paging round

Option 5: the round length is adaptive. The round can be adjusted by increasing its length or terminating earlier upon detection of too high collision.

The following terms were proposed by the email rapporteur: Access Occasion, Access round and Paging round as shown in the figure below.

According to the options listed above, we prefer to Option3 as the baseline, since it is the same as RFID. However, Option2 maybe works too, for example, the Ambient device can still randomly select an access occasion out of multiple following up random access occasions, and the reader may extend access occasion more than as configured via Q value.

Proposal-2: RAN2 study converge to Option 3 and Option2 for the device to try to re-access after initial access contention failure i.e., via another access occasion within the current access round or after the current access round.

In addition, we did not see the need to apply the backoff mechanism of NR to the Ambient IoT device.

One possible solution is to introduce the second trigger message within an access round triggered by the initial trigger message. Such a second trigger message only allows the following devices to respond:

- matching the filter criteria provided in the initial trigger message and

- experiencing contention failure during its access attempt responding to the initial trigger message

Proposal-3: Introduce second trigger message following the initial trigger message within the same access round.

To determine whether the device is allowed to respond to the second trigger message, the device needs to acknowledge its status, i.e., whether it experienced contention failure during access attempt responding to the initial trigger message. Such an status acknowledge can be supported upon receiving subsequent R2D transmission of “ACK/NACK” indication after D2R transmission (Msg.1 and Msg.3). Since Msg.2 R2D transmission can be treated as a subsequent R2D transmission of “ACK/NACK” indication after D2R transmission of Msg.1, we propose to support subsequent R2D transmission of “ACK/NACK” indication after D2R transmission of Msg.3 when the second trigger message is supported.

Proposal-4: Support subsequent R2D transmission of “ACK/NACK” indication after D2R transmission of Msg.3 when the second trigger message is supported.

## AS ID for scheduling

Last meeting, there is a discussion on the possibility to use the random ID generated by the device for the scheduling. There is a FFS on if reader assigns the AS ID for scheduling purposes. The negative point to use the random ID generated by the device for scheduling for the AIoT device is that there is a possibility to generate the same ID for different devices, since the communication of the AIoT devices may last for a long time, and then the probability for random ID collision may increase. For this reason we do not think scheduling for the data transmission for the AIoT device based on the ID generated by the AIoT devices is a reliable approach. In addition, in legacy NR system, the ID for scheduling (e.g. C-RNTI) is allocated by the base station. From our perspective, we did not see a reason to differ the AIoT design from legacy NR system design.

Proposal-5: AIoT reader assigns the AS ID (e.g. C-RNTI alike) to AIoT devices for scheduling purposes.

## RACH resource selection for Ambient IoT device

In order to select a time slot to respond to the reader via random access procedure, the AIoT device needs to know the exact time slots that can be used to initiate its random access. In addition, according to the discussion at RAN1, the FDM approach was also discussed on top of the TDM based access. FDM based approach may be used to afford large number of AIoT device access, which allows different devices to access the reader via a different frequency bandwidth at the same time slot. Based on this background, we discuss below what type of parameters the reader should announce before the AIoT device can initiate its random access.

The initial triggering message sent by the reader should include the time slots allocated for RA (e.g. M slots) within one frame or one access round and also the set of frequency location for each time slot. Assuming intended devices information (i.e., filter criteria from CN) is provided in the initial trigger message, each  device matching the filter criteria may generate a random number according to the number of the available time slots (e.g. M) in the initial triggering message for random access. The random number should be in the range [0, (M-1)]. It should be noted that here we do not intend to use Q value based slotted ALOHA due to its drawbacks discussed in the previous section.

Proposal-6: Each AIoT device matching the filter criteria generates a random number according to the number of the available time slots in the initial triggering message for random access if slotted ALOHA based access is chosen for AIoT device.

Based on the random number generated, the AIoT device may start “RACH-slot” counting down since receiving “initial triggering message” command.

For RFID, the slotted ALOHA access mechanism highly relies on the run of “QUERYREP” command, which allows the tag to count down when receiving such command at every slot. If AIoT device uses the similar approach, the AIoT device may need to count down whenever receiving a “Repeat” command from the reader following the initial triggering message. The “Repeat” command may be used by the device to maintain the synchronization or clock tracking. It should be noted that the run of “Repeat” command is a big signalling overhead for the AIoT air interface and also it requires the device to monitor the message from the reader at every time slot, which may cause energy consumption for AIoT device. RAN2/RAN1 needs to study if we adopt the similar approach for AIoT device for the purpose of RA slot count down, and synchronization or clock tracking.

Observation-1: the run of “QUERYREP” command (for RFID) is a big signalling overhead for the AIoT air interface and also it requires the device to monitor the message from the reader at every time slot, which may cause energy consumption for AIoT device.

Proposal-7: RAN2/RAN1 needs to study if we adopt the similar approach (as “QUERYREP” command for RFID) for AIoT device for the purpose of RA slot count down, and synchronization or clock tracking.

In any case, the abovementioned mechanism can only handle the RA slot selection but not the frequency location selection. So then in addition to the RA slot selection, the device may need to randomly selects one frequency location among the available frequency locations for that “RA slot” to send MSG-1 to the reader.

Proposal-8: in addition to the RA slot selection, the device may need to randomly selects one frequency location among the available frequency locations for that “RA slot” to send MSG-1 to the reader.

Meanwhile, it is very important to note that when there are frequency domain resources for each RA slot, RAN2 may need to study additional mechanisms or enhancements for AIoT device access, since anyway the slotted ALOHA mechanism can not be simply reused any more.

As discussed above, an RO in A-IoT can be defined as a combination of a time slot and a frequency location for each time slot (if FDM access is supported). RAN2#126 agreed that “AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message).”, therefore it is nature to assume that the total available ROs (for example, M slot and N frequency locations) is indicated by AIoT paging message.

Currently, both 2-step and 3-steps RA are supported in A-IoT, on the association between available ROs and 2-step and/or 3-steps RA, we have following options:

Option 1: available ROs are associated to 2-step RA only;

Option 2: available ROs are associated to 3-step RA only;

Option 3: available ROs are associated to both 2-setp and 3-steps

On how to indicate the association between available ROs and 2-step and/or 3-steps RAs, we see following two alternatives:

Alternative 1: AIoT paging message explicitly indicates association between available ROs and 2-setp and/or 3-steps ROs;

Alternative 2: Pre-define association between available ROs and 2-step and/or 3-steps RA in the specification.

Since Alternative 1 is much more flexible, we propose to support Alternative 1 in A-IoT system.

Proposal-9: AIoT paging message explicitly indicates following associations between available ROs and 2-setp and/or 3-steps ROs:

- available ROs are associated to 2-step RA only;

- available ROs are associated to 3-step RA only;

- available ROs are associated to both 2-setp and 3-steps.

# Conclusion and Proposal

We have the following observations and proposals:

Observation-1: the run of “QUERYREP” command (for RFID) is a big signalling overhead for the AIoT air interface and also it requires the device to monitor the message from the reader at every time slot, which may cause energy consumption for AIoT device.

Proposol-1: In case of D2R data transmission failure, device shall follow the received R2D to transmit the D2R re-transmission for general D2R data transmission, and Re-access in another RO opportunity for CBRA MSG1.

Proposal-2: RAN2 should converge to Option 3 and Option2 for the device to try to re-access after initial access contention failure i.e., via another access occasion within the current access round or after the current access round.

Proposal-3: Introduce second trigger message following the initial trigger message within the same access round.

Proposal-4: Support subsequent R2D transmission of “ACK/NACK” indication after D2R transmission of Msg.3 when the second trigger message is supported.

Proposal-5: AIoT reader assigns the AS ID (e.g. C-RNTI alike) to AIoT devices for scheduling purposes.

Proposal-6: Each AIoT device matching the filter criteria generates a random number according to the number of the available time slots in the initial triggering message for random access if slotted ALOHA based access is chosen for AIoT device.

Proposal-7: RAN2/RAN1 needs to study if we adopt the similar approach (as “QUERYREP” command for RFID) for AIoT device for the purpose of RA slot count down, and synchronization or clock tracking.

Proposal-8: in addition to the RA slot selection, the device may need to randomly selects one frequency location among the available frequency locations for that “RA slot” to send MSG-1 to the reader.

Proposal-9: AIoT paging message explicitly indicates following associations between available ROs and 2-setp and/or 3-steps ROs:

- available ROs are associated to 2-step RA only;

- available ROs are associated to 3-step RA only;

- available ROs are associated to both 2-setp and 3-steps.

# Reference

- Report of [POST127][033][AIoT] Random Access (Huawei)

- TS38.300