3GPP TSG-RAN WG2 Meeting #127bis	R2-2408551

Hefei, China, 14-18 Oct 2024

Agenda item:	8.3.3

Title:		Discussion on measurement event prediction

Source:		NEC

Document for:	Discussion and Decision

# 1. Introduction

For AI/ML mobility study, the following agreements were reached for the discussion of measurement event predictions at the previous RAN2 meetings:

Agreements:

- At least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction is also allowed.

- Clarifications on what is being as input should be provided with results

- Start with A3 as a baseline.

- Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

This document discusses the scenarios of measurement event predictions and the inputs for the AI/ML model for measurement event prediction based on the agreements RAN2 made for this subtopic.

# 2. Discussion

2.1 Target Scenarios

In RAN2#125bis meeting, it was agreed that the work of measurement event prediction evaluation will start with A3 as a baseline.

As a typical cell deployment scenario, the A3 measurement event is particularly useful for homogeneous cell deployments. Although A3 measurement event could also be applicable to heterogeneous cell deployments, it would be simple to basically assume homogeneous cell deployments in this SI.

Proposal-1: RAN2 to consider homogenous cell deployment scenarios as baseline for the study of measurement event predictions.

Although at last meeting, the discussion of measurement event predictions only focused on the occurrence of a certain measurement event (e.g. A3, A5, etc), we think the mobility event predictions are also in the scope, since the handover procedure will be triggered based on a certain measurement event. Correct prediction on a certain mobility event can not only improve the handover performance but also the system throughput.

In legacy L3-based HO, there are many events for L3 measurement reporting, e.g. A3 or A5 which would be used normally for homogenous cell deployments. In some heterogeneous cell deployments, another event could be useful, e.g. A2 or A4. These events can work well in the L3 HO based on event triggered L3 measurement reporting in most cases especially when the UE is moving slowly or is almost stationary.

However, when the UE is moving faster or cell coverages are not uniformly spread (e.g. cell coverages among neighbouring cells are complicated), the L3 HO based on those events might not work sufficiently well. For example, even though the HO is successfully performed, the UE may stay in the target cell for short period and then perform the HO again. The UE may continue this behaviour subsequently. The issue is known as “short time of stay” or frequent HO. One specific case of frequent HO is a ping-pong HO, where the UE comes back to the source cell right after the HO completion to the target cell.

In the abovementioned mobility scenarios, the UE may be able to stay in the source cell for some more time and skip one HO procedure causing the short time of stay, if the UE’s mobility can be predicted in advance. Note that another well-known problem is HO failure or RLF during the L3 HO and it is to be discussed separately (AI. 8.3.4 HOF/RLF prediction).

Fig.1: short time of stay example (Cell 01->11->02->21)

Proposal-2: RAN2 to consider at least the short time of stay scenario as target scenario, for mobility event prediction.

Proposal-2a: The short time of stay scenario can be further broken down to frequent HO and ping-pong HO as target scenarios, for mobility event prediction.

2.2 Effect of measurement prediction on network performance

Frequent handovers and constant signal strength monitoring in dense network environments can lead to unnecessary power consumption, especially for power-constrained devices. UE-based AI/ML models can predict A3 and A5 events by analysing real-time mobility patterns (e.g., user velocity, direction) and network conditions (e.g., RSRP, SINR, RSRQ). For example, if a user is moving through a densely populated area, the model can predict whether the UE is likely to require a handover based on A3, A5 event prediction. If the device's battery level is low, UE will not send the reports associated with the AI/ML model, preventing unnecessary handover triggers and reducing power consumption. This approach aligns closely with the previously outlined framework for predictive measurement event handling.

Proposal-3: RAN2 can leverage AI/ML prediction of measurement events based on a device’s battery status. If the battery is low, the device will refrain from sending reports associated with AI/ML model, helping to avoid unnecessary handovers, especially in dense network environments.

2.3 Impact of measurement event prediction on HO performance

If the A3 event is predicted ahead of time, the network can proactively prepare for the handover. The prediction could initiate handover preparation steps, such as pre-selecting the target cell, allocating resources in the neighbouring cell, preparing the necessary signalling channels, pre-buffering data in the target cell (for time-sensitive services).This reduces the time between the actual trigger and the completion of the handover, ensuring minimal service interruption and faster transitions between cells.

With proactive A3 event prediction, the network can optimize resource allocation by pre-emptively reserving resources in the target cell. This ensures that when the handover occurs, the necessary radio resources are already available, reducing the likelihood of handover failure due to resource contention.

By predicting A3 events in advance, the network can adjust handover decisions more intelligently. Instead of reacting to every minor signal fluctuation, predictive algorithms can consider the long-term signal trends, mobility patterns, and interference conditions. This reduces the frequency of unnecessary handovers and prevents the UE from oscillating between cells, ultimately improving overall user experience.

Proposal-4: RAN2 need to discuss how the network can make use of the outcome of measurement event prediction to optimize handover performance.

2.4 Metric/KPI for measurement event prediction

During the email discussion for RRM measurement predictions, there are discussions on how to define a prediction accuracy metric. The RSRP difference is assumed to be the prediction accuracy metric for RRM measurement. For measurement event prediction and mobility event prediction, we may use a bit different metric to weigh the prediction accuracy. Since measurement event or mobility event occurs at a certain timeframe, one of the possible metric can be considering the timing difference between the predicted occurrence of the measurement event or HO event and the real time of the occurrence of the event. At some extreme case, the predicted occurrence of the event may never happen, so then basically, the occurrence of the event should be also one of the metrics. A CDF curve may be used to express the trend of the timing difference for multiple instances of event predictions.

Proposal-5: The occurrence of the event, and the timing difference between the predicted occurrence of the event and the real time of the occurrence of the event can be defined as the prediction accuracy metric for measurement event (and mobility event) predictions.

Prediction Efficiency (PE) quantifies how early an A3 handover event is predicted in advance of its occurrence, aiming to optimize resource management and minimize packet drops. This metric is crucial in ensuring that the system reacts promptly to avoid any disruptions or degraded QoS during handovers, particularly in dense and dynamic network environments. Mathematically, PE is the ratio of the time difference between when the A3 event is predicted and when it would naturally trigger based on conventional thresholds, to the time window within which the handover can be successfully executed without packet loss or service interruption.

PE= Tpredicted-TtriggerThandover_window

Where:

- ​Tpredicted is the time at which the A3 event is predicted by the AI/ML model.

- Ttrigger  is the actual time at which the A3 event is triggered based on the standard measurement report.

- Thandover_window  represents the time window during which a handover can be safely initiated and completed without packet drops or service degradation. The Thandover_window  includes TTT, Handover preparation time and handover execution time.

High PE means the system can predict the A3 event well in advance, providing sufficient time to manage resources and execute a handover smoothly. Low PE indicates late predictions, potentially causing packet drops or failed handovers due to insufficient time for preparation. This metric is valuable for evaluating the performance of AI/ML models used in handover management, particularly when optimizing user experience in dense urban scenarios.

Proposal-6: The ratio of the time difference between the A3 prediction time and trigger time to the time window within which handover can be successfully executed without service interruption can be termed as prediction efficiency metric for measurement (and mobility event) predictions.

2.5 parameters for AI/ML model input/output for measurement predictions

We discuss the set of suitable parameters for input and output for the AI/ML model for measurement event prediction. These are discussed as follows.

Input model parameters:

Measured RSRP, RSRQ, SINR, CQI for SSB/CSI-RS:

Prediction Configuration:

- Events to be predicted

- Time instances

Measurement event parameters:

  - Threshold

  - Hysteresis

  - Time-to-Trigger

Output model parameters:

Predicted Triggered Event: The event that is triggered on the real-time and historical data.

Predicted Event Triggering Time: The expected time when the event will occur.

Proposal-7: RAN2 to agree on the following input/output parameters for measurement event prediction:

Input model parameters:

(1)Measured RSRP, RSRQ, SINR, CQI for SSB/CSI-RS

(2)Prediction Configuration:

- Events to be predicted

- Time instances

(3)Measurement event parameters:

- Threshold

- Hysteresis

- Time-to-Trigger

Output model parameters:

(1)Predicted Triggered Event: The event that is triggered on the real-time and historical data.

(2)Predicted Event Triggering Time: The expected time when the event will occur.

# 3. Conclusion

In this contribution we discussed on target scenarios for measurement event prediction and relevance of them in enhancing HO performance along with some input parameters for the AI/ML model and then made the following proposals.

Proposal-1: RAN2 to consider homogenous cell deployment scenarios as baseline for the study of measurement event predictions.

Proposal-2: RAN2 to consider at least the short time of stay scenario as target scenario, for mobility event prediction.

Proposal-2a: The short time of stay scenario can be further broken down to frequent HO and ping-pong HO as target scenarios, for mobility event prediction.

Proposal-3: RAN2 can leverage AI/ML prediction of measurement events based on a device’s battery status. If the battery is low, the device will refrain from sending measurement reports, helping to avoid unnecessary handovers, especially in dense network environments.

Proposal-4: RAN2 need to discuss how the network can make use of the outcome of measurement event prediction to optimize handover performance.

Proposal-5: The occurrence of the event, and the timing difference between the predicted occurrence of the event and the real time of the occurrence of the event can be defined as the prediction accuracy metric for measurement event (and mobility event) predictions.

Proposal-6: The ratio of the time difference between the A3 prediction time and trigger time to the time window within which handover can be successfully executed without service interruption can be termed as prediction efficiency metric for measurement (and mobility event) predictions.

Proposal-7: RAN2 to agree on the following input/output parameters for measurement event prediction:

Input model parameters:

(1)Measured RSRP, RSRQ, SINR, CQI for SSB/CSI-RS

(2)Prediction Configuration:

- Events to be predicted

- Time instances

(3)Measurement event parameters:

- Threshold

- Hysteresis

- Time-to-Trigger

Output model parameters:

(1)Predicted Triggered Event: The event that is triggered on the real-time and historical data.

(2)Predicted Event Triggering Time: The expected time when the event will occur.

# References

[1] RAN2#125bis meeting minutes

[2] R2-2404222 Target scenarios for measurement event prediction

[3] R2-2407360 Discussion on AI aided measurement events prediction