3GPP TSG-RAN WG2 Meeting #127bis	R2-2408553

Hefei, China, 14 – 18 October 2024

Agenda item:	8.1.2.2

Source:	Nokia

Title:	LCM for UE-side Beam Management

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this paper, we focus on the issues related to functionality-based LCM for UE-side AI/ML beam management, which have not been discussed in RAN2 yet and are independent from the LS sent out to RAN1 [1]. We first review the definitions of functionality states and then discuss the procedural impacts for reporting applicable functionalities and for controlling functionalities, e.g., activating, deactivating, switching, or falling back from functionalities. Next, we discuss the retention of applicable functionalities through RRC state transitions in the same cell, functionality performance monitoring, and the configuration of multiple functionalities simultaneously.

# 2	Discussion

## 2.1	Functionality-Based LCM

### 2.1.1	Definitions

During RAN2#127 [2], we agreed to the following.

- Activated functionalities refers to functionalities already enabled for performing inference

We think that from the NW perspective, it is important to include that an activated functionality is not only performing inference, but that it is also reporting the results to the NW. Therefore, we propose to update the definition.

Proposal 1: RAN2 to agree that “Activated functionalities refer to functionalities already enabled for performing inference and reporting their outputs to the NW.

We have yet to agree on definitions for deactivation, fallback, and switching.

A deactivated functionality is an AI/ML functionality for which the UE has received a configuration via a CSI-ReportConfig, but which is neither configured to perform inference nor to report its outputs to the NW.

Proposal 2: A deactivated functionality is an AI/ML functionality for which the UE has received a radio configuration via a CSI-ReportConfig, but which is neither configured to perform inference nor to report its outputs to the NW.

Functionality fallback to non-AI/ML is the procedure of simultaneously deactivating one functionality and activating a non-AI/ML feature.

Proposal 3: Functionality fallback to non-AI/ML means deactivating one AI/ML-enabled functionality and activating a non-AI/ML feature of the same type, e.g., Beam Management.

Functionality switching is the procedure of simultaneously deactivating one functionality and activating another.

Proposal 4: Functionality switching means simultaneously deactivating one AI/ML-enabled functionality and activating another of the same type, e.g., Beam Management.

### 2.1.2	Procedures

The following agreements from RAN2#125bis [3] will be used as a basis for the next part of our discussion, namely functionality management for a UE-sided (functionality) based on the “network decision, network-initiated” approach, which was defined in the TR 38.843 [4], shown following the agreements.

Agreements:

1	For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline.  The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

2	“UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

The following figure, Figure 1, extracted from TR 38.843 [4], Section 7.2.1.1, illustrates “network decision, network-initiated” AI/ML LCM. Of note in this call flow is that the UE is not pre-configured with any sort of event triggered LCM actions and that all aspects of configuration originate from the network, including measurement and reporting configuration and functionality activation, deactivation and switching.

Figure 1: Figure 7.2.1.1-1: Network decision, network-initiated AI/ML management [4]

Observation 1: In “network decision, network-initiated” AI/ML LCM, all aspects of functionality configuration originate from the NW, including measurement and reporting configuration and functionality activation, deactivation and switching.

Sometime after a functionality has been activated, it might become inapplicable. The change in applicability would have triggered a new applicable functionality report from the UE to the gNB, on which the gNB would need to make a decision. Depending on whether the functionality is crucial for NW operation, the gNB could take one of a few different actions:

- Deactivate an activated AI/ML functionality.

- Deactivate an activated AI/ML functionality, simultaneously activating a different applicable AI/ML functionality.

- Deactivate an activated AI/ML functionality, simultaneously activating a non-AI/ML feature.

As written in our RAN1 contribution [5], CSI reports for both BM-Case1 and BM-Case2 based on configurations for aperiodic CSI reporting (e.g., CSI-AssociatedReportConfigInfo) can already be triggered on-demand using a DCI, and CSI reports based on configurations for semi-persistent CSI reporting can already be triggered on-demand using a MAC-CE. In both cases, the functionality is activated through a DCI or MAC-CE, the functionality is deactivated implicitly at the end of reporting, and the switching or fallback of functionalities is implemented implicitly through the choice of which report to activate.

Observation 2: The specifications already support functionality activation, deactivation, switching, and fallback for aperiodic and semi-persistent CSI reporting using DCI and MAC-CE, respectively.

Periodic CSI reporting configurations can be activated or deactivated through RRC Reconfiguration, e.g., by adding or modifying, or removing a CSI-ReportConfig. The typical RRC Reconfiguration procedure requires the transmission of two messages: RRCReconfiguration and RRCReconfigurationComplete, while for the aperiodic and semi-persistent, only one message, a DCI or MAC-CE, which can be transmitted alongside normal data traffic, is required.

When using the RRCReconfiguration message to configure a BM functionality, the ToAddMod and ToRemove structures can be used, wherein new CSI-ReportConfigs can be added or modified, or removed.

Observation 3: The specifications support functionality activation, deactivation, switching, and fallback for periodic CSI reporting using RRC Reconfiguration of CSI-MeasConfig by adding, modifying, and/or releasing CSI-ReportConfigs.

Proposal 5: Legacy procedures for activating, deactivating, and switching between aperiodic, semi-persistent, and periodic CSI Reports using DCI, MAC-CE, and RRC Reconfiguration (csi-ReportConfigToAddModList, csi-ReportConfigToReleaseList) can be reused to implement functionality activation, deactivation, switching, and fallback.

### 2.1.3	Fallback

As written in Section 2.1.1, functionality fallback means deactivating one AI/ML-enabled functionality and activating a non-AI/ML feature. The purpose of fallback is to mitigate performance losses by reverting to a non-ML method for beam management. However, the NW may fall into unnecessary ping-pong between non-ML and ML-enabled beam management configurations, which could lead to signalling overhead to disabling/enabling ML functionality on one hand and imposing unnecessary activation delays on the other hand.

Because the performance of the AI/ML functionality was satisfactory prior to the decision to fall back, it could be reasonably assumed that the AI/ML functionality will perform well once again.

Observation 4: Because the performance of the AI/ML functionality was satisfactory prior to the decision to fall back, it could be reasonably assumed that the AI/ML functionality will perform well once again.

Therefore, monitoring an AI/ML functionality during fallback could be useful to help the NW determine when to exit fallback and resume the use of the AI/ML functionality to make beam management decisions. During this type of fallback, the AI/ML functionality would still be active, but it could be configured with a lower reporting frequency.

Proposal 6: Discuss signalling to enable fallback without deactivating an AI/ML beam management functionality such that performance monitoring of an AI/ML functionality can continue during fallback.

## 2.2 Retention of Applicable Functionalities in IDLE and INACTIVE

Apart from identifying the basic framework for reporting applicable functionalities between the network and the UE, changes to the UE’s RRC state, i.e., transitioning between CONNECTED, IDLE, and INACTIVE, may impact the procedure for applicable functionality reporting. As an example, it is unclear whether the UE should be mandated to perform the applicable functionality reporting procedure after Step3   when the UE transitions to IDLE or INACTIVE and returns to CONNECTED mode.

Proposal 7: RAN2 to discuss whether applicable functionalities are stored prior to transitioning to RRC_INACTIVE or RRC_IDLE and whether applicable functionality reporting is supported while UE transitions from RRC_INACTIVE or RRC_IDLE to RRC_CONNECTED.

## 2.3	Performance Monitoring

### 2.3.1	Overview

During the Release 18 SI, two types were introduced for performance monitoring of UE-side functionalities.

Type 1, Option 1, enables the NW to make LCM decisions based on performance monitoring metrics calculated at the NW. Type 1, Option 2, also known as UE-assisted performance monitoring, enables the NW to make LCM decisions based on performance monitoring metrics calculated and reported by the UE to the NW. Type 1 performance monitoring, including Option 1 and Option 2, may require signalling enhancements such that the UE can be configured to report additional measurements for use by the NW in calculating a performance metric or such that the UE can be configured to calculate and report a performance metric to the NW.

Type 2 performance monitoring enables the UE to make LCM decisions based on performance monitoring metrics calculated by the UE. Type 2 does not require any RAN2-related signalling enhancements as it is mainly applicable to model-level performance monitoring.

In RAN1#118 [6], four alternatives were agreed as performance monitoring metric for further study. As such, any discussions on the specific signaling or triggering of additional measurement reporting or KPI calculation in RAN2 should be discussed after RAN1 progresses on deciding the performance monitoring metric.

Proposal 8: RAN2 agrees that signalling is only needed to support Type 1 performance monitoring metrics. There is no impact to RAN2 for Type 2 performance monitoring metrics.

### 2.3.2	System-Level Performance Monitoring

Monitoring Configuration 0 (System-Level Performance Monitoring). Configure a BM functionality, which outputs spatial or temporal beam predictions and perform monitoring solely based on inference output and other metrics, e.g., system performance (Figure 2).

- L1-RSRP Measurements for Inference include measurements of Set B for BM Case-1 or Set A measurements within observation window  for BM Case-2.

- Functionality output includes spatial beam predictions of Set A beams for BM Case-1 or temporal beam predictions of Set A beams for BM Case-2, which are required as inputs to the BM functionality.

- The NW uses the output of System-Level Performance Monitoring to make functionality-based LCM decisions.

Figure 2: System-Level AI/ML Functionality Performance Monitoring

Observation 5: There is no specification impact for the use of NW-based system-level performance monitoring to enable BM functionality performance monitoring.

### 2.3.3	Type 1 Performance Monitoring

The following Monitoring Configurations and block diagrams illustrate potential variations of NW-based monitoring with and without UE assistance. In RAN1, this is called Type 1 performance monitoring. In every case of Type 1 performance monitoring, an AI/ML functionality is activated, and additional information is used to determine the quality of the performance of the AI/ML functionality.

#### 2.3.3.1	Type 1, Option 1 Performance Monitoring

Monitoring Configuration 1 (Performance Monitoring Type 1, Option 1). Configure a BM functionality, which outputs spatial or temporal beam predictions and perform monitoring based on additional measurements of Set A considering a procedure as follows (Figure 3):

- L1-RSRP Measurements for Inference are configured by the NW and include measurements required as inputs to the BM functionality, e.g., measurements of Set B for BM Case-1 or measurements of Set A within the observation window for BM Case-2.

- The BM Functionality Outputs, e.g., L1-RSRP and/or RS index, are reported to the NW.

- L1-RSRP Measurements for Monitoring, required for NW-based KPI calculation, are reported by the UE.

- KPI calculation is based on the measurement results obtained in (3) and the functionality output obtained in (2).

- The NW uses the output of the NW-based KPI calculation to make NW-based functionality-based LCM decisions.

Figure 3: Type 1, Option 1 AI/ML Functionality Performance Monitoring

#### 2.3.3.2	Type 1, Option 2 Performance Monitoring

Monitoring Configuration 2 (Performance Monitoring Type 1, Option 2). Configure a BM functionality, which outputs spatial or temporal beam predictions and perform monitoring based on assistance provided by the UE, e.g., UE-based KPI calculation considering a procedure as follows (Figure 4):

- L1-RSRP Measurements for Inference are configured by the NW and include measurements required as inputs to the BM functionality, e.g., measurements of Set B for BM Case-1 or measurements of Set A within the observation window for BM Case-2.

- The BM Functionality Outputs, e.g., L1-RSRP and/or RS index, are reported to the NW.

- L1-RSRP Measurements for Monitoring are used for UE-based KPI calculation,

- UE-based KPIs are calculated and reported to the NW and used to make NW-based functionality-based LCM decisions.

Figure 4: Type 1, Option 2 AI/ML Functionality Performance Monitoring

Observation 6: Based on discussions on candidates for KPI calculation for BM functionality performance monitoring in RAN1, for all Type 1 performance monitoring options, the outputs of the activated BM functionality are used by the NW (Option 1) or by the UE (Option 2) for KPI calculation.

Observation 7: Based on discussions on candidates for KPI calculation for BM functionality performance monitoring in RAN1, for Type 1 Option 1, additional L1-RSRP measurements are reported to the NW alongside the functionality outputs, and for Type 1 Option 2, performance monitoring KPIs are reported to the NW alongside the functionality outputs.

Proposal 9: RAN2 only to support performance monitoring of an activated functionality through reporting, e.g., additional L1-RSRP measurements or performance monitoring KPIs for both BM-Case1 and BM-Case2.

## 2.4 Reporting of Applicable Functionalities for neighbouring cell

In [7], the Rapporteur provided an open question based on companies’ contributions on whether applicable functionalities should be considered for neighbour cells. Highlighting our views, UAI framework can only report UE’s condition/information changes on serving cell (i.e. the UE is only required to monitor condition changes in serving cell in existing UAI framework). Evaluation of applicable functionalities is per-cell, and we don’t consider necessarily keeping the same functionality active through handover (e.g., to be used during the handover process). Since the existing UAI procedure can only work for reporting applicable functionalities of the serving cell reporting, if conditions in neighbour cells change, the UE will not be able to use UAI for applicable functionality reporting. As such, it is worth discussing the feasibility of existing UAI on reporting applicable functionalities of neighbour cells in RAN2.

Proposal 10: RAN2 to discuss whether applicable functionality reporting for neighbour cells is supported in Release 19.

# 3	Conclusion

This document has made the following observations:

Observation 1: In “network decision, network-initiated” AI/ML LCM, all aspects of functionality configuration originate from the NW, including measurement and reporting configuration and functionality activation, deactivation and switching.

Observation 2: The specifications already support functionality activation, deactivation, switching, and fallback for aperiodic and semi-persistent CSI reporting using DCI and MAC-CE, respectively.

Observation 3: The specifications support functionality activation, deactivation, switching, and fallback for periodic CSI reporting using RRC Reconfiguration of CSI-MeasConfig by adding, modifying, and/or releasing CSI-ReportConfigs.

Observation 4: Because the performance of the AI/ML functionality was satisfactory prior to the decision to fall back, it could be reasonably assumed that the AI/ML functionality will perform well once again.

Observation 5: There is no specification impact for the use of NW-based system-level performance monitoring to enable BM functionality performance monitoring.

Observation 6: Based on discussions on candidates for KPI calculation for BM functionality performance monitoring in RAN1, for all Type 1 performance monitoring options, the outputs of the activated BM functionality are used by the NW (Option 1) or by the UE (Option 2) for KPI calculation.

Observation 7: Based on discussions on candidates for KPI calculation for BM functionality performance monitoring in RAN1, for Type 1 Option 1, additional L1-RSRP measurements are reported to the NW alongside the functionality outputs, and for Type 1 Option 2, performance monitoring KPIs are reported to the NW alongside the functionality outputs.

And the following proposals:

Proposal 1: RAN2 to agree that “Activated functionalities refer to functionalities already enabled for performing inference and reporting their outputs to the NW.

Proposal 2: A deactivated functionality is an AI/ML functionality for which the UE has received a radio configuration via a CSI-ReportConfig, but which is neither configured to perform inference nor to report its outputs to the NW.

Proposal 3: Functionality fallback to non-AI/ML means deactivating one AI/ML-enabled functionality and activating a non-AI/ML feature of the same type, e.g., Beam Management.

Proposal 4: Functionality switching means simultaneously deactivating one AI/ML-enabled functionality and activating another of the same type, e.g., Beam Management.

Proposal 5: Legacy procedures for activating, deactivating, and switching between aperiodic, semi-persistent, and periodic CSI Reports using DCI, MAC-CE, and RRC Reconfiguration (csi-ReportConfigToAddModList, csi-ReportConfigToReleaseList) can be reused to implement functionality activation, deactivation, switching, and fallback.

Proposal 6: Discuss signalling to enable fallback without deactivating an AI/ML beam management functionality such that performance monitoring of an AI/ML functionality can continue during fallback.

Proposal 7: RAN2 to discuss whether applicable functionalities are stored prior to transitioning to RRC_INACTIVE or RRC_IDLE and whether applicable functionality reporting is supported while UE transitions from RRC_INACTIVE or RRC_IDLE to RRC_CONNECTED.

Proposal 8: RAN2 agrees that signalling is only needed to support Type 1 performance monitoring metrics. There is no impact to RAN2 for Type 2 performance monitoring metrics.

Proposal 9: RAN2 only to support performance monitoring of an activated functionality through reporting, e.g., additional L1-RSRP measurements or performance monitoring KPIs for both BM-Case1 and BM-Case2.

Proposal 10: RAN2 to discuss whether applicable functionality reporting for neighbour cells is supported in Release 19.

# 4	References

[1] R2-2407848, “LS on applicable functionality reporting for beam management UE-sided model”, TSG RAN WG2, August 2024

[2] R2-24079xx, “Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands (R2#127 Chair Notes)”, ETSI MCC, October 2024

[3] R2-2404102, “Report of 3GPP TSG RAN WG2 meeting #125bis, Changsha, China”, ETSI MCC, April 2024

[4] TR 38.843 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 3GPP, 2023-12

[5] R1- (R1 contribution on BM)

[6] R1-2407554, “FL summary #5 for AI/ML in beam management, 3GGP TSG RAN WG1 #118 – August 19th – August 23rd”, August 2024

[7] R2-2406381– Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2.