3GPP TSG-RAN WG2 Meeting #127bis	R2-2408554

Hefei, China, 14 – 18 October 2024

Agenda item:	8.1.2.3

Source:	Nokia

Title:	LCM for UE-side Positioning

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This contribution addresses the following agenda item.

8.1.2.3	LCM for Positioning use case

Contributions should focus on LCM for UE-sided model, but can discuss NW-sided model and should focus on 1st priority positioning use cases.  Aspects related to data collection should be covered in 8.1.3

In this paper, we discuss and propose a call flow for applicable functionality determination, reporting, and configuration for the AI/ML positioning use case for UE-side functionalities.

# 2	Discussion

## 2.1	Status on Agreements for UE-side Positioning

During RAN2#126 [1], the following agreements were made on AI/ML positioning.

Agreements

- The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities.  FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality.

- wait for RAN1 for associate ID discussion

- At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation.  FFS further details on signaling enhancements

Based on these agreements, we can at least exchange supported functionalities through RequestCapabilities and ProvideCapabilities and configure an applicable functionality for inference with RequestLocationInformation and report functionality outputs to the LMF through ProvideLocationInformation.

## 2.2	Related Agreements from Beam Management

During RAN2#127 [2], a call flow for applicable functionality determination, reporting, and activation was agreed for the beam management use case. The details are summarized in this section to avoid duplicating the discussion for AI/ML positioning for aspects agreed for beam management that can be transferred to the positioning use case in principle.

The first two steps in the call flow were uncontroversial. The RRC capability exchange, i.e., UECapabilityEnquiry and UECapabilityInformation, is used for the UE to report supported functionalities, where the complete set of supported functionalities can be inferred from the valid combinations of the reported capability parameters.

Step 3 covers three main aspects: configuration for the transmission of UAI carrying an applicable functionality report; provision by the NW of NW-side additional condition(s); and an FFS for the provision by the NW of configuration details related to the previously reported supported functionalities, where the FFS relates to the level of detail, if any, of the configuration provided in this step, which could range from a partial configuration to a full configuration.

Step 4 enables the UE to report applicable functionalities in two scenarios: as an update to previously reported applicable functionalities or as a response to a NW configuration or request, e.g., as an initial applicable functionality report.

Step 5 enables the NW to configure the UE with a functionality which has been determined to be applicable. There is an FFS on this item because it is still unclear to what extent configuration details are provided in Step 3. It is also left FFS whether the configuration in this step activates the functionality.

## 2.3	Applicable Functionality Determination for Positioning

Based on the agreements discussed in Section 2.1, we have yet to agree on a mechanism to provide the UE with NW-side additional condition(s) and any subset of configuration information that the UE could use to determine applicable functionalities.

Observation 1: RAN2 has yet to agree on a mechanism to provide the UE with NW-side additional condition(s) and any subset of configuration information that the UE could use to determine applicable functionalities.

Proposal 1: ProvideAssistanceData will be used to transfer NW-side additional condition(s) and any subset of configuration information that the UE could use to determine applicable functionalities. FFS on the content of NW-side additional condition(s) and any subset of configuration information, pending RAN1 discussion.

As discussed in Section 2.2, for the beam management use case for UE-side models, a call flow for applicable functionality determination and reporting was agreed [2]. The UECapabilityEnquiry and UECapabilityInformation were uncontroversial, and it was agreed that the UECapabilityInformation would provide information on “supported functionalities”. We suggest using the same approach for UE-side AI/ML positioning.

Proposal 2: For applicability reporting for UE-side AI/ML positioning, adopt LPP RequestCapabilities as “Step 1” to request supported functionalities.

Proposal 3: For applicability reporting for UE-side AI/ML positioning, adopt LPP ProvideCapabilities as “Step 2” to provide supported functionalities.

Step 3 diverges from the beam management use case. In beam management, it was agreed that applicable functionalities could be updated with UAI signalling, which does not exist in the LPP protocol. However, we can address the provision of NW-side additional condition(s) and the FFS on the nature of the assistance data. LPP supports the transmission of assistance data from the LMF to the UE via the ProvideAssistanceData message. Therein, at least NW-side additional condition(s) could be provided, and it could be left FFS to what level of detail any additional configuration information is provided.

Proposal 4: For applicability reporting for UE-side AI/ML positioning, adopt LPP ProvideAssistanceData as “Step 3” to provide NW-side additional condition(s) to be defined by RAN1, leaving FFS the level of detail of the assistance data provided to the UE. The UE would use this information to determine the applicability of a functionality.

In Step 4, the UE reports its applicable functionalities. The applicable functionalities are determined based on the supported functionalities provided in “Step 2” and the assistance data provided in “Step 3”. This step provides the initial report of applicable functionalities, but it could be repeated as an update to applicable functionalities.

Proposal 5: For applicable functionality reporting for UE-side AI/ML positioning, UE transmits an applicable functionality report as “Step 4” to report applicable functionalities. The message can be sent as a response to assistance data used in the determination of applicability, or unsolicited, based on functionalities becoming inapplicable or applicable. FFS the LPP message used to carry the applicable functionality report.

In Step 5, the NW can optionally provide the UE additional AssistanceData containing, for example, the PRS configuration to be used during inference, if not previously provided. The nature of the AssistanceData provided in this step depends on further RAN1 progress.

Proposal 6: For applicable functionality configuration for UE-side AI/ML positioning, the LMF can optionally provide the UE additional AssistanceData containing, for example, the PRS configuration to be used during inference in “Step 5”, if not previously provided. FFS on the content of the AssistanceData provided during this step, based on RAN1 progress.

In Step 6, the NW uses the applicable functionality report to determine a configuration for UE-side AI/ML positioning and transmits an LPP RequestLocationInformation containing a configuration for an applicable functionality. For the beam management use case, the initial state, i.e., activated or deactivated, was left FFS. For positioning, it seems that based on legacy procedures, the UE should immediately start to perform measurements and report measurements (case 2a) or positioning estimates (case 1), but as it is still under discussion for the beam management use case, and we are striving for as much uniformity in the procedures as possible, we think that it is worth discussing first.

Proposal 7: For applicable functionality configuration for UE-side AI/ML positioning, adopt LPP RequestLocationInformation as “Step 6” to configure the UE with an applicable functionality for AI/ML positioning. FFS whether the functionality is initially activated.

The following call flow incorporates each of the steps 1 – 6 proposed.

Proposal 8: Adopt the call flow for applicability determination and reporting for UE-side AI/ML positioning.

Another very important issue for the positioning use case that needs to be resolved by consulting with RAN1 is whether AI/ML positioning is considered as one or more new positioning method(s) or if it is considered as enhancements to existing LPP positioning methods that are based on inference done using trained model or models with the new measurements defined by RAN1 as inference data (input to the model(s)). This is important to know since LPP signaling are positioning method specific. Knowing this, we can decide whether we need to introduce a set of AI/ML position specific Information Elements as positioning methods IEs or if we need to extend the existing positioning methods IEs.

Proposal 9: RAN2 should send an LS to RAN1 with a list of questions on LCM for positioning use case similar to what was done for beam management use case. We can ask RAN1 about definition of functionalities for positioning use case, whether AI/ML positioning is one or more new positioning method(s) or if it is an enhanced implementation of existing positioning methods using models, about applicability of NW-side additional conditions, associated ID use etc.

# 3	Conclusion

This document has made the following observations:

Observation 1: RAN2 has yet to agree on a mechanism to provide the UE with NW-side additional condition(s) and any subset of configuration information that the UE could use to determine applicable functionalities.

And the following proposals:

Proposal 1: ProvideAssistanceData will be used to transfer NW-side additional condition(s) and any subset of configuration information that the UE could use to determine applicable functionalities. FFS on the content of NW-side additional condition(s) and any subset of configuration information, pending RAN1 discussion.

Proposal 2: For applicability reporting for UE-side AI/ML positioning, adopt LPP RequestCapabilities as “Step 1” to request supported functionalities.

Proposal 3: For applicability reporting for UE-side AI/ML positioning, adopt LPP ProvideCapabilities as “Step 2” to provide supported functionalities.

Proposal 4: For applicability reporting for UE-side AI/ML positioning, adopt LPP ProvideAssistanceData as “Step 3” to provide NW-side additional condition(s) to be defined by RAN1, leaving FFS the level of detail of the assistance data provided to the UE. The UE would use this information to determine the applicability of a functionality.

Proposal 5: For applicable functionality reporting for UE-side AI/ML positioning, UE transmits an applicable functionality report as “Step 4” to report applicable functionalities. The message can be sent as a response to assistance data used in the determination of applicability, or unsolicited, based on functionalities becoming inapplicable or applicable. FFS the LPP message used to carry the applicable functionality report.

Proposal 6: For applicable functionality configuration for UE-side AI/ML positioning, the LMF can optionally provide the UE additional AssistanceData containing, for example, the PRS configuration to be used during inference in “Step 5”, if not previously provided. FFS on the content of the AssistanceData provided during this step, based on RAN1 progress.

Proposal 7: For applicable functionality configuration for UE-side AI/ML positioning, adopt LPP RequestLocationInformation as “Step 6” to configure the UE with an applicable functionality for AI/ML positioning. FFS whether the functionality is initially activated.

Proposal 8: Adopt the call flow for applicability determination and reporting for UE-side AI/ML positioning.

Proposal 9: RAN2 should send an LS to RAN1 with a list of questions on LCM for positioning use case similar to what was done for beam management use case. We can ask RAN1 about definition of functionalities for positioning use case, whether AI/ML positioning is one or more new positioning method(s) or if it is an enhanced implementation of existing positioning methods using models, about applicability of NW-side additional conditions, associated ID use etc.

# 4	References

[1] R2-2406202, “RAN2#126 Meeting Report”, ETSI MCC, August 2024

[2] R2-2407902, “Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands (R2#127 Chair Notes)”, ETSI MCC, October 2024