3GPP TSG-RAN WG2 Meeting #127bis	R2-2408555

Hefei, China, 14 – 18 October 2024

Agenda item:	8.1.5

Source:	Nokia

Title:	Model transfer/delivery

WID/SID:	NR_AIML_air-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This paper analyses the limitations of model transfer/delivery options discussed in TR 38.843 and proposes an additional alternative option to mitigate the issues.

# 2	Discussion

In RAN1, it has been discussed that pending further study, model delivery/transfer case z4, an open format model with known structure, could be supported for 2-sided CSI compression. Information that are part of the known model to be transferred are: model parameters (in Option 3), dataset (in Option 4) and reference model (Option 5). RAN1 will continue further discussion on each of these options with potential identified issues. From the RAN2 viewpoint, information exchange protocols and signalling could be considered agnostic to the ML model and data being transferred and it should be compatible to any model transfer case studied in future. It is up to the higher layers (in the UE and NW) to decide how to interpret the incoming information towards the entity. We also see benefits of having unified discussion instead of having separate solutions and discussion for model transfer, for each use case separately.

Note the following agreements from RAN1#118 [1].

Agreement From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects - Model type/backbone (e.g., Transformer, CNN and so on) - In case model type is a neural network - Number of layers - Layer types/structure (e.g., full connected, activation layer and so on) - Layer size (e.g., the number of parameters of a layer) - Connection between different layers - model input/output related information Conclusion From RAN1 perspective, model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported) Agreement RAN1 is recommending extending the study of the Model identification, and Model transfer/Model delivery based on RAN1 understanding the study is not completed.

And for reference, the following are the Model Delivery/Transfer Cases [2].

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top. | Outside 3GPP Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format. | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format. | 3GPP Network | NW-side |
| z3 | model transfer in open format. | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known. | 3GPP Network | NW-side |
| Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. |  |  |  |

Additionally, 4 solutions for model transfer/delivery, each with two sub-cases were discussed during the SI [2].

Solutions 1-3 define gNB-based, CN-based, and LMF-based transfer/delivery, and the sub-options a, and b, distinguish between control plane and user plane, respectively. Solutions 4 are user plane solutions where 4a is defined for an OTT server and solution 4b is defined for OAM. When the topic was discussed, it was assumed that control and data transfer would happen on the same plane, i.e., control plane for solutions 1a, 2a, and 3a, and user plane for solutions 1b, 2b, 3b, 4a, and 4b.

## 2.1	Control Plane Model Transfer

### 2.1.1	Study Item Outcomes

Each of the control plane solutions: 1a; 2a; and 3a, identified issues with model sizes > 45kBytes based on the existing number of RRC segments. Furthermore, the specification only allows for the use of RRC segmentation in specific cases, and there are downsides to RRC segmentation, which are explained in the next section. For case 1a, the RRC segment size and segmentation limitation are unresolvable. For case 2a and 3a, respectively, it is proposed to use NAS layer and LPP segmentation, respectively.

Observation 1: Control plane model transfer/delivery solutions are limited by maximum message sizes, even when enabling segmentation on their respective protocols.

Because each of these solutions are implemented over the control plane, they are inherently controllable by the NW.

Observation 2: Control plane model transfer/delivery solutions enable NW control.

### 2.1.2	Evaluation

To support control plane model transfer, two main possibilities have been proposed: increase the maximum number of RRC segments; and introduce a new low-priority SRB.

Low-Priority SRB

Increasing the number of RRC segments could resolve the issue of supporting the transfer of larger ML models. However, the following excerpt from 38.300 [3] exposes an issue with increasing the number of segments.

“An RRC message may be segmented in case the size of the encoded RRC message PDU exceeds the maximum PDCP SDU size. Segmentation is performed in the RRC layer using a separate RRC PDU to carry each segment. The receiver reassembles the segments to form the complete RRC message. All segments of an RRC message are transmitted before sending another RRC message. Segmentation is supported in both uplink and downlink as specified in TS 38.331 [12].”

Note that “all segments of an RRC message are transmitted before sending another RRC message.” That means that once a model transfer begins, no other RRC messages can be transmitted, regardless of the SRB’s priority, so a new low-priority SRB will not help with adapting the CP to model transfer.

The behaviour for the transmission of an app layer measurement report [4] further illustrates the uninterruptible nature of segmented RRC traffic. Even when reporting is paused, it is required that remaining segments of a “segmented MeasurementReportAppLayer message” are submitted “to lower layers for transmission.”

Observation 3: A new low-priority SRB will not overcome the issue that a segmented RRC message must be completely transmitted prior to transmitting a new RRC message.

RRC Segmentation

According to 38.323 [5], “the maximum supported size of a PDCP SDU is 9000 bytes.” And, according to 38.306 [6], “the RRC buffer size is defined as the maximum overall RRC configuration size that the UE is required to store. The RRC buffer size is 45Kbytes.”. Therefore, 5 segments are allowed by the current specification.

Of course, 5G supports high download speeds, exceeding 1gbps under very good channel conditions. However, a diverse, and sometimes fast changing, set of channel conditions and bandwidths are experienced by UEs. For example, a UE could be connected to a cell with a 10 or 20MHz bandwidth compared to a cell with a 50MHz or 100MHz bandwidth, and based on the channel conditions could be configured with a single layer through four-layer reception and an MCS with a low bit density, such as QPSK, or one with a high bit density, such as 256QAM. Due to the highly dynamic nature of the UE’s channel, available cells, and cell loading, we make some basic assumptions to estimate an average throughput and evaluate the time to download a model.

Here, we can consider some scenarios related to different model sizes. For simplicity, we will use models of 1MB, 10MB, and 100MB as examples. We will be liberal with our estimate and assume the following for the reception.

- 2-layer MIMO

- MCS 16 with 64QAM

- 100MHz, 30kHz SCS TDD (70% DL) / 20MHz, 15kHz SCS FDD

- 2 symbols allocated to the control channel

- Ignore the resources taken by reference symbols

- The UE is allowed 20% (54RB / 20RB) of the resource blocks per slot.

- The UE will receive a transmission per slot.

- No retransmissions required.

Table 5.1.3.1-2, MCS index table 2 for PDSCH [7], indicates a modulation order of 6 (64QAM) and code rate of 719 for MCS 16. Each 64QAM symbol represents 6 bits. Given our assumption about the control channel occupying 2 OFDM symbols, we are left with the following bits per resource block.

bprb= 6*7191024*14 OFDM Symbols-2 OFDM Symbols*12RE1RB=607bitsRB

In the case of our TDD cell, where 70% of the resources are dedicated to the DL, we have 2000 slots, 54RB available to the UE per slot and 2 layers to transmit on. The following throughput is calculated.

70%*2000 slots*54RBslot*2 layers*607bitsRB=91.8Mbps

And in the case of our FDD cell, we have 1000 slots, 20RB available to the UE per slot, and 2 layers to transmit on. The following throughput is calculated.

1000 slots*20RBslot*2 layers*607bitsRB=24.3Mbps

In Table 1 below, the throughputs, based on the assumptions provided previously, are applied to models of sizes 1MB, 10MB, and 100MB for the 100MHz and 20MHz cells to determine the model transfer time.

Table 1: Calculation of Number of Segments and Time to Transfer Models for CP-based Model Transfer

| Model Size | Number of Segments | Time to Transfer (100MHz) | Time to Transfer (20MHz) |
| --- | --- | --- | --- |
| 1MB | 112 | 87ms | 329ms |
| 10MB | 1120 | 870ms | 3.29s |
| 100MB | 11200 | 8.71s | 32.9s |

Observation 4: Under relatively good channel conditions and while connected to a cell with a high bandwidth, a small data transfer, e.g., a 1MB model transfer, could take nearly 100ms, and larger data transfers, e.g., 10-100MB model transfers, could take nearly 10 seconds.

Observation 5: Under relatively good channel conditions and while connected to a cell with a medium bandwidth, a small data transfer, e.g., a 1MB model transfer, could take more than 300ms, and larger data transfers, e.g., 10-100MB model transfers, could take more than to 33s.

The RRC message transmission delay associated with using the control plane to transfer models is unacceptable even when the model is small and channel conditions are good. During the time to transfer a model over RRC, a handover command or a voice call could be delayed.

Proposal 1: Do not increase the number of RRC segments to support control plane model transfer, e.g., to support control plane model transfer/delivery in cases 1a and 2a.

## 2.2	User Plane Model Transfer

Model transfer over the user plane resolves the model size, continuity, and QoS problems. However, the following evaluations were provided for network controllability for the UP cases.

| Case | A3. Network Controllability |
| --- | --- |
| 1b (gNb) | Management and interaction between UE and gNB appear to be feasible but not supported |
| 2b (CN) | gNB cannot perform model management directly |
| 3b (LMF) | Management and interaction between UE and gNB is not supported |
| 4a (OTT) | Model transfer/delivery is transparent to RAN |
| 4b (OAM) | gNB cannot perform model management directly |

The currently relevant use case is CSI compression for a two-sided model. Therefore, we will focus on 1b, 2b, 4a, and 4b, since 3b considers an LMF-based model transfer, which wouldn’t be a relevant entity for CSI compression. It is clear that from the analysis during the SI, control is an unresolved problem when it comes to model transfer over UP.

Observation 6: The user plane model transfer/delivery solutions discussed during the SI do not satisfy network controllability requirements.

## 2.3	Hybrid model transfer

### 2.3.1	Overview

While it may be feasible to push CP data volume in the short term for low complexity models, this route might limit model size in the future, or require further expansion to CP data capacity. The UP has sufficient capacity to transfer/deliver larger models, but the UP does not natively support control signalling. For network-controlled model delivery, these two main options have emerged in solutions developed through discussions in the past meetings: CP-based and UP-based model delivery. For CP-based model delivery, the drawback is the data volume limitation, and its advantage is its native control signalling. For UP-based model delivery, the drawback is its lack of control signalling, and its advantage is that it is capable of a much larger data volume. To support UP-based model transfer/delivery, a control mechanism must be defined, which could be a UP model transfer/delivery supported by CP signalling, such as RRC, NAS, or LPP signalling. The basic operation is shown in Figure 2.4-1.

Observation 7: The main drawback of CP-based model delivery is its data volume limitation.

Observation 8: The main drawback of UP-based model delivery is its lack of a native control mechanism.

Figure 2.2.1-1 implements the control through RRC signalling, NAS signalling, or LPP signalling in the case of model transfer/delivery from the gNodeB, core network, or the LMF, respectively. Authentication and PDU session establishment would be handled through existing means, and the data transfer could use existing data transfer protocols.

For example, the “Model Download Control” could include at least a host, e.g., an IP address, a transfer protocol, e.g., FTP, a filename of the model to download.

Figure 2.2.1-1: User Plane Model Transfer/Delivery to a UE with control.

Observation 9: A CP-controlled UP-based ML model transfer/delivery would use the existing authentication and PDU session establishment procedures, and preexisting data transfer protocols could facilitate the transfer.

### 2.3.2	Specification impact

Based on the discussion in this section, we propose the following table to capture the specification of a hybrid model transfer/delivery solution. We would also like to note that given the different model transfer/delivery options that support the transfer of model parameters, datasets, reference models, or models, criteria A1 could be updated to reflect “model-related size” to cover each of these options.

| Supporting criteria | Impact on specification |
| --- | --- |
| A1. Large, no upper limit model-related size | No impact. UP supports large size models. |
| A2. Model transfer/delivery continuity | No impact. Continuity supported in the UP. |
| A3. NW controllability | RAN2 impact. The NW needs to command the UE to start a model transfer. |
| A4. Flexible model transfer/delivery QoS | No impact. Existing QoS mechanisms can be used. |

Observation 10: Given the different model transfer/delivery options that support the transfer of model parameters, datasets, reference models, or models, criteria A1 could be updated to reflect “model-related size” to cover each of these options

Proposal 2: Update criteria A1 to reflect model-related size instead of model size to support all options for model transfer/delivery, including, e.g.: model parameters, datasets, reference models, and models.

Observation 11: The impact of using the control plane to control a model transfer requires the addition of new RRC messages to control the data transfer.

Observation 12: The impact of using the user plane to carry a model transfer requires that the UE supports a file transfer protocol available at the UE and the server containing the models.

Observation 13: A hybrid control plane and user plane solution have minimal specification impact.

Proposal 3: Add the hybrid control plane and user plane solution to the model transfer comparison table.

# 3	Conclusion

This document has made the following observations:

Observation 1: Control plane model transfer/delivery solutions are limited by maximum message sizes, even when enabling segmentation on their respective protocols.

Observation 2: Control plane model transfer/delivery solutions enable NW control.

Observation 3: A new low-priority SRB will not overcome the issue that a segmented RRC message must be completely transmitted prior to transmitting a new RRC message.

Observation 4: Under relatively good channel conditions and while connected to a cell with a high bandwidth, a small data transfer, e.g., a 1MB model transfer, could take nearly 100ms, and larger data transfers, e.g., 10-100MB model transfers, could take nearly 10 seconds.

Observation 5: Under relatively good channel conditions and while connected to a cell with a medium bandwidth, a small data transfer, e.g., a 1MB model transfer, could take more than 300ms, and larger data transfers, e.g., 10-100MB model transfers, could take more than to 33s.

Observation 6: The user plane model transfer/delivery solutions discussed during the SI do not satisfy network controllability requirements.

Observation 7: The main drawback of CP-based model delivery is its data volume limitation.

Observation 8: The main drawback of UP-based model delivery is its lack of a native control mechanism.

Observation 9: A CP-controlled UP-based ML model transfer/delivery would use the existing authentication and PDU session establishment procedures, and preexisting data transfer protocols could facilitate the transfer.

Observation 10: Given the different model transfer/delivery options that support the transfer of model parameters, datasets, reference models, or models, criteria A1 could be updated to reflect “model-related size” to cover each of these options

Observation 11: The impact of using the control plane to control a model transfer requires the addition of new RRC messages to control the data transfer.

Observation 12: The impact of using the user plane to carry a model transfer requires that the UE supports a file transfer protocol available at the UE and the server containing the models.

Observation 13: A hybrid control plane and user plane solution have minimal specification impact.

And the following proposals:

Proposal 1: Do not increase the number of RRC segments to support control plane model transfer, e.g., to support control plane model transfer/delivery in cases 1a and 2a.

Proposal 2: Update criteria A1 to reflect model-related size instead of model size to support all options for model transfer/delivery, including, e.g.: model parameters, datasets, reference models, and models.

Proposal 3: Add the hybrid control plane and user plane solution to the model transfer comparison table.

# 4	References

[1] “RAN1 Chair’s Notes”, 3GPP TSG RAN WG1 #118, Maastricht, NL, August 19 – 23, 2024

[2] TR 38.843 “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”, V18.0.0, 3GPP, 2023-12

[3] TS 38.300, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; NR and NG-RAN Overall Description; Stage 2 (Release 18)”, V18.2.0, 3GPP, 2024-07

[4] TS 38.331, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Radio Resource Control (RRC) protocol specification (Release 18)”, V18.2.0, 3GPP, 2024-07

[5] TS 38.323, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Packet Data Convergence Protocol (PDCP) specification (Release 18)”, V18.2.0, 3GPP, 2024-07

[6] TS 38.306, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; User Equipment (UE) radio access capabilities (Release 18)”, V18.2.0, 3GPP, 2024-07

[7] TS 38.214, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical layer procedures for data (Release 18)”, V18.2.0, 3GPP, 2024-03