3GPP TSG-RAN WG2 Meeting #127bis													     R2-2408556

Hefei, China, October 14th – 18th, 2024

Agenda item:	8.3.2

Source:	Apple

Title:	RRM measurement prediction results using field and simulated data

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we provide evaluation results for several measurement prediction scenarios, which are based on field and simulated data.

In addition to the proposals to include the results in the TR 38.744 [6], we have several proposals to guide the continuation of this study.

# 2   	Discussion

### 2.1		Field Data Results

In [3], submitted to RAN#127, we provided initial results based on field data. At the time we didn’t yet propose to include the results in the TR 38.744 [6] due to certain limitations we had in the dataset we used at the time. In this contribution those issues have been addressed, and we provide results based on a larger data set, which also includes mobility information. The data set we use this time is large and diverse enough to allow AI/ML model training and therefore we provide results, based on field data, with the intention to include them in the TR 38.744 [6].

The dataset contains about 1 million L3 RSRP measurements in FR1 with the following breakdown by UE speeds:

- Stationary: 919K samples

- Pedestrian (up to 4 km/h): 16K samples

- Medium mobility (5-29 km/h): 48K samples

- High mobility (above 30 km/h): 17K samples

The measurements have been collected from a few hundreds of cells and they are all intra-frequency.

Note: we plan to provide additional results based e.g. for inter-frequency, but in this contribution we studied case 2 “FR1 to FR1 intra-frequency temporal domain case B”. This being a random moderately large sample already allows us to make an importance observation relevant to the current study. As a reminder, the study has two goals:

- Goal 1: measurement overhead reduction

- Goal 2: mobility performance improvement

For the latter goal, we are interested in extreme cases (e.g. high mobility, FR2). However, for the former goal we are primarily interested in the most common scenario – which appears to be stationary.

Observation 1: the majority of UEs in the field are stationary, which is an important observation that needs to be considered for the study goal 1 of measurement overhead reduction.

With regards to the compliance to the simulation assumptions agreed in RAN2 for the study, it is perhaps obvious but still worth pointing out, that field data is not collected in a controlled environment (unlike simulations). Therefore, it may not be possible to stick to all the assumptions agreed for simulations. Here is a short list of the most important differences:

- UE trajectory – completely random in our data as opposed to artificial mobility models in the assumptions

- UE trajectory boundary processing – not applicable

- UE speeds – see breakdown above

- Sampling period – 40ms in our data as opposed to 200ms in the assumptions

Furthermore, for some assumptions, we obviously cannot disclose the details (e.g. the details of L1 filtering implementation, etc). There are many more differences, of course, in particular all the assumptions related to channel models don’t apply.

Despite, or perhaps because, of these differences between field data and simulation assumptions, we think it is extremely important to include field data in the study to make sure the conclusions are applicable to real deployments and are not just an academic exercise. The differences between field deployments and simulation assumptions demonstrate that the gap between our “tools” (simulations) and the reality is rather large and therefore it is important to include field data results in the study.

Observation 2: naturally, there are differences between simulations assumptions and field data; on the one hand, these differences make our analysis somewhat harder but on the other hand they demonstrate the importance of using field data.

In this evaluation we studied the following models:

- Autoregressive Average (AR)

  - Configuration: ARIMA(1,1,0), differenced first-order autoregressive model, Measurement window size w=10, Ŷt  =  μ  + Yt-1  +  ϕ1 (Yt-1 - Yt-2) where Yt is L3-RSRP measurement at time index t

- eXtreme Gradient Boosting (XGBoost)

  - Configuration: Input window size w = 10, Number of estimators t= 200, Max depth d = 10

- Feedforward NN (multi-layer perceptron)

  - Configuration: 3 layer FCN with input size w =10, hidden layer size h = 64 and output o=1

Note: strictly speaking, AR is not an AI/ML model. Nevertheless, we thought it is important to include it in the study and indeed, as we show below, it is very relevant and at least in some cases AR provides reasonably good results. We would like to further stress that “non-AI/ML” approaches are not limited to trivial  “sample and hold”), which seems to be the assumption many companies are making.

Observation 3: non-AI/ML approaches are not limited to “sample and hold”.

The complexity calculations for these models are provided in Table 1 below, which should be self-explanatory.

| Model | Number of parameters | Model size | FLOPs |
| --- | --- | --- | --- |
| AR | 2 | 8Bytes | 4 FLOPs |
| XGBoost | # of nodes*(feature index + threshold + child node indexes) + # of leaves = ~1M | ~2MB | 82200 FLOPs O(td) |
| Feedforward NN | O(w*h+h*h+h*h+h) | 17.7KBytes | 8896 FLOPs |

Table 1: model complexity (time-domain evaluation with field data)

We’ve evaluated measurement reduction rates of 50%, 67% and 75% with the results shown in Figures 1, 2 and 3 (respectively) below.

Figure 1: 50% MRR results with field data

The first observation we can make looking at the 50% MRR results, which is also confirmed in the other cases below, is that FNN model outperforms simpler models with a significant margin, that grows larger when the speeds increase. Having said that, simple models do perform reasonably OK (with the disclaimer that we are yet to establish what should be considered “reasonable accuracy”).

Figure 2:67% MRR results with field data (different prediction windows)

Figure 3: 75% MRR results with field data (different prediction windows)

In the Figures 2 and 3 above, in addition to looking MRRs of 67% and 75%, we’ve also considered different prediction windows of 1, 2 and 3 steps (in units of sampling rate) ahead.

The previous conclusion that FNN outperforms other models still holds, with an additional observation that the accuracy gap is larger for larger prediction windows.

Observation 4: FNN outperforms other models; the performance gap is larger for larger speeds and larger prediction windows. However, simple models do perform reasonably OK (with the disclaimer that we are yet to establish what should be considered “reasonable accuracy”).

Additionally, as other companies already reported, prediction accuracy degrades with higher speeds. This observation is further illustrated by Figures 4, 5, 6 and 7 below, where we provide separate accuracy figures for the following mobility cases:

- All UEs

- Pedestrian UEs

- Medium mobility UEs

- High mobility UEs

Figure 4: all UE speeds

Figure 5: pedestrian speeds

Figure 6: medium mobility

Figure 7: high mobility

### 2.2		Simulated Data Results

In [2], submitted to RAN2#127, we provided some initial results based on simulated data. In this contribution those results are further revised and refined, based on the agreements in RAN2#127 and to cover new cases. With this understanding, in the present contribution we focus on differences compared to our assumptions and results in [2] and new findings.

For brevity, we don’t describe all the simulations assumptions and options chosen, as these are reported in the attached spreadsheets with the results.

#### 2.2.1	Temporal Domain

In [2] previously submitted to RAN2#127, we had provided results for case 2 “FR1 to FR1 intra-frequency temporal domain case B” and in the present contribution we cover the same case, with some revisions and adjustments to the simulations assumptions, as outlined below.

The first change is that we’ve implemented Soft LOS, as we’ve observed that many companies provided results with Soft LOS and to be consistent, we’ve aligned our simulations to what the majority seem to be using. Soft LOS is implemented in accordance with TR 38.901 [4]. And indeed, after implementing this change, we’ve observed that it does impact the results, in particularly we’ve observed a certain increase in prediction accuracy.

Additionally, we’ve implemented L1 filtering. Between the two options discussed by email in [5]:

- Sliding window, and

- Non-sliding (block) window

We’ve implemented the second (non-sliding/block window) approach. With this change we’ve observed further increase in prediction accuracy. This is likely because L1 filtering removes certain amount of noise and introduces more “smoothing” to the time series of RSRP measurements.

Observation 5: implementation of Soft LOS and introduction of L1 filtering improves RSRP prediction accuracy.

Another observation we had made in [2], which was also confirmed by our new results, is that at least in this case (temporal domain intra-frequency prediction), simple models (e.g. XGBoost) provide adequate results and relatively simple NN models provide good results, and so high complexity models such as LSTM are not necessarily justified. This observation is consistent with the results based on field data (see observation 4). With these observations we’ve reduced the number of models in our evaluation to the following:

- Autoregressive Average (AR)

- eXtreme Gradient Boosting (XGBoost) with number of estimators t=10, max depth d=7

- Feedforward NN (multi-layer perceptron) with n inputs (n being the number of measurements in the observation window)

- Random Forest (number of trees t = 100, average tree depth: d = 10)

The complexity calculations for these models are provided in Table 1 below.

| Model | Number of parameters | Model size | FLOPs |
| --- | --- | --- | --- |
| AR | n + 2 n=4: 6 | (n + 2) * 8 Bytes N=4: 48 Bytes | 2*n+1 |
| XGBoost | 63600 | 0.25 MB | 800 |
| Feedforward NN | 64*n+65 for n=4: 321 | (64*n + 65)*8 For n=4: 2.5 KB | ~128n+192 |
| Random Forest | 100x1023 = 102,300 | ~1,37 MB t×(2^d−1)×32 | ~1 KFLOPs |

Table 2: model complexity (time-domain evaluation)

All the results are provided in the excel spreadsheet in the agreed format and are attached to this contribution. Figure 11 illustrates some of the results for visualization and further discussions.

Figure 8: prediction accuracy (time domain evaluation)

A note about model inputs: in the time domain evaluation we’ve only used historical L3 RSRP measurements to predict L3 RSRP. The number of “lags” in Figure 1 indicates the length of the observation window, so that for example 4 lags equals 800ms. This in turn affects model complexity, as the number of lags becomes the variable “n” in Table 1 (where applicable).

In general, the approach we’ve taken in this contribution is not necessarily to cover the widest possible range of options, cases, models and parameters (which might produce an unreasonably high number of results, which in turn would make it difficult to draw conclusions), but rather use the preliminary results (ours and other companies’ from RAN#127) to filter out certain options to produce a reasonable number of high quality and high relevance results. In particular, as we have already established the impact of speed on the prediction accuracy, that is – that accuracy reduces somewhat with speed increase (this result makes sense and was indeed confirmed by many other companies and our own results for field data), this time we limited the evaluation to 30km/h.

The accuracy of all the models appears to be acceptable, with the caveat that RAN2 is yet to establish what an acceptable prediction accuracy is. We discuss this in more detail in section 2.4 below. Furthermore, similarly to what we’ve already observed in [2], also with these new results even simple models such as AR (which is not AI/ML) the prediction is adequate and in fact very close that of Feedforward NN. Naturally, the complexity of such simple models is significantly lower than that of NN let alone LSTM. Here, however, we observe an important difference between the results based on field and simulated data – while for simulated data AR performs well and indeed close to FNN, that is not quite the case for field data, where FNN significantly outperforms AR.

Observation 6: while for simulated data AR performs well and indeed close to FNN, that is not quite the case for field data, where FNN significantly outperforms AR.

#### 2.2.2	Frequency Domain

For frequency domain, we provide results for (previously not covered in [2]) case 3 “FR1 to FR1 inter-frequency (frequency domain)”. In this evaluation, the model takes the RSRP of cell X at frequency f1 (2 GHz) as input, predicts the RSRP at frequency f2 (4 GHz) of the inter-frequency cell Y at the same location and the same site. Furthermore, as we show below, the prediction accuracy turned out to be rather good even with a single input, i.e. the observation window is one sample.

We’ve evaluated several models in the following input/output model configurations:

- Input: L3 filtered, output: L3 filtered RSRP

- Input: L1 filtered, output: L1 filtered RSRP

- Input: L1 filtered, output: L3 filtered RSRP

The same 4 models (which as we’ve already established perform well) as the ones used time-domain prediction were evaluated for frequency domain prediction. As we’ve already explained above, the number of inputs here is just 1 sample and therefore the complexity is different (compared to time domain), as shown in Table 3 below.

| Model | Number of parameters | Model size | FLOPs |
| --- | --- | --- | --- |
| AR | n + 2 n=1: 3 | (n + 2) * 8 Bytes N=1: 24 Bytes | 2 |
| XGBoost | 63600 | 0.25 MB | 800 |
| Feedforward NN | 64*n+65 for n=1: 129 | (64*n + 65)*8 For n=1: 1 KB | ~320 |
| Random Forest | 100x1023 = 102,300 | ~1,37 MB t×(2^d−1)×32 | ~1 KFLOPs |

Table 3: model complexity (frequency-domain evaluation)

The results for the three input/output (L1 vs. L3) configurations for the four models are provided in the spreadsheet and shown in Figure 12 below for illustration and further discussion.

Figure 9: Frequency domain prediction, L1 vs. L3 model input/output configuration accuracy

Two immediate preliminary observations can be made from the graph in the Figure 2:

- L3 input, L3 output provides the best accuracy

- Accuracy of frequency domain prediction is adequate, but somewhat lower than the accuracy of time domain prediction

Observation 7: L3 input, L3 output model configuration provides the best accuracy for frequency domain prediction

Observation 8: Accuracy of frequency domain prediction is adequate, but somewhat lower than the accuracy of time domain prediction.

### 2.3		Model Generalization

An important aspect of this study is model generalization. And indeed, there is an editor’s note in TR 38.744 [6] stating:

Editor Note 2: AI/ML model generalization could be studied after sufficient evaluation of different use cases.

We think that at this stage the study has progressed enough to start discussing model generalization. This is an important aspect, which in many ways can “make or break” the feature [of AI/ML prediction for mobility], or at least make it practical (or not) to deploy.

Consider, for example, a potential conclusion that AI/ML is only accurate enough with cell specific models (this is only a hypothetical, we are not actually making this claim). This would either mean that it is not practical to deploy the feature at all, or at the very least, that it would require complex LCM. If, on the other hand (yet another hypothetical) it turns out that it is feasible to train a generalized mobile applicable to many cells with good performance, this would mean that the LCM impacts are very minimal and indeed such feature would be relatively easy to deploy. Furthermore, as we’ve listed only two extreme potential outcomes of the model generalization study, it is obvious that other “in between” conclusions are possible.

Observation 9: model generalization is an important objective which should be studied, preferably from the next meeting.

One potential approach to model generalization study (which is by no means the only possible approach, just the one we prefer) is to rely on field data. In a way, this is the ultimate generalization test. Having said that, as previously noted, using simulated data (where we can fully control certain parameters of interest, which are yet to be discussed) can also be helpful in the generalization study.

Observation 10: field data can be very useful for the study objective of model generalization.

With this in mind, we’ve provided some initial results of the model generalization study, using field data. Note: the results provided here are not yet complete nor are comprehensive and shall be viewed only as an illustration of how field data can be used for model generalization.

For the purpose of the model generalization exercise, we considered two models (trained and verified using field data described in section 2.1):

- Multi-cell model (Figure 10)

- Single-cell model (Figure 11)

Figure 10: model generalization, multi-cell model

Figure 11: model generalization, single cell model

The important observation (that needs to be further studied and confirmed) is that there is little performance degradation for with a generalized (multi-cell) model compared to a single cell model. This conclusion, if confirmed by additional studies, is important as it would greatly simplify the LCM framework for the AI/ML model prediction (yet to be discussed).

Observation 11: initial results demonstrate that AI/ML models trained with field data do not suffer from generalization problems; this observation needs to be further studied and confirm.

### 2.3		Conclusions

In this section we discuss the results (both field and simulated data), propose conclusions (both for inclusion in the TR 38.744 [6] and to advance the study) and raise a few questions that need to be clarified.

To recap, we’ve agreed that there are two goals for this study:

- Goal 1: measurement overhead reduction

- Goal 2: mobility performance improvement

As we’ve pointed out already, for the second goal it is important to address the most extreme scenarios (e.g. high mobility speeds, FR2, etc), however the first goal is different. Extreme [e.g. mobility] scenarios are important, but rare. Therefore, if the intention is to reduce overhead (e.g. to save UE power and/or radio resources), we need to also focus (in the context of addressing the first goal) on the most common scenarios, which so far have been overlooked.

Proposal 1: to address the first study goal of measurement overhead reduction, in addition to extreme mobility scenarios, RAN2 needs to also focus on the most common scenarios of low mobility and stationary UEs.

Since we now have both results based on field data and simulated data, it is worth comparing the results and discussing what conclusions can be drawn based on such comparison. Our first observation is that, in many aspects the results based on field and simulated data are reasonably well aligned, which is a good sign.

Then it’s interesting to compare the prediction accuracy of models trained on field and simulated data. Taking scenario 2 of temporal domain cased B with 50% MRR prediction as an example, with Feedforward NN models, and looking at Mean Average Error of L3 RSRP (in dB) we can roughly see the following accuracy results

- Field data: about 0.1 dB

- Simulated data: about 0.3 dB

One might jump to conclusions that AI/ML performance better with field data than simulated data, but that is of course a premature conclusion (e.g. because there are differences in mobility patterns between those data sets and for many other reasons). Another immediate (and equally) premature conclusion might be that as list of this particular scenario 3 AI/ML “performs well”. Such accuracy results indeed “look” good, but this raises the question of what is a good accuracy?

This issue was briefly discussed before, in particular there were proposals in RAN2#127, but due to lack of time no conclusions were reached. Similarly to our previous position, which we think is now reinforced by these new results, we propose to consider system level KPIs. We believe that relatively simple system level KPIs such as HO failure rate would provide a good objective indication of the AI/ML performance with reasonable evaluation cost. Some companies previously proposed to use a baseline for comparison, such as “sample and hold”, but in our view such baseline is rather meaningless as in practice no implementation would rely on such an approach. Even without AI/ML, implementations can do much better than sample and hold (in particular, we’ve shown in this paper that very simple Auto Regressive models can provide a reasonable accuracy).

Proposal 2: to consider limited set of simple system level KPIs such HO failure rate as a KPI in this study.

And finally, we propose to include in the TR 38.744 [6] the results provided in this contribution (see attachments), for both field and simulated data.

Proposal 3: include prediction results based on field data for case 2 “FR1 to FR1 intra-frequency temporal domain case B” (attached) in the TR.

Proposal 4: include prediction results for case 2 “FR1 to FR1 intra-frequency temporal domain case B” and 3 “FR1 to FR1 inter-frequency (frequency domain)” (attached) in the TR.

# 3	Proposals

### 3.1 	Observations

Observation 1: the majority of UEs in the field are stationary, which is an important observation that needs to be considered for the study goal 1 of measurement overhead reduction.

Observation 2: naturally, there are differences between simulations assumptions and field data; on the one hand, these differences make our analysis somewhat harder but on the other hand they demonstrate the importance of using field data.

Observation 3: non-AI/ML approaches are not limited to “sample and hold”.

Observation 4: FNN outperforms other models; the performance gap is larger for larger speeds and larger prediction windows. However, simple models do perform reasonably OK (with the disclaimer that we are yet to establish what should be considered “reasonable accuracy”).

Observation 5: implementation of Soft LOS and introduction of L1 filtering improves RSRP prediction accuracy.

Observation 6: while for simulated data AR performs well and indeed close to FNN, that is not quite the case for field data, where FNN significantly outperforms AR.

Observation 7: L3 input, L3 output model configuration provides the best accuracy for frequency domain prediction

Observation 8: Accuracy of frequency domain prediction is adequate, but somewhat lower than the accuracy of time domain prediction.

Observation 9: model generalization is an important objective which should be studied, preferably from the next meeting.

Observation 10: field data can be very useful for the study objective of model generalization.

Observation 11: initial results demonstrate that AI/ML models trained with field data do not suffer from generalization problems; this observation needs to be further studied and confirm.

### 3.1 	Proposals

Proposal 1: to address the first study goal of measurement overhead reduction, in addition to extreme mobility scenarios, RAN2 needs to also focus on the most common scenarios of low mobility and stationary UEs.

Proposal 2: to consider limited set of simple system level KPIs such HO failure rate as a KPI in this study.

Proposal 3: include prediction results based on field data for case 2 “FR1 to FR1 intra-frequency temporal domain case B” (attached) in the TR.

Proposal 4: include prediction results for case 2 “FR1 to FR1 intra-frequency temporal domain case B” and 3 “FR1 to FR1 inter-frequency (frequency domain)” (attached) in the TR.

# 4	Attachments – Evaluation Results

See attached spreadsheets:

- Results for scenario 2 based on field data:

  - Apple_field_data_Scenario2_RRM Measurement Prediction Evaluation results for caseB_v1.xlsx

- Results for scenario 2 based on simulated data:

  - Apple_Scenario2_RRM Measurement Prediction Evaluation results for caseB_v1.xlsx

- Results for scenario 3 based on simulated data:

  - Apple_Scenario3_RRM Measurement Prediction Evaluation results for frequency_v1.xlsx

# 5	References

[1]		RAN2#127 and RAN#126-bis meeting minutes

[2]		R2-2406664, RRM measurement prediction results for case B, Apple

[3]		R2-2406665, Field data for RRM measurement prediction, Apple

[4]		TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz

[5]		R2-2407849, Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions (OPPO)

[6]		TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR