3GPP TSG-RAN WG2 Meeting #127bis                                      R2-2408558

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.3.2

Source:	Mediatek Inc.

Title:	Evaluation and Simulation Results for AIML RRM Prediction

Document for:	Discussion, Decision

Introduction

In RAN2 #126 meeting, the agreement on the use cases and the corresponding priority are given as follows.

| Case number | Prioritization | Evaluation scenario combination | Study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this contribution, we present the simulation results for RRM prediction for those 4 marked cases. Observations from the simulation results are provided for each use case. Additionally, we discuss the importance of non-AI benchmarks and system-level KPIs.

Discussion

Case 2: Intra-frequency temporal domain case B

In this case, we examine the AI approaches for reducing measurement overhead in the temporal domain. For a given measurement reduction ratio (MRRT), RAN2 allows companies to consider their own observation and prediction patterns. The following observation/prediction pattern is chosen in our simulation results. This pattern uniformly allocates the observation and prediction time slots, minimizing the distance between the last observation in OW and the last prediction in PW.

Figure 1: observation and prediction pattern for (a) MRRT = 0.5 (b) MRRT = 0.8

We consider two different cases: sliding and non-sliding, with varying MRRT and OW lengths. For the sliding case, the L1 filter and L3 filter measurements are updated every sampling period, i.e., 40ms. For the non-sliding case, they are updated every measurement period, i.e., 200ms. The PW is then determined based on the update frequency and MRRT as shown in Figure 1. For the sliding case, when MRRT = 0.5, PW = 40ms (contain one predicted measurement). When MRRT = 0.8, PW = 160ms (contain 4 predicted measurements). For the non-sliding case, when MRRT = 0.5, PW = 200ms (contain one predicted measurement), and when MRRT = 0.8, PW = 800ms (contain 4 predicted measurements). We consider sub-use case 2 with cluster-based approach where both AI model input and output are L3 cell level RSRP of 21 cells. The approach selection is because the sub-use case 2 can achieve the highest accuracy among all sub-use cases and the cluster approach generally outperforms the cell-specific approaches in our simulation result. Detailed AI model parameters and settings are given in the below table. The general simulation setting can be found in the Appendix.

| Setting/Parameters | Value |
| --- | --- |
| AI model | CNN |
| Flop | 1.26M |
| Model parameters | 304KB |
| Cluster approaches setting | Input: 21 cells, output: 21 cells |
| AI input/output | Input: L3 cell level RSRP, output: L3 cell level RSRP |

Table 1:AI model parameters and settings for intra-frequency temporal domain case B

In the following simulation result, we examine the impact of different OW lengths, MRRT, UE speed, and sliding/non-sliding cases by comparing the result with non-AI sample and hold approaches, which directly replace the predicted measurement with the last observation measurement.

| Intra-freq: FR1, Sliding | Abs L3 Cell RSRP diff (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | MRRT = 0.5, PW=40ms (1 point) |  | MRRT = 0.8, PW=160ms (4 points) |  |
|  | OW = 40ms (1 point) | OW =1600ms (20 points) | OW = 40ms (1 point) | OW=1600ms (8 points) |
| Sample and hold (90km/hr) | 0.198 | 0.198 | 0.482 | 0.482 |
| AI (90km/hr) | 0.198 | 0.08 | 0.482 | 0.384 |
| Sample and hold (30km/hr) | 0.128 | 0.128 | 0.305 | 0.305 |
| AI (30km/hr) | 0.127 | 0.064 | 0.305 | 0.253 |

Table 2:  intra-frequency temporal domain case B with sliding case

| Intra-freq: FR1, Non-sliding | Abs L3 Cell RSRP diff (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | MRRT = 0.5, PW=200ms (1 point) |  | MRRT = 0.8, PW=800ms (4 points) |  |
|  | OW = 200ms (1 point) | OW =2000ms (5 points) | OW = 200ms (1 point) | OW=2000ms (2 points) |
| Sample and hold (90km/hr) | 0.986 | 0.986 | 1.958 | 1.958 |
| AI (90km/hr) | 0.986 | 0.908 | 1.969 | 1.959 |
| Sample and hold (30km/hr) | 0.63 | 0.63 | 1.277 | 1.277 |
| AI (30km/hr) | 0.633 | 0.58 | 1.279 | 1.278 |

Table 3:  intra-frequency temporal domain case B with non-sliding case

Our result shows that with a short OW, especially in cases containing only 1 observed measurement, there is almost no gain compared with non-AI cases. In contrast, when OW contains more observed measurements, the AI outperforms non-AI cases. This is reasonable since more AI input samples enable AI model to learn better and provide more accurate predictions. Additionally, we notice that the performance in the sliding case is generally better than in the non-sliding case. One reason is that the interval between two samples is larger in the non-sliding case, leading to greater channel variation between samples. Another reason is that, for a similar OW length, there are more observed measurements in the sliding case.

From our result, we can also observe that higher accuracy is achieved at lower UE speed and lower MRRT. However, it should be noticed that for intra-frequency temporal domain case B, the non-AI approach, i.e., sample and hold, provides good performance, even when compared with AI approaches. This is because, in the observation/ prediction pattern given in Figure 1, the PW is relatively short in terms of sample point (at most 4 points in MRRT=0.8 case). To fully exploit the advantage of AI approaches, we may need to consider the case with more long-term predictions; otherwise, the improvement from AI is limited.

Observation 1: For intra-frequency temporal domain case B, the prediction accuracy is affected by OW length, MRRT, and UE speed. Specifically, higher accuracy can be achieved with a longer OW, lower MRRT, and lower UE speed.

Observation 2: For intra-frequency temporal domain case B, the AI enhancement is limited compared to simple non-AI approaches, e.g., sample and hold. Particularly, when PW is short, non-AI approaches already provide very good performance, thus, the margin for AI enhancement is limited.

Case 3: Inter-frequency prediction

In this section, we examine the simulation result of co-located inter-frequency prediction, where 4GHz and 2GHz cells are deployed at the same place. The AI approach is applied to predict the measurement result of 4GHz cells based on the observation of the co-located 2Ghz cell. Both cell-specific and cluster approaches are considered in our simulation results. Detailed AI model parameters and settings are given in the following table.

| Setting/Parameters | Value |
| --- | --- |
| AI model | CNN |
| AI input/output | Input: 2GHz measurement, output: 4GHz measurement Input: L3 cell level RSRP, output: L3 cell level RSRP |
| Cell-specific approaches | Input: 1 cell, output: 1 cell, total 21 models |
| Cluster approaches setting | Input: 21 cells, output: 21 cells |
| Flop | Cluster: ~3M. Cell-specific: 0.14*21 = ~2.9M |
| Model parameters | Cluster: ~201KB, Cell-specific:27.8*21= ~585KB |

Table 4: AI model parameters and settings for inter-frequency prediction

To consider the AI enhancement, we compare the AI results with a non-AI approach that assumes the measurement result of 4GHz cells can be derived by the result of co-located 2GHz cell minus the path loss difference, i.e., the path loss of 4GHz – the path loss of 2GHz. In Table 5, we show that sub-use case 2, i.e., both input and output of AI model are L3 cell level RSRP, can achieve higher accuracy compared to other sub-use cases. The cluster approach which considers all 21 cells has significant improvement compared to the cell-specific approach and non-AI approach.

| Inter-freq: FR1, 30km/hr | AI Input/ output | Abs L3 Cell RSRP diff (dB) |
| --- | --- | --- |
| Non-AI: 4GHz measurement = 2 GHz measurement – path loss difference | NA | 2.34 |
| AI: Cluster | L3 to L3 | 1.64 |
|  | L1 to L3 | 1.93 |
|  | L1 to L1 | 1.97 |
| AI: Cell-specific | L3 to L3 | 2.29 |
|  | L1 to L3 | 3.26 |
|  | L1 to L1 | 2.53 |

Table 5: Simulation results for inter-frequency prediction with different sub-use cases (30km/hr)

In Table 6, we consider the AI performance at different UE speeds. Simulation results show that the UE speed has no significant impact on the predicted accuracy.

| Inter-freq: FR1, L3 to L3 | UE speed |  |  |  |
| --- | --- | --- | --- | --- |
|  | 30km/hr | 60km/hr | 90km/hr | 120km/hr |
| Non-AI | 2.34 | 2.30 | 2.31 | 2.21 |
| AI: Cluster | 1.64 | 1.55 | 1.49 | 1.40 |

Table 6: Simulation results for inter-frequency prediction with different UE speeds

Observation 3: In frequency domain prediction cases, cell-specific approaches achieve limited gain compared to the non-AI approach without the help of neighbor cell measurement. The cluster-based approach outperforms cell-specific approaches across all sub-use cases.

Observation 4: For the inter-frequency prediction in the co-located deployment, the UE speed does not affect the prediction accuracy.

Case 4: Intra-frequency temporal domain case A

For intra-frequency temporal domain case A, we use continuous historical measurement results to predict continuous future measurement results without considering any measurement reduction. The following is the default setting in our simulation if no further specification.

| Setting/Parameters | Value |
| --- | --- |
| AI model | CNN |
| Cell-specific approaches | Input: 1 cell, output: 1 cell, total 21 models. |
| UE speed | 60km/hr |
| Sliding/non-sliding | Sliding |
| OW/PW | OW=160ms, PW=640ms |
| AI input/output (sub-use cases) | Sub-use case 2, input: L3 cell RSRP; output: L3 cell RSRP |

Table 7: Default settings for intra-frequency temporal domain case A

We first examine the results of 3 different sub-use cases as shown in the following table. Under the same setting, the sub-use case 2, i.e., L3 cell to L3 cell, can achieve the highest prediction accuracy.

| Intra-freq: FR2 | Abs L3 Cell RSRP diff (dB) | AI Complexity: FLOPs(M) / Parameter size (K Bytes) |
| --- | --- | --- |
| Sub-use case 1: L1 beam to L1 beam | 2.49 | 16.06M/359.2KB |
| Sub-use case 2: L3 cell to L3 cell | 0.86 | 0.46M/96.3KB |
| Sub-use case 3: L1 beam to L3 cell | 1.94 | 14.79M/104.2KB |

Table 8: Simulation result for intra-frequency temporal domain case A with different sub-use cases.

In the next table, we consider different OW/PW rates (1/4, 1, 4). Since the OW and PW length directly affect the number of samples that can be used as AI model input and the number of samples needed to predict, a lower OW/PW rate would increase the prediction difficulty and vice versa. Our results also verify this observation.

| Intra-freq: FR2 | Abs L3 Cell RSRP diff (dB) | AI Complexity: FLOPs(M) / Parameter size (K Bytes) |
| --- | --- | --- |
| OW/PW=1/4: OW= 160, PW=640 | 0.86 | 0.46M/96.3KB |
| OW/PW=1: OW=400, PW=400 | 0.40 | 0.30M/123.9KB |
| OW/PW=4: OW=640, PW=160 | 0.16 | 0.37M/151.4KB |

Table 9: Intra-frequency temporal domain case A with different OW/PW ratios

According to the email discussion [1], the L1 and L3 filter measurements can be updated every sampling period, e.g., 20*4=80ms, or every measurement period, e.g., 400ms. The formal is the case with sliding filtering and the later is the case with non-sliding filtering. The following table compares the result of these two cases under the same OW and PW length (note that, the number of measurements in OW and PW are different for these two cases). Our result show that under the same OW and PW length, the sliding case can achieve higher prediction accuracy.

| Intra-freq: FR2 | Abs L3 Cell RSRP diff (dB) | AI Complexity: FLOPs(M) / Parameter size (K Bytes) |
| --- | --- | --- |
| Sliding filtering | 1.94 | 0.46M/96.3KB |
| Non-sliding filtering | 3.38 | 0.60M/124.4KB |

Table 10: Intra-frequency temporal domain case A with sliding and non-sliding cases.

Next, we examine the AI improvement under different UE speeds by comparing the result with the non-AI approach, i.e., sample and hold, which is described in section 2.1. For both 60km/hr and 120km/hr, the AI outperforms the non-AI approach and can achieve less than 1dB RSRP difference. The performance gain in the more challenging case, i.e., 120km/hr, is slightly larger than in the 60km/hr case.

| Intra-freq: FR2 | Abs L3 Cell RSRP diff (dB) | AI improvement | AI Complexity: FLOPs(M) / Parameter size (K Bytes) |
| --- | --- | --- | --- |
| Non-AI: sample and hold (60km/hr) | 1.07 | 1.07-0.86 =0.21(dB), 0.21/1.07=19.6% | NA |
| AI (60km/hr) | 0.86 |  | 0.46M/96.3KB |
| Non-AI: sample and hold (120km/hr) | 1.23 | 1.23-0.93=0.3 (dB) 0.3/1.23=24.3% | NA |
| AI (120km/hr) | 0.93 |  | 0.46M/96.3KB |

Table 11: AI enhancement in intra-frequency temporal domain case A with different UE speeds.

Observation 5: For intra-frequency temporal domain case A, the sub-use case 2, i.e., the L3 cell to L3 cell prediction, exhibits the smallest cell-level RSRP difference compared to other sub-use cases.

Observation 6: For intra-frequency temporal domain case A, a higher OW/PW rate leads to higher prediction accuracy.

Observation 7: For intra-frequency temporal domain case A, under the same OW and PW length, the sliding filtering case has higher prediction accuracy compared to the non-sliding filtering case.

Observation 8: For intra-frequency temporal domain case A, AI can provide ~20% gain (in terms of L3 cell RSRP difference) compared to the non-AI approach, i.e., sample and hold. The improvement is more significant in the high UE speed case, e.g., 120km/hr, than in the low UE speed cases, e.g., 60km/hr.

Case 6: Intra-frequency spatial domain prediction

In this case, AI is applied to reduce measurement overhead in the spatial domain. For MRRS=50%, only half of Tx beam RSRP is observed, the rest of the beams are predicted based on the observation. However, we notice that under the same MRRS, the observation beam selection will affect the prediction accuracy. In the following simulation, we consider three different observation beam patterns, e.g., uniform odd index, uniform even index, and upper half index. We also compare the result with the non-AI approach which directly derives L3 cell level RSRP based on the set of observation beams (i.e., only considering half of the beams). Our results show that the selection of observation beams significantly affects the performance of both AI and non-AI approaches. Once the selection of observation beams contains strong beams, the non-AI approach will perform well (even better than AI approaches).

| 30km/hr, MRRS=0.5, Sub-use case 1 (L1 to L1) | Abs L3 Cell RSRP diff (dB) |  |  | AI Complexity: FLOPs(M) / Parameter size (K Bytes) |
| --- | --- | --- | --- | --- |
|  | Uniform Odd index | Uniform Even Index | Upper Half index |  |
| AI (CNN) | 0.52 | 0.48 | 0.99 | 9.789M/164.9KB |
| Non-AI | 0.18 | 0.18 | 1.19 | NA |

Table 12: Intra-frequency spatial domain prediction with different observation beam selection.

Observation 9: The selection of observed beam patterns can significantly affect the prediction accuracy of the spatial domain prediction.

Proposal 1: RAN2 capture the above observations in the TR.

AI Gain Verification with Aligned Non-AI Baseline

One of the important goals of this SID is to examine the AI performance gain of each use case such that one can determine the potential way to enhance mobility with AI. To obtain a concrete and consistent conclusion from different companies’ results, a fair comparison is necessary. However, it is challenging since the implementation of the simulation environment and the assumption of UE/gNB behavior in HO procedure vary from different companies even with considering aligned simulation settings. A more practical and simple way is to consider an aligned non-AI baseline as a performance benchmarking which helps us to

- Show the AI performance gain by comparing with the non-AI baseline

- Offer a consistent baseline for performance benchmarking that remains comparable across different simulation implementations from different companies

Whether addressing intermediate KPIs, such as RSRP difference, or system-level KPIs, like HOF, establishing a baseline for these metrics is essential to provide meaningful implication by the simulation results. For example, considering temporal-domain prediction for Goal 1, observing a low RSRP difference value from an AI approach does not mean a benefit from AI, if a non-AI baseline, e.g., sample and hold, can achieve a lower (or similar) RSRP difference value. It just means that the channel does not change much between two measurement instances.

For the second purpose described above, even though RAN2 has agreed on many simulation settings before starting simulation work, due to differences in implementation, results are still expected to vary significantly from company to company. Similar observations can be found in the evaluation results in R18 AI-BM [2]. Even with considering the alignment of the evaluation setting and assumption, the results provided by each company still vary. Thus, it is necessary to consider an aligned non-AI baseline that provides a consistent baseline for performance benchmarking such that we can identify the bias due to different companies' simulation environments and implementation.

For example, if we observe the results of a company are always better than others,  regardless of whether non-AI or AI methods are used, this may imply the dataset, channel, and/or simulation environment provided by this company is easier for prediction, but it may not imply the AI can provide such good performance. Also, since both AI and non-AI approach results from the same company are generated through the same simulation environment, it is more reasonable to compare the AI performance gain, i.e., the difference between AI and aligned non-AI approaches, instead of comparing AI performance directly. This can reduce the impact of implementation differences.

Observation 10: In R18 AI-BM [2] the evaluation results vary in different companies due to the different simulation implementations, even with considering the aligned evaluation settings. A non-AI baseline can provide a consistent baseline for performance benchmarking that helps to identify the bias from different companies' simulations.

The selection of non-AI baseline is different from each sub-use case. Since the non-AI baseline is only considered as a reference, we can start from the simplest approach and do not need to consider too advanced approaches. For Goal 1, the baseline of temporal domain prediction can reuse the approach in R18 AI-BM [2], i.e., sample and hold approach, where the measurement result in prediction instance is replaced by the result in the nearest observation instance directly. For the spatial domain prediction, we can modify the method in [2], the predicted cell level result can be derived from the observation set B only. For frequency domain prediction, if co-located deployment is considered, a simple way is to assume the non-AI prediction to be equal to observation with pathloss modification, e.g., prediction (4GHz cell) = observation (co-located 2GHz cell) – pathloss difference (~6dB). For Goal 2, the baseline of temporal domain prediction could be the normal measure pattern, e.g., measure all instances. Other non-AI approaches are FFS.

Proposal 2: RAN2 discuss the non-AI baseline for performance benchmarking for each use case. The baseline in Table 2 can be the starting point. Other options could be FFS.

| Use case | Non-AI baseline |
| --- | --- |
| Case 2: Intra-frequency temporal domain case B | Sample and hold |
| Case 3: Inter-frequency prediction | Predict based on co-located cell measurement with pathloss modification |
| Case 4: Intra-frequency temporal domain case A | Sample and hold |
| Case 6: Intra-frequency spatial domain prediction | Derive cell level quality from observation set B |

Table 13: Non-AI baseline for different use cases.

System KPI

In RAN plenary # 102, the SID on AI mobility [3] agree to study the system performance KPIs. Although from the previous meeting, RAN2 agrees to study the prediction accuracy, e.g., intermediate KPI, at the first step, and the need of system performance will be FFS, we still think the system KPI plays an important role in the evaluation and is necessary for each use case.

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

From our perspectives, intermediate KPIs, e.g., RSRP difference, are designed to examine the predicted capability of AI model. To identify the real AI benefit, system KPIs, e.g., HOF, RLF rate, throughput, are necessary. For example, in the Goal 1, i.e., measurement overhead reduction, we cannot check the tradeoff between the measurement reduction and performance degradation through the intermediate KPI such as RSRP difference. For Goal 2, i.e., HO performance enhancement, it is also impossible to realize, through RSRP difference, the tradeoff between the benefit from early awareness of the future measurement by prediction and the degradation from prediction error. The intermediate KPIs, only help us to understand the potential of AI approaches in terms of their prediction capability, but the real benefit should be examined by the system KPIs. Due to the above reason, we propose to consider the system KPIs in AI mobility use case.

Observation 11: Intermediate KPIs are designed to examine the predicted capability of AI model. To examine the real benefit for AI approaches, system KPIs are necessary. For example, to examine (1) the tradeoff between measurement reduction and performance degradation (2) the tradeoff between the benefit from predicted future measurement and the degradation due to the prediction error.

One of the challenges for RAN2 to study and evaluate system-level performance comes from the overhead of aligning system-level simulation settings. Considering the time budget and workload, we propose RAN2 to study the system-level simulation with aligned non-AI baselines as a reference, so that detailed alignment and calibration of system-level simulation, e.g., the interference model, the HO procedure, the msg transmission model, the HOF, RLF mechanism, are not mandatory. RAN2 can directly compare the AI gain by examining the difference between AI and aligned non-AI results from the same company. Since both AI and non-AI results are obtained from the same simulation environment, comparing the AI/nonAI performance difference is fairer than directly comparing the AI performance, and thus, it is more likely to achieve concrete conclusions.

Proposal 3: RAN2 consider the system KPIs, HOF/RLF/ToS…etc, for each use case.

Proposal 4: To reduce the workload of system-level simulation alignment, RAN2 can compare the system-level performance through AI performance gain, i.e., the difference between AI and non-AI approach used as the performance benchmarking.

Conclusion

Observations:

Simulation results: Intra-frequency temporal domain case B

Observation 1: For intra-frequency temporal domain case B, the prediction accuracy is affected by OW length, MRRT, and UE speed. Specifically, higher accuracy can be achieved with a longer OW, lower MRRT, and lower UE speed.

Observation 2: For intra-frequency temporal domain case B, the AI enhancement is limited compared to simple non-AI approaches, e.g., sample and hold. Particularly, when PW is short, non-AI approaches already provide very good performance, thus, the margin for AI enhancement is limited.

Simulation results: Inter-frequency prediction

Observation 3: In frequency domain prediction cases, cell-specific approaches achieve limited gain compared to the non-AI approach without the help of neighbor cell measurement. The cluster-based approach outperforms cell-specific approaches across all sub-use cases.

Observation 4: For the inter-frequency prediction in the co-located deployment, the UE speed does not affect the prediction accuracy.

Simulation result: Intra-frequency temporal domain case A

Observation 5: For intra-frequency temporal domain case A, the sub-use case 2, i.e., the L3 cell to L3 cell prediction, exhibits the smallest cell-level RSRP difference compared to other sub-use cases.

Observation 6: For intra-frequency temporal domain case A, a higher OW/PW rate leads to higher prediction accuracy.

Observation 7: For intra-frequency temporal domain case A, under the same OW and PW length, the sliding filtering case has higher prediction accuracy compared to the non-sliding filtering case.

Observation 8: For intra-frequency temporal domain case A, AI can provide ~20% gain (in terms of L3 cell RSRP difference) compared to the non-AI approach, i.e., sample and hold. The improvement is more significant in the high UE speed case, e.g., 120km/hr, than in the low UE speed cases, e.g., 60km/hr.

Simulation result: Intra-frequency spatial domain prediction

Observation 9: The selection of observed beam patterns can significantly affect the prediction accuracy of the spatial domain prediction.

Proposals:

Proposal 1: RAN2 capture the above observations in the TR.

Non-AI benchmarking

Proposal 2: RAN2 discuss the non-AI baseline for performance benchmarking for each use case. The baseline in Table 2 can be the starting point. Other options could be FFS.

Necessity of SLS

Proposal 3: RAN2 consider the system KPIs, HOF/RLF/ToS…etc, for each use case.

Proposal 4: To reduce the workload of system-level simulation alignment, RAN2 can compare the system-level performance through AI performance gain, i.e., the difference between AI and non-AI approach used as the performance benchmarking.

Reference

[1] 	R2-2407849 summary of [POST127][030][AI mobility] RRM simulation assumptions - Phase 1

[2]	TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[3]	RP-234055 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

Appendix: Simulation Setting

| parameters | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz,30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model (7 sites, 3 sectors/cells per site) | 2-tier model (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and LOSsoft; Inter-frequency correlation model is optional. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and LOSsoft; |
| System BW | 20MHz | 80MHz |
| UE speed | 30,90 km/h for study targeting measurement reduction 60,120 km/h for study targeting HO performance improvement | 30,,90 km/h for study targeting measurement reduction 60,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ | (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) | Antenna setup and port layouts at UE: (1, 4, 2, 1, 1, 1, 1) |
| UE Antenna pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| Tx/Rx beam | Tx: 1, Rx:1 | Tx:32, Rx:4 |
| BS Tx Power | 44dBm | 40 dBm |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | Procedure A in TR38.901 with coarse update | Procedure A in TR38.901 with coarse update |
| UE trajectory model | Option 1 | Option 1 |
| UE trajectory boundary processing model | Option 2: circle-bouncing model, | Option 2: circle-bouncing model, |
| Sampling period | 40ms | 20ms *4 = 80ms |