# 3GPP TSG RAN WG2 Meeting #127b      	                           R2-2408562
Hefei, China, 14th – 18th Oct., 2024

Agenda item:	8.5.4

Source:	Apple

Title:	Further discussion on common signal transmission adaptation

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

The Rel-19 WID of Network Energy Savings (WID RP-242354) was updated in RAN#105 [1]. The WI objective of adaptation of common signal transmission includes paging adaptation, PRACH adaptation and SSB adaptation, as copied below:

In RAN2#125b [2], only paging adaptation was discussed, and below agreement was made. SSB adaptation and RACH adaptation were not discussed due to time limitation.

Agreements on paging adaptation:

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

And RAN2#127 further discussed paging adaptation and made below agreements [3].

Agreements on paging adaptation

- Option-a) is about the bundling of PF for R19 NES UEs.

- R2 observe that the option-a) and option-b) can be designed to configure the PO:s at same time position.

Meanwhile, there is on-going post-meeting email discussion ([POST127][109][NES]) to analyze the identified two solutions and perform down-selection between them [4]:

In this contribution, we further discuss RAN2 work on adaptation of common signal transmission.

# 2 Discussion

## 2.1 Adaptation of paging occasions

### 2.1.1 Legacy UE compatibility

We first discuss below legacy UE compatibility issue identified in RAN2#125b [2]:

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

- Option 1: Prevent the access of legacy UE via barring;

- Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

Option 1 means MNOs need to deploy Rel-19 NES cells which only support Rel-19 UEs capable of paging adaptation. We don’t think it makes sense especially at late release of NR because it may lead to coverage holes and it goes against the intention of introducing NES.

Observation 1: It doesn’t make sense to deploy Rel-19 NES cells which only support Rel-19 UEs capable of paging adaptation especially at late release of NR because it may lead to coverage holes and it goes against the intention of introducing NES.

Thus, we propose RAN2 to focus on Option 2, which implies that Rel-19 NES cells provide separate paging resources via a new PCCH-Config for Rel-19 UEs capable of paging adaptation.

Proposal 1: Don’t introduce new legacy UE barring mechanism for paging adaptation.

Proposal 2: Rel-19 NES cells provide separate paging resource for Rel-19 UEs capable of paging adaptation via a new PCCH-Config different from the legacy one.

### 2.1.2 RRC and legacy impact of option-b

In summary report of ([POST127][109][NES]) [4], option-a and option-b are analyzed. And based on majority view, the following summary proposals are suggested by email discussion Rapporteur:

Proposal 1: Select option-b as baseline for R19 NES paging enhancement.

Proposal 2: R2 further discuss the impact of RRC configuration and the impact of legacy UE for option-b.

As follows, we discuss the legacy impact and RRC impact, respectively.

#### 2.1.2.1 Legacy UE impacts

We think there are two specific issues to discuss:

- Whether legacy PF and Rel-19 PF can be overlapped?

- How Rel-19 UE and legacy UE derive their PO for monitoring?

For 1), it is obvious that it works if legacy PF and Rel-19 PF are not overlapped. However, RAN2 has not discussed whether they can be overlapped as illustrated in Figure. 1, i.e. whether both legacy POs and Rel-19 POs can be allocated to the same PF (PF1 in the figure).

Figure.1 Overlapping between legacy PF and Rel-19 PF

We prefer to allow this overlapping due to below considerations:

- If it is not allowed, how to achieve NES gain is not clear because NW has to wake up in all the possible POs configured by the two PCCH-Config.

- It will be difficulty for NW to always avoid such overlapping by its implementation because one PO can be spanned to a frame after the PF, especially when Ns is large (e.g. increased to 16).

Observation 2: For option-b, it should allow Rel-19 PF to overlap with legacy PF because:

- If it is not allowed, how to achieve NES gain is not clear.

- It will be difficulty for NW to always avoid such overlapping by its implementation because one PO can be spanned to a frame after the PF, especially when Ns is large (e.g. increased to 16).

For 2), it is clear that legacy UEs and Rel-19 UEs will monitor different POs in non-overlapped PF(s). However, one ambiguity may happen that the same Rel-19 UE may derive different PO to monitor via legacy PCCH-Config and Rel-19 PCCH-Config. As illustrated in Figure. 2, one Rel-19 UE with UE_ID=17 will derive to monitor 2nd PO (i_s=1) with legacy PCCH-Config and derive to monitor 1st PO (i_s=0) with Rel-19 PCCH-Config because Ns values are different in the two PCCH-Config.

Figure.2 Example of ambiguity issue in overlapped PF

Observation 3: In overlapped PF, the same Rel-19 UE may derive different PO to monitor via legacy PCCH-Config and Rel-19 PCCH-Config because Ns values are different in the two PCCH-Config.

To resolve the issue, we think the simplest solution is to specify that Rel-19 UEs always monitor the PO(s) derived from Rel-19 PCCH-Config, although it may lose some flexibility and NES gain (e.g. align both types of PO in overlapped PF). Please note that it is not correct to use terminology of “Rel-19 PO” in overlapped PF because two overlapped PO will use the same PMO(s).

Thus, based on observation 1 and observation 3, we propose:

Proposal 3a: For option-b, RAN2 confirm that it allows Rel-19 PF to overlap with legacy PF.

Proposal 3b: For option-b, Rel-19 UEs only monitor the PO(s) derived from Rel-19 PCCH-Config, including in overlapped PF with legacy UEs.

#### 2.1.2.2 RRC impacts

During [POST127][109][NES], the concern on RRC signaling overhead was raised [4]. In details, according to Table. 1, we need to increase N from 16 to 256. Then, if we need to fully compensate the decrease in number of PFs, Ns needs to increase from 4 to 64.

| Value of 1/N | Number of PFs in one paging cycle |  |  |  | Required Ns if fully compensate |
| --- | --- | --- | --- | --- | --- |
| 1/N | rf32 | rf64 | rf128 | rf256 |  |
| 1/16 | 2 | 4 | 8 | 16 | 4 (no need to extend Ns) |
| 1/8 | 4 | 8 | 16 | 32 | 8 |
| 1/4 | 8 | 16 | 32 | 64 | 16 |
| 1/2 | 16 | 32 | 64 | 128 | 32 |
| 1 | 32 | 64 | 128 | 256 | 64 |

Table.1 All possible cases of PF number in one paging cycle

Then, large number of Ns will significantly increase RRC signaling overhead of PMO and PEI-O configuration because these configurations depend on maxPO-perPF (i.e. Ns) and maxPEI-perPF (up to Ns), as illustrated below:

PMO configuration

PCCH-Config ::=             SEQUENCE {

omit

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT            SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

omit

}

PEI-O configuration

pei-ConfigBWP-r17      SEQUENCE {

pei-SearchSpace-r17                 SearchSpaceId,

firstPDCCH-MonitoringOccasionOfPEI-O-r17  CHOICE {

sCS15KHZoneT                                                     SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                       SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT  SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919),

sCS480KHZoneEighthT                                           SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                                        SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..71679)

}

}

Then, we provide an analysis on required RRC signalling overhead under different Ns values in table.2.

|  | Ns=4 (baseline) | Ns=8 | Ns=16 | Ns=32 | Ns=64 |
| --- | --- | --- | --- | --- | --- |
| Payload of PMO configuration (bit) | 18 | 36 | 72 | 144 | 288 |
| Payload of PEI-O configuration (bit) | 21 | 42 | 84 | 168 | 336 |

Table.2 Required RRC signaling overhead under different Ns

As can be observed, total payload size of PMO and PEI-O configuration will increase to 624bit, which seems not to be acceptable because maximum payload size of SIB1 is limited to 2976 bit [5].

NOTE 2:	The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

On the other hand, we think the increasing Ns to 64 may be an overdesign because paging adaptation is target for low loading scenario. In case of high loading case, we believe NW should not enable paging adaptation.

Observation 4: If we need to fully compensate the decrease in number of PFs, Ns needs to increase up to 64. It will result in a large RRC payload size of PMO and PEI-O configuration (up to 624bit). However, it is an overdesign because paging adaptation is target for low loading scenario.

Thus, we propose to only consider extended Ns values to 8 and 16. But N value can be extended to 32, 64, 128, and 256, to allow only one PF per paging cycle.

Proposal 4: For option-b, RAN2 is kindly suggested to only consider extending Ns value to 8 and 16 (i.e. no need to introduce large Ns vale like 32 or 64) while N value can be extended with 32, 64, 128, and 256.

## 2.2 Adaptation of PRACH in time domain

RAN1#116, RAN1#116b and RAN1#117 had discussed RACH adaptation [6][7][8], and a lot of agreements were made. Based on RAN1 agreements, we understand that RAN1 progress can be highlighted as below:

- Introduce adaptation mechanism between legacy PRACH resources and one new additional PRACH resources for NES-capable UE in all RRC states.

  - RAN1 agreed the adaptation signaling can be L1/DCI signaling and RRC/SIB.

- The additional PRACH resources are provided via a prach-ConfigurationIndex and a separate SSB-RO mapping is performed for the additional PRACH resources.

  - It is still under discussion whether to allow any overlapping between legacy PRACH resources and the additional PRACH.

  - It is still under discussion whether prach-ConfigurationIndex for the additional PRACH resources is same as the one for the legacy resource.

Based on latest RAN1 progress, we share our view on potential RAN2 work in the followed sub-sections.

### 2.2.1 Legacy UE compatibility

First, we discuss the legacy compatibility issue. Because RAN1 agreements have considered legacy UE impacts, our understanding is that RAN2 don’t need to introduce new legacy UE barring mechanism for RACH adaptation. Meanwhile, same as paging adaptation, we don’t think it makes sense to deploy Rel-19 NES cells which only support Rel-19 UEs capable of RACH adaptation because it may lead to coverage holes and goes against the intention of NES.

Proposal 5: Don’t introduce new legacy UE barring mechanism for RACH adaptation in time domain, i.e., Rel-19 NES cells support both Rel-19 UEs capable of RACH adaptation and legacy UEs.

### 2.2.2 Clarification on RACH framework

We have one general clarification question on the RACH framework to integrate RACH adaptation. In Rel-17, general RACH framework supporting multiple RACH partitions was introduced. As illustrated in Figure.3, a list of up to 255 RACH partitions (i.e. AdditionalRACH-Config) can be configured in SIB1, and the Network can configure different RACH partitions (i.e. AdditionalRACH-Config) associated with different features (e.g., redcap) or feature combination (e.g., SDT+Msg1-repetition).

Figure.3 Illustration of RRC configuration of multiple RACH partitions

Observation 5: In Rel-17, general RACH framework supporting multiple RACH partitions was introduced. A list of up to 255 RACH partitions (i.e. AdditionalRACH-Config) can be configured in SIB1, and the Network can configure different RACH partitions associated with different features (e.g., redcap) or feature combination (e.g., SDT+Msg1-repetition).

Firstly, we discuss whether RACH adaptation is applicable to CBRA or CFRA. Because CFRA may use UE dedicated signaling, the benefit of RACH adaptation seems not quite clear. Thus, we propose to first study CBRA.

Proposal 6: RAN2 first study CBRA in RACH adaptation.

Secondly, even for CBRA, we think it is not clear what is the role of RACH adaptation in the general RACH framework. Specifically, there are two different understandings:

- Understanding 1: RACH adaptation is applied to only Rel-15 RACH resources (i.e. only support adaptation between rach-ConfigCommon and additional PRACH resources).

- Understanding 2: RACH adaptation is applied to all/part of RACH features or feature combination (i.e. support adaptation between rach-ConfigCommon and additional PRACH resources, and adaptation between AdditionalRACH-Config and additional PRACH resources). FFS whether/which RACH features or feature combination are not applicable.

In our understanding, it seems that RAN1 so far only discussed adaptation between rach-ConfigCommon and additional PRACH resources (i.e. Understanding 1). However, we also think it makes sense to support RACH adaptation in at least some RACH features or feature combination (e.g., redcap + additional PRACH resource), but some RACH feature may further discussion (e.g., msg1-repetition + additional PRACH resource). That is why we put an FFS in understanding 2.

We are open to either understanding, but RAN2 should make a conclusion about it from beginning because it will impact the workload.

Proposal 7: RAN2 discuss which is the correct understanding of RACH adaptation in time domain:

- Understanding 1: RACH adaptation is applied to only Rel-15 RACH resources (i.e. only support adaptation between rach-ConfigCommon and additional PRACH resources).

- Understanding 2: RACH adaptation is applied to all/part of RACH features or feature combination (i.e. support adaptation between rach-ConfigCommon and additional PRACH resources, and adaptation between AdditionalRACH-Config and additional PRACH resources). FFS whether/which RACH features or feature combination are not applicable.

### 2.2.3 Potential RAN2 work

Based on latest RAN1 progress, we think that at least the following RRC and MAC impacts are foreseen:

RRC impacts:

- Introduce a separate SSB-RO mapping configuration for additional PRACH resource, e.g. ssb-perRACH-OccasionAndCB-PreamblesPerSSB-NES in RACH-ConfigCommon.

- How to integrate additional PRACH resource set into general RACH framework, which couples with which understanding in Proposal 7 is agreed.

MAC impacts:

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in initial Msg1 transmission.

  - For example, Rel-19 UE may always start from additional PRACH resource, considering legacy UE can only use legacy PRACH resource.

  - As another example, if target for RACH latency minimization, Rel-19 UE may prioritize legacy PRACH resource or additional PRACH resource depending on when preamble transmission is triggered (e.g. the UE may prioritize legacy resource when preamble transmission is triggered in T0, and the UE may prioritize additional PRACH resource when preamble transmission is triggered in T1 in Figure.4).

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in Msg1 retransmission.

  - RAN2 need to discuss whether only the same PRACH resource set (i.e. legacy vs additional) as initial transmission can be selected.

- Whether / how to support fallback from one PRACH resource to another PRACH resource.

Figure. 4 Illustration of RO selection in RACH adaptation in time domain

Thus, we propose:

Proposal 8: To support additional PRACH resource adaptation, RRC spec impacts at least include:

- Introduce a separate SSB-RO mapping configuration for additional PRACH resource, e.g. ssb-perRACH-OccasionAndCB-PreamblesPerSSB-NES in RACH-ConfigCommon.

- How to integrate additional PRACH resource set into general RACH framework, which couples with which understanding in Proposal 7 is agreed.

Proposal 9: To support additional PRACH resource adaptation, MAC spec impacts at least include:

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in initial Msg1 transmission.

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in Msg1 retransmission.

- Whether / how to support fallback from one PRACH resource to another PRACH resource.

## 2.3 Adaptation of SSB in time domain

Agreement

For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study

- Adaptation of SSB burst periodicity

- Adaptation based on two SSB configurations where up to two configurations can be active

- Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration

- Adapting the transmitted number of SSBs within a SSB burst

- Cell DTX for SSB adaptation

- Whether to support new SSB burst periodicity value(s)

- Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst)

  - New compact SSB burst(s)

  - Adapting the position of SSBs within a SSB burst

- Other mechanisms/combinations are not precluded

RAN1#116 discussed adaptation of SSB, and below agreement on adaptation mechanism was made [6]:

In above agreement, we think there are two groups of mechanisms which are highlighted as yellow and cyan respectively:

- The highlighted ones are essentially adaptation of SSB periodicity (e.g. ssb-PeriodicityServingCell in SIB1) and/or transmitted beam(s) in one SSB burst (i.e. ssb-PositionsInBurst in SIB1) within IE ServingCellConfigCommonSIB, as illustrated in Figure.5.

  - We think they may have RAN2 impacts related to L3 measurement, including RRM, RLM and BFD:

    - RRM: In legacy RRM based on SSB, the NW need to configure UEs SMTC and SSB_ToMeasure in MeasObjectNR for RRC_CONNECTED UEs and in SIB2/4 for RRC_IDLE/RRC_INACTIVE UEs. However, the legacy RRC reconfiguration of SMTC and/or SSB_ToMeasure may not work because RRC signaling is slower than DCI / MAC-CE which may lead to failure of UE performing RRM.

    - RLM: In legacy RLM, several SSB(s) can be configured as RLM-RS in RRC. However, the legacy RRC reconfiguration of RadioLinkMonitoringConfig may not work because RRC signaling is slower than DCI / MAC-CE which may lead to failure of UE performing RLM (e.g. one actually transmitted beam configured as RLM-RS is turned off after SSB adaptation).

    - BFD: In legacy BFD in SCell, several SSB(s) can be configured as BFD-RS in RRC. However, the legacy RRC reconfiguration of RadioLinkMonitoringConfig may not work because RRC signaling is slower than DCI / MAC-CE which may lead to failure of UE performing BFD (e.g. one actually transmitted beam configured as BFD-RS is turned off after SSB adaptation).

Figure. 5 Illustration of SSB adaptation mechanisms

- The highlighted one is enhancement of Cell DTX for SSB adaptation.

  - Please note that Cell DTX is only applied to the UEs in CONNECTED state in Rel-18 NES, and Cell DTX is not in scope of WID. Thus, we think RAN2 can wait RAN1 input.

Observation 6: Adaptation of SSB periodicity and/or transmitted beam(s) in one SSB burst may lead to failure of UE performing L3 measurement, including RRM, RLM and BFD.

Agreement

Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s):

- For cell with both legacy UEs and Rel-19 NES-capable UEs

  - Rel-19 NES-capable UE’s PCell (Connected mode)

    - Study from the following options:

      - Option A1: adaptation for CD-SSB

      - Option A2: adaptation for SSB that is not CD-SSB

      - Option A3: adaptation for SSB not on sync raster

  - Rel-19 NES-capable UE’s SCell

    - Study from the following options:

      - Option B1: adaptation for CD-SSB

      - Option B2: adaptation for SSB that is not CD-SSB

      - Option B3: adaptation for SSB not on sync raster

  - FFS: Rel-19 NES-capable UE in idle/inactive mode

- Note: Impact to idle/inactive UEs shall be minimized

Meanwhile, RAN1#116b also made below agreement on applicable scenario of SSB adaptation [7]:

With this agreement, it is not clear for now whether SSB adaptation is only applicable to CONNECTED UE and whether it can be CD-SSB. We think one potential RAN2 impact is to introduce a new legacy UE barring mechanism if RAN1 agree SSB adaptation is applicable to IDLE / INACTIVE UEs and CD-SSB, i.e., Rel-19 NES cell can be configured to bar legacy UE meanwhile allow Rel-19 NES capable UEs to camp.

Observation 7: It is not clear whether SSB adaptation is only applicable to CONNECTED UE and whether it can be CD-SSB. If RAN1 agree SSB adaptation is applicable to IDLE / INACTIVE UEs and CD-SSB, one potential RAN2 impact is to introduce a new legacy UE barring mechanism.

Agreement

For indication of adaptation of SSB in time-domain,

- Support at least SSB adaptation provided by gNB without UE trigger

Finally, RAN1#116b made below agreement on signaling of SSB adaptation [7].

As usual, potential RAN2 work is to design MAC-CE format if RAN1 agree to use MAC-CE to notify UE SSB adaptation. For now, RAN2 can wait RAN1 input.

Based on above discussion, we identify below potential RAN2 work on SSB adaptation.

Proposal 10: On SSB adaptation, potential RAN2 work includes below 3 aspects:

- How the UE performs L3 measurement upon SSB adaptation, including RRM, RLM and BFD.

- If RAN1 agree it is applicable to IDLE / INACTIVE UEs and CD-SSB, RAN2 may need to introduce a new legacy UE barring mechanism, i.e., Rel-19 NES cell bars legacy UE but allows Rel-19 UEs to camp.

- If RAN1 agree to use MAC-CE to notify UE SSB adaptation, RAN2 design MAC-CE format.

# 3 Conclusion

In this contribution, we further discuss RAN2 work on common signal transmission adaptation. Our observations are:

Observation 1: It doesn’t make sense to deploy Rel-19 NES cells which only support Rel-19 UEs capable of paging adaptation especially at late release of NR because it may lead to coverage holes and it goes against the intention of introducing NES.

Observation 2: For option-b, it should allow Rel-19 PF to overlap with legacy PF because:

- If it is not allowed, how to achieve NES gain is not clear.

- It will be difficulty for NW to always avoid such overlapping by its implementation because one PO can be spanned to a frame after the PF, especially when Ns is large (e.g. increased to 16).

Observation 3: In overlapped PF, the same Rel-19 UE may derive different PO to monitor via legacy PCCH-Config and Rel-19 PCCH-Config because Ns values are different in the two PCCH-Config.

Observation 4: If we need to fully compensate the decrease in number of PFs, Ns needs to increase up to 64. It will result in a large RRC payload size of PMO and PEI-O configuration (up to 624bit). However, it is an overdesign because paging adaptation is target for low loading scenario.

Observation 5: In Rel-17, general RACH framework supporting multiple RACH partitions was introduced. A list of up to 255 RACH partitions (i.e. AdditionalRACH-Config) can be configured in SIB1, and the Network can configure different RACH partitions associated with different features (e.g., redcap) or feature combination (e.g., SDT+Msg1-repetition).

Observation 6: Adaptation of SSB periodicity and/or transmitted beam(s) in one SSB burst may lead to failure of UE performing L3 measurement, including RRM, RLM and BFD.

Observation 7: It is not clear whether SSB adaptation is only applicable to CONNECTED UE and whether it can be CD-SSB. If RAN1 agree SSB adaptation is applicable to IDLE / INACTIVE UEs and CD-SSB, one potential RAN2 impact is to introduce a new legacy UE barring mechanism.

Based on observations, our proposals are:

Adaptation of paging occasions

Proposal 1: Don’t introduce new legacy UE barring mechanism for paging adaptation.

Proposal 2: Rel-19 NES cells provide separate paging resource for Rel-19 UEs capable of paging adaptation via a new PCCH-Config different from the legacy one.

Proposal 3a: For option-b, RAN2 confirm that it allows Rel-19 PF to overlap with legacy PF.

Proposal 3b: For option-b, Rel-19 UEs only monitor the PO(s) derived from Rel-19 PCCH-Config, including in overlapped PF with legacy UEs.

Proposal 4: For option-b, RAN2 is kindly suggested to only consider extending Ns value to 8 and 16 (i.e. no need to introduce large Ns vale like 32 or 64) while N value can be extended with 32, 64, 128, and 256.

Adaptation of PRACH in time domain

Proposal 5: Don’t introduce new legacy UE barring mechanism for RACH adaptation in time domain, i.e., Rel-19 NES cells support both Rel-19 UEs capable of RACH adaptation and legacy UEs.

Proposal 6: RAN2 first study CBRA in RACH adaptation.

Proposal 7: RAN2 discuss which is the correct understanding of RACH adaptation in time domain:

- Understanding 1: RACH adaptation is applied to only Rel-15 RACH resources (i.e. only support adaptation between rach-ConfigCommon and additional PRACH resources).

- Understanding 2: RACH adaptation is applied to all/part of RACH features or feature combination (i.e. support adaptation between rach-ConfigCommon and additional PRACH resources, and adaptation between AdditionalRACH-Config and additional PRACH resources). FFS whether/which RACH features or feature combination are not applicable.

Proposal 8: To support additional PRACH resource adaptation, RRC spec impacts at least include:

- Introduce a separate SSB-RO mapping configuration for additional PRACH resource, e.g. ssb-perRACH-OccasionAndCB-PreamblesPerSSB-NES in RACH-ConfigCommon.

- How to integrate additional PRACH resource set into general RACH framework, which couples with which understanding in Proposal 7 is agreed.

Proposal 9: To support additional PRACH resource adaptation, MAC spec impacts at least include:

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in initial Msg1 transmission.

- Whether / how the UE prioritizes between legacy PRACH resource and additional PRACH resource in Msg1 retransmission.

- Whether / how to support fallback from one PRACH resource to another PRACH resource.

Adaptation of SSB in time domain

Proposal 10: On SSB adaptation, potential RAN2 work includes below 3 aspects:

- How the UE performs L3 measurement upon SSB adaptation, including RRM, RLM and BFD.

- If RAN1 agree it is applicable to IDLE / INACTIVE UEs and CD-SSB, RAN2 may need to introduce a new legacy UE barring mechanism, i.e., Rel-19 NES cell bars legacy UE but allows Rel-19 UEs to camp.

- If RAN1 agree to use MAC-CE to notify UE SSB adaptation, RAN2 design MAC-CE format.

# 4 References

[1] RP-242354, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN2#125b, Chair Notes.

[3] RAN2#127, Chair Notes.

[4] R2-24xxxx, Summary of [109], OPPO

[5] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[6] RAN1#116, Chair Notes.

[7] RAN1#116b, Chair Notes.

[8] RAN1#117, Chair Notes.