# 3GPP TSG RAN WG2 Meeting #127b      	                           R2-2408566
Hefei, China, 14th – 18th Oct., 2024

Agenda item:	8.1.5

Source:	Apple

Title:	Discussion on Model transfer / delivery

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Model transfer/delivery [RAN2/RAN1]:

  - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

The updated Rel-19 WID of AI/ML for air interface (WID RP-242399) was agreed in RAN#105 [1]. The WI objective on model transfer / delivery can be found below:

This objective had not been discussed in RAN2 in Rel-19 yet. Meanwhile it was discussed in RAN1 for several meetings and some progress was made.

In this contribution, we first provide a brief review on RAN1 progress, and then we analyze model transfer / delivery for one-sided model and two-sided model, respectively

- Review of RAN1 progress

- Signaling / Protocol of model transfer / delivery

  - Model transfer / delivery for one-sided model

  - Model transfer / delivery for two-sided model

# 2 Discussion

## 2.1 Review of RAN1 progress

During Rel-18 study item phase, RAN1 identified below model delivery / transfer cases:

| Case | Model delivery/transfer | Model storage location | Training location |
| --- | --- | --- | --- |
| y | model delivery (if needed) over-the-top. | Outside 3GPP Network | UE-side / NW-side / neutral site |
| z1 | model transfer in proprietary format. | 3GPP Network | UE-side / neutral site |
| z2 | model transfer in proprietary format. | 3GPP Network | NW-side |
| z3 | model transfer in open format. | 3GPP Network | UE-side / neutral site |
| z4 | model transfer in open format of a known model structure at UE, i.e., an exact model structure as has been previously identified between NW and UE and for which the UE has explicitly indicated its support. | 3GPP Network | NW-side |
| z5 | model transfer in open format of an unknown model structure at UE, i.e., any other model structure not covered in z4, including any model structure that is only partially known. | 3GPP Network | NW-side |
| Note: The definition of various Cases is only for the purpose of facilitating discussion and does not imply applicability, feasibility, entity mapping, architecture, signalling nor any prioritization. |  |  |  |

During Rel-19, RAN1 further discussed above identified cases of model transfer / delivery under the AI of “Other aspects on data and models”, where the key agreements / conclusions in each meeting were summarized in Table.1.

| Meeting | Key agreements / conclusion on model transfer / delivery |
| --- | --- |
| RAN1#116 | From RAN1 perspective, the model transfer/delivery Case z5 is deprioritized for Rel-19. |
| RAN1#116b | From RAN1 perspective, the model transfer/delivery Case z2 is deprioritized at least for UE-sided model in Rel-19 due to the following reasons: - Risk of proprietary design disclosure - Burden of offline cross-vendor collaboration From RAN1 perspective, the model transfer/delivery Case z3 is deprioritized for Rel-19 due to the following reasons (compared to Case y): - No much benefit compared to Case y - Risk of proprietary design disclosure - Large burden of offline cross-vendor collaboration - Additional burden on model storage within in 3GPP network |
| RAN1#117 | From RAN1 perspective, for model delivery/transfer Case z4, further study the following alternatives (including the necessity/feasibility/benefits): - Alt. A - Step A-1: UE reports the supported known model structure(s) to network - Step A-2: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step A-1 - FFS: whether some additional step(s), and/or whether other information is needed - Alt. B - Step B-0: UE reports to NW its support of model transfer/delivery case z4 - Note: Step B-0 may be before or after Step B-1, or not necessary - Step B-1: NW indicates to UE the candidate known model structure(s) - Step B-2: UE reports to NW which model structure(s) out of the candidate known model structure(s) indicated in Step B-1 is supported - Step B-3: NW transfers to UE the parameters for one or more of supported known model structure(s) reported in Step B-2 - FFS: whether some additional step(s), and/or whether other information is needed - Note: Other alternative(s) is not precluded - Note: Other method(s) of parameter exchange from NW to UE side is a separate discussion. |
| RAN1#118 | From RAN1 perspective, the “known model structure(s)” of the model transfer/delivery Case z4 at least include known information on the following aspects - Model type/backbone (e.g., Transformer, CNN and so on) - In case model type is a neural network - Number of layers - Layer types/structure (e.g., full connected, activation layer and so on) - Layer size (e.g., the number of parameters of a layer) - Connection between different layers - model input/output related information From RAN1 perspective, model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported) |

Table.1 Summary of RAN1 agreements / conclusions on model transfer / delivery

Based on Table. 1, we can see:

- RAN1 deprioritize the model transfer/delivery Case z5/z2/z3 in Rel-19.

- RAN1 identify two options (option A and B) for the model transfer/delivery Case z4, where the NW and UE needs to align on the “known model structure(s)” and NW sends UE the parameters.

- RAN1 only agree that model transfer is needed for CSI compression with two-sided model.

Observation 1: Key RAN1 model transfer progress in Rel-19 can be summarized as below:

- RAN1 deprioritize the model transfer/delivery Case z5/z2/z3 in Rel-19.

- RAN1 identify two options (option A and B) for the model transfer/delivery Case z4, where the NW and UE needs to align on the “known model structure(s)” and NW sends UE the parameters.

- RAN1 only agree that model transfer is needed for CSI compression with two-sided model.

## Signalling / Protocol of Model transfer / delivery

In TR 38.864 [2], the following 8 solutions of model transfer / delivery were identified:

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

- Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

- Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

- Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data.

- Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data.

- Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP).

- Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

But there was no conclusion or prioritization being captured in TR 38.864 [2].

Since RAN1 has discussed model transfer for one-sided model and two-sided model respectively, we think RAN2 also need to analyze the above signaling for them, respectively.

### 2.2.1 Model transfer / delivery for one-sided model

As observed in Table.1, RAN1#118 agreed that model transfer is needed for CSI compression with two-sided model [3] because it requires inter-vendor training collaboration.

Conclusion

From RAN1 perspective, model transfer is needed at least for some (e.g., Option 3b) of inter-vendor training collaboration option(s) of CSI compression using two-sided model (if supported)

Please note that there is no conclusion whether 3GPP standardized model transfer / delivery solution is needed for one-sided model.

Observation 2: RAN1 agreed that model transfer is needed for CSI compression with two-sided model because it requires inter-vendor training collaboration. But there is no conclusion whether 3GPP standardized model transfer / delivery solution is needed for one-sided model.

In our understanding, below use cases of 3GPP standardized solutions were raised in Rel-18 study item phase:

- Use case 1: NW (e.g. gNB/LMF/OAM) or 3rd party may help train UE-sided model, and then transfer trained model to the UE

  - We don’t think it is a valid use case because which model(s) UE can run is hardware/firmware dependent (e.g. power status, UE memory) which can’t be shared with 3rd party. As NW or 3rd party is not aware of one UE’s specific hardware/firmware environment, we believe only UE vendor can ensure the performance of UE-sided model at least in Rel-19.

- Use case 2: UE’s OTT server trains UE-sided model, and then transfer trained model to the UE

  - This use case is valid, but it is not necessary to introduce 3GPP standardized signaling because 3GPP transparent solution (option 4a / Case y) can work and it has been widely used in industrial.

Observation 3: For one-sided model, the only valid use case is OTT server transfer trained UE-sided model to the UE. But it is not necessary to introduce 3GPP standardized signaling because 3GPP transparent solution (option 4a / Case y) can work and it has been widely used in industrial.

Thus, we propose:

Proposal 1: For one-sided model, only 3GPP transparent solution (i.e. RAN2 solution 4a and RAN1 Case y) is supported in Rel-19.

### 2.2.2 Model transfer / delivery for two-sided model

Agreement

- For Option 3, further define the two sub-options:

  - 3a: Parameters received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing.

  - 3b: Parameters received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

- For Option 5, further define the two sub-options:

  - 5a: Model received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing.

  - 5b: Model received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

In CSI compression with two-sided model, model transfer / delivery between UE and gNB is needed because it needs training collaboration. For example, in case of training collaborations Type 3, the UE may first train model for encoder-part, and then transfers trained model to gNB for its further training of model for decode-part. Meanwhile, as illustrated below, RAN1 identified option 3b which requires the UE to directly use received model parameter for inference. It is obvious that 3GPP standardized solution is needed in this case.

Among all the RAN2 identified 8 solutions, we think solution 1a (i.e. CP between UE and gNB) can be prioritized. Our considerations are:

Solution 1a (i.e. CP tunnel between UE and gNB)

- It is aligned with RAN1 model transfer/delivery Case z4 which is prioritized in RAN1, where the NW and UE needs to align on the “known model structure(s)” and NW sends UE the parameters.

- Meanwhile, solution 1a is between UE and gNB, which is also aligned with the use case of CSI compression.

Solution 1b (i.e. UP tunnel between UE and gNB)

Solution 1b is the only solution which companies can't achieve consensus on how it works during Rel-18. We prefer to preclude this solution due to below reasoning:

- It may change the basic concept of PDU session (i.e. UP tunnel) and basic UP protocol stack, which needs significant spec change in late stage of 5G.

- Its benefit over Solution 1a (i.e. CP solution between UE and gNB) is not clear.

Solutions between UE and CN (i.e. solution 2a/2b/3a/3b)

These solutions are signaling between UE and CN/LMF. We think these solutions are not suitable for CSI compression which is just between UE and gNB. If 3GPP adopt these solutions, it means that the model transfer route will be like UE→CN→gNB. Compared with solution 1a, it will introduce one extra hop from UE to CN which is unnecessary and may not work for CSI compression with strict latency requirement. Thus, we prefer to preclude these solutions in Rel-19.

Observation 4: For signaling solutions between UE and CN (i.e. solution 2a/2b/3a/3b), it means that the model transfer route will be like UE→CN→gNB. Compared with solution 1a, it will introduce one extra hop from UE to CN which is unnecessary and may not work for CSI compression with strict latency requirement.

Solutions between UE and OAM (i.e. solution 4b)

Like the solutions between UE and CN, this solution will introduce extra hop in model transfer route like UE→OAM→gNB, it will introduce one extra hop from UE to CN which is unnecessary and may not work for CSI compression with strict latency requirement. Meanwhile, please note that there is no standardized interface between UE and OAM, which may need significant spec work. Thus, we prefer to preclude this solution in Rel-19.

Observation 5: There is no existing standardized interface between UE and OAM, which may need significant spec work.

Based on above analysis, we propose:

Proposal 2: For two-sided model (CSI compression), RAN2 prioritize solution 1a due to below justifications:

- Solution 1a is aligned with model transfer/delivery Case z4 which is prioritized in RAN1.

- Solution 1b can’t work in Rel-19 because it challenges the basic concept of PDU session (i.e. UP tunnel) and basic UP protocol stack.

- Other signaling solutions between UE and CN/LMF/OAM (i.e. solution 2a/2b/3a/3b/4b) are precluded because they are not aligned with use case of CSI compression which is only between UE and gNB.

# 3 Conclusion

In this contribution, we share our view on signaling options of model transfer / delivery. Our observations are:

Observation 1: Key RAN1 model transfer progress in Rel-19 can be summarized as below:

- RAN1 deprioritize the model transfer/delivery Case z5/z2/z3 Rel-19.

- RAN1 identify two options (option A and B) for the model transfer/delivery Case z4, where the NW and UE needs to align on the “known model structure(s)” and NW sends UE the parameters.

- RAN1 only agree that model transfer is needed for CSI compression with two-sided model.

Observation 2: RAN1 agreed that model transfer is needed for CSI compression with two-sided model because it requires inter-vendor training collaboration. But there is no conclusion whether 3GPP standardized model transfer / delivery solution is needed for one-sided model.

Observation 3: For one-sided model, the only valid use case is OTT server transfer trained UE-sided model to the UE. But it is not necessary to introduce 3GPP standardized signaling because 3GPP transparent solution (option 4a) can work and it has been widely used in industrial.

Observation 4: For signaling solutions between UE and CN (i.e. solution 2a/2b/3a/3b), it means that the model transfer route will be like UE→CN→gNB. Compared with solution 1a, it will introduce one extra hop from UE to CN which is unnecessary and may not work for CSI compression with strict latency requirement.

Observation 5: There is no existing standardized interface between UE and OAM, which may need significant spec work.

Based on observations, our proposals can be found below:

Proposal 1: For one-sided model, only 3GPP transparent solution (i.e. RAN2 solution 4a and RAN1 Case y) is supported in Rel-19.

Proposal 2: For two-sided model (CSI compression), RAN2 prioritize solution 1a due to below justifications:

- Solution 1a is aligned with model transfer/delivery Case z4 which is prioritized in RAN1.

- Solution 1b can’t work in Rel-19 because it challenges the basic concept of PDU session (i.e. UP tunnel) and basic UP protocol stack.

- Other signaling solutions between UE and CN/LMF/OAM (i.e. solution 2a/2b/3a/3b/4b) are precluded because they are not aligned with use case of CSI compression which is only between UE and gNB.

# 4 References

[1] RP-242399, New WID: New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] TR 38.843-v2.0.1, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 2023-12.

[3] RAN1#118, Chair Notes.