3GPP TSG-RAN WG2 Meeting #127bis	R2-2408575

Hefei, China, 14 – 18 October 2024

Agenda item:	8.7.3

Source:	Apple

Title:	Views on Enhancements Relating to RRM Measurement Gaps

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

For Rel-19 XR, we have the following objective [1]:

-	Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

-	Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In this meeting RAN2 will discuss the topic for the first time, and this paper provides some of our views about potential RAN2 impacts.

# Discussions

We have identified a few aspects that RAN2 may discuss for enhancements that enable transmission/reception in gaps and/or restrictions for RRM measurements, including:

- UE Assistance Information (UAI)

- Control Signaling to enable transmission/reception in gaps/restrictions.

- MAC behaviors upon measurement gap skipping

Our views on these aspects are discussed below.

## UE Assistance Information (UAI)

According to the LS from RAN1 [2], several types of UAI to enable transmission/reception in gaps or restrictions have been suggested. The type of information ranges from the number of measurement occasions to the traffic characteristics such as packet importance. However, RAN1 has decided not to continue the discussion on UAI, mainly because there is no consensus about whether such UAI is needed or feasible, and other working groups (e.g. RAN4) may have a better understanding about the role that UAI can play in this case. If any UAI is eventually adopted, it is indeed RAN2’s scope to specify it in TS 38.331. Nevertheless, currently RAN4 is still discussing if such UAI is necessary or beneficial. The detailed RAN1 agreements listed in [2] are shown below:

Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.

Based on RAN4’s latest work plans, it seems RAN4 will start to discuss the need of UAI in Oct/2024, and they aim to conclude in Apr/2025. Thus, we do not expect any RAN2 work about UAI until RAN4 draws the conclusions by then. One may think that some types of the UAI identified by RAN1 may require RAN2’s involvement, such as UAI relevant to PSI; but in our views RAN2 should anyway wait until the impacts to RRM performance of measurement gap skipping is thoroughly analyzed by RAN4, before we can discuss whether PSI matters. It is also worth noting that, RAN1’s LS [2] does not indicate any action item for RAN2.

Proposal 1: RAN2 does not discuss UAI for enabling transmission/receptions in gaps/restrictions until it is concluded by RAN4.

## Control Signaling

Another important issue is how the network control this feature. That is, how is the gNB going to let the UE know which gap can be skipped? In our understanding, RAN1 discussions have boiled down to two main options:

- Dynamic indication via DCI

- Semi-Static patterns configured via RRC

Recently RAN1 has made the following working assumption:

For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap(s)/restriction(s); - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

Essentially, the 1-bit in the DCI indicates whether the UE should skip the very first gap/restriction occasion after the PDCCH, with a minimum time offset constraint. The feasibility of such solution should be evaluated by RAN4, and RAN1 has therefore sent RAN4 an LS (R1-2407561) to confirm if RAN4 sees any issue about such working assumption.

From RAN2 point of view, we think there are some open problems with DCI-based approach that should be clarified, especially when a UE is configured with multiple measurement gap configurations. Since only 1-bit is used in the DCI, it is insufficient to indicate whether skipping is applicable to any of the measurement gap configuration, or only restricted to specific measurement gap configuration(s). If this is applicable to any measurement gap, we are not sure about the impacts to RRM performance – but we believe this is up to RAN4 to sort out. On the other hand, if this is only applicable to a subset of measurement gap configurations, it is questionable how much gains we can have in terms of XR capacity.

In contrast, we tend to think that the approach based Semi-Static configuration via RRC provides the UE a more explicit guidelines about which measurement gap occasions should be skipped at what time. And since such knowledge is shared between network and UE (as it is configured by gNB), we believe it is also advantageous for scheduling policy.

With this in mind, we think RAN2 can also analyze the pros and cons of both approaches (i.e. DCI-based approach and RRC-based approach), while RAN4 is still evaluating the feasibility of DCI-based approach.

Proposal 2:  RAN2 should also analyze both DCI-based control and RRC-based control for measurement gap skipping, while RAN4 is still evaluating the feasibility of DCI-based control.

## MAC Behaviors

In TS 38.321, Clause 5.14 describes how an activated measurement gaps should be handled at MAC layer. In general, the UE does not perform PUSCH/PUCCH transmissions and PDSCH/PDCCH reception, during the activated measurement gap. In Rel-19, if the gNB instructs the UE to skip a particular measurement gap/restriction, then basically there is no more restriction about any transmission or reception, the UE can consider this gap as non-existing. We propose RAN2 to confirm this is the intended behaviors of MAC.

Proposal 3: From MAC perspective, when a measurement gap is skipped, the UE can perform any transmission or reception without restriction during the interval of the skipped measurement gap.

# Conclusions

In this paper, we have provided some of our views about measurement gaps skipping, and we propose the following:

Proposal 1: RAN2 does not discuss UAI for enabling transmission/receptions in gaps/restrictions until it is concluded by RAN4.

Proposal 2:  RAN2 should also analyze both DCI-based control and RRC-based control for measurement gap skipping, while RAN4 is still evaluating the feasibility of DCI-based control.

Proposal 3: From MAC perspective, when a measurement gap is skipped, the UE can perform any transmission or reception without restriction during the interval of the skipped measurement gap.

# References

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), Sept. 2024, Melbourne, Australia.

- R2-2406216 / R1-2405736, LS on UE Assistance Information