3GPP TSG-RAN WG2 Meeting #127bis	R2-2408577

Hefei, China, 14 – 18 October 2024

Agenda item:	8.7.4.2

Source:	Apple

Title:	Views on DSR Enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

Based on the study in Rel-19 XR Phase 3, the following objective has been added to the work item [1]:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

For DSR enhancements, we have made the following agreements in previous meetings [2,3]:

RAN2 #126 Agreements: Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. FFS whether DSR triggering is impacted FFS whether PDU set importance needs to be included RAN2 #127 Agreements: Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

This paper presents some of our views on DSR enhancement.

# Discussions

## Illustration of Rel-19 DSR Concept

In this section, we would like to outline the concept of Rel-19 DSR. The multiple pairs of remaining time and buffer size information in Rel-19 DSR would provide a more comprehensive picture of buffer delay status to the network. With multiple thresholds, this is analogous to dividing all the data buffered in an LCG into multiple portions in accordance with which threshold the remaining time of each PDCP SDU satisfies. In other words, the buffered PDCP SDUs are partitioned into different groups/portions based on their remaining time. For each of the portion, Rel-19 DSR should report the corresponding buffer size, and the shortest remaining time among the PDCP SDUs in this portion. An illustrative example is depicted in Figure 1 based on the assumptions where three remaining time thresholds (namely T0, T1 and T2, where T2>T1>T1>0) are configured for the LCG. With the three threshold levels in this example, all the buffered data can be divided into 4 portions. Only the buffer sizes and shortest remaining time of 3 out of these 4 portions will be reported, because DSR does not need to report the information relating to any data that has remaining time larger than the highest threshold (e.g. T2 in this example). Thus, three “pairs” of remaining time and buffer sizes are reported in this example.

Figure 1 An illustrative example of information to be reported with Rel-19 DSR for an LCG.

Proposal 1: For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple remaining time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information:

- Buffer size of data volume in each portion

- Shortest remaining time among PDCP SDUs buffered in each portion.

## Remaining Time Threshold for DSR Triggering

RAN2 has agreed to support DSR with multiple pairs of remaining time and buffer size information. In other words, the DSR for one LCG is derived based on multiple remaining time threshold levels that are configured for one LCG, in contrast to single remaining time threshold level as in Rel-18. However, we have also agreed that there is only one remaining time threshold for DSR triggering.

From our point of view, the network can simply choose one of the multiple remaining time threshold levels for an LCG, as the triggering threshold. Thus, this can be entirely up to network implementation to decide how DSR should be triggered. We do not see a need to impose any constraints in terms of network configuration.

Proposal 2: The network can select and configure one of the multiple remaining time thresholds of an LCG as the remaining time threshold for DSR triggering.

## Definition of Delay-Critical Data

In Rel-18, we have defined delay-critical PDCP SDUs, which are basically the PDCP SDUs with remaining time smaller than or equal to the remainingTimeThreshold. Since Rel-18 DSR focuses on the reporting of volume of delay-critical data, such definition facilitates the data volume calculation procedure.

For Rel-19, the UE needs to report the buffer size corresponding to multiple remaining time thresholds, not just the data with remaining time under one specific threshold. Hence, we think the differentiation between delay-critical data and non-delay-critical data does not matter that much for Rel-19 DSR. The UE should only calculate the buffer size and remaining time in accordance with each of the configured remaining time threshold. Whether “non-delay-critical” data is included in DSR becomes a network implementation issue, as it depends on how the threshold levels are set by the network.

Proposal 3: The definition of delay-critical data does not matter for Rel-19 DSR, as the UE simply calculate and report buffer size corresponding to each remaining time threshold configured by the network for this LCG.

## Non-Delay Critical Data Ahead of Delay Critical Data

A few companies have argued that, some less critical data may block the more urgent data. Therefore, it has been suggested that the UE can include non-delay critical data ahead of delay-critical data in buffer size calculation for DSR. We think this is a corner case, as typically the queue is based on a first-in first-out model, so the packets that are queued in front are naturally more urgent. Furthermore, in most scenarios we believe we can rely on UE implementation to resolve this issue (e.g. sorting the packets in the queue based on their level of urgency). With this in mind, we do not see the need to change the data volume calculation rule.

Proposal 4: There is no need to include non-delay critical data ahead of delay-critical data in the buffer size calculation for DSR.

## PDU Set Importance in DSR MAC CE

It was also suggested that the importance level of buffered data should be reported in the DSR, which can assist delay-aware scheduling. However, in our understanding, PDU Set Importance (PSI) is mainly used for the purpose of discarding when congestion is present according to TS 23.501, rather than scheduling:

The NG-RAN may use the Priority Level (see clause 5.7.3.3) across QoS Flows and PDU Set Importance within a QoS Flow for PDU Set level packet discarding in presence of congestion.

Basically, PSI is not so meaningful when congestion is absent. In our opinions, by default the gNB should anyway strive to allocate UL resources to the UE in accordance with the BSR/DSR information, regardless of how important the buffered data is. Furthermore, even if the gNB can allocate the UL-SCH resource based on its knowledge of data volume associating to specific importance level, eventually we cannot guarantee the more important data in a LCH will be allocated to the UL-SCH more immediately at the UE side, as this is up to UE implementation. Thus, we do not see a clear benefit of including packet importance information in the DSR.

Proposal 5: There is no need to include importance information in the DSR MAC CE.

## Other Issues Relating to Rel-19 DSR

Even though an LCG could be configured with multiple remaining time thresholds for Rel-19 DSR, not all the LCGs of the same MAC entity would have to be configured with the same number of remaining time thresholds. For instance, it should be possible that one LCG of a UE has three remaining time thresholds, while another LCG of the UE has only one remaining time threshold as in Rel-18. We should have such flexibility in Rel-19, such that the network can tailor the configuration properly based on the expected traffic characteristics for each LCG, and the UE complexity in terms of DSR information derivation could also be optimized.

Proposal 6: In Rel-19, the number of configured remaining time thresholds for DSR could be different among LCGs. It is still possible to configure some LCGs with single remaining time threshold as in Rel-18.

Moreover, for an LCG that is configured with multiple remaining time thresholds, it may not be necessary for the UE to always report DSR corresponding to every remaining time threshold that is configured for this LCG (i.e. the UE does not need to always report the buffer size and shortest remaining time for every single data portion as shown in Figure 1). For examples, if the buffer size corresponding to a particular threshold is zero (i.e. no PDCP SDU with remaining time satisfying a particular threshold), the UE can simply skip the reporting for this threshold in the DSR MAC CE, which can in turn reduce some signalling overheads. In general, we think the UE should be conditionally allowed to only report delay information corresponding to a subset (instead of all) of remaining time thresholds configured for an LCG.

Proposal 7: For an LCG configured with multiple remaining time thresholds, the UE should be conditionally allowed to only report delay information (i.e. remaining time and buffer size) corresponding to a subset of remaining time thresholds configured for the LCG.

It does not make sense to ditch away Rel-18 DSR even if a new DSR is to be introduced in Rel-19. For some traffics with relatively small delay budget, Rel-18 DSR based on single remaining time threshold would be sufficient. Moreover, from UE complexity point of view, the burden of generating Rel-18 DSR MAC CE is also lighter, since only one pair of remaining time and buffer size information needs to be derived per LCG. In our view, Rel-19 DSR should be modeled as a new DSR format, and RAN2 should study the co-existence issue of Rel-18 and Rel-19 DSR MAC CE formats (e.g. how does the UE select between Rel-18 and Rel-19 DSR MAC CE format).

Proposal 8: RAN2 should study the co-existence of Rel-18 DSR and Rel-19 DSR MAC CE formats.

# Conclusions

In this paper, we have presented our views on Rel-19 DSR enhancements and proposed the following:

Proposal 1: For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple remaining time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information:

- Buffer size of data volume in each portion

- Shortest remaining time among PDCP SDUs buffered in each portion.

Proposal 2: The network can select and configure one of the multiple remaining time thresholds of an LCG as the remaining time threshold for DSR triggering.

Proposal 3: The definition of delay-critical data does not matter for Rel-19 DSR, as the UE simply calculate and report buffer size corresponding to each remaining time threshold configured by the network for this LCG.

Proposal 4: There is no need to include non-delay critical data ahead of delay-critical data in the buffer size calculation for DSR.

Proposal 5: There is no need to include importance information in the DSR MAC CE.

Proposal 6: In Rel-19, the number of configured remaining time thresholds for DSR could be different among LCGs. It is still possible to configure some LCGs with single remaining time threshold as in Rel-18.

Proposal 7: For an LCG configured with multiple remaining time thresholds, the UE should be conditionally allowed to only report delay information (i.e. remaining time and buffer size) corresponding to a subset of remaining time thresholds configured for the LCG.

Proposal 8: RAN2 should study the co-existence of Rel-18 DSR and Rel-19 DSR MAC CE formats.

# References

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), Sept. 2024, Melbourne, Australia.

- 3GPP RAN2 #126 Chair’s Notes, Fukuoka, Japan, May 2024.

- 3GPP RAN2 #127 Chair’s Notes, Maastricht, Netherlands, August 2024.