3GPP TSG-RAN WG2 Meeting #127bis	R2-2408578

Hefei, China, 14 – 18 October 2024

Agenda item:	8.7.5

Source:	Apple

Title:	Views on RLC-AM Enhancements for Rel-19 XR

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

For Rel-19 XR Phase 3, we have the following objective [1]:

-	Specify the following user plane enhancements [RAN2]

-	RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

According to the agreements made in RAN2 #125bis, it is confirmed that RAN2 will consider the following directions:

RAN2 #125bis Agreements: RAN2 will analyse solutions to ensure timely RLC retransmission(s) for XR RAN2 will analyse how to avoid unnecessary retransmissions (e.g. to avoid reTx of out-dated packets)

For each of these directions, we have some extensive discussions during RAN2 #126 and RAN2 #127. The following agreements have been reached [2,3]:

RAN2 #126 Agreements on Timely RLC Retransmissions: To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic RAN2 #126 Agreements on Avoidance of Unnecessary Retransmissions: For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned RAN2 #127 Agreements on Avoidance of Unnecessary Retransmissions: Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first. In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer. In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach

This paper aims to provide some of our views on the directions that RAN2 has identified for RLC-AM enhancements.

# Discussions

## Timely Retransmission

### Autonomous Retransmission

To achieve faster retransmission, several companies have proposed that transmitter RLC entity may autonomously perform retransmission even if no feedback is received from the receiver (namely autonomous retransmission). From the latency perspective, such scheme may be beneficial as a RLC SDU can be retransmitted more proactively and quickly. However, some concerns on impacts to the system capacity have also been brought up, especially in uplink scenarios. The UE may not know the network status in general, blind retransmission may cause further congestions to other traffic flows and/or users.

In order to minimize the potential impacts to the system capacity, we think RAN2 should only consider autonomous retransmission by enhancing the existing mechanisms that are already well-established. It is undesirable and risky if autonomous retransmission is performed based on various types of new triggers that RAN2 has not extensively studied. Besides, given that the features of PDCP duplication and MAC/PHY repetitions are also available, this is questionable if there is a need to introduce any more brand-new mechanisms for such purposes.

We have noted that, RLC-AM of 5G NR already the behaviors of autonomous retransmission based on the expiration of t-PollRetransmit, as specified in TS 38.322:

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or - consider any RLC SDU which has not been positively acknowledged for retransmission. - include a poll in an AMD PDU as described in clause 5.3.3.2.

It is clear that, the current specification has provided some guidelines about when a UE can trigger autonomous retransmission, and which RLC SDU (that has been submitted to lower layer) can be selected to be considered for retransmission. Hence, in our opinions, reusing such framework makes a lot of sense in terms of performance benefits and specification impacts. In particular, there will be less concerns on impacts to capacity as this mechanism has existed in the specification for several releases already.

With the considerations above, we think expiry of t-PollRetransmit should be the baseline for any further enhancement for autonomous retransmission triggering in Rel-19. For instance, the transmitter may consider a delay-critical RLC SDU for retransmission when t-PollRetransmit expires, to ensure a more urgent packet is retransmitted more quickly. Alternatively, the transmitter can also consider a RLC SDU corresponding to a more important PDU Set for retransmission in such cases, especially if PSI-based discarding is activated for this DRB.

Proposal 1: The enhancements for autonomous retransmission should be based on the extensions of the existing trigger, i.e. the expiry of t-PollRetransmit. For instance, the transmitter considers a delay-critical RLC SDU for retransmission upon expiry of t-PollRetransmit.

Moreover, to make sure autonomous retransmission based on t-PollRetransmit can be triggered faster when there is a need, while minimizing autonomous retransmission that can unnecessarily impact capacity, we think RAN2 can consider an adaptive t-PollRetransmit timer value to strike a balance. More specifically, the transmitter may apply different t-PollRetransmit timer values, in accordance with whether an urgent packet is present. If there is a RLC SDU in the queue that needs to be delivered urgently (e.g. the remaining of which is small), a t-PollRetransmit timer with a shorter duration can be used in a hope to trigger autonomous retransmission more rapidly. Conversely, if none of the RLC SDUs in the queue (or the RLC SDUs that are waiting for acknowledgement) is considered urgent, the default t-PollRetransmit timer can be applied to minimize unnecessary triggering of autonomous retransmission.

Proposal 2: RAN2 can further discuss if different t-PollRetransmit timer values should be applied under different conditions relating to the presence of urgent RLC SDUs.

### Polling Enhancements

Polling is used by the transmitter side to trigger status feedback from the receiver side. The transmitter side of AM RLC entity can only move the transmitting window forward when the packets with SNs within the window are positively acknowledged. Otherwise, the new packets with SN outside the window cannot be transmitted. Thus, polling is a proactive method for the transmitter to solicitate the receiver about the status of transmitted packets, as an attempt to push the window forward.

However, currently the triggering of polling is only allowed under certain conditions, such as when no new RLC SDU can be transmitted (e.g. due to window stalling), when PDU_WITHOUT_POLL is greater than or equal to pollPDU, or when BYTE_WITHOUT_POLL is greater than or equal to pollByte.

To speed up RLC-AM operation to accommodate the traffics with small delay budget, especially for the packets that are already considered as delay-critical, we think the triggering conditions of polling can be extended by taking delay-criticality of RLC SDUs into account. The transmitter may be able to trigger a poll when there is a critical need. In particular, the conditions may include:

- When a delay-critical RLC SDU is present in the queue,

- When the number of RLC PDUs corresponding to delay-critical RLC SDUs sent since the last status report is received satisfies a threshold,

- When the number of bytes RLC PDUs corresponding to delay-critical RLC SDUs sent since the last status report is received satisfies a threshold.

Proposal 3: RAN2 can consider poll triggering based on conditions relating to the presence of more urgent RLC SDUs.

### Feedback Enhancements

The receiver side of AM RLC entity should provide a STATUS PDU to indicate positive and/or negative acknowledgements of RLC SDUs. However, the receiver side of AM RLC entity would only initiate the STATUS PDU when a poll is received, or when a reception failure is detected (and t-Reassembly is expired). Furthermore, even if the STATUS report is triggered, the receiver side of AM RLC entity must wait until the expiration of t-StatusProhibit (if running) before it can send the STATUS PDU.

Owing to such limitations, we think RAN2 can consider some more proactive mechanisms for the receiver side of AM RLC entity to trigger/send the STATUS PDU more timely. For examples, the new triggering events for status report can be introduced, such as:

- Whenever a RLC SDU is completely received, or

- Whenever the state variable RX_Next is updated.

Additionally, under specific triggering events of status reports, the receiver side of AM RLC entity may ignore or stop the t-StatusProhibit if it is running, in order to ensure the STATUS PDU can be included in the very next transmission opportunity.

Proposal 4: RAN2 can consider introducing conditions for the receiver side of the AM-RLC to trigger status report more proactively. The t-StatusProhibit (if running) could be stopped early to ensure more rapid transmission of STATUS PDU.

## Avoidance/Minimization of Unnecessary Retransmission

For avoidance of unnecessary retransmission, RAN2 has identified the following three approaches:

- Transmitter (TX)-Initiated Approach

- Receiver (RX)-Initiated Approach

- Combined Approach

During the online discussions in RAN2 #127, quite a few companies agree that TX-Initiated approach is more advantageous than RX-Initiated in terms of latency, as the transmitter knows exactly when the packets are discarded, it can stop retransmission immediately. However, regarding the issue relating to window synchronization, some companies think we should follow the baseline where the receiver should send the status report to update the transmitting window. Eventually, the Combined Approach has been identified as a compromise, in which the transmitter can stop retransmission immediately, while window synchronization still relies on the status report from the receiver.

For RX-Initiated approach (and Combined Approach), we have agreed that the receiver would need to use a timer to determine that a packet has been discarded by the transmitter. In our understanding, the timer may start when a SN gap is detected, and it may consider that the packet(s) corresponding to the SN(s) of this gap as discarded when these missing packets are not received upon timer expiry. However, in XR the transmitter may discard a large number of packets simultaneously (e.g. due to PDU Set discard), which can result in a number of non-consecutive SN gaps at the same time. In such cases, the receiver can only detect these SN gaps one by one sequentially with the timer. Consequently, it may need to send one status report for each of the subsequent SN gap it has identified. In contrast, with TX-Initiated approach, only one notification message from the transmitter may be sufficient even if more than one non-consecutive SN gaps are created due to discarding. Thus, in addition to the benefits of latency performance, TX-Initiated Approach also seems to more efficient from signalling overhead point of view. With this in mind, we think RAN2 should support TX-initiated approach.

Proposal 5: RAN2 should prioritize TX-initiated approach for avoidance of unnecessary retransmission, as it is more advantageous in terms of both latency and signaling overhead.

Alternatively, when the remaining time budget for a RLC SDU is already quite small, it is unlikely that the number of retransmissions for this RLC SDU would reach maxRetxThreshold before its delay budget runs out. Hence, a different (smaller) threshold for maximum number of retransmission (i.e. maxRetxThreshold ) may be applied to such RLC SDUs (or segments) to reduce the number of unnecessary retransmission. In particular, when RETX_COUNT reaches the alternative maxRetxThreshold (which is smaller than the default value of maxRetxThreshold ) for a delay-critical RLC SDU, the transmitter may stop performing retransmission of this RLC SDU and may directly consider it as positively acknowledged.

Proposal 6: RAN2 can consider an alternative (smaller) value of maxRetxThreshold for RLC SDUs (or segments) with remaining time smaller than a threshold. The RLC SDU can be directly considered as positively acknowledged when its RETX_COUNT reaches this alternative maxRetxThreshold.

# Conclusions

In this paper, we have presented our views on RLC-AM enhancements and proposed the following:

Proposal 1: The enhancements for autonomous retransmission should be based on the extensions of the existing trigger, i.e. the expiry of t-PollRetransmit. For instance, the transmitter considers a delay-critical RLC SDU for retransmission upon expiry of t-PollRetransmit.

Proposal 2: RAN2 can further discuss if different t-PollRetransmit timer values should be applied under different conditions relating to the presence of urgent RLC SDUs.

Proposal 3: RAN2 can consider poll triggering based on conditions relating to the presence of more urgent RLC SDUs.

Proposal 4: RAN2 can consider introducing conditions for the receiver side of the AM-RLC to trigger status report more proactively. The t-StatusProhibit (if running) could be stopped early to ensure more rapid transmission of STATUS PDU.

Proposal 5: RAN2 should prioritize TX-initiated approach for avoidance of unnecessary retransmission, as it is more advantageous in terms of both latency and signaling overhead.

Proposal 6: RAN2 can consider an alternative (smaller) value of maxRetxThreshold for RLC SDUs (or segments) with remaining time smaller than a threshold. The RLC SDU can be directly considered as positively acknowledged when its RETX_COUNT reaches this alternative maxRetxThreshold.

# References

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), Sept. 2024, Melbourne, Australia.

- 3GPP RAN2 #126 Chair’s Notes, Fukuoka, Japan, May 2024.

- 3GPP RAN2 #127 Chair’s Notes, Maastricht, Netherlands, August 2024.