# 3GPP TSG RAN WG2 Meeting #127bis      	                                                  R2-2408580
Hefei, China, October 14-18, 2024

Agenda item:	8.2.2

Source:	Apple

Title:	Functional Aspects of Ambient IoT

WID/SID:	FS_Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# 1	Introduction

In prior RAN2 meetings [1][2][[3],  the following function aspects on A-IoT have been agreed:

RAN2#125bis agreements: 1 RRC connection management is not supported. FFS how the resource configuration is provided to the device (if needed based on RAN1 progress) 2 RRM L3 measurement reporting is not supported by Ambient IoT devices. 3 RAN2 assumes, AIoT devices are not required to support ASN.1 encoding/decoding. 4 Periodical System information and MIB are not supported by AIoT devices. This doesn’t preclude any RAN1 defined broadcast signals. 5 RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC 6. SDAP is not supported for UP protocol stack. 7. PDCP layer is not needed. FFS how to handle AS security (if needed pending SA3 discussion) and any other really needed functionalities. 8. RLC layer is not needed. FFS how to handle segmentation (if needed and depending on RAN1 design and upper layer packet size). RAN2 considers segmentation and reassembly would add complexity, however further discussions are needed. 9. No HARQ and RLC AM 10 FFS about the level of visibility required by the reader and what information is necessary for AS layer operations. 11 RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL). FFS how to handle the general QoS requirements from SA2 RAN2#126 agreements: 1. Multiple “AIoT logical channels” for upper layer data are not supported. FFS if AIoT logical channel concept is used depending on final modeling issue. 2 legacy NR BSR/SR is not needed for A-IoT communication. 3 FFS whether further indication of device message size/status is needed 4 AS-layer (above PHY layer) RLC-like retransmission/repetition is not supported. This doesn’t preclude the reader and device sending the payload again as new transmission from MAC perspective. FFS how we handle segmentation case (if needed) RAN2#127 agreements: - Send an LS to RAN1 regarding TBS size. Ask SA2 on the max and typical upper layer packet size. - RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages. FFS details on signaling - RAN2 will study the need and use of a simple message “size” reporting to the reader - At least the following information are considered useful to be visible to the reader from CN - The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful - targeted for one or more than one devices; - approximate number of target devices (if available). FFS on mandatory/optional - RAN2 assumes that commands (e.g., read/write/disable) and/or inventory are carried over the AIOT interface as upper layer data.

In this contribution we provide our views on the outstanding stage-2 issues and functional aspects of Ambient IoT design.

# 2   	Discussion

### 2.1	A-IoT device types

RAN1 consider 3 types of A-IoT devices [4]:

- Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is generated internally by the device.

While the details of this device categorization do not directly impact RAN2 discussion, the fact there are different A-IoT devices with different capabilities does! Therefore, it makes sense to also consider different device capabilities in terms of their baseband (i.e. RAN2-related) capabilities.

Observation 1 	RAN1 agreed to consider different A-IoT device types and therefore it makes sense to consider different device types in terms of their baseband/RAN2-related capabilities.

One way to approach this problem is to start from the RAN1 device type definitions and assumptions and make our way up from there to discuss their baseband related capabilities. This approach is likely to be difficult, though, as it is not clear how we could agree on those. One point to keep in mind though is that if we consider various RFID EPC Class 1 Gen 2 devices in the market (see our contribution submitted to RAN2#125bis in [5]), their capabilities vary rather radically: from read-only devices to devices with writable memory, from sticker form factor to cards, etc. Needless to say, those devices also vary radically in terms of cost.

Observation 2 	Even RFID EPC Class 1 Gen 2 devices come in different types in terms of their baseband capabilities, form factors and cost.

Finally, SA2 have recently agreed on Solution #33 in TR 23.700-13 [6] where two Ambient IoT device types are considered:

- Type A AIoT device that may not need to address privacy requirements and may also use a fixed permanent

- device identifier;

- Type B AIoT device that would use a temporary identifier, to address privacy concerns.

Observation 3 	SA2 have recently agreed to Solution #33 introducing two device types (type A and type B).

This discussion eventually boils down to questions of cost and complexity, which incidentally would affect very much other important topics under discussion, such as security. It is rather obvious that RAN2 has two conflicting goals for the A-IoT design:

- A-IoT device is extremely cost-sensitive, which suggests the simplest possible design.

- There are already RFID EPC C1G2 [7] (and many other) low-cost devices in the market. In order to compete with them, 3GPP solution must provide differentiation – this suggests a somewhat more advanced design with features not currently supported by the devices on the market.

One possible way out of this conundrum is to consider (as RAN1 did!) at least two A-IoT device types: low-end A-IoT device and high-end A-IoT device. Naturally, there can be more than two types, but we think it is sufficient and is definitely a good starting point. Not only would this align with RAN1 design approach, but would also provide a way to resolve many of the conflicting requirements and opinions on the design. For example:

- the low-end A-IoT device type many not need to address privacy requirements and may also use permanent device identifier

- the high-end A-IoT device would use a temporary identifier, to address privacy concerns

This is just one example of such device categorization provided here for illustrative purposes only. All we are proposing at the moment is:

Proposal 1	RAN2 to consider two A-IoT device types (e.g. low-end and high-end) in terms of their baseband/RAN2-related capabilities.

If this is agreeable, eventually we may need to discuss how the RAN1 device types “map” to the RAN2 A-IoT device types, but this issue can be addressed later.

### 2.2 Multiplexing in A-IoT MAC PDUs

RAN2 has agreed that “Multiple AIoT logical channels” for upper layer data are not supported.” But it is still regarded as an open modeling issue whether multiplexing is still allowed in MAC layer. This is because MAC PDU design may or may not support different types of “upper layer” traffic. Although the upper layer traffic may be diverse with multiple different use cases (inventory, command, etc.) to support,  we think some abstraction from MAC layer perspective can be beneficial to clarify this design aspect.

For DL traffic, we can simply classify all R2D traffic as follows :

- R2D data that should trigger response

  - R2D data triggering response with CBRA

  - R2D data triggering response w/o CBRA (e.g., CFRA)

- R2D data (no ACK/response needed)

If all above are legitimate, there are at most only three above message types to be potentially supported in A-IoT MAC layer design for DL traffic. Moreover, for each “destination ID” (device ID, group ID, etc), there should be only one valid type of R2D data at one time. For example, there is no such a case that an A-IoT reader sends “ R2D data triggering response” and “R2D data (no ACK/response needed)” simultaneously to the same device, because aggregating those two upper-layer R2D data shall be merged by CN as a single “super-command” in need of device response, and such “aggregation” operation is transparent to the reader and A-IoT air interface, and should be out of RAN2 scope.

Therefore, we think the multiplexing issue in DL is equivalent to determine whether a MAC PDU that can be allowed to contain different R2D data towards different A-IoT destinations. For example, whether a R2D MAC PDU should contain multiple “Paging records”, with each of which intends to reach a different set of devices? If yes, then multiplexing is need in A-IoT DL. Otherwise, multiplexing is not needed. Thus, RAN2 need first discuss whether multiple R2D data to different destinations are allowed to be included in a single R2D transmission or not.

Proposal 2	Multiplexing support in DL A-IoT MAC PDU depends on whether multiple R2D data to different destinations are allowed to be included in one R2D transmission or not.

For UL traffic, we can also have three D2R traffic defined as the following:

- CBRA Msg 1

- CBRA Msg 3

- Other D2R response for DO-DTT (included response transmitted via CFRA)

Note that if CBRA Msg1 contains random ID, then it is supposed to be terminated in the reader and not reach the CN. But for all the other cases, the upper layer D2R data (including device ID) encapsulated in D2R transmissions from a certain A-IoT device will be terminated at CN. Logically, the A-IoT UL MAC PDU can be very simple, it will be always one of the following two cases:

- Msg1 (Random ID)

- Device ID and/or any upper layer “D2R response” data

Logically, the above two cannot be transmitted at the same time by an A-IoT device. The only exception is the Msg1 in 2-step RACH which includes both Random ID, device ID and D2R data, which is agreed in RAN2#127 meeting[3]. But that is supported as a special message format design which is not regarded as aggregating multiple D2R MAC SDUs. Therefore, there is no multiplexing needed in UL MAC PDU.

Proposal 3	Not support Multiplexing in UL A-IoT MAC PDU.

### 2.3 Visibility to AS layer of reader

RAN2 have the following agreement regarding the visibility issue:

At least the following information are considered useful to be visible to the reader from CN

  - The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

  - targeted for one or more than one devices;

  - approximate number of target devices (if available). FFS on mandatory/optional

First, we believe device id(s) or device group id(s) are always provided to AS layer for each A-IoT procedure. This is obviously needed for “command” procedure, as a command usually targets a specific device or a group of devices. Also, this may also be beneficial for “inventory” procedure, as it is reasonable to assume that in many scenarios the application would need to inventory specific devices and/or groups of devices. The alternative of always performing “inventory” for all the devices in coverage of a reader and then filtering out the devices the application is not interested in is clearly a waste of resources and time.

Based on this,  we think  “targeted for one or more than one devices” is somehow overlaps with “device ID/group ID” information, as obviously whether the inventory/command targets one or more devices can be deduced from the list of device/group id(s) made visible to the reader. Thus, we think RAN2 may need to clarify this understanding that this agreement does not necessarily mean adding a new indication in CN-reader signalling or cross-layer indication for this.

Proposal 4: 	RAN2 clarify that “whether it targets one more than one device” can be realized by providing “group ID/device ID” to the AS layer of reader w/o new explicit indication.

Although the agreement above says “at least the following information…” , we don’t think any further discussion of additional information beyond those is needed in the study phase. Naturally, some additional information (e.g. expected D2R data size) can be beneficial in certain cases, but it is just an optimization – no additional information is strictly needed. In fact, some of the already agreed information can also be considered an optimization. For instance, the “approximate number of devices” may or may not be provided by CN. If not provided by CN, the A-IoT reader can allocate resource based on its own estimation of the number of devices and performs the procedure(s).

To help SA2 understand the core necessary information which needs to be provided by CN, we think RAN2 need to point out which information is mandatory, and which is optional. In general, we think A-IoT optimizations should be considered in the next release, and the first release should be dedicated to the required functionality.

Proposal 5	Only the following information are mandatory to be visible to AS layer: 1) The service type of A-IoT (e.g. inventory, command), 2) targeted for one or more than one device.

### 2.4 Ambient IoT Protocol Stack

For Ambient IoT user plane support, only PHY layer and MAC layer are needed. As agreed in RAN2#125bis meeting [1] that:

RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

For Control Plane design, there could be a dedicated A-IoT CP protocol (e.g. for paging and initial access) on top of MAC layer, but it can also be implemented as MAC Control PDUs. Thus, the presence of “AIoT CP Protocol” is not really needed. For the sake of time efficiency in Rel-19 study, we propose to remove this layer and RAN2 can focus on MAC layer design only for both UP and CP altogether.

Proposal 6	No need for a dedicated AS control plane protocol (i.e. control plane functions are part of the MAC protocol).

On top of data transmission protocol in MAC layer, there could be an upper layer designated as “AIoT layer” (e.g., AIoT-NAS for CP, FFS UP) to communicate with Core Network for both UP and CP, which is to be studied by SA2 and CT1, but out of RAN2 scope, as those data contents may be transparent to the gNB reader.

We think RAN can consider the protocol stack in Figure 2 as a baseline. Of course, depending on the inputs from other 3GPP WGs (RAN1, SA2 or SA3), certain changes would still be considered.

Figure 2. Protocol Stack for A-IoT Topology 1

Proposal 7	RAN2 consider use the above protocol stack as baseline for Ambient IoT Topology 1.

### 2.5 Energy Status Report and D2R Message Size Indication

RAN2 have agreed to study [3]:

- the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages

- the need and use of a simple message “size” reporting to the reader

First off, we’d like to point out that these are clearly optimizations, and they are not required for the basic A-IoT functionality. That being said, since we are going to study them anyway, we might as well spend the time on making them simple and useful, rather than discussing the need over and over again. It’s important to note that in general we still think RAN2 should spend the time on the basic A-IoT functionality rather than optimizations, we are just being constructive on this particular topic.

With regards to the energy status report, the main issue we need to discuss is whether a simple 1-bit indication (e.g. “no more energy”) is sufficient or something more complex is needed. In general, we think it would be rather hard for an A-IoT device to predict what the remaining energy is sufficient for – whether it’s enough for one message or more, what message length its sufficient for, etc. Such prediction is not trivial for a simple device and also depends on many factors being the device knowledge and control, such as CW characteristics, interference, distance to the reader, etc. Therefore, we propose to support 1-bit energy status indication. The details of what that 1-bit means can be discussed in the normative phase. Clearly, the indication should be optional, as not every device may have the complexity to support it. Which device types would support this can also be further discussed in normative phase.

Proposal 8 	To support 1-bit optional energy status indication. The details of what that 1-bit means can be discussed in the normative phase.

With regards to the message “size” reporting, we follow the same logic and propose the simplest solution, which at least some devices would be able to implement (and even that should be made optional as we can’t expect every device to support it). Such 1-bit indication may be interpreted as “I have more data”, but these stage-3 level details can be discussed in the normative phase.

Proposal 9	To support 1-bit optional “more data” D2R indication. The details of what that 1-bit means can be discussed in the normative phase.

# 3	Conclusions and Proposals

In this paper, we have discussed the functional aspects of A-IoT.  We have the following observations:

Observation 1 	RAN1 agreed to consider different A-IoT device types and therefore it makes sense to consider different device types in terms of their baseband/RAN2-related capabilities.

Observation 2 	Even RFID EPC Class 1 Gen 2 devices come in different types in terms of their baseband capabilities, form factors and cost.

Observation 3 	SA2 have recently agreed to Solution #33 introducing two device types (type A and type B).

Then, we have the following proposals:

Proposal 1	RAN2 to consider two A-IoT device types (e.g. low-end and high-end) in terms of their baseband/RAN2-related capabilities.

Proposal 2	Multiplexing support in DL A-IoT MAC PDU depends on whether multiple R2D data to different destinations are allowed to be included in one R2D transmission or not.

Proposal 3	Not support Multiplexing in UL A-IoT MAC PDU.

Proposal 4 	RAN2 clarify that “whether it targets one more than one device” can be realized by providing “group ID/device ID” to the AS layer of reader w/o new explicit indication.

Proposal 5	Only the following information are mandatory to be visible to AS layer: 1) The service type of A-IoT (e.g. inventory, command), 2) targeted for one or more than one device.

Proposal 6	No need for a dedicated AS control plane protocol (i.e. control plane functions are part of the MAC protocol).

Proposal 7	RAN2 consider use the above protocol stack as baseline for Ambient IoT Topology 1.

Proposal 8 	To support 1-bit optional energy status indication. The details of what that 1-bit means can be discussed in the normative phase.

Proposal 9	To support 1-bit optional “more data” D2R indication. The details of what that 1-bit means can be discussed in the normative phase.

# 4	References

[1] 		RAN2#125bis meeting minutes

[2] 		RAN2#126 meeting minutes

[3] 		Chairman’s Notes, RAN2#127

[4] 		RP-240826, Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR

[5] 		R2-2402896, A-IoT device hardware capabilities, Apple

[6]		TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things

[7]		EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID Standard