3GPP TSG-RAN2 Meeting #127bis	R2-2408585

# Hefei, China, October 14th – 18th, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5019 | rev |  | Current version: | 16.18.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on UE behaviour upon reception of RRCReconfigCompleteSL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2024-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current procedure specified in 5.8.9.1.9 is quite insufficient. Although the UE shall “consider the configurations in the corresponding RRCReconfigurationSidelink message to be applied” in this clause, the “to be applied” means the TX UE assuming the peer RX UE has applied the provided configuration, but the Tx UE behaviour is still unspecified. Literally, there are actions to be taken by the UE when the Tx UE of RRCReconfigruaitonSidleink message receives the “RRCReconfigruaitonSidleinComplete” message, such as: - release of SL-DRB (include release Sidelink SDAP/PDCP/RLC/LogicalChannel ) - additional modification of SL-DRB (include (re)configuration of Sidelink SDAP/PDCP/RLC/LogicalChannel) In the current RRC spec, the above Tx UE actions are spread in several different procedures, with the text like “ ….. after receiving the RRCReconfigurationCompleteSidelink message, …“ appearing in different subclauses, such as 5.8.9.1a.1, 5.8.9.1a.2. It is worth nothing that the UE actions according to those sections have been elaborated in 5.8.9.1.3 and referred to the above sections for the RX UE side, but there is no corresponding text in 5.8.9.1.9 for the TX UE side. Based on the above analysis, we think the UE behaviour in 5.8.9.1.9 should also be elaborated to match the procedures in other sections. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 5.8.9.1.9, added “by the peer UE” in “consider the configurations in the corresponding RRCReconfigurationSidelink message to be applied “. - In 5.8.9.1.9, added the missing SL-DRB add/modification/release actions. Impact analysis Impacted functionality: UE Reception of RRCReconfigurationSidelinkComplete. Inter-operability: - If Sidelink UE implements this change and gNB does not, there is no inter-operability issue between UE and gNB. - If Sidelink TX UE implements this change, but the Sidelink RX UE is not implemented this change, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong UE implementation of processing RRCReconfigurationCompleteSL message. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.9.1.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

BEGIN OF CHNAGES

##### 5.8.9.1.9	Reception of an RRCReconfigurationCompleteSidelink by the UE

The UE shall perform the following actions upon reception of the RRCReconfigurationCompleteSidelink:

1>	stop timer T400 for the destination, if running;

1>	consider the configurations in the corresponding RRCReconfigurationSidelink message to be applied by the peer UE;

- if the configurations in the corresponding RRCReconfigurationSidelink message includes the slrb-ConfigToReleaseList:

2>	for each entry value included in the slrb-ConfigToReleaseList:

3>	perform the sidelink DRB release procedure, according to clause 5.8.9.1a.1;

- if the configurations in the corresponding RRCReconfigurationSidelink message includes the slrb-ConfigToAddModList:

2>	for each slrb-PC5-ConfigIndex value included in the slrb-ConfigToAddModList:

3>	perform the sidelink DRB addition/modification procedure, according to clause 5.8.9.1a.2.

END OF CHNAGES