3GPP TSG-RAN WG2 Meeting #127bis		R2-2408599

Hefei, China, 14th – 18th Oct. 2024

Agenda Item: 	8.6.4

Source:	Apple

Title:	Scoping Conditional LTM for Rel-19

Document for:		Discussion and Decision

# Introduction

In RAN-105, the conditional LTM objective was confirmed, but also revised (down-scoped rather) to only intra-CU LTM. In this contribution, we scope the objective and list several important initial topics and propose resolution to allow RAN2 to progress on this objective.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

# Open topics

## Configuration and preparation

### General Configuration

LTM has been specified from Rel-18 for intra-CU and in Rel-19 inter-CU is also being added. Since conditional LTM is limited to intra-CU for Rel-19, the first aspect to discuss and resolve is how the UE can be configured with conditional LTM in relation to the configuration of Rel-18 intra-CU and Rel-19 inter-CU LTM.

Observation 1: From UE perspective, conditional LTM is another defining factor, but still applies to legacy Rel-18 and Rel-19 LTM candidate cells. Support of conditional LTM would only mean that the UE can now evaluate a “bit differently” these candidate LTM cells, compared to traditional LTM.

Observation 2: From NW perspective, if it has under it’s deployment a set of candidate LTM cells that are potential neighbor cells to the UE being served, there should be no restriction on having the NW control the mobility of the UE to a particular cell via a NW triggered MAC CE or have the UE select conditionally to that cell, with some triggering condition. In other words, we see no reason to have the same candidate cell that allows the UE to select conditionally, to be to triggered to the UE to switch to, with LTM MAC CE.

Proposal 1: An LTM candidate cell configured to the UE as part of Rel-18/Rel-19 potential NW triggered LTM cell, can also be configured to the UE as a conditional LTM candidate cell.

With the above, atleast from the UE perspective, it should be clear which candidate LTM cells are allowed for conditional LTM evaluation and which are not.

Observation 3: From UE perspective, it should be clear which candidate LTM cells are allowed for conditional LTM evaluation and which are not.

Proposal 2: UE can be configured with a separate cell of LTM candidate cells that are considered for conditional LTM evaluation. The traditional Rel-18/Rel-19 LTM candidates which are configured to the UE for NW triggered LTM, can also be present in the conditional LTM candidate list.

This raises the question of whether the UE can have more than 8 LTM candidates, now that we can have a separate list of conditional LTM candidates. But it is better to stick to the way RAN2 has been going with number of LTM candidate – keep the total number to 8.

Observation 4: Introduction of conditional LTM does not mean the number of potential candidate cells are increased for the UE from a location.

Proposal 3: Conditional LTM does NOT bring any change to the number of supported candidate LTM cell configuration, we follow Rel-18 (configuration and capability).

Having progressed so far with the configuration, we think it is better to go ahead and simplify the stage-3 design as well in terms of how the conditional LTM candidates are configured to the UE in relation to the traditional LTM candidates.

Proposal 4: Existing LTM candidate configuration is re-used for conditional LTM configuration, where additional information is configured for each candidate cell where conditional LTM is also allowed.

### Configuration from inter-CU perspective

Since Rel-19 WI limits conditional LTM to only intra-CU, among the candidate LTM cells configured to the UE, if some of them are configured for conditional LTM, it can result in more than one set of potential conditional LTM cells belonging to different CUs. This is something RAN2 needs to address and guide RAN3 as well.

Observation 5: In case of inter-CU LTM configuration to the UE, if among the candidate LTM cells configured to the UE, if some of them are configured for conditional LTM, it can result in more than one set of potential conditional LTM cells belonging to different CUs.

Observation 6: UE can then find in cases where a set of conditional LTM candidate cells belonging to CU ‘x’ while another set of conditional LTM candidate cells belong to CU ‘y’. It would be logical for the UE to then only evaluate the conditional LTM candidates that belong to the CU the UE currently has the serving cell in.

It is better to first agree the configuration that results in UE having conditional candidate LTM cells across CUs.

Proposal 5: If inter-CU LTM is configured to the UE, the UE can be configured with conditional LTM candidate cells where some of them belong to a particular CU while the others belong to other CUs.

Proposal 6: Based on the CU that the UE has it’s serving cell in, the UE evaluates the triggering conditions for the conditional candidate LTM cells that only belong to the same CU. Conditional LTM configuration should provide the UE with the CUs to which the conditional LTM candidates belong to.

Another aspect that needs resolution is what happens to the candidate LTM configuration based on the LTM cells switch (conditional or NW triggered).

Observation 7: If the UE is configured with traditional inter-CU LTM, then subsequent LTM back to the source CU after a switch to a candidate LTM belong to different CU, is always possible and allowed, as the UE keeps all the candidate LTM configurations. There is no reason to prevent this for conditional LTM, and is also a bit implied by proposal 6.

Proposal 7: The UE keeps the conditional LTM configuration after a conditional LTM switch. In case the NW triggered LTM switch results in an inter-CU LTM, the conditional configuration of the source CU candidate LTM cell is also kept by the UE. Unless the UE is performing recovery or re-establishment, the UE only releases conditional LTM configuration by explicit RRC reconfiguration.

This also bring another important point. The WID sub-objective scope limits conditional LTM to only SA case while DC case is still to be evaluated if it is part of Rel-19.

Proposal 8: It is upto the NW to ensure that the UE configuration does not result in conditional LTM candidates having a DC configuration, or does not result in a UE with DC current configuration being configured with conditional LTM candidates.

## Evaluation and Triggering aspects

The most important aspect of conditional LTM is the criterion that results in the UE triggering the conditional LTM. In this respect we think that the framework that we are developing for event triggered L1 measurements is the one we should build on for UE triggering evaluation.

Observation 8: Event triggered L1 measurement framework already provides the basic infrastructure for configuration, evaluation and triggering aspects that are also needed for conditional LTM. We should reuse the same functionality where it is usable instead of creating multiple but similar procedures/frameworks at the UE and the NW.

Proposal 9: For triggering condition for conditional LTM, event triggered L1 measurement framework would be used as the baseline – configuration/evaluation and triggering aspect can be reused for conditional LTM evaluation and triggering. FFS on the details.

## Early UL sync and execution

The early sync phase on LTM (Rel-18 intra-CU or Rel-19 inter-CU) needs more clarity in case of conditional LTM.

In NW triggered LTM, the UL early sync part is driven by the NW, in that the NW provides the TA to use, or the UE estimates the TA. For the case of NW provided TA, the UE should have RACHed earlier.

Observation 9: Since there is no DL in the case of conditional LTM, the UE should obtain UL sync at every conditional LTM switch. The easiest way is for the UE to RACH. More complex ways are possible with early RACH (and TA delivery for all conditional LTM candidates) or with UE UL TA estimation.

Observation 10: It is prudent to focus on the most practical and basic usecase first – UL sync at the time of conditional LTM switch using RACH, and come back to other ways later.

Proposal 10:  RACH based conditional LTM switch is the default way for conditional LTM operation. FFS on CFRA.

We can evaluate on the usefulness of UE RACHing in early sync phase to allow the NW to provide the TA to the UE. But this is not efficient.

Observation 11: If RACH is to be avoided by early delivery of TA to the UE, the UE needs to RACH to all candidate conditional LTM cells and allow for the delivery of the TA for each of these by the NW, and for the UE to maintain the TA values for each of these.

Observation 12: While maintaining TA values for each of the potential conditional LTM candidates itself is an additional complexity at the UE (and the NW), the fact that the UE would need to RACH on all of the conditional LTM candidates itself is very power inefficient.

Proposal 11: RACHless conditional LTM execution using pre-delivery of TA is not supported in Rel-19.

Observation 13: The other way to avoid RACH is via UE TA estimation. To start with, this is only possible in certain deployments and RAN4 hasn’t yet specified requirements for the UE for Rel-18 UE estimated TA. It would be prudent to defer this aspect until Rel-18 progress is made.

Proposal 12: Whether to support UE estimated TA based conditional LTM is to be discussed only after the RACH based conditional LTM aspects are specified.

With the above agreements in place, the open item with UL sync/execution is whether to support CFRA RACH (on top of CBRA RACH).

Observation 14: For CFRA based RACH for conditional LTM, the NW needs to pre-allocate and reserve resources across different participating conditional LTM cells with the anticipation that the UE might RACH to this cell. This is very inefficient.

Observation 15: Beside, the whole aim of conditional LTM is “mobility robustness” than latency reduction. So it is better to focus on aspects that allow robustness.

Proposal 13: RAN2 to discuss if CFRA RACH is configured as part of conditional LTM.

# Conclusion

Observation 1: From UE perspective, conditional LTM is another defining factor, but still applies to legacy Rel-18 and Rel-19 LTM candidate cells. Support of conditional LTM would only mean that the UE can now evaluate a “bit differently” these candidate LTM cells, compared to traditional LTM.

Observation 2: From NW perspective, if it has under it’s deployment a set of candidate LTM cells that are potential neighbor cells to the UE being served, there should be no restriction on having the NW control the mobility of the UE to a particular cell via a NW triggered MAC CE or have the UE select conditionally to that cell, with some triggering condition. In other words, we see no reason to have the same candidate cell that allows the UE to select conditionally, to be to triggered to the UE to switch to, with LTM MAC CE.

Observation 3: From UE perspective, it should be clear which candidate LTM cells are allowed for conditional LTM evaluation and which are not.

Observation 4: Introduction of conditional LTM does not mean the number of potential candidate cells are increased for the UE from a location.

Observation 5: In case of inter-CU LTM configuration to the UE, if among the candidate LTM cells configured to the UE, if some of them are configured for conditional LTM, it can result in more than one set of potential conditional LTM cells belonging to different CUs.

Observation 6: UE can then find in cases where a set of conditional LTM candidate cells belonging to CU ‘x’ while another set of conditional LTM candidate cells belong to CU ‘y’. It would be logical for the UE to then only evaluate the conditional LTM candidates that belong to the CU the UE currently has the serving cell in.

Observation 7: If the UE is configured with traditional inter-CU LTM, then subsequent LTM back to the source CU after a switch to a candidate LTM belong to different CU, is always possible and allowed, as the UE keeps all the candidate LTM configurations. There is no reason to prevent this for conditional LTM, and is also a bit implied by proposal 6.

Observation 8: Event triggered L1 measurement framework already provides the basic infrastructure for configuration, evaluation and triggering aspects that are also needed for conditional LTM. We should reuse the same functionality where it is usable instead of creating multiple but similar procedures/frameworks at the UE and the NW.

Observation 9: Since there is no DL in the case of conditional LTM, the UE should obtain UL sync at every conditional LTM switch. The easiest way is for the UE to RACH. More complex ways are possible with early RACH (and TA delivery for all conditional LTM candidates) or with UE UL TA estimation.

Observation 10: It is prudent to focus on the most practical and basic usecase first – UL sync at the time of conditional LTM switch using RACH, and come back to other ways later.

Observation 11: If RACH is to be avoided by early delivery of TA to the UE, the UE needs to RACH to all candidate conditional LTM cells and allow for the delivery of the TA for each of these by the NW, and for the UE to maintain the TA values for each of these.

Observation 12: While maintaining TA values for each of the potential conditional LTM candidates itself is an additional complexity at the UE (and the NW), the fact that the UE would need to RACH on all of the conditional LTM candidates itself is very power inefficient.

Observation 13: The other way to avoid RACH is via UE TA estimation. To start with, this is only possible in certain deployments and RAN4 hasn’t yet specified requirements for the UE for Rel-18 UE estimated TA. It would be prudent to defer this aspect until Rel-18 progress is made.

Observation 14: For CFRA based RACH for conditional LTM, the NW needs to pre-allocate and reserve resources across different participating conditional LTM cells with the anticipation that the UE might RACH to this cell. This is very inefficient.

Observation 15: Beside, the whole aim of conditional LTM is “mobility robustness” than latency reduction. So it is better to focus on aspects that allow robustness.

Proposal 1: An LTM candidate cell configured to the UE as part of Rel-18/Rel-19 potential NW triggered LTM cell, can also be configured to the UE as a conditional LTM candidate cell.

Proposal 2: UE can be configured with a separate cell of LTM candidate cells that are considered for conditional LTM evaluation. The traditional Rel-18/Rel-19 LTM candidates which are configured to the UE for NW triggered LTM, can also be present in the conditional LTM candidate list.

Proposal 3: Conditional LTM does NOT bring any change to the number of supported candidate LTM cell configuration, we follow Rel-18 (configuration and capability).

Proposal 4: Existing LTM candidate configuration is re-used for conditional LTM configuration, where additional information is configured for each candidate cell where conditional LTM is also allowed.

Proposal 5: If inter-CU LTM is configured to the UE, the UE can be configured with conditional LTM candidate cells where some of them belong to a particular CU while the others belong to other CUs.

Proposal 6: Based on the CU that the UE has it’s serving cell in, the UE evaluates the triggering conditions for the conditional candidate LTM cells that only belong to the same CU. Conditional LTM configuration should provide the UE with the CUs to which the conditional LTM candidates belong to.

Proposal 7: The UE keeps the conditional LTM configuration after a conditional LTM switch. In case the NW triggered LTM switch results in an inter-CU LTM, the conditional configuration of the source CU candidate LTM cell is also kept by the UE. Unless the UE is performing recovery or re-establishment, the UE only releases conditional LTM configuration by explicit RRC reconfiguration.

Proposal 8: It is upto the NW to ensure that the UE configuration does not result in conditional LTM candidates having a DC configuration, or does not result in a UE with DC current configuration being configured with conditional LTM candidates.

Proposal 9: For triggering condition for conditional LTM, event triggered L1 measurement framework would be used as the baseline – configuration/evaluation and triggering aspect can be reused for conditional LTM evaluation and triggering. FFS on the details.

Proposal 10:  RACH based conditional LTM switch is the default way for conditional LTM operation. FFS on CFRA.

Proposal 11: RACHless conditional LTM execution using pre-delivery of TA is not supported in Rel-19.

Proposal 12: Whether to support UE estimated TA based conditional LTM is to be discussed only after the RACH based conditional LTM aspects are specified.

Proposal 13: RAN2 to discuss if CFRA RACH is configured as part of conditional LTM.