3GPP TSG-RAN WG2 #127bis	R2-2408602

Hefei, China, 14 – 18 October 2024

Agenda item:		8.8.4	Support of Broadcast service

Title:		Further details on intended service area for MBS and ETWS

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the approved Rel-19 WID [1] there is the objective for on-demand SIB1 transmission for Idle/Inactive UEs:

RAN has approved a Rel-19 work item (WI) aimed at defining Phase 3 enhancements to Non-Terrestrial Networks (NTN). There was a minor revision of the WID at RAN#103. Among the objectives captured in the WID [1] the following can be found:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

In RAN2 #126, it was agreed:

Agreements:

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

In RAN2 #127, it was agreed:

Agreements:

1.	For RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

2.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

3.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

In this contribution, we discuss further details of intended service area.

# 2. Discussion

2.1	Further details on intended service area

It would be too demanding from a technical point of view to have beams physically target only a service area, as was agreed during the RAN2 #126 meeting. UEs outside of these areas may be within coverage, and it would be up to upper layers to ensure that they cannot decode such broadcast.

However, regulatory concerns outside of RAN2’s jurisdiction may decide what UEs may be within coverage of what broad/multi-cast.

Observation 1: It may not be possible to only broadcast signal within a service area.

Observation 2: It is up to any given country’s laws what may be broadcasted on their geographical territory.

Hence, RAN2 needs more feedback from other WGs and potentially regulatory bodies. Without explicit instruction, it seems reasonable to avoid very fine granularity targeted service area.

Proposal 1: RAN2 should not aim for physically targeted broadcasting or precise service area definition, unless regulatory requirement out of the scope of RAN2.

During an offline discussion [2], it was also discussed how to signal MBS intended service area. Two main options were discussed: SIB20 or MCCH.

Using SIB20 will induce signalling overhead and extra modification of SIB20 caused by service area info update, while using MCCH could cause extra UE power consumption to always monitor MCCH update. However, UE may not need to (re)acquire up-to-date MCCH.

Proposal 2: MBS intended service area is broadcasted via MCCH and measurement relaxations can be introduced for UEs in and outside of the service area.

2.2	Intended service area and ETWS

Intended service area for MBS may be as simple as possible and, as agreed in the last meeting, it is associated with an MBS session. For ETWS, this is different. Indeed, this type of broadcasting is not encrypted or associated with any connection requirement, such as for MBS. Furthermore, some area information can already be indicated as part of the ETWS notification.

While a more targeted broadcast of PWS is always beneficial to avoid wrongly alerting people, it is unclear how MBS service area information can be used for ETWS.

Proposal 3: RAN2 to discuss whether the intended service area can be applied to ETWS.

During RAN2 #127, an LS was sent to SA1 [3] to ask whether the capability to geofence emergency messages to an area should be supported for ETWS in a NTN cell, considering the delay requirements for ETWS primary notification.

Proposal 4: RAN2 to discuss RAN2 impacts if SA1 decides that enhancements specific to ETWS are necessary.

# 3. Conclusion

In this contribution we propose:

Proposal 1: RAN2 should not aim for physically targeted broadcasting or precise service area definition. Unless regulatory requirement out of the scope of RAN2.

Proposal 2: MBS intended service area is broadcasted via MCCH and measurement relaxations can be introduced for UEs in and outside of the service area.

Proposal 3: RAN2 to discuss whether the intended service area can be applied to ETWS.

Proposal 4: RAN2 to discuss RAN2 impacts if SA1 decides that enhancements specific to ETWS are necessary.

# References

[1] RP-241667 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

[2] R2-2407619 Summary of [Offline 307] [R19 NR NTN] service area information (Apple)

[3] R2-2407823 LS on requirements for ETWS primary notification