3GPP TSG-RAN WG2 Meeting #127bis	R2-2408608

Hefei, China, Oct 14th – 18th, 2024

| Agenda Item: | 8.6.4 (NR_Mob_Ph4-Core) |
| --- | --- |
| Source: | LG Electronics Inc. |
| Title: | Discussion on conditional LTM |
| Document for: | Discussion and Decision |

1.	Introduction

In RAN#105 meeting, the objective for conditional LTM (CLTM) is revised as follows [1]:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

  - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this paper, we discuss intra-CU CLTM. Specifically, the followings are covered: overall signaling flows for CLTM and CLTM execution condition.

2.	Discussion

2.1.	Overall signaling flows for CLTM

Similar to CHO, CLTM may be a procedure for an LTM cell switch that is executed by the UE when one or more LTM cell switch execution conditions are met, where the execution conditions are provided to the UE within the candidate cell configurations in advance. Since Rel-18 LTM is a pre-configuration (i.e., candidate cell configuration) based mobility, Rel-18 LTM signaling procedure can be a baseline for discussion on CLTM signaling procedure.

All information required for handover in CHO is included in the RRCReconfiguration message to configure CHO. Hence, it is sufficient that the UE starts to evaluate the execution conditions for CHO candidate cells upon receiving CHO configuration via RRC. On the other hand, all information required for handover (i.e., cell switch) in LTM is seperately included in the RRCReconfiguration message to configure LTM and LTM Cell Switch Command (CSC) MAC CE, which means that, for LTM cell switch, the UE needs the integrated information between RRC LTM configuration and LTM CSC MAC CE. Based on CHO and LTM procedures, we can design a CLTM procedure in two kinds as follows: 1) RRC configuration initiated CLTM; and 2) LTM CSC MAC CE initiated CLTM.

RRC configuration initiated CLTM (see Figure 1): The network configures the UE with one or more CLTM candidate cells (i.e., RRC CLTM configuration) by the RRC reconfiguration procedure. The UE starts to evaluate the execution conditions for CLTM candidate cells upon receiving RRC CLTM configuration. If an execution condition is met, the UE performs the LTM cell switch.

Figure 1. Signaling procedure for CLTM (RRC configuration initiated option)

LTM CSC MAC CE initiated CLTM (see Figure 2): The network configures the UE with one or more CLTM candidate cells (i.e., RRC CLTM configuration) by the RRC reconfiguration procedure. The UE may perform early DL/UL synchronization to one or more CLTM candidate cells and performs L1 measurement/reporting of CLTM candidate cells. The network sends a CLTM CSC MAC CE. The UE starts to evaluate execution conditions for CLTM candidate cells upon receiving CLTM CSC MAC CE. If an execution condition is met, the UE performs the LTM cell switch.

Figure 2. Signaling procedure for CLTM (LTM CSC MAC CE initiated option)

Observation 1. Based on CHO and LTM procedures, a CLTM procedure can be designed in two kinds as follows: 1) RRC configuration initiated CLTM; and 2) LTM CSC MAC CE initiated CLTM.

- RRC configuration initiated CLTM (see Figure 1): The network configures the UE with one or more CLTM candidate cells (i.e., RRC CLTM configuration) by the RRC reconfiguration procedure. The UE starts to evaluate the execution conditions for CLTM candidate cells upon receiving RRC CLTM configuration. If an execution condition is met, the UE performs the LTM cell switch.

- LTM CSC MAC CE initiated CLTM (see Figure 2): The network configures the UE with one or more CLTM candidate cells (i.e., RRC CLTM configuration) by the RRC reconfiguration procedure. The UE may perform early DL/UL synchronization to one or more CLTM candidate cells and performs L1 measurement/reporting of CLTM candidate cells. The network sends a CLTM CSC MAC CE. The UE starts to evaluate execution conditions for CLTM candidate cells upon receiving CLTM CSC MAC CE. If an execution condition is met, the UE performs the LTM cell switch.

In case of RRC configuration initiated CLTM, the network should reserve relevant resources (e.g., RA preamble) provided in the CLTM configuration after CLTM configuration until reconfiguration of CLTM or CLTM release. Since CLTM supports subsequent mobility, the resource reservation time may become too long, which may result in poor resource utilization. On the contrary, the network can reserve the resources for cell switch (e.g., RA preamble or CG) at an appropriate time in case of LTM CSC MAC CE initiated CLTM, which may lead to prevent waste of resources.

Moreover, signaling overhead may occur between the network and the UE due to RRC reconfiguration to dynamically control resources for cell switch during subsequent mobility. On the other hand, with LTM CSC MAC CE initiated CLTM, resources for cell switch can be controlled by the LTM CSC MAC CE, which is lighter than RRC messages. Hence, LTM CSC MAC CE initiated CLTM is a good way to support subsequent mobility.

Observation 2. In terms of resource utilization and signaling overhead during subsequent mobility, LTM Cell Switch Command MAC CE initiated CLTM is superior to RRC configuration initiated CLTM.

As described in Annex 5.1, RACH-less cell switch is beneficial for CLTM in terms of mobility latency. In RRC configuration initiated CLTM, there may be several difficulties to support RACH-less cell switch. First, the information required for RACH-less cell switch (e.g., beam indication and TA information) may need to be frequently changed because of UE movement. For example, the network frequently should indicate beam indication by the RRC reconfiguration procedure for RACH-less CLTM cell switch when the UE moves fast, which can lead to signaling overhead and large interruption. Second, the opportunity of RACH-less cell switch may be reduced, i.e., most of CLTM cell switches may be executed with the RA procedure because the information about RACH-cell switch included in RRC CLTM configuration becomes invalid at CLTM cell switch execution, which can reduce the usefulness of CLTM. Third, as mentioned in the observation 2, the reservation time of UL resources for RACH-less cell switch, which is configured in RRC CLTM configuration, may become too long to result in poor resource utilization.

Basically, configurations that change dynamically, such as the best beam, are better controlled by MAC rather than RRC because MAC signal is lighter and faster than RRC signal. For this reason, dynamic information about LTM cell switch (e.g., beam indication, TA information, RA preamble, etc.) is included in the LTM CSC MAC CE in Rel-18 LTM. In this context, it seems beneficial to provide the dynamic information about LTM cell switch into the LTM CSC MAC CE. In other words, frequent RRC reconfiguration can be avoided by utilizing LTM CSC MAC CE to dynamically reconfigure the information for LTM cell switch.

Observation 3. In terms of RACH-less cell switch, RRC configuration initiated CLTM may have the following difficulties: frequent RRC reconfiguration for updating information about RACH-less cell switch; reduced usefulness of CLTM; and poor resource utilization. The difficulties can be addressed by LTM Cell Switch Command initiated CLTM.

With LTM CSC MAC CE initiated CLTM, most of the existing Rel-18 LTM procedures can be reused for CLTM, i.e., preparation, early DL/UL synchronization, LTM cell switch execution, and LTM cell switch completion. Consequently, specification work can be minimized. Additionally, the existing Rel-18 LTM RRC signaling model can be reused because of similarity between LTM and CLTM. Based on the similarity, the RRC signaling for CLTM configuration may be designed by adding new information for CLTM on top of the RRC signaling for Rel-18 LTM. Then, the network can configure CLTM based on the existing LTM configuration by delta configuration and the network can flexibly indicate to the UE LTM or CLTM cell switch toward a candidate cell based on a single LTM candidate cell configuration, which result in flexibility for the network and signaling overhead reduction. Further, the UE can save its capability for LTM configuration. In other words, the UE may not need to store the CLTM configuration separately from the LTM configuration. As a result, the UE complexity can be reduced.

Observation 4. LTM Cell Switch Command MAC CE initiated CLTM has the following advantages: 1) less specification work; 2) flexibility for the network; 3) signaling overhead reduction; and 4) UE complexity reduction.

Based on the observations 2, 3, and 4, we suggest that the signaling procedure for CLTM with LTM CSC MAC CE initiated option given in Figure 2 as a baseline for stage-2 level discussion on CLTM.

Proposal 1. Consider the signaling procedure for conditional LTM given in Figure 2 as a baseline for stage-2 level discussion on CLTM.

For CLTM preparation, overall signaling flow for Rel-18 LTM can be reused, where the network configures the UE with CLTM by the RRC reconfiguration procedure based on the measurement report of neighbor cells. Further study is needed on additional information for CLTM configuration beyond the Rel-18 LTM configuration. For example, each candidate cell configuration includes one or more execution conditions for LTM cell switch execution.

Proposal 2. LTM preparation procedure for Rel-18 LTM is reused for CLTM preparation procedure. FFS on additional information for CLTM configuration beyond the Rel-18 LTM configuration.

As abovementioned, RACH-less cell switch can further increse the benefits from CLTM and the difficulties to support RACH-less cell switch can be addressed by LTM CSC MAC CE initiated CLTM. Therefore, we think RACH-less cell switch needs to be supported for CLTM.

Proposal 3. Support RACH-less cell switch for CLTM.

In order to support RACH-less cell switch for LTM CSC MAC CE initiated CLTM, Rel-18 DL/UL early synchronization can be reused. Namely, the UE can acquire the information of DL/UL synchronization of one or more candidate cells between CLTM preparation and CLTM cell switch execution, i.e., the DL synchronization is acquired by candidate TCI activation MAC CE and the UL synchronization is acquired by RA based early TA or UE based TA.

Proposal 4. Rel-18 early DL/UL synchronization procedure is used for early DL/UL synchronization to candidate cells for CLTM.

Now, we need to discuss how to provide to the UE with the UL synchronization acquired by RA based early TA. In RAN2#126 meeting, some companies think RAR based option may be needed for CLTM. However, the RAR based option produces many issues, e.g., as follows:

- FFS which cell sends RAR to the UE, i.e., the current serving cell or the candidate cell

- FFS the UE needs to evaluate the validity of candidate cell TA by, e.g., TA timer

- FFS the UE action if the candidate cell TA becomes invalid

The issues listed above seems controversial so discussion on RAR based early TA for CLTM may take a lot of time to conclude. Further, RAN1 may need to be involved in order to resolve the issues, which makes it difficult to reach a conclusion.

Observation 5. RAR based early TA for CLTM may take a lot of time to conclude because it can produce many controversial issues.

To save the time for discussion, it seems like a good idea to reuse the Rel-18 way of providing to the UE with the TA of candidate cell, i.e., the TA of candidate cell is included in LTM CSC MAC CE.

Proposal 5. In CLTM, RAR based option for RA based early TA is not pursued, i.e., TA is included in the LTM Cell Switch Command MAC CE as in Rel-18 LTM.

If the Rel-18 way is reused for provision of candidate cell TA (i.e., Proposal 4 is agreed), the following question is: how can the UE distinguish whether the LTM CSC MAC CE is for CLTM or not? It is simple and straightforward that LTM CSC MAC CE includes the information about whether or not the LTM cell switch is conditional LTM. To this end, we think, for example, one reserved bit in Rel-18 LTM CSC MAC CE can be used for indicating whether or not the LTM cell switch is conditional LTM.

Proposal 6. LTM Cell Switch Command MAC CE includes the information indicating that the LTM cell switch is CLTM.

If the LTM CSC MAC CE indicates non-conditional LTM (i.e., Rel-18 LTM), then the UE performs the LTM cell switch according to Rel-18 LTM cell switch execution procedure. If the LTM CSC MAC CE indicates conditional LTM, then the UE starts to evaluate the execution condition(s) associated with the candidate cell indicated by the LTM CSC MAC CE. If an execution condition is met, the UE performs the LTM cell switch according to Rel-18 LTM cell switch execution procedure.

Proposal 7. If the LTM Cell Switch Command MAC CE indicates CLTM, then the UE starts to evaluate the execution condition(s) associated with the candidate cell indicated by the LTM Cell Switch Command MAC CE. If the execution condition is met, the UE performs the LTM cell switch according to Rel-18 LTM cell switch execution procedure.

2.2.	CLTM execution condition

RAN2 agreed to introduce Event LTM2 to Event LTM5, which are respectively similar to A2 to A5 event. Among those, Event LTM3 and Event LTM5 are suitable for CLTM execution conditions.

Proposal 8. Event LTM3 and Event LTM 5 being introduced for non-CLTM are reused as CLTM execution conditions.

RAN2 agreed that for network-triggered LTM, a single beam quality is used for event evaluation. It is FFS event evaluation based on quality related to multiple beams for conditional LTM.

The motivation of considering quality of multiple beams for event evaluation is to trigger cell switch based on more representative quality of a concerned candidate cell, rather than a single beam quality, because this is for cell switch, not merely an intra-cell beam switch. If the event evaluation based on the consolidated results of multiple RSs is supported, the UE can avoid triggering CLTM to candidate cell where there is only one good beam.

One way of considering quality of multiple beams for CLTM is simply to calculate average value of the candidate cell beams which satisfy the CLTM execution condition and use the average value for evaluation CLTM execution condition.

Another way is that instead of UE calculating the consolidated quality, network configures the minimum number of beams, say N, to satisfy the CLTM execution condition, and UE considers the CLTM execution condition to be met if at least N beams of the candidate cell satisfy the CLTM condition respectively.

In our view, the latter is more desirable because derivation of the consolidated result is not needed and network can configure N from 1 to some positive integer value.

Proposal 9. Network can configure value N indicating  minimum number of beams to satisfy CLTM execution condition. Then, UE considers the CLTM execution condition to be met if at least N beams of the candidate cell satisfy the CLTM condition respectively.

3.	Conclusion

In this paper, we have made the following observations and proposals:

Overall signaling flows for CLTM

Observation 1. Based on CHO and LTM procedures, a CLTM procedure can be designed in two kinds as follows: 1) RRC configuration initiated CLTM; and 2) LTM CSC MAC CE initiated CLTM.

- RRC configuration initiated CLTM (see Figure 1): The network configures the UE with one or more CLTM candidate cells (i.e., RRC CLTM configuration) by the RRC reconfiguration procedure. The UE starts to evaluate the execution conditions for CLTM candidate cells upon receiving RRC CLTM configuration. If an execution condition is met, the UE performs the LTM cell switch.

- LTM CSC MAC CE initiated CLTM (see Figure 2): The network configures the UE with one or more CLTM candidate cells (i.e., RRC CLTM configuration) by the RRC reconfiguration procedure. The UE may perform early DL/UL synchronization to one or more CLTM candidate cells and performs L1 measurement/reporting of CLTM candidate cells. The network sends a CLTM CSC MAC CE. The UE starts to evaluate execution conditions for CLTM candidate cells upon receiving CLTM CSC MAC CE. If an execution condition is met, the UE performs the LTM cell switch.

Observation 2. In terms of resource utilization and signaling overhead during subsequent mobility, LTM Cell Switch Command MAC CE initiated CLTM is superior to RRC configuration initiated CLTM.

Observation 3. In terms of RACH-less cell switch, RRC configuration initiated CLTM may have the following difficulties: frequent RRC reconfiguration for updating information about RACH-less cell switch; reduced usefulness of CLTM; and poor resource utilization. The difficulties can be addressed by LTM Cell Switch Command initiated CLTM.

Observation 4. LTM Cell Switch Command MAC CE initiated CLTM has the following advantages: 1) less specification work; 2) flexibility for the network; 3) signaling overhead reduction; and 4) UE complexity reduction.

Proposal 1. Consider the signaling procedure for conditional LTM given in Figure 2 as a baseline for stage-2 level discussion on CLTM.

Proposal 2. LTM preparation procedure for Rel-18 LTM is reused for CLTM preparation procedure. FFS on additional information for CLTM configuration beyond the Rel-18 LTM configuration.

Proposal 3. Support RACH-less cell switch for CLTM.

Proposal 4. Rel-18 early DL/UL synchronization procedure is used for early DL/UL synchronization to candidate cells for CLTM.

Observation 5. RAR based early TA for CLTM may take a lot of time to conclude because it can produce many controversial issues.

Proposal 5. In CLTM, RAR based option for RA based early TA is not pursued, i.e., TA is included in the LTM Cell Switch Command MAC CE as in Rel-18 LTM.

Proposal 6. LTM Cell Switch Command MAC CE includes the information indicating that the LTM cell switch is CLTM.

Proposal 7. If the LTM Cell Switch Command MAC CE indicates CLTM, then the UE starts to evaluate the execution condition(s) associated with the candidate cell indicated by the LTM Cell Switch Command MAC CE. If the execution condition is met, the UE performs the LTM cell switch according to Rel-18 LTM cell switch execution procedure.

CLTM execution condition

Proposal 8. Event LTM3 and Event LTM5 being introduced for non-CLTM are reused as CLTM execution conditions.

Proposal 9. Network can configure value N indicating minimum number of beams to satisfy CLTM execution condition. Then, UE considers the CLTM execution condition to be met if at least N beams of the candidate cell satisfy the CLTM condition respectively.

4.	References

[1] RP-242356 Revised Work Item: NR mobility enhancements Phase 4, 3GPP TSG RAN meeting #105

[2] TS 38.300 NR and NG-RAN Overall Description (Release 18)

[3] TS 38.133 Requirements for support of radio resource management (Release 18)

[4] TS 37.340 Multi-connectivity Overall Description (Release 18)

5.	Annex

5.1.	Potential gains from RACH-less cell switch in CLTM

In [2], the mobility latency charts for RACH-based LTM and RACH-less LTM are presented as follows (i.e., Figure G-2 and Figure G-3).

Figure G-2: Mobility Latency for RACH-based LTM

Figure G-3: Mobility Latency for RACH-less LTM

Basically, in terms of mobility latency, the main difference between RACH-based LTM and RACH-less LTM comes from the latency due to random access. In [3], the LTM cell switch delay is expressed as

DLTM = Tcmd + TLTM-interrupt,

where Tcmd = THARQ + 3ms, and TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU. Based on the mobility latency charts, Tfirst-RS, TRS-proc, and TLTM-IU can be reduced by RACH-less LTM cell switch, where Tfirst-RS and TRS-proc are respectively the time to the first SSB transmission on the target cell after Tcmd and the time for SSB processing, and TLTM-IU is the interruption uncertainty during LTM cell switch. Note that TRS-proc = 2 ms for RACH-based LTM cell switch.

For RACH-based LTM cell switch, TLTM-IU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TLTM-IU can be up to the summation of SSB to PRACH occasion association period and 10 ms, where SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3] and it can be up to 160 ms. Further, in case of RACH-based LTM cell switch, TRAR is added to the mobility latency as shown in Figure G-2, where the worst case TRAR can be calculated by multiplying the maximum value of RAR window size (i.e., 10 ms) and the maximum number of RA preamble transmission (i.e., 200), i.e., TRAR can be up to 2 seconds. Finally, the increased mobility latency due to RACH is expressed as

ΔRACH = DLTM(RACH-based) - DLTM(RACH-less) = Tfirst-RS + TRS-proc + TLTM-IU + TRAR,

where ΔRACH can be larger than 2 seconds based on the analysis.

Observation A1. RACH-based LTM has less benefit than RACH-less LTM in terms of mobility latency.

In Rel-18, LTM is introduced in order to reduce the latency, overhead, and interruption time. For instance, L2 reset skipping for intra-DU LTM cell switch and RACH-less LTM cell switch based on early DL/UL synchronization are introduced in order to reduce latency and interruption. This means that the conditional LTM (CLTM) has smaller latency/interruption than CHO. Compared to CHO, RACH-based conditional LTM (CLTM) shows the latency gain due to skipping L2 reset in intra-DU LTM case. For CHO and LTM, the maximum value of UE processing delay (i.e., TProcessing or TLTM-Processing) is 20 ms for intra frequency case and 40 ms for inter frequency case [3]. This implies that the latency gain from skipping L2 reset is within 20 ms for intra frequency case and 40 ms for inter frequency case, respectively. In our view, the latency gain from skipping L2 reset is marginal compared to the latency gain from skipping RACH because the latency gain from skipping RACH is up to 2 seconds in the worst case. Hence, RACH-based CLTM may have a marginal latency gain compared to the CHO.

Observation A2. The potential latency gain of RACH-based CLTM compared to the CHO is marginal.

Based on the observations A1 and A2, we observe that the gains from CLTM can be further increased by supporting RACH-less cell switch.

Observation A3. The gains from CLTM can be further increased by supporting RACH-less cell switch.