3GPP TSG-RAN WG2#127bis											　 R2-2408609

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.2.5 Topology 2 considerations

Source: 	NEC

Title:	        Topology 2 related aspects

Document for:	 Discussion

- Introduction

Rel-19 Ambient IoT (A-IoT) SI was approved at RAN#102. Following two deployment scenarios are included in the SID, RP-234058.

- Deployment scenario 1 with Topology 1

- Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control

In RAN2#127, the following agreements were made [1]:

1 RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2. 2 A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device. 3 RAN2 assumes that intermediate UE authorization is performed by upper layers. RAN2 impact can be studied after further SA/RAN3 progress. 4 RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied) 5 NW is in control of resources to be used by AIoT air interface. 6 Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO). May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

This contribution provides our views on topology 2 related aspects between reader and BS.

- Discussion

  - Radio Resource Allocation

RAN2#126 agreed that NW is in control of resources to be used by AIoT air interface. On the method of informing resource to be used for AIoT air interface, generally there are two alternatives:

Alt.1 indicate the assigned resource via a dedicated RRC signaling, e.g., RRCReconfiguration.

Alt.2 indicate the available resource candidates (e.g., resource pool-like) via SIB signaling.

With Alt.1, the resource allocation for AIoT air interface is fully under NW control. The NW is able to dynamically assign resources to be used by AIoT air interface based on A-IoT and Uu traffic, which may increase the resource utilization efficiency. Moreover, it is expected that following interferences/collisions could be avoided/minimized with Alt.1:

- the interference between the Uu interface and the AIoT Uu interface for a certain UE reader.

- the interference/collision among different UE readers when performing AIoT operation over AIoT air interface.

Therefore, we observe that Alt.1 is beneficial for inference/collision control and increasing the resource utilization efficiency.

Observation 1: Indicating the assigned resource to be used by AIoT air interface via a dedicated RRC signaling is beneficial for inference/collision control and increasing the resource utilization efficiency.

The basic assumption of Alt.2 seems that a UE reader is allowed to autonomously select resource among the indicated available resource candidates for its AIoT operation over AIoT air interface, i.e., in a distributed way. If NW has no knowledge on whether the resource pool is utilized by UE reader(s), it would be difficult to dynamically adjust resource assignment between A-IoT air interface and Uu interface, therefore a semi-static AIoT resource pool assignment seems realistic. Given that AIoT operations may happen infrequently, Alt. 2 with semi-static AIoT resource pool assignment may decrease the resource utilization efficiency. On the other hand, when there are multiple UE readers under one BS, with Alt.2, inference/collision among different UE readers when performing AIoT operation over AIoT air interface may become out of control.

Observation 2: Indicating the available resource candidates (e.g., resource pool-like) via SIB signaling may decrease resource utilization efficiency and result in the out-of-control inference/collision.

In our understanding, whether Alt.2 is needed also depends on UE reader RRC states discussion (see details in 2.3). Based on above, our proposals are

RAN2 agree to indicate the assigned resource via a dedicated RRC signaling as baseline.

RAN2 further study whether to support indicating the available resource candidates (e.g., resource pool-like) via SIB signaling.

One more issue of Alt.1 is the granularity of assigned resource controlled by the NW, i.e., either NW controls the resource assignment per UE reader or per AIoT procedure/sub-procedure/message performed by a certain UE reader. The first one means that how to use the assigned resource for different AIoT procedure/sub-procedure/message is determined by the UE reader. While for the second one, the NW (BS) may need to understand the AIoT procedure/sub-procedure/message and/or grasp progress/situation of them at UE reader side which may result in a complicated BS implementation and/or additional information exchange between the UE reader and the NW (BS).

RAN2 agree to assign/control resources to be used by AIoT air interface per UE reader as baseline.

Upon receiving the allocated resource to be used for AIoT air interface from the BS, the UE reader can start its AIoT procedure/sub-procedure/message. However, AIoT procedure/sub-procedure/message duration may be variable since many uncertain factors (e.g., contention resolution during random access, failure handling, etc.) exist. To assist NW’s A-IoT resource assignment, some assistance information (e.g., the AIoT service completion indication) from the UE reader to the NW may be needed since only the UE reader knows details of the progress/situation of AIoT operation over AIoT air interface.

To assist NW’s A-IoT resource assignment, RAN2 agree to further study necessary assistance information (e.g., the AIoT service completion indication) from the UE reader to the NW.

2.2 Reader selection

Generally, there are two options for reader selection:

Option 1: the CN performs reader selection itself (for example, solution 11, 23 in TR 23.700-13).

Option 2: the BS performs readers selection from authorized intermediate UE reader candidates provided by the CN (i.e., the BS determines UE readers to be involved for a certain A-IoT service requested by the upper layer)

It is obviously that Option 1 is up to other WG discussion whereas Option 2 is in RAN2 scope. To perform reader selection precisely at BS, some assistant information from the UE reader candidates (for example, the information related to UE reader location) are beneficial.

To perform reader selection precisely at BS, RAN2 further study assistant information from the UE reader candidates (for example, the information related to UE reader location).

Furthermore, to finally determine minimum required UE readers to be involved for a certain A-IoT service requested by the upper layer, following two-way mechanism may be considered:

- the BS indicates/configures criteria to UE reader candidates.

- the UE reader candidates determine and indicate whether expected to be involved based on criteria indicated/configured by the BS.

For Topology 2 with UE as intermediate reader, RAN2 consider following two-way mechanism to reader selection to finally determine minimum required UE readers to be involved for a certain A-IoT service requested by the upper layer:

- the BS indicates/configures criteria to UE reader candidates.

- the UE reader candidates determine and indicate whether it is expected to be involved based on criteria indicated/configured by the BS.

2.3 RRC state of the UE Reader

For RRC_CONENCTED UE, both NAS connection and RRC connection are established, so this should be a baseline for an RRC_CONENCTED UE to be selected as reader. However, for Topology 2 protocol stacks, there will be scenarios that UEs operating in RRC states other than RRC_CONNECTED are also suitable to be a UE reader. For these UE in RRC_IDLE/RRC_INACTIVE, at least paging can be used to notify the start of a AIoT procedure.

Support UE in RRC_CONENCTED, RRC_INACTIVE and RRC_IDLE to be selected as reader.

For UE in RRC_IDLE/RRC_INACTIVE, paging can be used to notify the start of an AIoT procedure.

Considering that there may be time period when there’s no service other than services for AIoT, maintaining the Reader UE in RRC_CONNECTED state during the entire AIoT session is not resource efficient. Therefore, transition from RRC_CONNECTED to RRC_INACTIVE/RRC_IDLE is likely to happen after an RRC_CONNECTED UE starting AIoT procedure.

For RRC_INACTIVE/RRC_IDLE UE reader, upon receiving the notification of the start of AIoT service, on one hand, it may transit to RRC_CONNECTED, for example, to receive further commands, or to provide collected AIoT data. On the other hand, the UE reader may stay in the current state before finishing the AIoT procedure.

RAN2 to study the RRC state transition of the UE reader during the AIoT procedure.

- Conclusion

Proposal 1:	RAN2 agree to indicate the assigned resource via a dedicated RRC signaling as baseline.

Proposal 2:	RAN2 further study whether to support indicating the available resource candidates (e.g., resource pool-like) via SIB signaling.

Proposal 3:	RAN2 agree to assign/control resources to be used by AIoT air interface per UE reader as baseline.

Proposal 4:	To assist NW’s A-IoT resource assignment, RAN2 agree to further study necessary assistance information (e.g., the AIoT service completion indication) from the UE reader to the NW.

Proposal 5:	To perform reader selection precisely at BS, RAN2 further study assistant information from the UE reader candidates (for example, the information related to UE reader location).

Proposal 6:	For Topology 2 with UE as intermediate reader, RAN2 consider following two-way mechanism to reader selection to finally determine minimum required UE readers to be involved for a certain A-IoT service requested by the upper layer:

a)	the BS indicates/configures criteria to UE reader candidates.

b)	the UE reader candidates determine and indicate whether it is expected to be involved based on criteria indicated/configured by the BS.

Proposal 7:	Support UE in RRC_CONENCTED, RRC_INACTIVE and RRC_IDLE to be selected as reader.

Proposal 8:	For UE in RRC_IDLE/RRC_INACTIVE, paging can be used to notify the start of an AIoT procedure.

Proposal 9:	RAN2 to study the RRC state transition of the UE reader during the AIoT procedure.

- References

[1] Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands, ETSI MCC